3GPP TSG-RAN WG2 Meeting #129	R2-2500791
Athens, Greece, February 17th – 21st, 2025

Agenda Item:	8.5.2

Source:	InterDigital

Title:	On demand SSB transmission for SCell

Document for:	Discussion, Decision

# Introduction

The Rel-19 WID on Network Energy Savings for NR has the following objective on on-demand SSB SCell [1]:

The WID includes the following objective accordingly:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In RAN2#127 and 127bis, the objective was discussed and the following was agreed:

Agreements on OD-SSB

Scenarios on OD-SSB:

- RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

OD-SSB transmission indication:

- RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

- Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

- NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

Measurement on OD-SSB:

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

- RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

- RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

Further, in the last RAN4 meeting, RAN2 has received the following LS from RAN4 [3]:

RAN4#112bis extensively discussed impact on RRM requirements from SSB adaptation. The following agreements were reached after discussion: Agreements: - RAN4 to discuss whether there are any impacts on at least the following RRM requirements due to SSB adaption: - L1-RSRP for serving cell - L1-SINR - BFD - CBD - SCell activation - TCI state switching - NR intra-frequency L3 measurement - NR inter-frequency L3 measurement However, L3 measurement requirement in RAN4 is defined based on SMTC configuration rather than SSB configuration in legacy. RAN4 would like to ask RAN2 that whether there is any SMTC impact due to SSB adaptation. 2. Actions: To RAN2. ACTION: RAN4 kindly asks RAN2 to answer the SMTC impact question due to SSB adaptation above.

Full agreements made in RAN1#116 meeting are provided in the appendix. In this contribution, we discuss the enhancements for enabling on-demand SSB SCell along with the MAC CE design to indicate it.

# Discussion

For CA deployments, SCells with SSB-less Scell has been supported since Rel-15 for both FR1 and FR2. In Rel-18 the support for SSB-less Scell is extended for collocated cells in FR1 that are in inter-band CA (e.g. within max receive time, frequency and power difference). In the inter-base CA case, the NW signals to the UE which cell is the reference cell (e.g. a Pcell or another Scell), where the reference cell acts as the timing reference and AGC source of the SSB-less Scell.

Supporting inter-band CA for FR2 comes with challenges, e.g. achieving synchronization, L1/L3 measurements and beam management, especially when the cells are non-collocated and only the Scells operate in FR2 while other cells in FR1. For energy saving purposes in CA, it is important for the Scells to operate in a NES mode as long as possible without or with limited SSB transmission.

## Signalling for OD-SSB reception

MAC CE signalling

During Rel-18 SI, it was discussed that the Scells which are used as capacity cells and operating in NES mode are not expected to transmit SSBs or only transmit sparse SSBs. gNB initiated triggering of OD-SSB in the context of Scell activation procedure has been agreed by R1.

Per R2 agreement, reception of on-demand SSB on an SCell is indicated by a new MAC CE signalling separate from the legacy MAC CE for SCell (de)activation. Further, since the activation of the SCell itself is based on the network’s decision, i.e. based on the reception of UE measurement and L1 CSI, the activation decision may lag the indication for the reception of an OD-SSB. For scenario 2A where the OD-SSB and SCell are indicated and activated together, R2 agreed to not introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication.

Per RAN1 agreements, MAC CE based signalling is used to indicate the (de)-activation of on-demand SSB transmission on an SCell supporting on-demand SSB SCell operation. RAN1 further agreed that for a cell supporting on-demand SSB SCell operation, multiple candidate values can be configured by RRC for the periodicity of the on-demand SSB, and the applicable periodicity value can be indicated by MAC CE. It is FFS whether there are any other relevant parameters. However, looking at the list of parameters from RAN1, it appears like there are no other parameters that need to be indicated dynamically.

Proposal 1:	The OD-SSB indication MAC CE signalling indicates OD-SSB (de)-activation per SCell, along with the applicable OD-SSB periodicity per activated SCell. No other parameters need to be included in the MAC CE.

Given R1 agreed to indicate a value associated with the OD-SSB periodicity, R2 can assume that there will be an additional field to indicate the periodicity index from the values configured in RRC. RAN1 agreed that no new periodicity configuration values will be introduced in Rel-19 for the OD-SSB, other than legacy values (5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms).

Observation 1:	RAN1 agreed to reuse legacy values for the configuration of the OD-SSB periodicity, including (5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms).

Therefore, 3 bits is sufficient to indicate which periodicity is applied for the OD-SSB.

Proposal 2:	3 bits is sufficient to indicate which periodicity is applied for the OD-SSB.

From a MAC CE design perspective, R2 agreed that NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE. Therefore, a bit map can be used to indicate the SCells for which the OD-SSB is indicated. For each indicated SCell, an additional (optional) field is added to indicate the applicable periodicity index. The OD-SSB MAC CE therefore has a variable size. A sample MAC CE is illustrated in Figure 2 for such assumption, at least for the case of up to 7 SCells.

Figure 1: illustration of a variable-size OD-SSB MAC CE short format

Provided that the MAC CE does not provide (de)-activation of SCells, it is beneficial to keep the MAC CE simple and fixed in size.

Proposal 3:	A bit map in a variable size MAC CE is used to indicate the SCells for which the OD-SSB is indicated. For each indicated SCell, an additional (optional) field is added to indicate the applicable OD-SSB periodicity.

Similar to the SCell Activation MAC CE, there can be two formats: one short variable size format, which indicates up to 7 SCells. Another long variable size format, which indicates up to 31 SCells. The short variable size format is illustrated in Figure 1, while the long variable size format is illustrated in Figure 2, where 3 bits are used to indicate the OD-SSB periodicity.

Figure 2: illustration of a variable-size OD-SSB MAC CE long format

Proposal 4:	Similar to SCell Activation MAC CE, the OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and another format for up to 31 SCells.

In terms of deactivation of the OD-SSB transmission, the following mechanisms were agreed:

- Deactivation by RRC. RAN2 agreed that “RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration.”

- Deactivation by MAC CE. RAN1 agreed to support explicit indication of deactivation via MAC-CE. The MAC CE design above is in line with this agreement, as the C bit can be used to activate/deactivate the OD-SSB transmission.

- Deactivation after N transmission occasions. RAN1 also agreed to support “Option 2: Configuration/indication of the number N of OD-SSB bursts”. While the agreement says “configuration/indication”, we think configuring the value N is sufficient and there is no need to also support the dynamic indication of N, given the gNB already has an option to dynamically deactivate the OD-SSB transmission by MAC CE and the value of N is not envisioned to change dynamically. This reduces the size of the MAC CE and complexity, especially since the MAC CE can indicate up to 32 cells.

Proposal 5:	For the agreed RAN1 “Option 2: Configuration/indication of the number N of OD-SSB bursts”, the value of N is configured in RRC and is not indicated in the OD-SSB MAC CE. FFS: if it is configured per SCell or per periodicity.

L3 measurements on OD-SSBs

Per the LS from RAN4 [3], RAN4 asks RAN2 whether there are any impacts on RRM requirements due to SSB adaptation relating to intra/inter-frequency L3 measurement. Adaptation of SSB periodicity can therefore lead to failure of UE performing L3 measurement. RAN2 discussed L3 measurements on OD-SSB, which can have a different periodicity to that of the always-on SSBs; the following two options were discussed with no consensus:

- Option1: Based on different measurement configuration when OD-SSB is transmitted

- Option2: Based on OD-SSB pattern ignoring SMTC when OD-SSB is transmitted

Option 1 may require multiple SMTCs for the UE to perform measurements. Option 2 is simpler, as the transmission of OD-SSB is changing dynamically, however it means that UE won’t measure the OD-SSB, unless UE measures OD-SSB based on OD-SSB transmission pattern and not based on SMTC configuration (as illuded to in R4 LS).

Option 1 can better to consider and should work for both OD-SSB and SSB adaptation. Essentially the understanding now is that the UE uses on-demand SSB for RRM measurements when they are transmitted in addition to periodic SSB for RRM measurements, but this may not be simple to do when they’re on different frequencies.

From a time domain perspective, the periodicity of OD-SSB can be configured separately from the periodicity of AO-SSB, per RAN1 agreements. It has been identified that the main use case is that the periodicity of OD- SSB is equal to or smaller than that of AO-SSB. A separate configuration can therefore align the measurement occasions with the OD-SSB occasions. RAN1 is discussing the frequency domain relation between periodicity of AO-SSB and OD-SSB.

Proposal 6:	For L3 measurements on OD-SSBs and SSB adaptation, UE performs measurements based on a measurement configuration used when OD-SSB is transmitted.

Proposal 7:	Reply to RAN4 explaining a common solution for L3 measurements for OD-SSB and SSB adaptation based on proposal 6.

Per the LS from RAN4 [3], RAN4 asks RAN2 whether there are any impacts on RRM requirements due to SSB adaptation relating to BFD, RRM, and RLM. The same question can be asked, i.e. whether RLM and BFD measurements can be done on OD-SSB occasions.

Proposal 8:	RAN2 to discuss and evaluate whether RLM and BFD measurements can be done on OD-SSB transmission occasions.

# Conclusion

The following proposals are provided for on demand SSB reception on SCells:

Proposal 1:	The OD-SSB indication MAC CE signalling indicates OD-SSB (de)-activation per SCell, along with the applicable OD-SSB periodicity per activated SCell. No other parameters need to be included in the MAC CE.

Observation 1:	RAN1 agreed to reuse legacy values for the configuration of the OD-SSB periodicity, including (5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms).

Proposal 2:	3 bits is sufficient to indicate which periodicity is applied for the OD-SSB.

Proposal 3:	A bit map in a variable size MAC CE is used to indicate the SCells for which the OD-SSB is indicated. For each indicated SCell, an additional (optional) field is added to indicate the applicable OD-SSB periodicity.

Proposal 4:	Similar to SCell Activation MAC CE, the OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and another format for up to 31 SCells.

Proposal 5:	For the agreed RAN1 “Option 2: Configuration/indication of the number N of OD-SSB bursts”, the value of N is configured in RRC and is not indicated in the OD-SSB MAC CE. FFS: if it is configured per SCell or per periodicity.

Proposal 6:	For L3 measurements on OD-SSBs and SSB adaptation, UE performs measurements based on a measurement configuration used when OD-SSB is transmitted.

Proposal 7:	Reply to RAN4 explaining a common solution for L3 measurements for OD-SSB and SSB adaptation based on proposal 6.

Proposal 8:	RAN2 to discuss and evaluate whether RLM and BFD measurements can be done on OD-SSB transmission occasions.

# References

RP-234065, “New WID: Enhancements of network energy savings for NR”, Ericsson

3GPP TR 38.864, Study on network energy savings for NR (Release 18), V18.1.0, Mar 2023

R4-2416911, “LS on SSB adaptation”, RAN4

# Appendix: Agreements on OD-SSB SCell operation

RAN1#116 Agreements

Agreement

Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study:

- Case #1: No always-on SSB on the cell

- Case #2: Always-on SSB is periodically transmitted on the cell

- FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted.

FFS: Which scenario the above applies for

Agreement

RAN1 to strive for a common design for on-demand SSB operation considering all applicable CA configurations.

Agreement

For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios.

- Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321)

- Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321)

  - This does not preclude SCell for which activation is completed

  - FFS: The case where SCell activation is completed

FFS: Application timing between NW triggering message and on demand SSB transmission

Agreement

Support on-demand SSB SCell operation triggered by gNB.

- FFS Details of associated signaling/indication/configuration provided to UE

Agreement

- For SSB burst(s) triggered by on-demand SSB SCell operation, study at least the following options.

  - Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A.

  - Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB

  - Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B.

  - Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted.

  - Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B.

  - FFS: The combination of above options

  - FFS: How to define time instance A/B and the value of N per option

  - FFS: Each option is applicable to which Cases or Scenarios (as per the previous agreement)

RAN1#116bis Agreements

- Scenarios where OD-SSB can be triggered by gNB

  - Scenario #2 with Case #1 (no always-on SSB) and Case #2 (always-on SSB is periodically transmitted)

  - Scenario #2A with Case #1 and Case #2

  - FFS: Scenario 3A with Case 1 & 2

  - FFS: Scenario 3B with Case 1 & 2

    - For Case #1, once OD-SSB is triggered, its transmission is in a periodic manner

    - Notes:

      - Scenario #2A: When UE receives SCell activation command

      - Scenario #3A: After UE receives SCell activation command until SCell activation is completed

      - Scenario #3B: When SCell activation is completed, or after SCell activation is completed

- Type of SSBs for OD-SSB

  - Down-select between the following:

    - Alt-1: It is up to gNB implementation whether OD-SSB is CD-SSB or not

    - Alt-2: OD-SSB is limited to NCD-SSB.

      - FFS: Further limitations to OD-SSB

- Type of measurements

  - L1 and/or L3 measurements based on OD-SSB is supported for the cell

  - FFS further details on L1 and/or L3 measurement (e.g. timing)

- Study signalling for indicating OD-SSB

  - Option 1: Separate signalling for providing SCell (de)activation with legacy signalling (e.g., RRC, MAC CE) and for providing OD-SSB transmission indication

  - Option 2: A single signalling providing both SCell (de)activation and OD-SSB transmission indication

RAN1#117 Agreements

- Type of signaling for indicating OD-SSB

  - Support RRC based signaling

  - Support MAC based signaling

  - FFS: DCI based signaling

    - DCI does not provide Scell (de)activation

    - If supported, details on whether to use UE-specific or group-common DCI

  - FFS: Scenarios where the signaling is applicable

- Configuration info provided for OD-SSB:

  - Frequency of the OD-SSB

  - SSB positions within an OD-SSB burst (by using signaling similar to ssb-PositionsInBurst)

  - Periodicity of OD-SSB

  - Sub-carrier spacing of the OD-SSB

  - Physical Cell ID of the OD-SSB

  - Location of on-demand SSB burst (e.g. offset, half frame)

  - Downlink transmit power of OD-SSB

  - FFS: Further config details:

    - Whether more than one OD-SSB configs can be configured for the cell to UE

    - Whether RRC parameters are newly introduced or existing are reused

  - FFS: Other parameters, e.g.

    - How many OD-SSB burst(s) are transmitted once indicated

    - Offset between HARQ-ACK corresponding to OD-SSB transmission indication signaling and OD-SSB transmission

    - Gap length between SSB bursts

  - FFS: Whether each of above parameters is configured/indicated explicitly or not

- Timing of OD-SSB transmission upon indication by MAC CE

  - Time instance A is [the slot boundary of] the first SSB time domain position [of actually transmitted OD-SSB burst] which is T [slots or symbols] after the [slot or symbol] where UE receives a signaling from gNB to indicate OD-SSB transmission

    - SSB time domain positions of OD-SSB burst are configured by gNB.

  - FFS: Details of value of T (≥ 0) including possibility of T comprising of multiple components

    - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation

  - FFS: Whether the value of T is predefined or indicated/configured by gNB

- Support L1 measurements on OD-SSB

  - Periodic, semi-persistent [and aperiodic] L1 measurements based on existing CSI framework

  - FFS: Potential enhancements of CSI report config and/or triggering/activation mechanisms for L1 measurements on OD-SSB

- RAN1#118

- Signaling of OD-SSB

  - Support RRC based signaling to indicate OD-SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides OD-SSB configuration.

    - FFS: Whether to support RRC based signaling for other cases.

  - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A.

  - Note: Deactivation and adaptation of on-demand SSB transmission to be separately discussed.

- Signaling of OD-SSB parameters

  - Multiple candidate values (of parameter(s) of OD-SSB) can be configured by RRC and the applicable value indicated by MAC CE

  - MAC CE indicates: Periodicity of OD-SSB

  - FFS: Other relevant parameters

- SSB type for OD-SSB

  - Support: OD-SSB on the cell is not located on synchronization raster

  - Support: OD-SSB on the cell is NCD-SSB

  - FFS: Additional support of OD-SSB for CD-SSB located on sync-raster

  - [JR] Companies (HW, vivo, Samsung, Apple, QC, Ericsson) were against supporting OD-SSB that is on sync-raster (i.e. CD-SSB) due to possible impacts on idle/inactive UEs.

- Timeline of OD-SSB transmission

  - Time instance A: Beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] of OD-SSB burst which is at least T slots after the slot where UE receives a signaling from gNB to indicate OD-SSB transmission

  - WA: T is not less than T_min = m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and Nslotsubframe,μ is as defined in current specification.

    - RAN4 to confirm that T_min can be equal to m+3Nslotsubframe,μ+1

  - WA: T=T_min

- L1 measurements on OD-SSB

  - Support aperiodic measurements based on existing CSI framework

  - FFS: potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB

  - Support of LTM is a separate discussion point

- L3 measurements on OD-SSB

  - Support. Further work is up to RAN2/RAN4

- Other topics discussed (with no agreements)

  - Scenarios for OD-SSB

    - Scenario 3A and 3B are objected by Apple and vivo

  - Whether/how to support multiple OD-SSB configs

    - All NW vendors wanted to support multiple OD-SSB configs (e.g. each containing different parameter set), but no consensus on the need for > one config.

    - Discussion direction then changed to first identify what is included in one OD-SSB config

  - Contents of OD-SSB configuration

    - Parameters identified: Frequency of the OD-SSB, SSB position in burst, periodicity, SCS, PCI, Tx power, number of bursts, gap between bursts, T value

  - Transmission pattern of OD-SSB

    - All time spent on discussing/clarifying views on timeline (T value, time instance A) than on the pattern given the multiple understandings on start time

  - L1/L3 measurements on OD-SSB

    - Whether to support OD-SSB for BFD-RS (LG, Goog, DCM, NEC are supportive)

    - Whether/how to configure OD-SSB in CSI-RS resource config

      - Companies mentioned to first discuss whether UE should measure always-on SSB and OD-SSB separately or jointly to derive one CSI (e.g. for Case #2)

  - UE-triggered OD-SSB

    - Tagged as low priority proposal in FL summary

RAN1#118bis Agreements

Agreement For a cell supporting on-demand SSB SCell operation, deactivation of on-demand SSB transmission is supported. In order to deactivate on-demand SSB transmission from a UE perspective, support at least one of the following options. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Option 1A: Explicit indication of deactivation for on-demand SSB via RRC for on-demand SSB transmission indication - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - Option 3: Configuration/indication of the duration of on-demand SSB transmission window - Option 4: On-demand SSB transmission, if any, is deactivated when UE receives SCell deactivation MAC-CE for the activated SCell - Option 4A: On-demand SSB transmission, if any, is deactivated when the timer for SCell deactivation is expired - Option 5: On-demand SSB transmission, if any, is deactivated when SCell activation is completed - Option 6: Explicit indication of deactivation for on-demand SSB via [group-common] DCI - FFS: Each option is applicable to which Cases or Scenarios - FFS: Details related to each of the above options Agreement For a cell supporting on-demand SSB SCell operation, support to provide at least the following parameters for on-demand SSB configuration by RRC at least for Case #1. - Sub-carrier spacing of the on-demand SSB - FFS if this can be absent - Physical Cell ID of the on-demand SSB - FFS: Time domain location of on-demand SSB burst such as SFN offset and half frame index - Downlink transmit power of on-demand SSB - FFS: The number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS whether the above parameters are configured by reusing legacy RRC parameters or new RRC parameters Agreement For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), consider only one or both of the following options for UE to perform L1 measurement based on on-demand SSB. - Option 1: A CSI report configuration is associated with both of on-demand SSB and always-on SSB - Option 2: A CSI report configuration is associated with one of always-on SSB and on-demand SSB - FFS: Whether OD-SSB and always on SSB have same beam or not Conclusion No consensus on the support of on-demand SSB SCell operation triggered by UE. Agreement The previous RAN1 agreement is partly confirmed and further revised as follows. - For SSB burst(s) indicated by on-demand SSB SCell operation via a MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows. - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] ofwithin the first “possible” on-demand SSB burst which is at least T slots after the slot where UE receives a signalling from gNB to indicate on-demand SSB transmission - The SSB time domain positions of on-demand SSB burst are configured by gNB. - The location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation - (Working assumption): T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and Nslotsubframe,μ is as defined in current specification. - RAN4 to confirm that T_min can be equal to m+3Nslotsubframe,μ+1 - (Working assumption) T=T_min - Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology. Agreement For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), study at least the following Mux-Cases. - Mux-Case #1: No time-domain overlap between always-on SSB and on-demand SSB - Mux-Case #2: Always-on SSB and on-demand SSB overlap at least in time or frequency domain