3GPP TSG-RAN WG2 Meeting #129	R2-2500792
Athens, Greece, February 17th – 21st, 2025

Agenda Item:	8.7.4

Source:	InterDigital

Title:	LCP enhancement for XR

Document for:	Discussion, Decision

# Introduction

The Rel-19 WID on XR the following objective on LCP enhancements:

Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]:

- Specify additional Logical Channel priority handling using delay/deadline information of packets;

This contribution address remaining open issues on how to realize the additional LCH priority in LCP when the UE has delay critical data to transmit.

# Discussion

The following agreements on LCP enhancements were made in RAN2#127bis and 128[2]:

Agreements on LCP enhancements - As a baseline, additional LCH priority is applied for an LCH in both 1st and 2nd Rounds of resource allocation procedure in LCP, as long as the LCH has delay-critical data available for transmission when starting the 1st Round. - Introduce an independent per-LCH remaining time threshold for applying delay-critical priority. - We do not introduce any setting restrictions of this new remaining time threshold with relation to DSR triggering threshold. Agreements on LCP prioritization - As a baseline, the additional LCH priority is applied to both the first round and the second round of the LCP procedure. The UE does not fallback to the default LCH priority in the second round even if there is no more LCH priority-adjusted data after the first round. - As an optional capability, the UE can also support to fallback to default priority in the 2nd round of LCP.

In the baseline UE behaviour, the additional LCH priority is applied for an LCH in both 1st and 2nd Rounds of resource allocation procedure in LCP, as long as the LCH has delay-critical data available for transmission when starting the 1st Round.

Proposal 1:	For the purpose of LCP enhancement for XR, the evaluated remaining time is the smallest remaining value of the running PDCP discardTimers, as specified in clause 7.3 in TS 38.323, among PDCP SDUs that are buffered for the logical channel and have not been transmitted in any MAC PDU.

Proposal 2:	For the baseline UE supporting LCP enhancement for XR, the UE applies a configured additional priority to both the first round and the second round of the LCP procedure if the remaining time is less than a configured threshold.

For the agreed optional capability behaviour, the additional LCH priority is applied for an LCH in 1st Round of resource allocation procedure in LCP, if the LCH has delay-critical data available for transmission when starting the 1st Round, and the additional LCH priority is applied for an LCH in 2nd Round of resource allocation procedure in LCP, if the LCH has residual delay-critical data available for transmission when starting 2nd Round.

Proposal 3:	For the UE supporting the additional optional capability for falling back to the default priority:

- the UE applies a configured additional priority to the first round and the second round of the LCP procedure if the remaining time is less than a configured threshold.

- the UE applies the default priority to the second round and the second round of the LCP procedure if the remaining time is larger than a configured threshold.

From a specification perspective, the baseline and the optional UE capability behaviours are shown in text proposals to TS 38.321 in Annex A.

Proposal 4:	Adopt TPs in Annex A for baseline and the optional UE capability behaviours for LCP.

Other remaining open issues:

One remaining FFS from RAN2#127 is “FFS whether/how additional priority impacts intra-UE prioritization (can be discussed in stage-3).” Intra-UE prioritization was introduced in R16 URLLC for handling overlapping grants and UCI/SR resources. Intra-UE prioritization can be a separate capability from XR, and is thus not necessary in the scope of this objective.

Proposal 5:	The configured additional priority for LCP is not considered in intra-UE prioritization, introduced for R16 URLLC.

Another open issue is to whether to also enhance PBR and Bj when the LCH’s priority is upgraded per above. One scenario that is possible is Bj can be negative for the logical channel, which results in not having the LCH served in LCP. in the current design, if Bj is negative, the delay-critical LCH will not be served/selected regardless of how the new value of priority, which can happen for bursty traffic with limited BSD. One proposed option is to temporarily increase the PBR while the remaining time is less than the threshold, to help more delay-critical data to be transmitted. However, it is ambiguous how Bj or PBR can be temporarily raised and how to configure a value it should be raised to. Increasing the PBR to infinity, also proposed by other companies, creates a fairness problem to other LCHs and is thus undesirable. Another option is to allow an LCH with an upgraded priority to be transmitted even if Bj is negative, while the remaining time is less the configured threshold.

Another alternative is to rely on the network to configure PBR to be large enough to consider bursty traffic from the get-go. If PBR is configured properly, this case may be viewed as corner case, as LCH has not been allocated much UL resources with the default/downgraded priority, and likely the Bj level has accumulated to a required rate once the priority is upgraded. Further, there is always the possibility for the network to allocate a larger grant, where allocating more resources in the round 2 of LCP is possible regardless of Bj, especially given RAN2 agreed to also use the upgraded priority in the second round of LCP.

Proposal 6:	RAN2 to discuss the following options to ensure an LCH with upgraded priority is served in LCP:

- allow an LCH with an upgraded priority to be transmitted even if Bj is negative, while the remaining time is less the configured threshold.

- rely on the network implementation, e.g. by configuring a proper value for PBR considering bursty traffic and dimensioning the grant size.

# Conclusion

The following observations and proposals are provided on LCP enhancement for XR:

# Annes A: Text proposals

### 5.4.3	Multiplexing and assembly

#### 5.4.3.1	Logical Channel Prioritization

##### 5.4.3.1.1	General

The Logical Channel Prioritization (LCP) procedure is applied whenever a new transmission is performed.

RRC controls the scheduling of uplink data by signalling for each logical channel per MAC entity:

-	priority where an increasing priority value indicates a lower priority level;

-	prioritisedBitRate which sets the Prioritized Bit Rate (PBR);

-	bucketSizeDuration which sets the Bucket Size Duration (BSD);

-	lchRemainingTimeThreshold which sets the threshold on remaining time for using AdditionalPriority for the logical channel;

-	AdditionalPriority which sets the priority for the logical channel when remaining time for an SDU is less than lchRemainingTimeThreshold.

RRC additionally controls the LCP procedure by configuring mapping restrictions for each logical channel:

-	allowedSCS-List which sets the allowed Subcarrier Spacing(s) for transmission;

-	maxPUSCH-Duration which sets the maximum PUSCH duration allowed for transmission;

-	configuredGrantType1Allowed which sets whether a configured grant Type 1 can be used for transmission;

-	allowedServingCells which sets the allowed cell(s) for transmission;

-	allowedCG-List which sets the allowed configured grant(s) for transmission;

-	allowedPHY-PriorityIndex which sets the allowed PHY priority index(es) of a dynamic grant for transmission;

-	allowedHARQ-mode which sets the allowed UL HARQ mode for transmission.

The following UE variable is used for the Logical channel prioritization procedure:

-	Bj which is maintained for each logical channel j.

The MAC entity shall initialize Bj of the logical channel to zero when the logical channel is established.

For each logical channel j, the MAC entity shall:

1>	increment Bj by the product PBR × T before every instance of the LCP procedure, where T is the time elapsed since Bj was last incremented;

1>	if the value of Bj is greater than the bucket size (i.e. PBR × BSD):

2>	set Bj to the bucket size.

NOTE:	The exact moment(s) when the UE updates Bj between LCP procedures is up to UE implementation, as long as Bj is up to date at the time when a grant is processed by LCP.

1>	if AdditionalPriority is configured and the remaining time, which is the smallest remaining value of the running PDCP discardTimers among PDCP SDUs that are buffered for the logical channel but have not been transmitted in any MAC PDU as specified in clause 7.3 in TS 38.323 [4], is less than lchRemainingTimeThreshold:

2>	set the priority for the logical channel to AdditionalPriority.

##### 5.4.3.1.2	Selection of logical channels

The MAC entity shall, when a new transmission is performed:

1>	select the logical channels for each UL grant that satisfy all the following conditions:

2>	the set of allowed Subcarrier Spacing index values in allowedSCS-List, if configured, includes the Subcarrier Spacing index associated to the UL grant; and

2>	maxPUSCH-Duration, if configured, is larger than or equal to the PUSCH transmission duration associated to the UL grant; and

2>	configuredGrantType1Allowed, if configured, is set to true in case the UL grant is a Configured Grant Type 1; and

2>	allowedServingCells, if configured, includes the Cell information associated to the UL grant. Does not apply to logical channels associated with a DRB configured with PDCP duplication within the same MAC entity (i.e. CA duplication) when CA duplication is deactivated for this DRB in this MAC entity; and

2>	allowedCG-List, if configured, includes the configured grant index associated to the UL grant; and

2>	allowedPHY-PriorityIndex, if configured, includes the priority index (as specified in clause 9 of TS 38.213 [6]) associated to the dynamic UL grant; and

2>	allowedHARQ-mode, if configured, includes the allowed UL HARQ mode for the HARQ process associated to the UL grant.

NOTE:	The Subcarrier Spacing index, PUSCH transmission duration, Cell information, and priority index are included in Uplink transmission information received from lower layers for the corresponding scheduled uplink transmission.

##### 5.4.3.1.3	Allocation of resources

Before the successful completion of the Random Access procedure initiated for DAPS handover, the target MAC entity shall not select the logical channel(s) corresponding to non-DAPS DRB(s) for the uplink grant received in a Random Access Response or the uplink grant for the transmission of the MSGA payload. The source MAC entity shall select only the logical channel(s) corresponding to DAPS DRB(s) during DAPS handover.

The MAC entity shall, when a new transmission is performed:

1>	allocate resources to the logical channels as follows:

2>	logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s);

2>	decrement Bj by the total size of MAC SDUs served to logical channel j above;

2>	if AdditionalPriority and [capability TBD] are configured and the remaining time, which is the smallest remaining value of the running PDCP discardTimers among PDCP SDUs that are buffered for the logical channel but have not been transmitted in any MAC PDU as specified in clause 7.3 in TS 38.323 [4], is greater than lchRemainingTimeThreshold:

3>	set the priority for the logical channel to priority.

2>	if any resources remain, all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally.

NOTE 1:	The value of Bj can be negative.

If the MAC entity is requested to simultaneously transmit multiple MAC PDUs, or if the MAC entity receives the multiple UL grants within one or more coinciding PDCCH occasions (i.e. on different Serving Cells), it is up to UE implementation in which order the grants are processed.

The UE shall also follow the rules below during the scheduling procedures above:

-	the UE should not segment an RLC SDU (or partially transmitted SDU or retransmitted RLC PDU) if the whole SDU (or partially transmitted SDU or retransmitted RLC PDU) fits into the remaining resources of the associated MAC entity;

-	if the UE segments an RLC SDU from the logical channel, it shall maximize the size of the segment to fill the grant of the associated MAC entity as much as possible;

-	the UE should maximise the transmission of data;

-	if the MAC entity is given a UL grant size that is equal to or larger than 8 bytes (when eLCID is not used) or 10 bytes (when eLCID is used) while having data available and allowed (according to clause 5.4.3.1) for transmission, the MAC entity shall not transmit only padding BSR and/or padding.

The MAC entity shall:

1>	if the MAC entity is configured with enhancedSkipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or if the MAC entity is configured with enhancedSkipUplinkTxConfigured with value true and the grant indicated to the HARQ entity is a configured uplink grant:

2>	if there is no UCI to be multiplexed on this PUSCH transmission as specified in TS 38.213 [6]; and

2>	if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and

2>	if the MAC PDU includes zero MAC SDUs; and

2>	if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR:

3>	not generate a MAC PDU for the HARQ entity.

1>	else if the MAC entity is configured with skipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the grant indicated to the HARQ entity is a configured uplink grant:

2>	if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and

2>	if the MAC PDU includes zero MAC SDUs; and

2>	if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR:

3>	not generate a MAC PDU for the HARQ entity.

Logical channels shall be prioritised in accordance with the following order (highest priority listed first):

-	MAC CE for C-RNTI, or data from UL-CCCH;

-	MAC CE for (Enhanced) BFR, or MAC CE for Configured Grant Confirmation, or MAC CE for Multiple Entry Configured Grant Confirmation;

-	MAC CE for Sidelink Configured Grant Confirmation;

-	MAC CE for LBT failure;

-	MAC CE for SL LBT failure according to clause 5.31.2;

-	MAC CE for Timing Advance Report;

-	MAC CE for Delay Status Report;

-	MAC CE for SL-BSR prioritized according to clause 5.22.1.6;

-	MAC CE for SL-PRS Resource Request;

-	MAC CE for (Extended) BSR, with exception of BSR included for padding;

-	MAC CE for (Enhanced) Single Entry PHR, or MAC CE for (Enhanced) Multiple Entry PHR or MAC CE for Single Entry PHR with assumed PUSCH, or MAC CE for Multiple Entry PHR with assumed PUSCH, or MAC CE for Enhanced Single Entry PHR for multiple TRP or MAC CE for Enhanced Multiple Entry PHR for multiple TRP, or MAC CE for Enhanced Single Entry PHR for multiple TRP STx2P or MAC CE for Enhanced Multiple Entry PHR for multiple TRP STx2P;

-	MAC CE for Positioning Measurement Gap Activation/Deactivation Request;

-	MAC CE for the number of Desired Guard Symbols;

-	MAC CE for Case-6 Timing Request;

-	MAC CE for (Extended) Pre-emptive BSR;

-	MAC CE for SL-BSR, with exception of SL-BSR prioritized according to clause 5.22.1.6 and SL-BSR included for padding;

-	MAC CE for IAB-MT Recommended Beam Indication, or MAC CE for Desired IAB-MT PSD range, or MAC CE for Desired DL Tx Power Adjustment;

-	data from any Logical Channel, except data from UL-CCCH;

-	MAC CE for Recommended bit rate query;

-	MAC CE for BSR included for padding;

-	MAC CE for SL-BSR included for padding.

NOTE 2:	Prioritization among MAC CEs of same priority is up to UE implementation.

The MAC entity shall prioritize any MAC CE listed in a higher order than 'data from any Logical Channel, except data from UL-CCCH' over NR sidelink transmission.

#### 5.4.3.2	Multiplexing of MAC Control Elements and MAC SDUs

The MAC entity shall multiplex MAC CEs and MAC SDUs in a MAC PDU according to clauses 5.4.3.1 and 6.1.2.

NOTE:	Content of a MAC PDU does not change after being built for transmission on a dynamic uplink grant, regardless of LBT outcome.