3GPP TSG-RAN WG2 Meeting #129	R2-2500793
Athens, Greece, February 17th – 21st, 2025

Agenda Item:	8.7.3

Source:	InterDigital

Title:	RRM measurement gap related enhancements for XR

Document for:	Discussion, Decision

# Introduction

The Rel-19 WID on XR has the following objective on RRM measurement gap related enhancements [1]:

Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4]

The topic was discussed the last two RAN2 meeting, and the following was agreed:

R2#127bis Agreements for RRM measurement gap skipping - RAN2 assumes that at least some impact on DSR from MG skipping can be avoided by NW implementation. FFS whether there is an impact which would require some specification changes/enhancements. - No need to have delay-aware LCP enhancements specific for MG skipping, i.e. MG skipping and delay-aware LCP are designed as independent features - RAN2 can further evaluate whether there is any impact on DRX from MG skipping. For the moment, the issue is unclear. - RAN2 will focus its work on supporting the solution chosen by RAN1/RAN4 - RAN2 can discuss whether there is a need to additionally have other solution (e.g. RRC-based) which can be decided after RAN1/RAN4 evaluation and knowing more details of DCI-based solution R2#128 Agreements on RRM measurement gaps impacts - No MG-specific enhancements is needed on DSR operation. - RAN2 assumes that UE follows DRX pattern as currently, even when MG is indicated as skipped No MG-specific enhancements is needed for DRX operation.

The contribution discusses the open L2 issues on RRM measurement gap related enhancements for XR.

# Discussion

In RAN2#127bis, this objective on RRM measurement gap related enhancements for XR was discussed. At the time it was agreed that RAN2 will only focus on any required support for solutions chosen by RAN1/RAN4. RAN1 sent an LS to RAN4 about the feasibility of their DCI-based solution for RRM measurement gap skipping, and to evaluate the need to introduce semi-static indication-based solutions for measurement gap skipping. RAN4 replied to RAN1 [1], confirming the feasibility of the DCI based solution from a RAN4 perspective. Further, RAN4 is still evaluating the need for a semi-static indication-based solution, and those discussions remain ongoing. Therefore, RAN2 should wait for RAN4 discussions to conclude on whether there is a need for semi-static indication-based solution for measurement gap skipping.

Proposal 1:	RAN2 waits for RAN4/RAN1 discussions to conclude on whether there is a need for semi-static indication-based solution for measurement gap skipping.

# Conclusion

The following proposals are provided on RRM measurement gap related enhancements for XR:

Proposal 1:	RAN2 waits for RAN4/RAN1 discussions to conclude on whether there is a need for semi-static indication-based solution for measurement gap skipping.

# References

[1] R4-2420198, Response LS on gaps/restrictions that are caused by RRM measurements, RAN4