3GPP TSG-RAN WG2 Meeting #129	R2-2500795

Athens, Greece, February 14 – 18 2025

Agenda item:	8.6.2

Source:	Nokia

Title:	Further analysis on open issues for inter-CU LTM

WID/SID:	NR_Mob_Ph4 - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In this contribution, we discuss the baseline procedure for inter-CU LTM support and identify the related key issues.

# 2	Discussion

We analyze the following key issues related to Inter-CU LTM and provide our proposals on way forward on these issues in this paper.

- LTM CSI Resource configuration and Reporting configuration changes needed for Inter-CU subsequent LTM scenarios.

- Remaining issues related to non-DC LTM

  - UE based TA related configuration

  - Support for non-subsequent LTM

  - Shared usage of configured grant for Inter-CU LTM

- Remaining Inter-CU DC LTM issues

  - Analysis on MCG-LTM with SCG addition scenario

  - Inter-SN SCG LTM configuration structure and associated impacts for co-existence with MCG-LTM

- Security Key Management for Inter-CU LTM

  - SK-counter configurations for Inter-SN SCG LTM and SK-counter configuration for Inter-MN MCG LTM.

  - Remaining issues for security key handling for Inter-CU Recovery.

## 2.1	LTM CSI Resource and Reporting related changes

### 2.1.1	Modifications for LTM-Report-configuration mapping to LTM-CSI-Resources

RAN2 and RAN3 have agreed the following for the CSI RS Resource configuration and report configuration in Inter-CU LTM scenario. We discuss further changes needed for Inter-CU scenarios with this agreement.

RAN2#127 agreements: CSI resource and report configuration - The Rel-18 signaling structure for LTM CSI resource and report configuration is reused for inter-CU LTM, i.e. a common CSI resource configuration and cell-specific CSI report configuration. - The source CU is responsible to generate the common CSI resource configuration. RAN3#126 agreement - The source CU can request candidate CU to provide CSI-RS configuration in HANDOVER REQUEST message, and candidate CU signals the CSI-RS configuration in HANDOVER REQUEST ACKNOWLEDGEMENT message. - The source CU generates common CSI-RS Resource Configuration and sends it to candidate CU in LTM CONFIGURATION UPDATE message, the candidate CU signals the CSI-RS Report Configuration in LTM CONFIGURATION UPDATE ACKNOWLEDGEMENT message.

In the baseline Rel-18 LTM preparation message sequence, the candidate cells provide the RS configurations (SSB configurations). The CU generates the LTM CSI resource sets that includes the list of SSB and candidate cell indices. The similar mechanism will be used for inter-CU scenario where the source CU will generate the common LTM CSI resource sets based on the RS configuration information from intra or inter-CU candidate cells. As the source CU only knows the RS configuration of candidate cell, the LTM-CSI-Resource configuration it generates needs to be based on the L3 measurement result(s) that triggered the preparation. The source CU is not aware of any information about the potential target cells of each candidate cell for subsequent mobility for generating candidate-cell specific resource-configuration.

Observation 1: LTM CSI resource configuration is generated by source CU based on the candidate RS configuration (SSB/CSI-RS) received and the L3 measurement results that triggered the LTM Preparation.

With L3 measurement results received from current serving cell as basis, the source CU can generate the LTM CSI Resource configuration in either of the following means.

- O1: Source CU generates single LTM CSI resource configuration that includes all the SSBs of all candidate cells prepared for LTM.

- O2: Source CU generates separate LTM CSI resource configuration for the SSBs of each candidate cell. In this case the source CU generates N LTM CSI resource configurations where N is the number of LTM candidate cells.

Observation 2: For source CU generation of LTM CSI resource configuration, either single resource configuration containing all candidate SSBs from all prepared candidate cells or separate resource configuration for each candidate are the possible options.

When the source CU provides the LTM CSI resource configuration to a candidate cell, the candidate CU generates a LTM report configuration that points to the LTM CSI resource configuration for the candidate cells controlled by this CU.

In case of O1 all the LTM report configurations need to point to the single LTM CSI resource configuration ID. This resource configuration is large resource set consists of all RS of all candidate cells. In this option, the report configuration maps to resource set that contains SSB of all candidate cells. However, all the LTM candidate cells are not potential candidate for subsequent mobility from given candidate cell. In this case measurement of all candidate cells and all beams is redundant from UE measurement perspective. The UE may need to measure all the candidate SSB in this case similar to L3 measurements. This is not efficient considering the regular L1 measurement result that needs to provide best L1 measurements periodically.

In case of O2, the candidate CU can decide to map its each of its report configuration to different LTM CSI resource configuration based on their preference for subsequent mobility. In this option candidate CU needs to have separate report configurations and also separate UL resources for reporting measurement results of each of the target cells. For making decision to select between two target cells, the candidate CU needs to receive reports associated with two separate report configurations. Because Rel-18 LTM report configuration can only be linked to one LTM CSI resource configuration.

In case of subsequent mobility, the LTM measurements and reporting needs to be determined based on the current serving cell and its preference for outward mobility. Hence, in both these options of source CU generation of LTM CSI resource configuration further information needed from candidate CU to modify the LTM measurements and reporting for subsequent mobility.

Observation 3: With current RAN2 agreements on LTM CSI resource configuration and re-use of Rel-18 LTM report configuration, inter-CU subsequent LTM measurements and reporting is not efficient from signalling and UE processing perspective.

Above problems can be mitigated in following ways.

- For O1, when the source CU generates a single LTM CSI resource configuration consists of all candidate beams, there can be two simple ways to solve the above issues:

  - Alternative 1 – Each candidate cell can provide additional information selecting a subset of cells/resources from the LTM CSI resource configuration (e.g., a bitmap of the cell or resource indices) to be considered for LTM measurements when it becomes serving cell. This information can be included within the LTM report configuration as additional information in addition to a pointer to LTM CSI resource config.

  - Alternative 2 – Each candidate cell should allow to dynamically select a subset of cells/SSBs form the LTM CSI resource configuration associated with a report configuration, and provide it to the UE, e.g., via MAC-CE. This approach may have additional benefit, as appropriate resources can be selected dynamically as the UE moves.

- For O2, when the multiple LTM CSI resource configurations are generated, each containing the SSBs of only one candidate cell, the report configuration can be optimised if the LTM report configuration can be mapped to more than one LTM CSI resource configurations.

Proposal 1A: For the NW implementation option when source CU generates a single LTM CSI resource configuration consisting of all candidate beams, in order to provide flexibility for each candidate cell to select its own SSBs/candidate cells for LTM measurements, RAN2 to consider at least one of the following options:

- Alternative 1 - An LTM report configuration includes additional information on the candidate cells /SSBs to measure from the linked LTM CSI resource configuration for subsequent mobility.

- Alternative 2 - Support dynamic selection of a subset of resources for measurements from the LTM resource config associated with a report configuration.

Proposal 1B: For NW implementation when source CU generates separate LTM CSI resource configurations, each containing SSBs of one candidate cell, RAN2 to consider that a LTM report configuration can be linked to more than one LTM CSI resource configurations.

## 2.2	Remaining Issues for non-DC LTM

### 2.2.1	UE-based TA estimation support for inter-CU LTM

In RAN2#125bis, the following was agreed

1. Early DL and UL sync is also supported for inter-CU LTM. Inform RAN3 of this. Early DL sync using CSI-RS should be considered, pending RAN1 approval. 2. PDCCH ordered early RACH is supported for inter-CU LTM.

In their LS (R4-2314455) RAN4 informed the other WGs that UE-based TA measurement can be supported in Rel. 18. From RAN2 viewpoint, UE knows that it is configured with UE-based TA measurements if the value of ltm-UE-MeasuredTA-ID within LTM-Candidate IE is equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID. Moreover, in case of RACH-less LTM, if the LTM CSC does not give a valid TA value, and UE is configured with UE-based TA measurement for the target cell, UE can use the TA value it has measured itself,

Observation 4: UE-based TA measurement is supported for Rel. 18 intra-CU LTM and can be used by the UE during RACH-less when the LTM CSC does not provide a valid TA value.

As Rel-18 only supports Intra-CU LTM, the serving CU determines the UE-based TA estimation configuration based on its serving cell.  According to the RAN4 LS (R4-2314455), the support for UE-based TA measurement is dependent on TAE, DL timing estimation error (of both serving and target cell), serving cell TA resolution and TA adjustment error.  For Inter-CU LTM in Rel-19 if this field needs to be extended across all the candidates across different CU, additional coordination will be required to exchange the information related to possibility of TA-estimation across Inter-CU cells.  Depending on support of such coordination NW may decide to restrict the UE-based TA within same CU or extend it across CU

Proposal 2: RAN2 to discuss how the UE-based-TA-measurement-ID is coordinated across Inter-CU candidate cells based on the knowledge of cells of candidate CU that can be considered for UA-based TA estimation.

### 2.2.2	Support for Non-Subsequent LTM

In RAN2#125bis the following were agreed with respect to subsequent LTM

Agreements on scenarios: - RAN2 first focus on inter-CU LTM in NR standalone scenario and use it as baseline for supporting inter-CU LTM in NR-DC scenarios. - Rel-19 inter-CU LTM also supports mixture of subsequent inter-CU LTM and subsequent intra-CU LTM after an inter-CU or intra-CU LTM switch. - UE can be configured with a mixture of intra-CU and inter-CU candidate LTM cells and irrespective of how the UE is configured with this mixture, UE measurement and reporting procedures will be the same for both intra-CU and inter-CU candidate LTM cells.

The Inter-CU LTM involves security key change and also PDCP re-establishment as additional steps. Subsequent LTM is beneficial for fast switching without any higher layer impacts. Usage of subsequent LTM in Inter-CU is required for cases where a high number of ping-pongs is expected. For instance, consider generic movement, wherein the UE may move between different cells without any pattern. In this case, the user may or may not go back to the same cell. In such cases, preparing multiple candidates would likely lead to inefficient resource usage.

However, in some scenarios the NW may need to support one time mobility towards some candidates and subsequent mobility across other candidates. This case is similar to support of CPAC and subsequent CPAC together in single configuration. In such cases non-subsequent LTM towards selected candidate is needed to avoid maintaining the other configurations for which no further subsequent mobility is expected.  This support becomes more relevant for Inter-CU LTM where there could be some issues in candidate CU supporting reference configurations and exchange of information related to Early-sync procedures. It is better to configure non-subsequent mobility towards these candidate cells

Observation 5:  In some Inter-CU LTM scenarios subsequent mobility is not needed at some candidate cells. Release of configuration of other candidates is preferred in this case.

Proposal 3:  Non-subsequent LTM towards selected candidates is supported for Inter-CU LTM.

### 2.2.3	Support of Configured Grant-based RACH-less LTM

In RAN2#126, the following agreement was made on the support of the first UL transmission for RACH-less inter-CU LTM

RAN2#126 Agreement: Support CG-based RACH-less and DG-based RACH-less procedures for inter-CU LTM.

Compared to dynamic grants (DGs), configured grants (CGs) enable faster access to the target cell, as the UE does not need to monitor PDCCH to receive the UL grant and can directly perform its first UL transmission on one of the valid occasions of the provided CG.

Nevertheless, a known issue of the use of CGs in LTM is the potential excessive resource reservation to support a single RACH-less LTM switch. For each UE prepared for RACH-less LTM and for each candidate cell, a CG must be included in the candidate’s RRC configuration provided to the UE during the preparation phase. Therefore, the PUSCH resources of these grants are reserved for a long period of time stretching from the preparation phase to the execution of one or multiple cell switches and they may never be used, if the UE does not switch to the corresponding cell. The issue may be even more pronounced in inter-CU LTM, as it cannot be solved or mitigated with intra-gNB resource scheduling optimizations.

Observation 6: Support of CG-based inter-CU LTM may lead to excessive PUSCH resource reservation.

. To optimise the PUSCH resource reservation issue,  the probabilistic nature of the use of the provided PUSCH resources can be exploited in a similar way as it is done for CFRA resources. PUSCH resources can be shared by multiple UEs, as it is unlikely that they switch to the same candidate cell at the same time. Still, it is possible that UEs sharing the same PUSCH resource will use the same CG occasion for their first UL transmission to the candidate. Therefore, coordination of the use of shared PUSCH resources is required. As cell switch execution is controlled by the source node, shared PUSCH resources can be provided by the candidate node to the source, which in turn can coordinate their use during cell switch.

Proposal 4: To avoid excessive resource reservation overhead for CG-based RACH-less LTM, PUSCH resources can be shared among multiple UEs. PUSCH resource sharing among UEs should be coordinated by the source node.

## 2.3	Remaining issues for DC LTM

### 2.3.1	RRC model for LTM configuration for inter-CU LTM DC

Unlike in Rel-18 Intra-CU MCG and Intra-CU SCG LTM wherein the LTM candidate configuration includes only one CG config (i.e. either MCG or SCG); In #127, RAN2 acknowledged that in Inter-CU MCG LTM, SCG configuration can change; likewise, in inter-CU SCG LTM, the MCG configuration might change as well based on the inter-CU SCG LTM candidate configuration. thus, the candidate configuration must include both MCG and SCG configuration.

[RAN2#127] Reuse the Rel-18 reference configuration structure and not support additional reference configurations. [RAN2#127bis] The reference configuration for inter-CU SCG LTM at least include SCG part, FFS on MCG part. [RAN2#127bis] FFS: Network ensures that when UE combines the reference and candidate configuration for inter-CU SCG LTM, the configuration generated by UE must contain both MCG and SCG part configurations. [RAN2#127bis] The candidate configuration and reference configuration are modeled as an MN RRCReconfiguration message. [RAN2#127bis] The MN generates the MCG part of the reference configuration (if any), while the SN (source or candidate) generates the SCG part of the reference configuration. [RAN2#127bis] The MN is responsible for the reference configuration generation for SN initiated inter-CU SCG LTM. It can be up to the NW implementation whether to include the MCG part [RAN2#127bis] The MN can request an SCG reference configuration from any of the involved SNs. [RAN2-128] 10. It’s up to NW to ensure that the complete configuration includes the MCG part and SCG part configuration when UE combines the reference and candidate configuration for inter-CU SCG LTM.

Considering the above listed RAN2 agreements, we see the following options to model the candidate RRC configuration for inter-CU SCG LTM and inter-CU MCG LTM scenarios:

Scenario 1: Inter-CU SCG LTM with same MCG

During Inter-CU SCG LTM preparation, it is possible that changes in candidate SCG configuration compared to the source SCG configuration may require changes to associated MCG configuration in each inter-CU SCG LTM candidate configuration; this can be handled via one of the following options.

- Option 1: The reference configuration includes MCG and SCG Part. Candidate configuration includes delta-configuration for both parts.

- Option 2: The reference configuration contains only SCG part. If there is any change in the MCG part of the inter-CU SCG LTM candidate configuration (due to the above stated reason), then the MCG part of the inter-CU SCG LTM candidate configuration needs to be provided as complete configuration.

- Option 3: In case if there is no change in the MCG part of the configuration in the Inter-SN scenario the current MCG configuration can be considered as MCG configuration

Option 1 allows the UE to construct the MCG and SCG part of the candidate target cell configuration using MCG and SCG reference + associated MCG and SCG delta configuration parts in the LTM candidate configuration; this option allows the UE to apply only the delta parts of MCG and SCG LTM candidate configuration at the time of LTM cell change execution, thus the impacts of UE having to apply the complete MCG configuration at the inter-CU SCG LTM cell switch may be avoided.

Option 2 is similar to Rel-18 with an extension that MCG can be provided as full configuration (if MCG part is modified); which may require the UE to perform reconfigurationWithSync even if there is no PCell HO  involved.

During inter-CU SCG LTM Preparation, the source MN knows if there is change in MCG configuration part for each of the prepared candidates; If there is no change, then the Rel-18 approach can be considered. Otherwise, option 1 mentioned above should be used. This allows network to have the flexibility to choose Rel-18 model(option 2) or the new model(option 1) to prepare inter-CU SCG LTM configuration.

Observation 7: Whether MCG configuration needs to be included in the reference configuration /candidate configuration for Inter-SN SCG LTM depends on the need for change in MCG configuration during preparation.

Proposal 5A: Whether the candidate configuration /reference configuration for Inter-SN SCG LTM includes MCG is determined by common configuration in LTM-Config.

Likewise, in the case of Inter-CU MCG LTM, the SCG configuration in each inter-CU MCG candidate configuration may change; this may be handled via one of the following options:

- Option 1: The reference configuration includes both MCG and SCG parts.

- Option 2: Reference configuration includes only MCG. And the SCG configuration can be added as full configuration in the LTM candidate configuration.

Observation 8: SCG configuration in the prepared inter-CU MCG LTM candidate configuration may change compared to the SCG configuration at the source, thus reference configuration must include MCG and SCG parts to avoid the overhead of UE having to apply full SCG configuration at the time of inter-CU MCG LTM execution.

Proposal 5B: RAN2 agree to include reference configuration for MCG and SCG in the LTM Reference configuration for Rel-19 Inter-MN MCG LTM.

### 2.3.2	Coexistence of Inter-CU and Intra-CU LTM DC scenarios

RAN2 made the following agreements related to inter-CU and intra-CU co-existence scenarios:

[RAN2#127bis] RAN2 confirms to support coexistence of following cases: Inter-MN LTM and intra-SN LTM Inter-SN LTM and intra-MN LTM it is up to network implementation to ensure simultaneous execution for both MCG and SCG will not happen. [RAN2#128]: RAN2 confirms that the inter-CU MCG LTM with intra-SN PSCell change is supported in Rel19. [RAN2#128]: RAN2 confirms the support of following coexistence cases in NR-DC: Case 1: Intra-CU MCG LTM + Inter-CU MCG LTM Case 2: Intra-CU SCG LTM + Inter-CU SCG LTM [RAN2#128]: In coexistence cases of inter-CU MCG/SCG LTM and intra-CU MCG/SCG LTM, when inter-CU MCG or SCG LTM is executed, it’s up to the NW to ensure that maintained LTM candidate configurations are valid, e.g. reconfigure or release invalid intra-CU MCG/SCG LTM candidate configurations. UE does not autonomously release invalid intra-CU candidate configurations. [RAN2#128]: RAN2 to support intra-CU SCG LTM in MN RRC message (i.e. MN RRCReconfiguration message), in addition to SN RRC message.

We have considered the co-existence of inter-CU SCG LTM with intra-CU MCG LTM as an example scenario to highlight some issues that RAN2 must consider.

If we assume that the UE is configured with intra-CU MCG LTM(with unchanged SCG) followed by inter-CU SCG LTM configurations; and as explained in the previous section, it is likely that the SCG configuration prepared by each inter-CU candidate SN may differ from the SCG configuration at the source SN; and if the UE executes inter-SN SCG LTM and if the SCG configuration at the new serving SN is different compared to the SCG configuration at the source SN, then the intra-CU MCG LTM configuration stored at the UE may be invalid because it was prepared by the source MN considering the SCG configuration and DBR configurations at the source SN at the time of preparation. Thus, after every inter-SN SCG LTM, if the SCG configuration of the new serving SN is different compared to the SCG configuration at the source, the source MN must re-prepare the intra-CU MCG LTM candidates considering the new serving SN and reconfigure the UE to ensure that subsequent Intra-CG MCG-LTM results in similar configuration of the current serving MCG configuration.

Observation 9: After every inter-SN SCG LTM, if the SCG configuration of the new serving SN is different compared to the SCG configuration at the source, the source MN must re-prepare the intra-CU MCG LTM considering the new serving SN and reconfigure the UE to enable intra-CU PCell to work.

Such a re-preparation of intra-CU MCG LTM configuration and reconfiguring the UE after every inter-SN SCG LTM when the new SN SN’s SCG configuration differs from the SCG configuration of the source SN considered by the source MN at the time of intra-CU MCG LTM preparation shall mean that the subsequent cell switch in the inter-CU + Intra-CU co-existences cannot be supported.

Observation 10: Subsequent cell switch in the case of inter-CU + Intra-CU Co-existence scenarios cannot be supported If the CG configuration is not uniform across all the prepared inter-CU candidates.

Proposal 6: RAN2 to discuss the impact to subsequent Intra-CU MCG LTM after Inter-CU SCG-LTM that modifies the MCG configuration as part of SCG cell switch

### 2.3.3 	Inter-CU MCG LTM – SCG Addition after SCG release

RAN2 made the following agreement related to SCG Addition in the case of inter-CU MCG LTM.

[RAN2#128]: RAN2 confirms that inter-CU MCG LTM with SCG addition is supported assuming no much specification effort is required. If there are much specification efforts, we will not have it.

In Inter-CU MCG LTM scenario, SCG may be released, if inter-CU MCG LTM (i.e. inter-MN LTM PCell switch) is executed to a candidate MN that has prepared MCG configuration only by remapping the bearers mapped to the SCG resources at the source to MCG resources; for the subsequent inter-CU MCG LTM execution, if the candidate target MN has prepared DC configuration for the UE (i.e. MCG + SCG), then it was agreed by RAN2 via the above listed agreement to allow SCG addition in such a scenario, as SCG reconfiguration was already agreed by RAN2 to be supported in inter-CU MCG LTM scenarios. However, it is likely that the candidate target MN that triggers SCG addition (after SCG release in one of the previous inter-CU MCG LTM executions) might have prepared multiple candidate PSCells associated with the target candidate PCell.

When there are multiple candidate PSCells prepared associated with a candidate inter-MN target PCell, then an inter-CU MCG LTM execution which may result in SCG or PSCell addition (after an earlier SCG release) may result in a blind PSCell addition, which may potentially result in RL failure of UE’s link to the PSCell or sub-optimal DC throughput.

Observation 11: In an inter-CU MCG LTM scenario, when there are multiple candidate PSCells prepared associated with a candidate inter-MN target PCell, then an inter-CU MCG LTM execution which may result in SCG or PSCell addition (after an earlier SCG release) may result in a blind PSCell addition, which may potentially result in RL failure of UE’s radio link to the serving PSCell or sub-optimal DC throughput.

Furthermore, when there are multiple candidate PSCells prepared associated with a candidate inter-MN target PCell, then an inter-CU MCG LTM execution may result in intra-CU SCG or PSCell change (PSCell HO), that may not necessarily be based on latest radio measurements of the candidate PSCells. Which may result in sub-optimal DC throughput.

Proposal 7: RAN2 to discuss and agree on signaling enhancements to prevent a blind PSCell addition and change when the LTM candidate configurations for MCG-LTM includes SCG configuration.

## 2.4	Security key change-related issues

### 2.4.1	SK Counter list configuration for Inter-SN SCG LTM and Inter-MN MCG LTM

SK-Counter configuration for Inter-SN SCG LTM

Based on the RAN2-127 agreemennts, the security key change solution for Inter-SN SCG LTM will reuse the SCPAC principle.  The security key change solution for SCPAC introduces new parameter security-cell-set ID in each candidate configuration that is used by UE to differentiate whether two candidate cells belongs to same SN or not. For each security cell set ID set of SK-counter values are provided as separate configuration within conditional-reconfiguration variable. Following the similar approach for Inter-SN SCG LTM, new parameter similar to security-cell-set-ID is introduced in each candidate configuration and new structure that provide sk-counter list for each security-cell-set-ID is introduced in the LTM-Config variable.

In this regard we propose following changes in the LTM Config parameter. These changes are applicable in the LTM-Config for SCG.

LTM-Config-r18 ::=   SEQUENCE {

ltm-ReferenceConfiguration-r18        SetupRelease {ReferenceConfiguration-r18}

ltm-CandidateToReleaseList-r18        SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-CandidateId-r18    OPTIONAL,   -- Need N

ltm-CandidateToAddModList-r18         SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-Candidate-r18      OPTIONAL,   -- Need N

ltm-ServingCellNoResetID-r18          INTEGER (1..maxNrofLTM-Configs-plus1-r18)                             OPTIONAL,   -- Need N

ltm-CSI-ResourceConfigToAddModList-r18  SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourceConfigurations-r18)) OF LTM-CSI-ResourceConfig-r18

OPTIONAL,   -- Need N

ltm-CSI-ResourceConfigToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourceConfigurations-r18)) OF LTM-CSI-ResourceConfigId-r18

OPTIONAL,   -- Need N

attemptLTM-Switch-r18                 ENUMERATED {true}                                                    OPTIONAL, -- Cond LTM-MCG

ltm-ServingCellUE-MeasuredTA-ID-r18   INTEGER (1..maxNrofLTM-Configs-plus1-r18)                             OPTIONAL,   -- Need N

...,

[[

ltm-ServingCellNoSecurityChangeID-r19       INTEGER (1..maxNrofLTM-Configs-plus1-r18)                           OPTIONAL,   -- Need N

sk-CounterConfiguration-LTM-r19          SK-CounterConfiguration-LTM-r19                        OPTIONAL    -- Need M

cltm-ServingCellExecutionConditions-r19     SetupRelease {CLTM-ExecutionConditions-r19}                         OPTIONAL    -- Need M

]]

}

SK-CounterConfiguration-LTM-r19      ::= SEQUENCE {

sk-CounterConfigToReleaseList-r18    SEQUENCE (SIZE (1..maxSecurityCellSet-r18)) OF SecurityCellSetId-r18    OPTIONAL,    -- Need N

sk-CounterConfigToAddModList-r18     SEQUENCE (SIZE (1..maxSecurityCellSet-r18)) OF SK-CounterConfig-r18     OPTIONAL     -- Need N

}

SK-CounterConfig-r18 ::=             SEQUENCE {

ltm-ServingCellNoSecurityChangeID-r19       INTEGER (1..maxNrofLTM-Configs-plus1-r18),

sk-CounterList-r18                   SEQUENCE (SIZE (1..maxSK-Counter-r18)) OF SK-Counter

}

Table 1 : LTM-Configuration for Inter-SN SCG LTM

Proposal 8: For Inter-SN SCG LTM, new SK-Counter configuration consists of sk-counter list for each no-change-security-cell-set-ID is introduced in LTM Config. Ref Table 1 for the proposed modification for LTM-Config.

SK-counter configuration for Inter-MN MCG LTM without SCG change.

The security key generation principles described in TS33.501 indicates that whenever master key associated with MCG is changed the network and UE need to regenerate new S-KgNB and the derived keys associated with SCG bearers.

In classic handover scenario, above requirement is achieved by MN including explicit SK-counter value in the RRC Reconfiguration message that includes master-key change or NCC change.

Observation 12: In classic Inter-MN handover scenario without SCG change, the SK-counter value needs to be included along with master-key change configuration to enable regeneration of new security keys for SN .

In case of Inter-MN MCG LTM with same SCG, whenever inter MN LTM is executed the UE and MN can apply the new K-GNB based on NCC value provided in the MAC CE command or based on stored NCC values. The actual solution is discussed in SA3. However, how the SN keys are regenerated in this scenario is not yet discussed in RAN2. This issue is not indicated as key issue in the LS to SA3.

Observation 13: Issues related to regeneration of SN-Keys during Inter-MN MCG LTM is not discussed within RAN2. Not included in the list of issues for SA3 to address in the previous LS to SA3.

There can be multiple solutions to enable regeneration of SN keys without additional RRC configuration. Some of the options are as follows:

- O1: LTM Configuration includes SK-counter value for each Rel-19 ID (the identifier to differentiate Inter-MN MCG LTM). Whenever inter MN MCG LTM is executed both UE and MN make use of this SK-counter for generation of SN keys. One SK counter value is sufficient to handle multiple MN changes because the master key is changed in this case and same SK-counter value can be reused. O2: Without explicit SK-counter value, for inter-MN LTM with SCG, the UE and MN should regenerate new SN-keys with default SK-counter value =1. For further inter-SN change without MN change, MN should use the value starting from 2. In this case LTM with SCG unchanged the LTM candidate configuration need not include any SK-counter value.

Proposal 9:  RAN2 discuss and agree upon the potential options to trigger SN key regeneration for Inter-MN MCG-LTM and seek SA3 views through LS.

### 2.4.2	Security key usage for Inter-CU LTM recovery

In RAN2#127bis, the following agreement was made:

RAN2#127bis agreement: The Rel18 handling on failure is reused in R19 if the UE selects an intra-CU LTM candidate cell after intra-CU LTM failure; for other cases, e.g. inter-CU LTM failure, the failure handling is FFS (related to SA3’s inputs).

The SA3 LS reply is discussed in section 2.5.1. In either case a new message is used to indicate the LTM security configuration to the UE. In case UE have received the security parameters from the agreed method, UE can use the received parameters for security update during LTM recovery procedure after RLF or L3 based HOF.

Proposal 10A: The UE can use the configured inter-CU LTM security parameters for inter-CU LTM recovery triggered after RLF or L3 based HOF.

For the scenario of LTM recovery after LTM cell switch failure we have the following scenarios,

- recovery to inter-CU LTM candidate after intra-CU LTM failure,

- recovery to inter-CU LTM candidate (with respect to the failed target and initial source cell) after inter-CU LTM failure,

- recovery to intra-CU LTM candidate (with respect to the failed target) after inter-CU LTM failure.

- recovery to intra-CU LTM candidate (with respect to initial source cell) after inter-CU LTM failure,

For scenario 1, UE can use the stored LTM security configuration. These configurations will be untouched by the UE as the failed configuration is intra-CU.

For scenario 2, the UE uses the LTM security configuration for the initial inter-CU LTM procedure that has failed, UE can re-use the stored LTM security configuration to generate the keys for the new candidate CU from scratch.

For scenario 3, the UE uses the LTM security configuration for the initial inter-CU LTM procedure that has failed. UE can re-use the same security configuration to generate the new key for the recovery target.

For scenario 4, UE can re-use the Rel. 18 procedure.

Proposal 10B: In case the LTM security configuration is configured from the current serving MN, the UE can re-use the LTM security configuration for inter-CU LTM recovery for all other cases.

Finally, if UE does LTM recovery before the security parameters are configured to the UE, the solution for scenario 1, 2 and 3 cannot work. For these scenarios there are two options.

- UE triggers re-establishment.

- UE is pre-configured to do horizontal key derivation as default behavior when the LTM security configuration is not available at the UE side.

Proposal 10C: RAN2 discuss and agree among the below options:

- Option 1: A default behavior for the UE to use horizontal key generation in case if new LTM security configuration is not received from current serving MN

- Option 2: A default behavior to trigger re-establishment when the LTM security configuration is not received from current serving MN for scenario 1, 2 and 3.

# 3	Conclusion

In this paper, we analyzed the remaining open issues for Inter-CU LTM and made the following observations and proposals.

LTM CSI Resource and Reporting related changes

Observation 1: LTM CSI resource configuration is generated by source CU based on the candidate RS configuration (SSB/CSI-RS) received and the L3 measurement results that triggered the LTM Preparation.

Observation 2: For source CU generation of LTM CSI resource configuration, either single resource configuration containing all candidate SSBs from all prepared candidate cells or separate resource configuration for each candidate are the possible options.

Observation 3: With current RAN2 agreements on LTM CSI resource configuration and re-use of Rel-18 LTM report configuration, inter-CU subsequent LTM measurements and reporting is not efficient from signalling and UE processing perspective.

Proposal 1A: For the NW implementation option when source CU generates a single LTM CSI resource configuration consisting of all candidate beams, in order to provide flexibility for each candidate cell to select its own SSBs/candidate cells for LTM measurements, RAN2 to consider at least one of the following options:

- Alternative 1 - An LTM report configuration includes additional information on the candidate cells /SSBs to measure from the linked LTM CSI resource configuration for subsequent mobility.

- Alternative 2 - Support dynamic selection of a subset of resources for measurements from the LTM resource config associated with a report configuration.

Proposal 1B: For NW implementation when source CU generates separate LTM CSI resource configurations, each containing SSBs of one candidate cell, RAN2 to consider that a LTM report configuration can be linked to more than one LTM CSI resource configurations.

Remaining Issues for non-DC LTM

Observation 4: UE-based TA measurement is supported for Rel. 18 intra-CU LTM and can be used by the UE during RACH-less when the LTM CSC does not provide a valid TA value.

Proposal 2: RAN2 to discuss how the UE-based-TA-measurement-ID is coordinated across Inter-CU candidate cells based on the knowledge of cells of candidate CU that can be considered for UA-based TA estimation.

Observation 5:  In some Inter-CU LTM scenarios subsequent mobility is not needed at some candidate cells. Release of configuration of other candidates is preferred in this case.

Proposal 3:  Non-subsequent LTM towards selected candidates is supported for Inter-CU LTM.

Observation 6: Support of CG-based inter-CU LTM may lead to excessive PUSCH resource reservation.

Proposal 4: To avoid excessive resource reservation overhead for CG-based RACH-less LTM, PUSCH resources can be shared among multiple UEs. PUSCH resource sharing among UEs should be coordinated by the source node.

Remaining issues for DC LTM

Observation 7: Whether MCG configuration needs to be included in the reference configuration /candidate configuration for Inter-SN SCG LTM depends on the need for change in MCG configuration during preparation.

Proposal 5A: Whether the candidate configuration /reference configuration for Inter-SN SCG LTM includes MCG is determined by common configuration in LTM-Config.

Observation 8: SCG configuration in the prepared inter-CU MCG LTM candidate configuration may change compared to the SCG configuration at the source, thus reference configuration must include MCG and SCG parts to avoid the overhead of UE having to apply full SCG configuration at the time of inter-CU MCG LTM execution.

Proposal 5B: RAN2 agree to include reference configuration for MCG and SCG in the LTM Reference configuration for Rel-19 Inter-MN MCG LTM.

Observation 9: After every inter-SN SCG LTM, if the SCG configuration of the new serving SN is different compared to the SCG configuration at the source, the source MN must re-prepare the intra-CU MCG LTM considering the new serving SN and reconfigure the UE to enable intra-CU PCell to work.

Observation 10: Subsequent cell switch in the case of inter-CU + Intra-CU Co-existence scenarios cannot be supported If the CG configuration is not uniform across all the prepared inter-CU candidates.

Proposal 6: RAN2 to discuss the impact to subsequent Intra-CU MCG LTM after Inter-CU SCG-LTM that modifies the MCG configuration as part of SCG cell switch.

Observation 11: In an inter-CU MCG LTM scenario, when there are multiple candidate PSCells prepared associated with a candidate inter-MN target PCell, then an inter-CU MCG LTM execution which may result in SCG or PSCell addition (after an earlier SCG release) may result in a blind PSCell addition, which may potentially result in RL failure of UE’s radio link to the serving PSCell or sub-optimal DC throughput.

Proposal 7: RAN2 to discuss and agree on signaling enhancements to prevent a blind PSCell addition and change when the LTM candidate configurations for MCG-LTM includes SCG configuration.

Security key change-related issues

Proposal 8: For Inter-SN SCG LTM, new SK-Counter configuration consists of sk-counter list for each no-change-security-cell-set-ID is introduced in LTM Config. Ref Table 1 for the proposed modification for LTM-Config.

Observation 12: In classic Inter-MN handover scenario without SCG change, the SK-counter value needs to be included along with master-key change configuration to enable regeneration of new security keys for SN .

Observation 13: Issues related to regeneration of SN-Keys during Inter-MN MCG LTM is not discussed within RAN2. Not included in the list of issues for SA3 to address in the previous LS to SA3.

Proposal 9: RAN2 discuss and agree upon the potential options to trigger SN key regeneration for Inter-MN MCG-LTM and seek SA3 views through LS.

Proposal 10A: The UE can use the configured inter-CU LTM security parameters for inter-CU LTM recovery triggered after RLF or L3 based HOF.

Proposal 10B: In case the LTM security configuration is configured from the current serving MN, the UE can re-use the LTM security configuration for inter-CU LTM recovery for all other cases.

Proposal 10C: RAN2 discuss and agree among the below options:

- Option 1: A default behavior for the UE to use horizontal key generation in case if new LTM security configuration is not received from current serving MN

- Option 2: A default behavior to trigger re-establishment when the LTM security configuration is not received from current serving MN for scenario 1, 2 and 3.