3GPP TSG-RAN WG2 Meeting #129	R2-2500796

Athens, Greece, 17th Feb -21st Feb 2025

Agenda item:	8.9.2

Source:	Nokia, Nokia Shanghai Bell

Title:	RAN impacts for IoT-NTN SF Operations

WID/SID:	IoT_NTN_Ph3 - Release 19

Document for:	Discussion and Decision

# 1	Introduction

Following are the agreements in RAN2 on the SF operation for IoT-NTN in RAN2-127 and RAN2-128.

RAN2-127 Agreements:

1.	RAN2 adopts the SA2 study conclusions on the possible S&F architectures as the baseline for further discussion (RAN2 will only consider the full CN and spit-MME payload options)

2.	RAN2 will consider both single satellite pass and multiple satellite pass scenarios

3.	RAN2 will consider both MO and MT data within scope

4.	UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature)

5.	RAN2 does not further discuss whether legacy UEs will always need to be barred in a S&F network, at least not before further progress in SA2 discussion. If there will be a need for this, mechanisms to bar legacy UEs are already in place and no further impact on RAN2 specs in expected.

7.	RAN2 agrees there will be a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow R19 S&F capable UEs. FFS on the exact solution (e.g. new barring bits or whether this is linked to some other indication)

RAN2-127bis Agreements:

1.	The dynamic indication that “the cell is operating in S&F mode” is called “S&F operation” indication (we can come back on the exact name when putting this in the spec, if needed)

2.	RAN2 assumes that if an indication that “the cell is operating in S&F mode” is not provided in a NTN cell, the UE should assume that the NTN cell is operating in real-time mode (i.e. default/normal/”not-S&F” mode). RAN2 assumes there should be no distinction between the case that the UE is served by a NTN cell that do not support S&F capability and the case that the UE is served by a NTN cell that does support S&F capability but indication that “the cell is operating in S&F mode” is not provided.

3.	An S&F explicit capability indication by the serving cell, conceived as a static indication of whether the S&F capability is supported or not by a specific satellite/NTN-cell, in addition to the indication that “the cell is operating in S&F mode”, is not needed

4.	If the “S&F operation” indication is not broadcast, a Rel-19 UE (regardless whether supporting S&F or not) shall follow the legacy barring procedure

5.	When present, the “S&F operation” indication has two possible settings:

‘1’: the cell is operating in S&F mode for all UEs (Rel-19 UEs supporting S&F are allowed to access the cell)

‘0’: the cell is operating in S&F mode for UEs in Connected mode (which are not required to monitor the “S&F indication”), but idle Rel-19 UEs supporting S&F are barred (Rel-19 UEs not supporting S&F will follow legacy barring procedure).

RAN2-128 Agreements:

1.	Time information can be broadcasted to indicate when the current satellite operation mode will transit from “S&F operation” mode to real-time/normal mode. RAN2 assumes that a R19 UE could use this information at least to delay some NAS procedures until the feeder link is resumed (FFS what we specify in RAN2 specs for this and if we differentiate the behaviour for R19 UEs supporting and not supporting S&F mode, also considering the relation to the Wait Timer). FFS which SIB is used (SIB1 or SIB31). FFS if absolute or relative time is used. FFS on the opposite transition (i.e. whether to indicate when the “S&F operation” will start)

2.	It is FFS whether we need to introduce a new release cause when the NW wants to release a R19 UE to RRC idle because the feeder link becomes unavailable.

3.	“S&F operation” indication is provided in SIB1

4.	RAN2 assumes that AS signalling is not needed to indicate the architecture option (Split MME or Full CN).

5.	RAN2 waits for further progress in SA2, if any, before working on RAN2 aspects of a CIoT-UP solution for SF operation

6.	RAN2 understands that the UE configured with a satellite ID list by MME is not prevented to camp on a satellite operating in normal IoT NTN mode (i.e. with feeder-link connection), and perform subsequent access and data/signalling communication with that satellite

7.	RAN2 understands the UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list.

In this paper we analyse the remaining open issues for SF operation related to mode transition and access /paging procedure impacts in SF operation.

# 2	Discussion

## 2.1 Remaining Issues related to SF Operation indication

On the information related to transition of SF mode from SFG to normal mode, RAN2 has made agreement in RAN2-128 that time information is included to indicate the transition from SF mode to normal mode. However, whether absolute time or relative time is not concluded. This parameter is mainly referred by idle mode UE to know the remaining duration of SF operation to decide on the pending NAS and real time data transmission in the current cell. As the idle mode UE may camp onto this cell any time after the start of SF mode operation the absolute time is preferred rather relative time. Hence the time information for SF mode can follow the similar definition of ‘T-service’. This information is related to the service duration for real-time mode operation, it needs to be included in the system information that is not linked to timer-based acquisition. Hence SIB1 is preferred as placeholder for this parameter.

Proposal 1: The timer for SF mode transition is defined as absolute time is included in SIB1.

This timer information along with the status of SF operation is indicated to higher layers. The higher layers can decide on further actions on whether to delay NAS signalling and connection setup procedure for real time services. Handling of this parameter need not be specified in AS. This behavior is applicable for Rel-19 UE that can read this information  irrespective of whether UE support SF mode operation or not.

Proposal 2: The UE (capable of SF operation) behaviour on SF mode and the timer status is decided by higher layers for UE in RRC-IDLE mode. RAN2 assumes no impact to AS layer operations.  FFS connected mode.

When the satellite operates in normal mode no additional indication is included in the system information as per current RAN2 agreements on the SF mode operation. On change of SF mode from normal to SF mode the gNB may need to include barring information in system information to avoid legacy UE camping to this cell. This update is optional and used by the NW if it wants to enable access barring for legacy UE. The SF capable UE will know the mode transition from SIB1 modification. This can be used to modify the NAS behaviour for new transactions. However, if the UE wants to start some real-time data transmissions which can be completed within the duration of normal mode operation is useful at the UE to defer such transactions that may lead to failure.  Hence, we propose that RAN2 needs to discuss the usefulness of the timer for normal mode based on application scenarios.

Proposal 3: RAN2 to discuss the scenarios for benefits of time for mode transition from normal to SF operation for decision on the need for timer for real-time mode operation.

Need for additional parameters in the system information to indicate the maximum expected delay for the services was proposed from companies during RAN2-127bis.  As the NAS level registration procedure for SF capable UE already includes information about wait timer for next cell access and also the list of satellite-ID for next access that can help the UE to estimate the time of storage this additional information is not essential. Moreover whether the response can be delivered for MO traffic in next access is not fully guaranteed and it depends on the storage and other UE traffic at the time of feeder link connectivity. So this information will not be accurate.

Proposal 4: Additional information related to maximum delay impact of SF operation is not broadcasted in system information. UE can deduce this information from other parameters related SF operation provided in NAS level procedures.

## 2.2 Access Control and Paging Changes for SF operation

When NTN node is serving new area, it may need to handle the new data transmissions from the coverage area and also complete the data transmission in downlink that was received and stored from CN at the time of earlier connectivity. In such cases the access control and data transmission across these two variants of users needs to be differentiated. Mainly to avoid the delivering the stored data becoming invalid or time-out at UE.  In the same way for the uplink transmissions depending on the storage available at NTN node to receive and buffer the packets the access control and scheduling of transmissions needs to be handled. This can be achieved by explicit indication of support for MO or MT traffic along with SF operation parameter in system information.

Proposal 5:  RAN2 to consider inclusion of additional information for access control for MO or MT in SF operation

When the mobile originated IoT data transmission that requires application level ack are sent to eNB operating in store and forward mode the UE may expect the corresponding downlink packets after connecting to the next eNB. In this case UE monitors the next gNB for receiving ACK and new MT packets if any stored in the eNB. The data packet available at this eNB may contain ACK for earlier MO traffic and MT traffic toward the UE. In some other cases, the next eNB may not have any downlink ACK for earlier transmission depending on its CN connection time. In different scenarios based on the network awareness of CN connectivity time and the actual stored downlink data to be delivered towards UE, it is possible to improve the paging monitoring towards UE.

Proposal 6:  RAN2 to consider paging enhancements in SF mode for delivering ACK for MO Traffic and MT Traffic towards IoT-NTN UE.

In addition to paging enhancements, it is also beneficial for the UE battery life to consider that a satellite may not have more MT data for the UE. The typical case is that if the feeder link is not available, the satellite has no more MT data for the UE if the satellite buffer is drained since the satellite will not receive further MT data from CN. Until the feeder link recovery, the UE will not benefit from continuing paging monitoring on the current cell, nor will it benefit from reselecting to and monitoring paging from other cells of the same satellite. Thus, RAN2 can consider whether Idle mode procedures can be enhanced when there is no more MT data to be delivered in S&F operation.

Proposal 7 RAN2 to consider idle mode procedure enhancements, when there is no MT data to be delivered by a store & forward satellite.

# 3	Conclusion

In this paper we analyse the radio signalling procedure impacts for store and forward operation. Following proposals are

made based on the analysis.

SF mode transition related remaining issues

Proposal 1: The timer for SF mode transition is defined as absolute time is included in SIB1.

Proposal 2: The UE (capable of SF operation) behaviour on SF mode and the timer status is decided by higher layers for UE in RRC-IDLE mode. RAN2 assumes no impact to AS layer operations.  FFS connected mode.

Proposal 3: RAN2 to discuss the scenarios for benefits of time for mode transition from normal to SF operation for decision on the need for timer for real-time mode operation.

Proposal 4: Additional information related to maximum delay impact of SF operation is not broadcasted in system information. UE can deduce this information from other parameters related SF operation provided in NAS level procedures.

Access control and Paging Procedure for SF mode

Proposal 5:  RAN2 to consider inclusion of additional information for access control for MO or MT in SF operation.

Proposal 6:  RAN2 to consider paging enhancements in SF mode for delivering ACK for MO Traffic and MT Traffic towards IoT-NTN UE.

Proposal 7 RAN2 to consider idle mode procedure enhancements, when there is no MT data to be delivered by a store & forward satellite.

# Reference

[1]	RP-240776 WID for IoT-NTN Enhancements