3GPP TSG-RAN WG2 Meeting #129						R2-2500821

Athens, Greece, Feb. 17th – 21st, 2025

Source:	Continental Automotive

Title:                       Discussion on AS ID

Agenda Item:         8.2.4

Document for:	Discussion and Decision

# Introduction

In the RAN2#128 meeting, the following agreements were made for the AS ID [1].

Agreements on AS ID

Capture the option that it is up to the reader whether to use the random ID as AS ID or assigns a new AS ID.   FFS what message is used

Capture in the TR that to down-select we need to consider which message need to consider whether we need the reader to handle the collision.

In this document, we discuss whether to use the random ID as the AS ID or assign a new AS ID by the reader.

# Discussion

According to the RAN2#128 agreement, our understanding is that a reader uses a random ID as the AS ID if only one A-IoT device is assigned that random ID. However, if multiple devices end up with the same random ID, the reader assigns a new AS ID to each device to ensure proper identification and smooth communication. However, when the reader assigns the AS ID, it becomes difficult for the A-IoT devices to determine which AS ID corresponds to which device as shown in figure 1. As a result, there is a high possibility of collisions, even when the reader assigns the AS ID.

Figure 1: Same Random ID selected by more than one UE

To differentiate the AS ID among devices, the reader must include additional information in Msg2 along with the new AS ID, as shown in Figure 2. For example, the resource information (time/frequency) used in Msg1 is included by the reader in Msg2. Upon receiving this information, the UE can correctly identify the assigned AS ID from the reader. Without this additional information in Msg2, there is a risk that UEs with the same random ID generated in Msg1 may use the same AS ID and the same resources, which may potentially lead to collision. However, this approach would increase device complexity and signaling overhead.

Figure 2: Additional information in Msg2

.

Observation 1: Without additional information in Msg2, UEs with the same random ID from Msg1 may use the same AS ID and resources, leading to potential collisions.

Proposal 1: We propose de-prioritizing the option where the reader assigns the AS ID to the A-IoT device, due to the increased device complexity and additional signaling overhead.

The alternative option is that when multiple A-IoT devices end up with the same random ID, the reader does not provide a Msg2 response upon receiving the same random ID from more than one A-IoT device. As a result, the device will retransmit Msg1 with a new random ID after the Msg2 time window expires. This option is simpler and more straightforward.

Proposal 2: We propose that when the reader receives the same random ID from more than one A-IoT device, this issue can be resolved by the reader not providing a Msg2 response to those devices.

# Conclusions

In this contribution, we have shared our views on whether to use the random ID as the AS ID or assign a new AS ID by the reader. Additionally, we request RAN2 to discuss the following observation and proposals:

Observation 1: Without additional information in Msg2, UEs with the same random ID from Msg1 may use the same AS ID and resources, leading to potential collisions.

Proposal 1: We propose de-prioritizing the option where the reader assigns the AS ID to the A-IoT device, due to the increased device complexity and additional signalling overhead.

Proposal 2: We propose that when the reader receives the same random ID from more than one A-IoT device, this issue can be resolved by the reader not providing a Msg2 response to those devices.

# References

[1] RAN2#128 meeting chair note