# 3GPP TSG RAN WG2 #129									 R2-2500862

# Athens, Greece, Feb. 17th – 21st, 2025

# Source:	Hanbat National University

# Title:		Discussion on RRM measurement operation for XR enhancements

# Agenda item:	8.7.3

# Document for:	Discussion/Decision

# Introduction

The Rel-19 work item on XR Phase 3 contains the RRM measurement gaps/restrictions related enhancements in its scope [1]. as follows:

Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

In this regard, in order to skip a particular gap/restrictions, RAN1 made the following agreements on the explicit indication by DCI in the 3GPP RAN1#119 meeting [2]:

Agreement For explicit indication by DCI to skip a particular gap/restriction, in addition to DCI formats 0_1/1_1 and 0_2/1_2, one bit indication can be included as a part of DCI formats 0_3/1_3. - Explicit indication can be configured for each DCI format individually by higher layer. - The skipping indication corresponds to the first gap/restriction occasion among co-scheduled cells by the DCI (Type-1C field) Above applies only for intra-band CA cases (same SCS for cells in the set of co-scheduled cells).

RAN2 made the following agreements on RRM measurement gap skipping in 3GPP RAN2#128 meeting [3]:

Agreements on RRM measurement gaps impacts - No MG-specific enhancements is needed on DSR operation. - RAN2 assumes that UE follows DRX pattern as currently, even when MG is indicated as skipped - No MG-specific enhancements is needed for DRX operation.

Meanwhile, timing offset between DCI and MG is also discussed in [4], particularly focused on the following issues:

- Since a partial cancellation/skipping of a measurement gap is not supported, the to-be-skipped measurement gap occurrence should start after the pre-defined minimum time offset.

- The minimum time offset between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI may be a parameter configured via RRC.

In this contribution, we discuss the operation regarding timing offset between DCI and measurement gap and the handling the operation for RRM measurement gap/restrictions related enhancement.

# Discussions

RAN1 made some progress on determining how to skip a particular gap/restriction for RRM measurements. It was agreed to employ explicit indication by DCI to skip a particular gap/restriction. Explicit indication by DCI can provide some flexibility needed for the XR traffic support such as packet arrival time, packet size, scheduling, etc. uncertainty handling.

Also, when MG is indicated as skipped (or not skipped) by DCI, UE shall or shall not skip the measurement gap, configured by RRC, after the predefined time offset between the last symbol of the PDCCH carrying the DCI. In particular, the measurement gap shall start after the predefined time offset. Otherwise if the measurement gap is already started prior to the predefined time offset, in the perspective UE, how to handle related to MG operation shall be clarified. Potential solutions shall be discussed, and we propose how to handle this issue by selecting among the solutions for XR enhancements. In addition, we propose the following alternatives:

- Alt1: UE stops the any operation related MG between DCI and pre-defined minimum time offset

- Alt2: UE does not skip the measurement gap if the measurement gap is already started prior to the predefined time offset from the last symbol of the PDCCH carrying the DCI regardless of indicating to skip the measurement gap

Proposal 1: Discuss potential RAN2 impacts and find the solution if any impact occurs by the introduction of DCI-based solution to skip a particular gap/restriction for RRM measurements, with following alternatives:

- Alt1: UE stops the any operation related MG between DCI and pre-defined minimum time offset

- Alt2: UE does not skip the measurement gap if the measurement gap is already started prior to the predefined time offset from the last symbol of the PDCCH carrying the DCI regardless of indicating to skip the measurement gap

# Conclusion

In this contribution, we discussed and suggest MG operation related XR enhancements. As a general design by the introduction of DCI-based solution to skip a particular gap/restriction for RRM measurements, we propose:

Proposal 1: Discuss potential RAN2 impacts and find the solution if any impact occurs by the introduction of DCI-based solution to skip a particular gap/restriction for RRM measurements, with following alternatives:

- Alt1: UE stops the any operation related MG between DCI and pre-defined minimum time offset

- Alt2: UE does not skip the measurement gap if the measurement gap is already started prior to the predefined time offset from the last symbol of the PDCCH carrying the DCI regardless of indicating to skip the measurement gap

# References

- RP-240791, “Revised WID on XR (eXtended Reality) for NR Phase 3,” Mar. 2024.

- 3GPP RAN1 Chair’s notes RAN1#119, Nov. 2024.

- 3GPP RAN2 Chair’s notes RAN1#128, Nov. 2024.

- R2-2409844, Discussions on measurement gap related enhancements, Fujitsu, Nov. 2024.