3GPP TSG-RAN WG2 #129	R2-2500873

# Athens, Greece, Feb 17th– 21st, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 2038 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to MAC on IUC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2025-02-17 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Sidelink Inter-UE Coordination Information reporting procedure can be triggered by Sidelink Inter-UE Coordination Request MAC CE only, both Sidelink Inter-UE Coordination Request MAC CE and SCI, or condition. In clause 5.22.1.10.1, it is only the MAC procedure to handle (e.g., generation) a Sidelink Inter-UE Coordination Request MAC CE triggered by an explicit request from UEB, that has been captured. The MAC procedures to handle below cases are missing Case 1: a Sidelink Inter-UE Coordination Request MAC CE trigger and generation at UEB Case 2: a Sidelink Inter-UE Coordination Information MAC CE trigger and generation at UEA by a condition |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause 5.122.1.10.1, include the MAC procedures to handle (e.g., generation) the MAC CEs including: Case 1: a Sidelink Inter-UE Coordination Request MAC CE trigger and generation at UEB Case 2: a Sidelink Inter-UE Coordination Information MAC CE trigger and generation at UEA by a condition Impact Analysis Impacted 5G architecture options: NR SA Impacted functionality: Sidelink IUC Inter-operability: f one UE implements the change according to the CR, but not the other UE, there are no inter-operability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the changes are not approved, the MAC procedures on handling (e.g., generation) the below MAC CEs including: Case 1: a Sidelink Inter-UE Coordination Request MAC CE trigger and generation at UEB Case 2: a Sidelink Inter-UE Coordination Information MAC CE trigger and generation at UEA by a condition are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.22.1.9, 5.22.1.10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

<<<<Skipped>>>>

#### 5.22.1.9	IUC-Request transmission

The Sidelink Inter-UE Coordination Request transmission procedure is used to trigger a peer UE to transmit Sidelink Inter-UE Coordination Information.

RRC configures sl-TransmissionStructureForPSCCHandPSSCH parameter to control the inter-UE coordination request procedure. For a resource pool in a SL BWP where sl-TransmissionStructureForPSCCHandPSSCH is configured as interlaceRB, the Enhanced Inter-UE coordination Request MAC CE is used. Otherwise, the Sidelink Inter-UE coordination Request MAC CE is used.

The MAC entity shall for each pair of the Source Layer-2 ID and the Destination Layer-2 ID corresponding to a PC5-RRC connection which has been established by upper layers:

1>	if a Sidelink Inter-UE Coordination Request has been triggered and the MAC entity has SL resources allocated for new transmission and the SL-SCH resources can accommodate the Sidelink Inter-UE Coordination Request MAC CE or the Enhanced Inter-UE coordination Request MAC CE and its subheader as a result of logical channel prioritization:

2>	instruct the Multiplexing and Assembly procedure to generate a Sidelink Inter-UE Coordination Request MAC CE or an Enhanced Inter-UE coordination Request MAC CE as defined in clause 6.1.3.54;

2>	cancel the triggered Sidelink Inter-UE Coordination Request.

#### 5.22.1.10	IUC-Information Reporting

##### 5.22.1.10.1	General

The Sidelink Inter-UE Coordination Information reporting procedure is used to provide a peer UE with inter-UE coordination information as specified in clause 8.1.4 of TS 38.214 [7]. The Sidelink Inter-UE Coordination Information reporting procedure can be triggered by Sidelink Inter-UE Coordination Request MAC CE only, both Sidelink Inter-UE Coordination Request MAC CE and SCI (See TS 38.214 [7]), or condition (See TS 38.331 [5] and TS 38.214 [7]). Unicast for preferred resource set and non-preferred resource set and Groupcast/Broadcast for non-preferred resource set are supported for inter-UE coordination information transmission triggered by a condition other than explicit request reception. Unicast for preferred resource set and non-preferred resource set is supported for inter-UE coordination information transmission triggered by an explicit request.

RRC also configures sl-TransmissionStructureForPSCCHandPSSCH parameter to control the inter-UE coordination Information reporting procedure. For a resource pool in a SL BWP where sl-TransmissionStructureForPSCCHandPSSCH is configured as interlaceRB, the Enhanced Inter-UE coordination Information MAC CE is used. Otherwise, the Sidelink Inter-UE coordination Information MAC CE is used.

RRC configures the following parameter to control the SL-IUC Information reporting procedure:

-	sl-LatencyBoundIUC-Report, which is maintained for each PC5-RRC connection.

The MAC entity maintains an sl-IUC-ReportTimer for each pair of the Source Layer-2 ID and the Destination Layer-2 ID corresponding to a PC5-RRC connection. sl-IUC-ReportTimer is used for an SL-IUC Information reporting UE to follow the latency requirement signalled from an IUC-Information triggering UE. The value of sl-IUC-ReportTimer is the same as the‎ latency requirement of the SL-IUC Information in sl-LatencyBoundIUC-Report configured by RRC.

The MAC entity shall for each pair of the Source Layer-2 ID and the Destination Layer-2 ID corresponding to a PC5-RRC connection which has been established by upper layers:

1>	if the SL-IUC Information reporting has been triggered by an Sidelink Inter-UE coordination Request MAC CE alone, or by both a Sidelink Inter-UE Coordination Request MAC CE and an SCI, and not cancelled:

2>	if the sl-IUC-ReportTimer for the triggered SL-IUC Information reporting is not running:

3>	start the sl-IUC-ReportTimer.

2>	if the sl-IUC-ReportTimer for the triggered SL-IUC Information reporting expires:

3>	cancel the triggered SL-IUC Information reporting.

2>	else if the MAC entity has SL resources allocated for new transmission and the SL-SCH resources can accommodate the Sidelink Inter-UE Coordination Information MAC CE and its subheader as a result of logical channel prioritization:

3>	instruct the Multiplexing and Assembly procedure to generate a Sidelink Inter-UE Coordination Information MAC CE as defined in clause 6.1.3.53;

3>	stop the sl-IUC-ReportTimer for the triggered SL-IUC Information reporting;

3>	cancel the triggered SL-IUC Information reporting.

The MAC entity may for each Destination Layer-2 ID:

1>	if a SL-IUC Information reporting has been triggered by a condition and the MAC entity has SL resources allocated for new transmission and the SL-SCH resources can accommodate the Sidelink Inter-UE Coordination Information MAC CE or the Enhanced Inter-UE coordination Information MAC CE and its subheader as a result of logical channel prioritization:

2>	instruct the Multiplexing and Assembly procedure to generate a Sidelink Inter-UE Coordination Information MAC CE or an Enhanced Inter-UE coordination Information MAC CE as defined in clause 6.1.3.53;

2>	cancel this SL-IUC Information reporting.

<<<<Skipped>>>>

End of changes