3GPP TSG-RAN WG2 #129	R2-2500875

# Athens, Greece, Feb 17th– 21st, 2025

Agenda Item:	7.6

Source:	Ericsson

Title:	Discussion on resource re-selection from SL LBT Failure indication

Document for:	Discussion, Decision

# 1	Introduction

we discuss the issues on resource re-selection from SL LBT Failure indication in this paper.

# 2	Discussion

In clause 5.22.1.2c of TS 38.321, how a UE performs resource re-selection if the UE has been configured with resource allocation mode 2 in an unlicensed carrier, is described as the below

If the MAC entity has been configured with Sidelink resource allocation mode 2 to transmit using pool(s) of resources in a carrier as indicated in TS 38.331 [5] based on sensing or random selection the MAC entity shall for each Sidelink process:

1>	if SL LBT failure indication is received from lower layers:

2>	for the resource(s) where SL LBT failure is detected from the selected sidelink grant associated to the Sidelink process:

3>	if transmission based on random selection is configured by upper layers:

4>	randomly select the time and frequency resources for one transmission opportunity from the resource pool excluding all RB sets for which Sidelink consistent LBT failures were detected and not cancelled, if configured according to the amount of selected frequency resources, the selected number of HARQ retransmissions and the remaining PDB of SL data available in the logical channel(s) by ensuring the minimum time gap between any two selected resources of the selected sidelink grant in case that PSFCH is configured for this pool of resources.

3>	else:

4>	randomly select the time and frequency resources for one transmission opportunity from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7], according to the amount of selected frequency resources, the selected number of HARQ retransmissions and the remaining PDB of SL data available in the logical channel(s) by ensuring the minimum time gap between any two selected resources of the selected sidelink grant in case that PSFCH is configured for this pool of resources.

The MAC entity is triggered to perform reselection for resources when SL LBT failure is detected.

As captured in the spec, the MAC entity uses the selected number of HARQ retransmissions as an input in the resource reselection procedure.

However, the selected number of HARQ retransmissions is only needed for resource (re)selection for retransmission opportunities. Therefore, the above texts are not correct for resource reselection for initial transmission opportunities in case of SL LBT failure.

In the existing MAC spec version, the selected number of HARQ retransmissions is only needed for resource (re)selection for retransmission opportunities.

All in all, we think the issue needs to be addressed.

Correct the resource re-selection from SL LBT Failure indication.

In order to address the issue, two options are feasible.

Option 1: include “if applicable” in the existing texts so that the MAC procedure doesn’t need to be specified separately for initial transmissions and retransmissions.

Option 2: update the existing texts so that the MAC procedure is separately for initial transmissions and retransmissions.

For both options, we have prepared corresponding text proposals in section 4.

Adopt one of the options and the corresponding TP captured in Appendix.

# 3 Conclusion

In the previous sections we made the following observations:

Observation 1	The MAC entity is triggered to perform reselection for resources when SL LBT failure is detected.

Observation 2	As captured in the spec, the MAC entity uses the selected number of HARQ retransmissions as an input in the resource reselection procedure.

Observation 3	In the existing MAC spec version, the selected number of HARQ retransmissions is only needed for resource (re)selection for retransmission opportunities.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Correct the resource re-selection from SL LBT Failure indication.

Proposal 2	Adopt one of the options and the corresponding TP captured in Appendix.

# 4 Appendix - Text proposal

## 4.1 Option 1 - Text proposal

#### 5.22.1.2c	Resource re-selection from SL LBT Failure indication

If the MAC entity has been configured with Sidelink resource allocation mode 2 to transmit using pool(s) of resources in a carrier as indicated in TS 38.331 [5] based on sensing or random selection the MAC entity shall for each Sidelink process:

1>	if SL LBT failure indication is received from lower layers:

2>	for the resource(s) where SL LBT failure is detected from the selected sidelink grant associated to the Sidelink process:

3>	if transmission based on random selection is configured by upper layers:

4>	randomly select the time and frequency resources for one transmission opportunity from the resource pool excluding all RB sets for which Sidelink consistent LBT failures were detected and not cancelled, if configured according to the amount of selected frequency resources, if applicable, the selected number of HARQ retransmissions and the remaining PDB of SL data available in the logical channel(s) by ensuring the minimum time gap between any two selected resources of the selected sidelink grant in case that PSFCH is configured for this pool of resources.

3>	else:

4>	randomly select the time and frequency resources for one transmission opportunity from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7], according to the amount of selected frequency resources, if applicable, the selected number of HARQ retransmissions and the remaining PDB of SL data available in the logical channel(s) by ensuring the minimum time gap between any two selected resources of the selected sidelink grant in case that PSFCH is configured for this pool of resources.

## 4.2 Option 2 - Text proposal

#### 5.22.1.2c	Resource re-selection from SL LBT Failure indication

If the MAC entity has been configured with Sidelink resource allocation mode 2 to transmit using pool(s) of resources in a carrier as indicated in TS 38.331 [5] based on sensing or random selection the MAC entity shall for each Sidelink process:

1>	if SL LBT failure indication is received from lower layers:

2>	for the resource(s) where SL LBT failure is detected from the selected sidelink grant associated to the Sidelink process:

3>	if transmission based on random selection is configured by upper layers:

4>	if the resource(s) were selected for initial transmission opportunities:

5>	randomly select the time and frequency resources for one transmission opportunity from the resource pool excluding all RB sets for which Sidelink consistent LBT failures were detected and not cancelled, if configured according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) by ensuring the minimum time gap between any two selected resources of the selected sidelink grant in case that PSFCH is configured for this pool of resources.

4>	else:

5>	randomly select the time and frequency resources for one transmission opportunity from the resource pool excluding all RB sets for which Sidelink consistent LBT failures were detected and not cancelled, if configured according to the amount of selected frequency resources, the selected number of HARQ retransmissions and the remaining PDB of SL data available in the logical channel(s) by ensuring the minimum time gap between any two selected resources of the selected sidelink grant in case that PSFCH is configured for this pool of resources.

3>	else:

4>	if the resource(s) were selected for initial transmission opportunities:

5>	randomly select the time and frequency resources for one transmission opportunity from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7], according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) by ensuring the minimum time gap between any two selected resources of the selected sidelink grant in case that PSFCH is configured for this pool of resources.

4>	else:

5>	randomly select the time and frequency resources for one transmission opportunity from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7], according to the amount of selected frequency resources, the selected number of HARQ retransmissions and the remaining PDB of SL data available in the logical channel(s) by ensuring the minimum time gap between any two selected resources of the selected sidelink grant in case that PSFCH is configured for this pool of resources.