3GPP RAN WG2 Meeting #129 	R2-2500876

# Athens, Greece, February 17th – 21st, 2025

Agenda Item:	8.9.2

Source:	InterDigital, Inc.

Title:	Support of Store and Forward.

Document for:	Discussion

# Introduction

A revised WID was approved to further enhance the support of NB-IoT and eMTC [1]

The following objective is provided

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

In RAN2#127 the following was agreed:

Agreements:

1.	RAN2 adopts the SA2 study conclusions on the possible S&F architectures as the baseline for further discussion (RAN2 will only consider the full CN and spit-MME payload options)

2.	RAN2 will consider both single satellite pass and multiple satellite pass scenarios

3.	RAN2 will consider both MO and MT data within scope

4.	UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature)

5.	RAN2 does not further discuss whether legacy UEs will always need to be barred in a S&F network, at least not before further progress in SA2 discussion. If there will be a need for this, mechanisms to bar legacy UEs are already in place and no further impact on RAN2 specs in expected.

7.	RAN2 agrees there will be a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow R19 S&F capable UEs. FFS on the exact solution (e.g. new barring bits or whether this is linked to some other indication)

In RAN2#127bis the following was agreed:

Agreements:

1.	The dynamic indication that “the cell is operating in S&F mode” is called “S&F operation” indication (we can come back on the exact name when putting this in the spec, if needed)

2.	RAN2 assumes that if an indication that “the cell is operating in S&F mode” is not provided in a NTN cell, the UE should assume that the NTN cell is operating in real-time mode (i.e. default/normal/”not-S&F” mode). RAN2 assumes there should be no distinction between the case that the UE is served by a NTN cell that do not support S&F capability and the case that the UE is served by a NTN cell that does support S&F capability but indication that “the cell is operating in S&F mode” is not provided.

3.	An S&F explicit capability indication by the serving cell, conceived as a static indication of whether the S&F capability is supported or not by a specific satellite/NTN-cell, in addition to the indication that “the cell is operating in S&F mode”, is not needed

4.	If the “S&F operation” indication is not broadcast, a Rel-19 UE (regardless whether supporting S&F or not) shall follow the legacy barring procedure

5.	When present, the “S&F operation” indication has two possible settings:

‘1’: the cell is operating in S&F mode for all UEs (Rel-19 UEs supporting S&F are allowed to access the cell)

‘0’: the cell is operating in S&F mode for UEs in Connected mode (which are not required to monitor the “S&F indication”), but idle Rel-19 UEs supporting S&F are barred (Rel-19 UEs not supporting S&F will follow legacy barring procedure).

In RAN2#128 the following was agreed.

Agreements:

1.	Time information can be broadcasted to indicate when the current satellite operation mode will transit from “S&F operation” mode to real-time/normal mode. RAN2 assumes that a R19 UE could use this information at least to delay some NAS procedures until the feeder link is resumed (FFS what we specify in RAN2 specs for this and if we differentiate the behaviour for R19 UEs supporting and not supporting S&F mode, also considering the relation to the Wait Timer). FFS which SIB is used (SIB1 or SIB31). FFS if absolute or relative time is used. FFS on the opposite transition (i.e. whether to indicate when the “S&F operation” will start)

2.	It is FFS whether we need to introduce a new release cause when the NW wants to release a R19 UE to RRC idle because the feeder link becomes unavailable

3.	“S&F operation” indication is provided in SIB1

4.	RAN2 assumes that AS signalling is not needed to indicate the architecture option (Split MME or Full CN).

5.	RAN2 waits for further progress in SA2, if any, before working on RAN2 aspects of a CIoT-UP solution for SF operation

6.	RAN2 understands that the UE configured with a satellite ID list by MME is not prevented to camp on a satellite operating in normal IoT NTN mode (i.e. with feeder-link connection), and perform subsequent access and data/signalling communication with that satellite

7.	RAN2 understands the UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list.

# Discussion

## 2.1 S&F Operation mode

According to the agreements from RAN2#127bis meeting:

5.	When present, the “S&F operation” indication has two possible settings:

‘1’: the cell is operating in S&F mode for all UEs (Rel-19 UEs supporting S&F are allowed to access the cell)

‘0’: the cell is operating in S&F mode for UEs in Connected mode (which are not required to monitor the “S&F indication”), but idle Rel-19 UEs supporting S&F are barred (Rel-19 UEs not supporting S&F will follow legacy barring procedure).

The behaviour described in the above agreement is identical to the behaviour used for feature specific barred/no barred indication. Idle mode UEs, when determining cell status and cell reservations according to 38.304 [3] section 5.3.1 may access the cell if the value is “not barred” and treat the cell as barred if the value is “barred”. UEs in RRC_CONNECTED don’t use the IE.

Observation: The agreed behaviour for “S&F operation” is identical to “cellBarred” and feature specific “cellBArred” IEs.

For consistency with all of the other IEs of this type, we propose that the IE is named in line with the other IEs of this type, and the behaviour in 38.304 is aligned with the other IEs of this type and captured in section 5.3.1 along with the others.

Proposal 1: To align with existing IEs of the same type and behaviour, “S&F operation” IE is named “cellBarredSandF” and has the value “barred” or “not barred”.

Typically, when a cell is barred (either using the legacy cellBarred IE or using a feature specific IE) then the UE is required to exclude the cell for cell selection/reselection for 300s. Note that there are some conditions whereby the UE “may” exclude the cell for up to 300s for power saving purpose, but these conditions are typically MIB or SIB1 reception failure, i.e. the error case. Conditions whereby the cell is explicitly barred result in the UE “shall” exclude the cell from cell selection/reselection for 300s (see 38.304 [3] section 5.3.1)

In most cases, cell barring occurs due to abnormal situations, and the status is not updated very frequently. Hence, UE does not need to re-check the status for 5 minutes to allow for power saving.

However, in the case of S&F operation, it is likely that the mode itself is likely to be updated more frequently than other cellBarred IEs. Applying 5 minutes barring may limit the amount of time a cell is usable, if the UE determined based on the status being barred and S&F operation being on. For example, the UE may determine that the cell is barred, but the gNB then changes S&F operation to normal operation sooner than 5 minutes. Similarly, if the gNB does not change the status for longer than 5 minutes, then the UE may need to perform SIB reception and determine the status again unnecessarily.

One additional reason for S&F mode to have an indicated timing other than fixed 5 minutes is related to the satellite movement. Due to the satellite movement, nature of the orbit and the number of satellites in the constellation, a satellite may likely go out of coverage for a UE in about 5 minutes. Thus, based upon the deployment, e.g., orbit and constellation, the operators may choose a suitable time after which the UE may need to check the S&F status/indication.

In the previous meeting, it was agreed to introduce a t-Service like parameter to indicate the remaining time of the current mode.

1.	Time information can be broadcasted to indicate when the current satellite operation mode will transit from “S&F operation” mode to real-time/normal mode. RAN2 assumes that a R19 UE could use this information at least to delay some NAS procedures until the feeder link is resumed (FFS what we specify in RAN2 specs for this and if we differentiate the behaviour for R19 UEs supporting and not supporting S&F mode, also considering the relation to the Wait Timer). FFS which SIB is used (SIB1 or SIB31). FFS if absolute or relative time is used. FFS on the opposite transition (i.e. whether to indicate when the “S&F operation” will start)

For simplicity and consistency, we think it is sufficient to re-use the t-Service format and values. In other words, use TimeOffsetUTC-r17 as the type for a new IE describing the remaining time of the current mode.

Proposal 2: Use TimeOffsetUTC-r17 as the format of the new time information describing the remaining time before the mode is switched between S+F/normal.

If the use of the parameter is simply to enable the UE to determine if and when to start a service, then only serving cell information is needed, and only an informative note would be needed to say what the UE may do with the information.

Proposal 3: Use an informative note to say that the UE may defer starting a connection if the remaining time in the current mode is not enough to complete the service.

If the parameter is also used to perform some cell reselection prioritisation then the information would obviously also be beneficial for neighbouring cells and normative requirements are needed. First the use-case of cell reselection prioritization needs to be discussed.

Proposal 4a: RAN2 to decide whether the new time remaining IE can be used to determine cell reselection prioritization.

Proposal 4b: If remaining time of the current mode of S&F is used for cell reselection prioritization, then this information is also broadcast for neighbour cells in addition to the current cell.

As discussed above, usually the UE excludes a barred cell for 5 minutes if it is barred. The fixed 5 minute timer may not be appropriate if the remaining time of S+F operation is used, but rather the UE should bar according to the remaining time parameter so that it may check the status when the mode changes (In this case we may also need to discuss whether this applies to both cases - by checking the legacy cellBarred when the mode changes from S&F to normal, and by checking the cellBarredSandF when the mode changes from normal to S&F).

Proposal 5: If the UE is barred due “cellBarredSandF” (or whatever the name is) = “barred” the UE uses the value of the remaining time parameter to determine how long to bar the cell. FFS the case of changing from normal to S&F mode.

# Conclusion

In this contribution we discuss support of PWS and make the following proposals for discussion:

Observation: The agreed behaviour for “S&F operation” is identical to “cellBarred” and feature specific “cellBArred” IEs.

Proposal 1: To align with existing IEs of the same type and behaviour, “S&F operation” IE is named “cellBarredSandF” and has the value “barred” or “not barred”.

Proposal 2: Use TimeOffsetUTC-r17 as the format of the new time information describing the remaining time before the mode is switched between S+F/normal.

Proposal 3: Use an informative note to say that the UE may defer starting a connection if the remaining time in the current mode is not enough to complete the service.

Proposal 4a: RAN2 to decide whether the new time remaining IE can be used to determine cell reselection prioritization.

Proposal 4b: If remaining time of the current mode of S&F is used for cell reselection prioritization, then this information is also broadcast for neighbour cells in addition to the current cell.

Proposal 5: If the UE is barred due “cellBarredSandF” (or whatever the name is) = “barred” the UE uses the value of the remaining time parameter to determine how long to bar the cell. FFS the case of changing from normal to S&F mode.

# References

- RP-242397 Revised WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”

- 36300-i00 , “E-UTRA/E-UTRAN;Overall description;Stage 2”

- 36304-i30, “User Equipment (UE) procedures in idle mode”