3GPP RAN WG2 Meeting #129 	R2-2500877

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	8.9.3

Source:	InterDigital, Inc.

Title:	EDT/PUR enhancements.

Document for:	Discussion

# Introduction

A revised WID was approved to further enhance the support of NB-IoT and eMTC [1]

The following objective is provided

- Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2] - Multi-tone support for 15 kHz SCS should also be considered Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an EDT transaction [RAN2]: - Msg3 transmission without msg1/RAR - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

In RAN2#126, the following was agreed:

Agreements: - Both NB-IoT and eMTC are within scope of uplink capacity enhancements - Both C-plane and U-plane solutions are within scope of uplink capacity enhancements. - Only CIoT EPS is within scope of uplink capacity enhancements

In RAN2#127, the following was agreed:

Agreements: - RAN2 will continue working on a CB-msg3 EDT-like mechanism - RAN2 assumes that a DSA based scheme would not have RAN1 impacts, while RAN2 thinks that a CRDSA based scheme would necessarily have RAN1 impacts - In the next meeting continue the comparison (e.g. in terms of packet loss ratio, usage of UL/DL radio resources) between existing CB mechanism (Slotted Aloha) and other mechanisms (DSA, CRDSA, others) and try to decide on which way to go and in case whether/what to ask to RAN1. - For DSA and CRDSA, RAN2 can consider in the evaluation how to integrate them with repetition.

In RAN2#127bis, the following was agreed:

Agreements:

1.	RAN2 will introduce support for DSA (i.e. the possibility to transmit more than one replica of CB-msg3, if configured by the NW). RAN2 does not intend to work on CRDSA in Rel-19.

2.	RAN2 will continue their work on CB-msg3 assuming that OCC2 might also apply to CB-msg3 transmission (“CB-NPUSCH”) (final decision whether this is feasible is up to RAN1). FFS whether an LS to indicate this to RAN1 is needed.

3.	At least system Information is used to provide CB-msg3 EDT cell-specific PUSCH resources for Msg3 transmission. FFS on signalling details.

4.	CB-msg3 EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level (FFS whether we have a CE level specific configuration for DSA)

5.	RAN2 assumes that CB-msg3 EDT cell specific PUSCH resources are associated with number of repetitions, RSRP selection threshold to determine the CE level and largest TBS for Msg3 transmission, but this has to be confirmed by RAN1. FFS if there is an RSRP threshold that determines whether CB-msg3 EDT cannot be used (the UE will have to use 4-step RA)

6.	At least in the cases confirmed by RAN1/RAN4, a running TAT is not needed to initiate a CB-msg3 EDT transmission

7.	The RNTI used at least to schedule Msg4 transmission is derived based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission (FFS on the details. FFS how this is impacted by DSA)

8.	CB-msg3 EDT procedures and any Msg4 enhancement are only introduced for IoT NTN.

. In RAN2#128 the following was agreed.

Agreements:

1.	Only system Information is used to provide cell-specific CB-Msg3 PUSCH resources (FFS if anything is needed in dedicated signalling for the TA validation parameters, if needed, for the case of 15kHz SCS NB-IoT and eMTC CE mode B)

2.	Reuse the existing CE level selection procedure for CB-Msg3, at least for the initial selection. FFS whether we can reuse the same thresholds. FFS on the number of levels

3.	The UE triggers CB-Msg3 only if the size of pending UL data is less than the configured maximum TBS (FFS if the maximum TBS is same or different for different CE levels)

4.	There will be a RSRP threshold that determines whether CB-Msg3 can be used (if the RSRP is below such threshold, PRACH will have to be used) (FFS if we need a separate set of thresholds, including a different minimum threshold, in case CB-msg3 EDT is combined with OCC)

5.	The number of replicas for DSA will be configured by the NW: 1 (SA), 2, 3, 4. The configuration of the number of replicas is CE-level specific

6.	There will be a configurable time window for DSA CB-Msg3 occasion selection. FFS on the details (e.g. when the time window starts)

7.	For SA case (single replica), after the end of all repetition of CB-Msg3 PUSCH transmission, UE starts a window for response reception taking UE-eNB RTT into account. FFS if we need to consider additional delay e.g. for the processing time

8.	For DSA case, FFS if we only have one or multiple PDCCH monitoring window(s) (i.e. one window per each replica) for response reception. FFS when the window(s) is/are started (or restarted) and stopped. FFS on the window length. FFS if the UE needs to monitor only one RNTI or multiple RNTIs)

9.	The UE stops the PDCCH monitoring window(s) once it receives a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4)

10.	Assuming that there will be scenarios where it’s possible to receive a CB-msg4 before the UE transmits some replicas, a UE stops transmitting the remaining replicas if it has received a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4)

11.	Within the configured time window, the UE shall select randomly different time domain occasions for transmitting different replicas. And for each time domain occasion, the UE shall select randomly a frequency domain resource.

12.	RAN2 understands that, for DSA, once the eNB successfully decodes one of the multiple replicas, it may respond without waiting for the remaining replica(s) (FFS when the response window(s) is/are started)

# Overview of existing functionality

## 2.1	MO-EDT

It was agreed that both CP and UP solutions, NB-IoT and eMTC, are in the scope of the study, but only EPS (and not 5GC).

The following show MO-EDT procedures for the CP and UP EPS optimizations.

The MO-EDT procedure for Control Plane CIoT EPS optimisation is illustrated in Figure 7.3b- of 36.300 [2]

Figure 7.3b-1: MO-EDT for Control Plane CIoT EPS Optimisation

The MO-EDT procedure for User Plane CIoT EPS optimisation is illustrated in Figure 7.3b-2 of 36.300 [2]

Figure 7.3b-2: MO-EDT for User Plane CIoT EPS Optimisation

## 2.2	Transmission using PUR

The procedure for transmission using PUR for the Control Plane CIoT EPS optimisation and for the Control Plane CIoT 5GS optimisation is illustrated in Figure 7.3d-2 of 36.300 [2].

Figure 7.3d-2: Transmission using PUR for the Control Plane CIoT EPS/5GS Optimisations

The procedure for PUR configuration request and PUR configuration is common to the Control Plane CIoT EPS/5GS optimisations and the User Plane CIoT EPS/5GS optimisations and is illustrated in Figure 7.3d-1 of 36.300 [2].

Figure 7.3d-1: PUR Configuration Request and PUR Configuration

# Msg3 transmission without msg1/RAR

In the previous meeting the following key agreements were made for contention based EDT transmission:

2. Reuse the existing CE level selection procedure for CB-Msg3, at least for the initial selection. FFS whether we can reuse the same thresholds. FFS on the number of levels

- There will be a RSRP threshold that determines whether CB-Msg3 can be used (if the RSRP is below such threshold, PRACH will have to be used) (FFS if we need a separate set of thresholds, including a different minimum threshold, in case CB-msg3 EDT is combined with OCC)

In terms of the number of coverage levels, we do not see any strong reason to have a different number than that supported for PRACH. In other words, there should be 3 coverage levels determined from 2 thresholds. If RSRP is > threshold A then coverage level 0 is used, else if RSRP is > threshold B then coverage level 1 is used, otherwise coverage level 2 is used.

Proposal 1: For CB-Msg3 there can be up to 3 coverage levels using 2 thresholds, same as for PRACH. If RSRP is > threshold A then coverage level 0 is used, else if RSRP is > threshold B then coverage level 1 is used, otherwise coverage level 2 is used.

Then the question is how to support the threshold for determining whether CB-Msg3 can be used at all, or whether PRACH is used. There seem to be 4 possible options:

Option 1: Re-use the same thresholds as PRACH, coverage level 2 is always based on PRACH (hence there are 2 usable coverage levels for CB-Msg3 and “fallback” to PRACH comes for free.

Option 2: Re-use the same thresholds as PRACH, introduce a new threshold to indicate whether CB-Msg3 can be used. If RSRP < new threshold, then use legacy PRACH coverage level selection.

Option 3: Introduce new thresholds for CB-Msg3 coverage levels, with coverage level 2 always based on PRACH (similar to option 1, but with new thresholds).

Option 4: Introduce 3 new thresholds. The first 2 are for selecting coverage level 0, 1, 2 for CB-Msg3, and the 3rd one indicated when UE has to use PRACH. If UE determines PRACH has to be used, then legacy PRACH coverage level selection thresholds shall be used to determine the repetition number etc. (It is very similar to option 3, but provides an additional usable coverage level for CB-Msg3).

In our opinion, the options are in order of simplicity (i.e. option 1 is the simplest, option 4 is the most complex). However, the options are also in the order of flexibility of configuration, option 4 being the most flexible.

Proposal 2: Selection from the following options on CB-Msg3 coverage level selection:

- Option 1: Re-use the same thresholds as PRACH, coverage level 2 is always based on PRACH.

- Option 2: Re-use the same thresholds as PRACH, introduce a new threshold to indicate whether CB-Msg3 can be used. If RSRP < new threshold, then use legacy PRACH coverage level selection.

- Option 3: Introduce new thresholds for CB-Msg3 coverage levels, with coverage level 2 always based on PRACH.

- Option 4: Introduce 3 new thresholds. The first 2 are for selecting coverage level 0, 1, 2 for CB-Msg3, and the 3rd one indicated when UE has to use PRACH. If UE determines PRACH has to be used, then legacy PRACH coverage level selection thresholds shall be used to determine the PRACH coverage level.

In addition we need to decide what happens in case of failure. For example if the UE selects coverage level 0 but the CB-Msg3 transmission is not successful, then what happens next. In our opinion there is already a well-defined fallback mechanism for PRACH and this should be re-used. In other words, if transmission fails using the selected coverage level then the UE selects the next coverage level and re-tries. Depending on the option selected in proposal 2, then either the “last” coverage level will be PRACH so this comes for free, or all of the CB-Msg3 coverage levels are tried, then UE uses PRACH in the end. For simplicity, option 1 or option 3 are best because we do not need to define anything special for PRACH fallback case, it will happen automatically based on the coverage level selection.

Proposal 3: Re-use the existing per-CE level PRACH failure handling for CB-Msg3 failures – if transmission fails using the selected CE level configuration, then select the next CE level configuration and retry.

- No need to introduce separate failure handling if option 1 or option 3(above) are selected, this comes for free.

- Some additional condition for fallback to PRACH is needed if option 2 or option 4 are selected.

# Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

As can be seen, only the MO-EDT procedure for CP solution uses “RRCEarlyDataComplete”. User data is attached to Msg3 and Msg4 during the RACH procedure. The procedure is terminated by Msg4.

The MO-EDT procedure for UP solution transfers data in RRC_CONNECTED, after completing random access (5 steps) and the procedure is terminated using RRCConnectionRelease.

The WI objective clearly states that the intention is to reduce overhead of Msg4/RRCEarlyDataComplete, so it is clear that this part of the objective is only applicable to the CP solution.

Proposal 4: Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete only applies to the C-plane solution.

As can be seen, if PUR is used, it is already possible to terminate EDT without using RRCEarlyDataComplete. As described in 36.300 [2] :

7a	If the (ng-)eNB is aware that there is no pending downlink data or signalling, the (ng-)eNB can send a Layer 1 ACK optionally containing a Time Advance Adjustment to the UE to update the TA and terminate the procedure.

7b	If the (ng-)eNB is aware that there is no further data or signalling, the (ng-)eNB can send a Time Advance Command to update the TA and terminate the procedure.

Observation 1: The existing PUR feature can terminate the EDT procedure without using RRCEarlyDataComplete, by using Layer 1 ACK or Timing advance MAC CE, if eNB is aware that there is no pending downlink data or signalling.

The main question as to whether early termination of EDT works in practise is “how does eNB know that there is no pending downlink data from the application layer?“

Proposal 5: RAN2 to discuss how eNB knows that there is no pending downlink data from the application layer.

The UE can transmit an indication (L1Ack) in PURConfigurationRequest, which informs the eNB a preference for using RRCEarlyDataComplete. Note that, although the figure states “L1 ACK” this is named “rrc-ACK” in the RRC specification and it indicates RRC response message is preferred by the UE for acknowledging the reception of a transmission using PUR. This is an indirect way to specify that the UE can send some information about whether it expects downlink data in response to the uplink. It is then up to the eNB implementation what (if anything) to do with that knowledge. The main issue with terminating EDT early is when there is some downlink application layer data to send in response. If the procedure is terminated early then the network must page the UE in order to transmit this, which is even more inefficient than waiting and terminating using RRC.

Observation 2: With the existing PUR feature it is possible for the UE to indicate in PURConfigurationRequest whether it expects a downlink response by RRCEarlyDataComplete.

Since for CB-Msg3 we will not be using PURConfigurationRequest, there needs to be another way to indicate from the UE whether RRC response message is preferred by the UE for acknowledging the reception of a transmission. The most convenient place would be in the RRCEarlyDataRequest.

Proposal 6: Introduce “rrc-ACK”, to RRCEarlyDataRequest to indicate whether an RRC Response Message is preferred by the UE for CB-Msg3, similar to PURConfigurationRequest

It was agreed in the last meeting:

Assuming that there will be scenarios where it’s possible to receive a CB-msg4 before the UE transmits some replicas, a UE stops transmitting the remaining replicas if it has received a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4)

In previous contributions, we listed various options for early termination of EDT. With the above agreement, it seems that the most suitable approach for terminating early is use of contention resolution MAC CE.

However, UE needs to know whether to terminate only the uplink transmissions and wait for an RRC response, or to terminate the procedure entirely and return to RRC_IDLE. For this purpose, an additional flag would be needed in the contention resolution MAC CE.

Proposal 7: Contention Resolution MAC CE includes a new flag indicating whether to terminate only the uplink transmissions, or to terminate the entire procedure and return to RRC_IDLE.

# Conclusion

In this contribution we discuss uplink capacity enhancements and make the following proposals for discussion:

Msg3 transmission without msg1/RAR

Proposal 1: For CB-Msg3 there can be up to 3 coverage levels using 2 thresholds, same as for PRACH. If RSRP is > threshold A then coverage level 0 is used, else if RSRP is > threshold B then coverage level 1 is used, otherwise coverage level 2 is used.

Proposal 2: Selection from the following options on CB-Msg3 coverage level selection:

- Option 1: Re-use the same thresholds as PRACH, coverage level 2 is always based on PRACH.

- Option 2: Re-use the same thresholds as PRACH, introduce a new threshold to indicate whether CB-Msg3 can be used. If RSRP < new threshold, then use legacy PRACH coverage level selection.

- Option 3: Introduce new thresholds for CB-Msg3 coverage levels, with coverage level 2 always based on PRACH.

- Option 4: Introduce 3 new thresholds. The first 2 are for selecting coverage level 0, 1, 2 for CB-Msg3, and the 3rd one indicated when UE has to use PRACH. If UE determines PRACH has to be used, then legacy PRACH coverage level selection thresholds shall be used to determine the PRACH coverage level.

Proposal 3: Re-use the existing per-CE level PRACH failure handling for CB-Msg3 failures – if transmission fails using the selected CE level configuration, then select the next CE level configuration and retry.

- No need to introduce separate failure handling if option 1 or option 3(above) are selected, this comes for free.

- Some additional condition for fallback to PRACH is needed if option 2 or option 4 are selected.

Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

Proposal 4: Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete only applies to the C-plane solution.

Observation 1: The existing PUR feature can terminate the EDT procedure without using RRCEarlyDataComplete, by using Layer 1 ACK or Timing advance MAC CE, if eNB is aware that there is no pending downlink data or signalling.

Proposal 5: RAN2 to discuss how eNB knows that there is no pending downlink data from the application layer.

Observation 2: With the existing PUR feature it is possible for the UE to indicate in PURConfigurationRequest whether it expects a downlink response by RRCEarlyDataComplete.

Proposal 6: Introduce “rrc-ACK”, to RRCEarlyDataRequest to indicate whether an RRC Response Message is preferred by the UE for CB-Msg3, similar to PURConfigurationRequest

Proposal 7: Contention Resolution MAC CE includes a new flag indicating whether to terminate only the uplink transmissions, or to terminate the entire procedure and return to RRC_IDLE.

# References

- RP-242397 Revised WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”

- 36300-i00 , “E-UTRA/E-UTRAN;Overall description;Stage 2”