3GPP TSG RAN WG2 #129	R2-2500904

Athens, Greece, 17-21 February 2025

Agenda Item:	8.8.5
Source:	Toyota ITC
Title:	Regenerative payload for NTN for NR Ph3

Document for:	Discussion and Decision

# 1	Introduction

The Work Item on NR for NTN Ph3 was initially approved in December 2023 and then revised lastly in RAN#106 in [1]. It includes the objective on support of regenerative payload as follows:

Support of regenerative payload [RAN3, RAN2, RAN4] - Specify the support of gNB on board in TS 38.300 - Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3] Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it.

For the regenerative payload objective, the following agreements were made in RAN2#127bis [2]. No new agreement were recorded at RAN2#128 [3] for this specific objective.

RAN2-127bis agreements:

- We don’t further consider this possible optimization.

Note: The above RAN2-127bis agreement refers to the following: After early contention resolution, i.e., after sending HARQ feedback of the message containing only contention resolution MAC CE, a UE waits UE-gNB RTT to monitor the PDCCH for further downlink message.

In addition, RAN2#126 made the following agreements (extract) [5]:

- RAN2 assumes that network verified UE positioning is supported for regenerative architecture. - (…) - Companies interested to support satellite switch with resync with regenerative payload are invited to bring this discussion to the RAN plenary

The agenda from the chairlady for RAN2#129 states the following for this agenda item:

Contributions, if any, should focus on the needed updates for Stage 2 description and on whether any other existing essential features (not considered so far) would be affected - and potentially need any modifications - in a regenerative payload architecture.

In this document, we discuss remaining issues for this objective considering the above, with impacts to stage 2 and potential features affected.

# 2	Discussion

## 2.1	Stage 2 impacts

## TS 38.300 [7] has a shared responsibility between RAN2 and RAN3. RAN3 has endorsed the 38.300 CR in R3-243949 [6] at RAN3#124. Note that no other general text proposal/CR was endorsed/agreed at RAN3#125 or RAN3#126 for the introduction of regenerative payload for NTN for Ph3, so this can be considered as the latest RAN3 proposed text.

## The latest version of TS 38.300 [7] includes a clause on Timing and Synchronization, 16.14.2. Figure 16.14.2.1-1 is depicted below:

Figure 16.14.2.1-1: Illustration of timing relationship (for collocated gNB and NTN Gateway).
Source: TS 38.300 [7] clause16.14.2.

## This figure has not been impacted by the endorsed RAN3 CR [6]. One reason may be that the Uu timing and synchronization in 38.300 [7] may have been considered out of RAN3 scope.  Since the figure above applies to transparent payload, it is suggested to clarify this in TS 38.300 [7].

## Observation 1: Figure 16.14.2.1-1 in TS 38.300 [7] applies to the transparent payload case, and has not been impacted by the earlier/latest endorsed RAN3 CR for introduction of regenerative payload in NR NTN Ph3.

## Proposal 1: Update Figure 16.14.2.1-1 in TS 38.300 [7] to state that it applies to transparent payload.

## For the regenerative payload case, RAN2 sent on LS to RAN1 and RAN4 on common TA in a regenerative payload scenario in [8]. RAN1 and RAN4 have replied in [9] and [10] respectively, stating the following:

## Extract from RAN1 reply LS [9]: “A majority of companies in RAN1 think that it would not be a problem, depending on gNB implementation in a regenerative payload scenario, to stick to 0 as minimum possible value for TA common without introducing the negative value for ta-Common”.

Extract from RAN4 reply LS [10]: “No consensus on whether there is an issue due to non-negative value for common TA or whether negative values should be introduced for ta-common. Note: No more discussion of this issue is expected in RAN4.”.

Which means that there were no consensus in RAN1 or RAN4 to introduce negative values for common TA parameters.

Observation 2: From the reply LSs [9] and [10], there were no consensus in RAN1 or RAN4 to introduce negative values for common TA parameters.

This means that the RP cannot be considered closer to the UE between the UE and on-board gNB. Therefore, the RP can only be considered at the satellite gNB, with values of common TA as zero. It is therefore suggested to introduce an additional figure 16.14.2.1-2 to reflect this for regenerative payload. To avoid restricting the implementation, it is suggested that this should be stated as an example.

Observation 3: Following reply LSs from RAN1 and RAN4 in [9] and [10], the RP cannot be considered closer to the UE between the UE and on-board gNB.

Proposal 2: Introduce a new Figure 16.14.2.1-2 in TS 38.300 [7] reflecting that for regenerative payload at the gNB, the RP is considered at the satellite gNB. Provide additional text reflecting that value of common TA may be considered as zero. To avoid restricting implementation, this will be stated as an example.

If the proposals above are agreed, then the draft CR in Annex A can be endorsed/agreed.

Proposal 3: Endorse/agree the draft 38.300 CR from Annex A in this document.

## 2.2	Feature impacts

## 2.2.1	Satellite switch with resynch

With the Rel-18 transparent architecture, satellite switch with resync is supported because the same PCI may apply to the two satellites as they may belong to the same cell. However, with the Rel-19 regenerative payload scenario, the two satellites belong to two different cells.

This means that there is likely to have SA3 impacts e.g. in terms of changing security keys. Depending on the impact potentially required by SA3, it is therefore possible that additional signalling would be required for the procedure. Having additional signalling for the satellite switch with resynch procedure would mean that the benefit of satellite switch with resynch compared to L3 handover may decrease.

Observation 4: In a regenerative payload scenario, for satellite switch with resynch, the two satellites belong to different cells. Due to security reasons, this may lead to additional signalling required for the satellite switch with resynch procedure. It is possible that this may reduce the usefulness of the satellite switch with resynch procedure compared to the L3 handover procedure.

Given the above, RAN2 should not pursue the work on satellite switch with resynch for regenerative payload until SA3 has clarified the impact required for the procedure, and RAN2 concluded in turn that there are still benefits for this procedure. While RAN2#126 recorded the following agreement [5]:

Companies interested to support satellite switch with resync with regenerative payload are invited to bring this discussion to the RAN plenary.

There is a need to state a decision about satellite switch with resync in the Rel-19.

Proposal 4: RAN2 should not pursue the work on satellite switch with resynch for regenerative payload until SA3 has clarified the impact required for the procedure and RAN2 has concluded in turn that there are still benefits for this procedure.

    - Network verified UE positioning

The UE location verification procedure may be initiated by the network and assisted by a positioning method.

NGSO satellites have limited in-service duration. Consequently, the positioning procedure may be interrupted when the satellite is involved in the positioning procedure.

For the regenerative scenario, it is important to guarantee that the UE can receive the positioning signal from the involved TRP(s). This was discussed in the contribution [4] from the RAN2-127bis meeting. One reasoning is that for the regenerative scenario, different gNBs are involved as TRPs in the positioning procedure. Therefore, longer interruptions may happen and inter-gNB interaction is required for the positioning measurement configuration update. This means that the positioning procedure may be delayed, or even terminated depending on the number of active TRP(s).

Observation 5: TRP change for regenerative scenario may have adverse effects on positioning procedure.

To solve this, reconfiguration of positioning measurements appears as an inefficient solution. Inter-gNB coordination may lead to additional overhead, or may not be feasible economically-wise in the network.

Observation 6: In a regenerative scenario which includes gNB change, inter-gNB coordination appears inefficient to reduce positioning procedure interruption effects.

In order to solve the above issue, contribution [4] from RAN2-128 suggested that the positioning procedure is not initiated if the time before gNB change would not be enough to complete it. This can be done by the UE reporting the gNB change timing to the LMF via LPP, or to the gNB via Uu. The LMF can in turn make the decision whether or not to initiate the positioning procedure. The gNB can also use this information to guarantee the completion of the incoming positioning procedure.

The gNB change timing may be reported in absolute or relative time. As from TS 38.331, coding absolute time requires 48 bits, while coding relative time requires 7 bits. Therefore, coding the latest seems much more efficient, i.e. the remaining time compared to when the message was sent.

Observation 7: To prevent interruption of positioning procedure, the UE reporting of incoming TRP expiration is useful. Using of relative time compared to the sending of the message appears the most efficient for this.

Given the above, the following is suggested:

Proposal 5: The UE indicates incoming TRP expiration to the LMF.

Proposal 6: The UE indicates incoming TRP expiration to the LMF in a relative time compared to the sending of the message.

# 3.	Conclusion

In this document, we analyse some remaining issues for regenerative scenario for NTN for NR Ph3, with the following observations and proposals:

Stage 2 impacts:

## Observation 1: Figure 16.14.2.1-1 in TS 38.300 [7] applies to the transparent payload case, and has not been impacted by the earlier/latest endorsed RAN3 CR for introduction of regenerative payload in NR NTN Ph3.

## Proposal 1: Update Figure 16.14.2.1-1 in TS 38.300 [7] to state that it applies to transparent payload.

Observation 2: From the reply LSs [9] and [10], there were no consensus in RAN1 or RAN4 to introduce negative values for common TA parameters.

Observation 3: Following reply LSs from RAN1 and RAN4 in [9] and [10], the RP cannot be considered closer to the UE between the UE and on-board gNB.

Proposal 2: Introduce a new Figure 16.14.2.1-2 in TS 38.300 [7] reflecting that the RP is considered at the satellite gNB for transparent payload. Provide additional text reflecting that value of common TA may be considered as zero. To avoid restricting implementation, this will be stated as an example.

Proposal 3: Endorse/agree the draft CR from Annex A in this document.

Satellite switch with resynch:

Observation 4: In a regenerative payload scenario, for satellite switch with resynch, the two satellites belong to different cells. Due to security reasons, this may lead to additional signalling required for the satellite switch with resynch procedure. It is possible that this may reduce the usefulness of the satellite switch with resynch procedure compared to the L3 handover procedure.

Proposal 4: RAN2 should not pursue the work on satellite switch with resynch for regenerative payload until SA3 has clarified the impact required for the procedure and RAN2 concluded in turn that there are still benefits for this procedure.

Network verified UE positioning:

Observation 5: TRP change for regenerative scenario may have adverse effects on positioning procedure.

Observation 6: In a regenerative scenario which includes gNB change, inter-gNB coordination appears inefficient to reduce positioning procedure interruption effects.

Observation 7: To prevent interruption of positioning procedure, the UE reporting of incoming TRP expiration is useful. Using of relative time compared to the sending of the message appears the most efficient for this.

Proposal 5: The UE indicates incoming TRP expiration to the LMF.

Proposal 6: The UE indicates incoming TRP expiration to the LMF in a relative time compared to the sending of the message.

# 4.	References

- RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3 3GPP TSG RAN Meeting #106 Madrid, Spain, 09-12 December 2024.

- R2-2409213, RAN2-127bis Report from Break-out session on NR-NTN and IoT-NTN. Session Chair (ZTE Corporation).

- R2-2410913, RAN2-128 Report from Break-out session on NR-NTN and IoT-NTN.  Session Chair (ZTE Corporation).

- R2-2410269, RAN2-128. UE location verification in NTN regenerative architecture, Lenovo.

- R2-2405703, RAN2-126 Report from Break-out session on NR-NTN and IoT-NTN  Session Chair (ZTE Corporation).

- R3-243949, RAN3-124, Fukuoka, Japan, 20-24 May 2024. CR for 38.300 on Support for Regenerative Payload in NR NTN. Source to RAN3: Ericsson, Thales, Deutsche Telekom, Nokia, ESA, CATT, ZTE, Sateliot, Huawei, Dish Networks, Echostar, Eutelsat Group, Xiaomi, Samsung, CMCC, LG Electronic.

- 3GPP TS 38.300 version 18.4.0 “NR; NR and NG-RAN Overall Description; Stage 2”.

- R2-2407623, LS on common TA in a regenerative payload scenario, RAN2.

- R1-2409258, Reply LS on common TA in a regenerative payload scenario, RAN1.

- R4-2416920, Reply LS on common TA in a regenerative payload scenario, RAN4.

# Annex A. CR to 38.300.

3GPP TSG-RAN WG2 Meeting #129	R2-24xxxxx

# Athens, Greece, 17-21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | - | Rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for Regenerative Payload in NR NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Toyota ITC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-01-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-19, gNB on board the satellite is supported. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify the stage 2 description in the clause related to scheduling and timing for regenerative payload with gNB on board. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No stage 2 description for scheduling and timing for the regenerative payload with gNB on board. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.4.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

### 16.14.2	Timing and Synchronization

#### 16.14.2.1	Scheduling and Timing

DL and UL are frame aligned at the uplink time synchronization reference point (RP) with an offset given by NTA,offset (see clause 4.2 of TS 38.213 [38]).

To accommodate the propagation delay in NTNs, several timing relationships are enhanced by a Common Timing Advance (Common TA) and two offsets Koffset and Kmac:

-	Common TA is a configured timing offset that is equal to the RTT between the RP and the NTN payload.

-	Koffset is a configured scheduling offset that needs to be larger or equal to the sum of the service link RTT and the Common TA.

-	Kmac is a configured offset (see TS 38.331 [12]) that is approximately equal to the RTT between the RP and the gNB.

The scheduling offset Koffset is used to allow the UE sufficient processing time between a downlink reception and an uplink transmission, see TS 38.213 [38].

The offset Kmac is used to delay the application of a downlink configuration indicated by a MAC CE command on PDSCH, see TS 38.213 [38], and in estimation of UE-gNB RTT, see TS 38.321 [6]. It may be provided by the network when downlink and uplink frame timing are not aligned at gNB. The Kmac is also used in the random access procedure, to determine the start time of RAR window/MsgB window after a Msg1/MsgA transmission (see TS 38.213 [38]).

The Service link RTT, Feeder link RTT, RP, Common TA, Kmac and TTA (see clause 16.14.2.2) are illustrated in Figure 16.14.2.1-1 and Figure 16.14.2.1-2.

Figure 16.14.2.1-1: Illustration of timing relationship (for collocated gNB and NTN Gateway with transparent payload)

Figure 16.14.2.1-2: Illustration of timing relationship (one example for collocated gNB and NTN Gateway with regenerative payload hosting a gNB)

For the case of collocated gNB and NTN Gateway with regenerative payload hosting a gNB, the value of Common TA may be considered as zero in one example.

The network may configure the HARQ operation as follows:

-	For downlink, HARQ feedback can be enabled or disabled per HARQ process (as specified in clause 5.3.2.2 and clause 5.7 of TS 38.321 [6]). Disabling HARQ feedback allows scheduling a HARQ process before one HARQ RTT has elapsed since last scheduled.

-	For uplink, HARQ mode (i.e. HARQ mode A or HARQ mode B) can be configured per HARQ process (as specified in clause 5.4.3.1 and clause 5.7 of TS 38.321 [6]). HARQ mode B allows scheduling a HARQ process before one HARQ RTT has elapsed since last scheduled.

NOTE:	For the HARQ processes configured with HARQ feedback enabled/disabled, it is up to network implementation to ensure a proper configuration of HARQ feedback (e.g., either all enabled or all disabled) for HARQ processes used by an SPS configuration. For the HARQ processes configured with HARQ mode, it is up to network implementation to ensure a proper configuration of HARQ mode (e.g., either all HARQ mode A or all HARQ mode B) for HARQ processes used by a CG configuration.

END OF CHANGES