3GPP TSG-RAN WG2 #129	R2-2500916

Athens, Greece, Feb 17 – 21, 2025

Agenda Item:	8.10.2.1

Source:	Ericsson

Title:	Further considerations on MRO for LTM

Document for:	Discussion/Decision

# Introduction

- MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]:

•	Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3]

•	Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

One of the main objectives of the SONMDT Rel-19 WID [RP-234038] is to enhance the MRO features to optimize the LTM cell switch procedure.

This contribution discusses considerations regarding SON enhancements for LTM and CHO with candidate SCGs.

# Discussion

## SON for LTM

Per the agreements from previous meetings, we will focus on MCG LTM, and discuss the enhancement for the RLF report, the SHR and the RA report for LTM optimization. Considering that the LTM cell switch procedure highly relies on underlying features such as early DL/UL synchronization and a RACH-less access procedure, we will also discuss the possible MRO enhancements from those aspects.

### Enhancement of RLF-Report

Logging beam information

The LTM cell switch command includes the target beam to be used by the UE when executing the LTM cell switch. However, the beam indicated in the LTM cell switch command may be sub-optimal and the LTM cell switch may fail due to a bad choice of target beam, the UE may re-establish/recover or reconnect to the same cell but in a better beam or in a different cell.

Currently the UE logs the target cell ID, the re-established cell ID and, when applicable, the reconnected cell ID in the RLF report, and at RAN2 #127 it was agreed to also include the LTM recovery cell ID (when applicable). Given that the LTM cell switch relies heavily on the beam information, the beam information can also help the network to optimize future LTM cell switch procedures. More specifically, together with the beam information indicated in the LTM cell switch command, the beam information the UE succeeded to recover/re-establish or reconnect in, and the beam level L1 or L3 measurement quantities, the NW can determine to prioritize the succeeded beam for early DL sync and LTM cell switch.

- Currently the UE, upon failure, logs some cell information such as failed cells ID, re-establishment Cell ID, reconnect cell ID and LTM recovery cell ID. This enables the network to figure out better candidate beam for the future LTM decisions.

In summary, the logging of the beam related information can help the NW in selecting the optimal beam for early DL sync and in the LTM cell switch command and also in the selection of LTM candidate beams.

Enhance the RLF report with beam information of.

  - the beam toward which the UE failed to perform LTM cell switch,

  - The beam indicated in the last successful LTM cell switch command, when failure occurs shortly after the LTM cell switch,

  - The beam the UE succeeded to recover/re-establish or reconnect in.

We also note that additional information that would be valuable for optimization includes whether the UE switched beam in the target cell prior to the RLF. To this end, it would be beneficial if the UE also includes information indicating whether the UE has switched the beam in target cell or not prior to the RLF. If the UE changes the beam in the target cell to a non-candidate beam shortly after a successful LTM cell switch , perhaps it is the target node that need to take actions to optimize the UE configuration, e.g. selection of candidate beams/TCI states, but if the UE has not changed the beam prior to the failure in the target cell, the DU controlling the source cell of the LTM cell switch should take counteractions to avoid failures for future LTM cell switches. In light of the above, we propose the following.

In case of RLF shortly after a successful LTM cell switch, the UE logs whether the UE has switched the beam in the target cell prior to the RLF. This enables the network to correctly classify Too Early HO failure from failure due to wrong beam configuration at the target cell.

Logging time info for PDCCH ordered TA acquisition

In RAN2#127bis meeting, the following was agreed:

1) If RA-based LTM failure happens the UE logs and reports RACH info in the RLF report. Additional information is TBD.

2) Unless RAN3 defines a NW-based solution: The UE logs and reports whether and how the UE got the TA value used for a failed LTM switch (gNB indicated or UE determined).

Unless RAN3 defines a NW-based solution, whether and how the UE got the TA value used for a failed LTM cell switch is agreed to be logged. It is also agreed that the UE logs the RACH info when a RA-based LTM failure happens. Note that as long as the UE got the TA value, no matter if the TA value was indicated from gNB or estimated by UE itself, RACH-less LTM was initiated. Thus, the presence or absence of RACH info (in the form of the ra-InformationCommon IE) in the RLF report in case of a failed LTM cell switch can be used to implicitly indicate whether the UE got the TA value or not, i.e. whether the UE attempted RACH-based or RACH-less LTM cell switch.

- The absence of RACH info implies that the UE attempted a RACH-less LTM cell switch. This in turn implies that the UE either received a TA value from the NW or that the UE estimated the TA itself.

However, how to log whether the TA value was indicated from gNB or estimated by UE itself is still open. A straightforward and simple solution would be to introduce a flag to indicate whether it is gNB indicated or UE determined. Taking a closer look at this problem, for PDCCH ordered TA estimation, the most important thing in the NW is to trigger the PDCCH ordered RACH procedure towards a right LTM candidate cell at a proper time. If there is a long time gap between the PDCCH ordered PRACH and LTM cell switch initialization, then the estimated TA value maybe not be accurate enough at the time of LTM cell switch. In contrast, if the PDCCH ordered RACH procedure have to be repeated (too frequently), it will waste radio resources and increase the UE energy consumption. Therefore, the timing information of when PDCCH ordered TA estimation is initiated can help the NW to optimize PDCCH ordered early UL sync procedure.

On the other hand, for the UE-based TA estimation, the UE performs TA measurement for the candidate cells after being configured by RRC but the exact time the UE performs TA measurement is up to UE implementation. Thus, NW cannot control or optimize when UE performs TA estimation. Therefore, there is no need to log the time information of when the UE performs TA estimation.

In summary, the following items of information can be used to indicate whether and how the UE got the TA value:

- The presence of RACH info: RACH-based LTM.

- The absence of RACH info, presence of time since the PDCCH ordered PRACH transmission till the reception of the LTM cell switch command: RACH-less LTM, UE got the TA by PDCCH order (network based).

- The absence of RACH info, absence of time since the PDCCH ordered PRACH transmission till the reception of the LTM cell switch command: RACH-less LTM, UE got the TA by UE autonomous TA estimation.

The UE logs and reports the time between the PDCCH ordered TA acquisition and the reception of the LTM Cell Switch Command MAC CE that triggered the execution of the failed LTM cell switch. For the UE-based TA estimation, there is no need to log this information.

Logging TA difference

To fulfill its purpose of being a mobility procedure with quick execution and short interruption, it is important that LTM as far as possible use RACH-less access in the target cell. RACH-less LTM is enabled by the early UL sync procedure, which thus is key for well-functioning LTM. A well-functioning early UL sync procedure is thus important for well-functioning LTM, and in the context of SON, this makes it natural to ways to design SON mechanisms for optimization of the early UL sync procedure. To this end, it has been proposed that the UE could log and report the TA it receives in a RAR during a random access procedure in conjunction with a reestablishment procedure or LTM recovery procedure following a failed LTM cell switch. Another possibility is that the UE reports the difference between such a TA and the TA it received in the LTM Cell Switch Command MAC CE. Which of these that is the most useful to report depends on the strategy for the target cell TA determination.

One way is to determine a target cell TA value that works sufficiently well and then use that TA value for all LTM cell switches towards that target cell. This is a rudimentary and rigid strategy, but may be pragmatic and work at least in small cells. To support this strategy, reporting of a TA received in a RAR (or MsgB) in close conjunction with the execution of the LTM cell switch is very useful information since it is a direct indication of the correct TA for the target cell. However, in this scenario, the reported TA does not provide any direct information about the early UL sync and it actually serves the purpose of enabling bypassing of the early UL sync procedure.

- Reporting of the TA received in a RAR in the target cell is useful when the goal of the source DU is to determine a static target cell TA value that can be used for all LTM cell switches towards that target cell.

- The TA received in a RAR in the target cell provides no direct information about the early UL sync.

Another approach for target cell TA determination, which is more in line with the intention of the specifications, is to fully rely on the TA measured during an early UL sync procedure. This will produce a more accurate target cell TA value as each early UL sync measurement accurately determines the TA for the UE in its particular location. In addition, this strategy works in all scenarios, e.g. regardless of the cell size. For this strategy, reporting of the TA received in a RAR in the target cell is of little value. Instead, what is needed is information that provides some indication of how well the early UL sync performed and in what way it could be optimized. For this purpose, the difference between the TA received in a RAR in the target cell and the TA the UE received in the LTM Cell Switch Command MAC CE is more useful.

If the size of the TA difference is large for a certain target cell (and UEs frequently reports this), this tells the source DU that UEs in the source cell, or in the part of the source cell close to the border to the target cell, are often moving rather fast towards or away from the target cell’s antenna site, which causes the measured TA to become outdated faster than expected. From this the source DU can conclude that it must be more careful to ensure that the measured TA is sufficiently fresh when the LTM cell switch is executed. The sign of the TA difference (positive or negative) informs the source DU of which the direction (towards or away from the target cell’s antenna site) the UEs typically move in. Furthermore, together with the time between the PDCCH order and the reception of the LTM Cell Switch Command MAC CE (which advantageously could be included in the RLF report) the TA difference informs the source DU of the rate of change of the RTT between the UE and the target cell’s antenna site which translates directly into the rate of change of the TA. If this rate of change is high, the source DU (as mentioned before) concludes that it must not let the measured TA become too old when triggering the LTM cell switch execution, and the source DU can also conclude that the timing of the LTM cell switch execution is crucial so that it can be triggered in the window when the target cell channel quality is good enough while the source cell channel quality is still not too bad. To ensure that the measured TA does not get too old, the source DU can trigger repeated early UL sync procedures if too long time elapses after the preceding early UL sync procedure without execution of the LTM cell switch, or in the worst case the source DU can choose to use RACH-based LTM towards the concerned target cell. To ensure good timing of the trigger of the LTM cell switch execution, the source DU may configure UEs with L1 measurement reporting with short periodicity.

- The difference between the TA received in a RAR in the target cell and the TA in the LTM Cell Switch Command MAC CE, in particular when combined with the time between the PDCCH order and the LTM Cell Switch Command MAC CE, can indicate to the source DU that a TA measured using the early UL sync procedure must be kept sufficiently new and that timing of the trigger of the LTM cell switch indication is critical.

- Repeating of the early UL sync if too long time elapses after the preceding early UL sync procedure without execution of the LTM cell switch and configuration of L1 measurement reporting are adaptations the source DU may perform if the UEs’ RLF reports indicate fast rate of change of the TA.

In conclusion, the difference between the TA received in a RAR in the target cell and the TA in the LTM Cell Switch Command MAC CE is more useful for optimization of the early UL sync procedure than the TA received in a RAR in the target cell.

- The difference between the TA received in a RAR in the target cell and the TA in the LTM Cell Switch Command MAC CE is more useful for optimization of the early UL sync procedure than the TA received in a RAR in the target cell.

The UE logs the difference between a TA received in a RAR (or a MsgB) in the target cell and the TA received in the LTM Cell Switch Command MAC CE in the RLF report.

### Enhancement of SHR

Successful LTM cell switch to wrong beam

As discussed above, the beam indicated in the LTM cell switch command may be wrong or suboptimal. With the wrong or suboptimal beam, the LTM cell switch can still be successful, but beam failure may occur shortly afterwards. The UE then executes the beam failure recovery procedure towards a better beam in the same cell. To capture this scenario, we envisage either a UE-based solution based on SHR, or a NW-based solution.

For the UE-based solution based on SHR, we could introduce a new triggering condition for the SHR generation, i.e. “BFD after successful LTM cell switch”. And if the SHR is generated, the UE could include in this SHR information related to the beam in which the UE performs the BFR (so that the network may consider this beam for future LTM cell switches), and also the time gap between the successful execution of the LTM cell switch and the BFD (to aid the network to figure out whether optimization is needed at the source or target DU). Furthermore, the UE could include the beam towards which the LTM cell switch was performed to facilitate for the source DU to identify the situations where the L1 measurements (which are also included in the SHR) imply that the selected LTM target beam is the best while the BFR target beam is actually the best.

For the NW-based solution instead, if BFR happens shortly after the successful LTM cell switch, the target DU informs the source DU about the LTM cell switch to wrong beam.

RAN2 to select one of the following solutions to solve the issue of “successful LTM cell switch to wrong beam”:

  - UE based solution: introduce new SHR triggering condition (a successful BFR occurs in the target cell shortly after a successful LTM cell switch) and include in the SHR the beam information associated to the beam in which the UE performs BFR as well as the target beam of the LTM cell switch, and time elapsed between the successful LTM cell switch and BFD.

  - NW based solution: the target DU notifies the source DU about BFR occurred in the target cell of the successful LTM cell switch.

    - If this solution is selected, RAN2 should send an LS to RAN3 asking them to implement it.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Currently the UE, upon failure, logs some cell information such as failed cells ID, re-establishment Cell ID, reconnect cell ID and LTM recovery cell ID. This enables the network to figure out better candidate beam for the future LTM decisions.

Observation 2	The absence of RACH info implies that the UE attempted a RACH-less LTM cell switch. This in turn implies that the UE either received a TA value from the NW or that the UE estimated the TA itself.

Observation 3	Reporting of the TA received in a RAR in the target cell is useful when the goal of the source DU is to determine a static target cell TA value that can be used for all LTM cell switches towards that target cell.

Observation 4	The TA received in a RAR in the target cell provides no direct information about the early UL sync.

Observation 5	The difference between the TA received in a RAR in the target cell and the TA in the LTM Cell Switch Command MAC CE, in particular when combined with the time between the PDCCH order and the LTM Cell Switch Command MAC CE, can indicate to the source DU that a TA measured using the early UL sync procedure must be kept sufficiently new and that timing of the trigger of the LTM cell switch indication is critical.

Observation 6	Repeating of the early UL sync if too long time elapses after the preceding early UL sync procedure without execution of the LTM cell switch and configuration of L1 measurement reporting are adaptations the source DU may perform if the UEs’ RLF reports indicate fast rate of change of the TA.

Observation 7	The difference between the TA received in a RAR in the target cell and the TA in the LTM Cell Switch Command MAC CE is more useful for optimization of the early UL sync procedure than the TA received in a RAR in the target cell.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Enhance the RLF report with beam information of.

a.	the beam toward which the UE failed to perform LTM cell switch,

b.	The beam indicated in the last successful LTM cell switch command, when failure occurs shortly after the LTM cell switch,

c.	The beam the UE succeeded to recover/re-establish or reconnect in.

Proposal 2	In case of RLF shortly after a successful LTM cell switch, the UE logs whether the UE has switched the beam in the target cell prior to the RLF. This enables the network to correctly classify Too Early HO failure from failure due to wrong beam configuration at the target cell.

Proposal 3	The UE logs and reports the time between the PDCCH ordered TA acquisition and the reception of the LTM Cell Switch Command MAC CE that triggered the execution of the failed LTM cell switch. For the UE-based TA estimation, there is no need to log this information.

Proposal 4	The UE logs the difference between a TA received in a RAR (or a MsgB) in the target cell and the TA received in the LTM Cell Switch Command MAC CE in the RLF report.

Proposal 5	RAN2 to select one of the following solutions to solve the issue of “successful LTM cell switch to wrong beam”:

a.	UE based solution: introduce new SHR triggering condition (a successful BFR occurs in the target cell shortly after a successful LTM cell switch) and include in the SHR the beam information associated to the beam in which the UE performs BFR as well as the target beam of the LTM cell switch, and time elapsed between the successful LTM cell switch and BFD.

b.	NW based solution: the target DU notifies the source DU about BFR occurred in the target cell of the successful LTM cell switch.

i.	If this solution is selected, RAN2 should send an LS to RAN3 asking them to implement it.

# References

R2-2406959, Discussion on MRO enhancements for R18 mobility features, RAN2, 3GPP TSG RAN WG2#127, Maastricht, Netherlands, 19th - 23rd August 2024.