3GPP TSG-RAN WG2 #129	R2-2500917

Athens, Greece, Feb 17 – 21, 2025

Agenda Item:	8.10.4

Source:	Ericsson

Title:	Discussion on the reply LS to RAN3 on SON-MDT enhancements for NTN

Document for:	Discussion/Decision

# Introduction

RAN#103 decided to support further data collection for Self-organizing Networks (SON). One of the topics that was endorsed was the SON and MDT enhancements for mobility in non-terrestrial networks.

RAN2 had no discussion on SON and MDT enhancement for NTN in previous meetings. Now, we think RAN2 should trigger the related discussion due to the LS[2] from RAN3 on this subject. An excerpt from the LS is listed here for reference:

- Regarding SON/MDT for NTN: On MRO for NTN, RAN3 agreed that UE reports the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition”, it is up to RAN2 to decide to reuse the existing IE or not. On MDT for NTN, RAN3 assumes that the geographical area and/or the mapped cell ID can be used as the Area Scope for the logged MDT for NTN over NGAP. Further, RAN3 would like to confirm with RAN2 whether the geographical area defined for MBS NTN can be reused for Area Scope of logged MDT over Uu for NTN, to help RAN3 decide whether to specify geographical area and/or mapped cell ID over NGAP. To RAN2 group. ACTION: RAN3 kindly asks RAN2 to take the above into consideration and update their specifications as needed. RAN3 also kindly requests RAN2 to provide feedback to the question on MDT for NTN.

# Discussion

## MRO for NTN

Joint trigger events with time-based/location-based trigger event and measurement based trigger event constituting the execution condition for CHO

RAN3 had discussed MRO enhancement for conditional handover. One scenario is joint trigger event with time-based/location-based trigger event and measurement-based trigger event constituting the execution condition for CHO. The agreement reached in RAN3 is:

UE reports the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition”, it is up to RAN2 to decide to reuse the existing IE or not.

In the current RLF-Report, the fulfilled trigger event(s) information at the time of handover failure or radio link failure is logged by firstTriggeredEvent and timeBetweenEvents in measResultNR when RRM measurements are available.

Therefore, if RRM measurements are available, the fulfilled CHO trigger events before RLF occurs in case of “time and measurement based trigger events” or “location and measurement based trigger events” can be logged in the RLF-Report naturally by firstTriggeredEvent and timeBetweenEvents.

It is up to UE implementation how the UE evaluates the time- or location-based trigger event together with the RRM measurement-based trigger event. A reasonable assumption is that UE will at least start the RRM measurement before or when the time- or location-based trigger event is fulfilled. Therefore, if the time- or location-based trigger event is fulfilled, RRM measurements should be available, and firstTriggeredEvent and timeBetweenEvents in measResultNR will be logged when a failure occurs. However, if the time- or location-based trigger event is unfulfilled, the UE may not do the RRM measurement depending on its implementation, then whether the measurement-based trigger event is fulfilled or not cannot be logged as there is no related measResultNR to put the information in. We think that this case can be acceptable as the UE actually has no available RRM measurements (and therefore cannot have determined whether the measurement-based trigger event was fulfilled), and the network can first tune the time- or location-based trigger event accordingly. Therefore, no additional modifications are needed.

Reuse the existing IE to report the fulfilled CHO triggering events in case of time- or location-based trigger event together with the RRM measurement-based trigger event.

## MDT for NTN

RAN2 agreements regarding the geographical area defined for MBS NTN are copied in the Annex B of the contribution. In summary, based on the MBM NTN agreements, a new SIB is to be introduced to indicate the MBS service area, which is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s). Furthermore, the tn-ReferenceLocation-r18 and tn-DistanceRadius-r18 is reused for the geographical area of the circle and the encoding of Polygon in TS37.355 is reused for the geographical area of the Polygon.

The definition of geographical area for MBS service can also apply to MDT area scope which can be implemented by tn-ReferenceLocation-r18 and tn-DistanceRadius-r18 or the encoding of Polygon in TS37.355.

RAN2 to confirm that the definition of geographical area for MBS NTN can apply to MDT area scope, i.e. a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s).

RAN2 to confirm that the implementation of geographical area for MBS NTN can apply to MDT area scope, i.e. the encoding of TN coverage introduced in Rel-18 in TS38.331 and the encoding of Polygon in TS37.355 are reused.

Regarding the signaling of the MBS service area, a new SIB is introduced to include a list of intended service areas, and each of the intended service area needs to be associated with an MBS session.

areaConfiguration included in LoggedMeasurementConfiguration is currently used to indicate the MDT area scope, which is in the form of cells, tracking area codes/ frequencies, PNI-NPN identities or the SNPN identities. Similarly, this IE can be extended to indicate the newly introduced geographical area.

RAN2 to extend areaConfiguration in LoggedMeasurementConfiguration with geographical area for MBS NTN.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Reuse the existing IE to report the fulfilled CHO trigger events in case of time- or location-based trigger event together with the RRM measurement-based trigger event.

Proposal 2	RAN2 to confirm that the definition of geographical area for MBS NTN can apply to MDT area scope, i.e. a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s).

Proposal 3	RAN2 to confirm that the implementation of geographical area for MBS NTN can apply to MDT area scope, i.e. the encoding of TN coverage introduced in Rel-18 in TS38.331 and the encoding of Polygon in TS37.355 are reused.

Proposal 4	RAN2 to extend areaConfiguration in LoggedMeasurementConfiguration with geographical area for MBS NTN.

# References

RP-234038, “New WID: Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4”, RAN#102, Edinburgh, Scotland, December 2023.

R3-247908, LS on RAN3 agreements with impact on UE related to the Rel-19 SON/MDT.

# Annex

## Annex.1 Draft LS reply to RAN3

3GPP TSG-RAN WG2 #129	R2-25xxxxx

Athens, Greece, Feb 17 – 21, 2025

Title:	[Draft] Reply LS on RAN3 agreements on SON/MDT for NTN:

Response to:	R3-247908

Release:	Rel-19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core

Source:	TSG RAN WG2

To:	TSG RAN WG3

Cc:	N/A

Contact Person:

#### Name:

E-mail Address:

Attachments:	None

# 1	Overall description

RAN2 would like to thank RAN3 for the LS on RAN3 agreements with impact on UE related to the Rel-19 SON/MDT.

Regarding the MRO for NTN, RAN2 confirms that the existing IE in RLF-Report can be reused for the UE to report the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement-based trigger condition” or “location and measurement-based trigger condition”.

Regarding the MDT for NTN, RAN2 confirms that same as the geographical area defined for MBS NTN, the MDT area scope can be extended to a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s). Furthermore, the encoding of TN coverage introduced in Rel-18 in TS38.331, including tn-ReferenceLocation-r18 and tn-DistanceRadius-r18, can be reused for the geographical area of the circle. And the encoding of Polygon in TS37.355 can be reused for the geographical area of the Polygon.

# 2	Actions

To RAN3

ACTION: 	RAN2 kindly requests RAN3 to take the above information into consideration.

# 3	Dates of next TSG WG2 meetings

TSG-RAN WG2 Meeting #129bis	7 - 11 April 2025	Wuhan, China

TSG-RAN WG2 Meeting #130	19 - 23 May 2025	St Julian, Malta

# Annex B

Agreements from NTN sessions in RAN2 meetings

RAN2 #126 When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details) RAN2 #127 1. The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s). RAN2 #128 Agreements: 1. The encoding of TN coverage introduced in Rel-18 in TS38.331, including tn-ReferenceLocation-r18 and tn-DistanceRadius-r18, is reused for the geographical area of the circle. 2. The encoding of Polygon in TS37.355 is reused for the geographical area of the Polygon. 3. The IntendedServiceArea is considered as the IE name of the geographical area (we can still update the name in the CR implementation if needed) 4. A signalled intended service area for a MBS BC service may include geographic areas across the current serving cell and overlapping neighbor cell(s). 5. RAN2 understands that the geographic area information for the intended service areas can be semi-static and not cause frequent updates. 6. Introduce a new SIB to include a list of intended service areas and related pointer (FFS if we point to the intended services areas via the index in the list or with an ID or another way) 7. The legacy SIB modification procedure is applied to update the intended service area information in the new SIB.