3GPP TSG RAN WG2 #129	R2-2500923

Athens, Greece, February 17 – 21, 2025

Agenda Item:	8.17
Source:	Toyota ITC
Title:	Signaling aspects of IoT-NTN TDD mode

Document for:	Discussion and Decision

# Introduction

The RAN plenary has approved a Release 19 work item to introduce IoT-NTN TDD mode [1]. This document discusses some control-plane signaling aspects of the feature.

# Discussion

The feature aims to save energy by making use of the time-slotted structure of the radio interfaces for data transfers every Nth frame. Therefore, the satellite needs to inform UEs about the support of this feature, e.g. with an indicator in the system information broadcast. Using a single indicator bit would be efficient, but it is worthwhile to consider current feature restrictions and possible future developments instead of opting for the simplest solution.

Release 19 captures the following restrictions in the work item description. The periodic pattern includes non-overlapping uplink and downlink frames, fixed guard periods every Nth radio frame, no blind detection is required at the UE side, and a fixed periodicity value N is set per frequency band. N is set to a fixed value 9 as a baseline which is yet to be confirmed. Without the need for blind detection, the UE uses the fixed N value instead of identifying it independently or blindly.

It should be noted that some restrictions may change in the future. For example, the guard period might become configurable, and other N values added. The same N value could remain while other restrictions may be relaxed. Release 19 UE should detect these changes, and future Release UEs should comply with Release 19 restrictions. Ensuring compatibility with UEs that do not understand newer signaling formats is crucial, so it is important to consider the semantics and extensibility of the indication.

It is suggested to separate the N value from the feature restrictions and explicitly indicate the N value, even if fixed in Release 19, since it may be extended in the future. In other words, it is better to avoid default values. This allows using the fixed Release 19 N value with different restrictions in future Releases and applying Release 19 restrictions while using another N value.

Proposal 1: Include the N value in the indication for the sake of future proofness, even if it has a fixed value in Release 19.

Proposal 2: Separate the indication of restrictions from the N value with an own indicator.

It is advisable to use extensible data types, so they can accommodate new values with the same data type. For example, define N as an enumerated code-point n9. This allows adding new values later with the same data type. It may also be useful to avoid using integer ranges because it can lead to awkward semantics when not all values in the range are used. For example, initially, UE categories were specified as integers, but later extensions added enumerations, complicating things for everyone involved. To avoid similar issues, it is advisable to consider using enumerations. It is similarly unlikely that the periodicity values will be utilized for any arithmetic calculations.

A possible signaling solution is to define a new extensible sequence with two indicators: periodicity-19 for the N value and restrictions-r19 for applied restrictions. The sequence can be incorporated into the existing signaling as an extension, or it can be specified as a separate information element. The semantics of the restrictions can be captured in the field description.

ntn-TDD-Info-r19 				SEQUENCE {

periodicity-r19						ENUMERATED {n9}				OPTIONAL,

restrictions-r19					ENUMERATED {rel19}			OPTIONAL,

...

}																	OPTIONAL

The presence of the ntn-TDD-Info-r19 field indicates feature support. New N values or restrictions can be added after the extension marker, omitting the optionally present periodicity-r19 or restrictions-r19 fields, respectively. If any of the Release 19 fields are absent and extensions are present, the Release 19 UE will recognize that an evolved version of the feature is used, and it should camp to another cell. This ensures that the signaling remains extensible and future proof.

# Conclusion

This contribution discusses signaling aspects of IoT-NTN TDD mode. Proposals are summarized as follows:

Proposal 1: Include the N value in the indication for the sake of future proofness, even if it has a fixed value in Release 19.

Proposal 2: Separate the indication of restrictions from the N value with an own indicator.

# Reference

- RP-243293, “Revised WID for introduction of IoT-NTN TDD mode”, Iridium Satellite LLC, December 9-12, 2024.