3GPP TSG-RAN WG2 Meeting #129	R2-2500929

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	8.9.2

Source:	Turkcell

Title:	Support of Store&Forward for IoT NTN

Document for:	Discussion, Decision

# 1	Introduction

The Internet of Things (IoT) Non-Terrestrial Network (NTN) was specified in 3GPP Radio Access Network (RAN) Release 17 and has been optimised in Release 18 since then. Commercial deployment is ongoing, and based on actual deployment plans, the IoT NTN needs to evolve further.

The preparatory discussions in 3GPP since the June 2023 RAN Release 19 workshop have converged to mainly two areas of further evolution [1]:

- Support of Store and Forward (S&F) based on regenerative payload

- Uplink capacity enhancements

Based on the conclusions from the 3GPP RAN2 #128 meeting [2], in this contribution, we discuss further improvements for the store and forward in the NTN IoT Phase 3 Work Item.

# 2	Discussion

The following items were agreed upon in the 3GPP RAN2 #128 meeting, which could impact the radio interface.

Agreements:

1.	Time information can be broadcasted to indicate when the current satellite operation mode will transit from “S&F operation” mode to real-time/normal mode. RAN2 assumes that a R19 UE could use this information at least to delay some NAS procedures until the feeder link is resumed (FFS what we specify in RAN2 specs for this and if we differentiate the behaviour for R19 UEs supporting and not supporting S&F mode, also considering the relation to the Wait Timer). FFS which SIB is used (SIB1 or SIB31). FFS if absolute or relative time is used. FFS on the opposite transition (i.e. whether to indicate when the “S&F operation” will start)

2.	It is FFS whether we need to introduce a new release cause when the NW wants to release a R19 UE to RRC idle because the feeder link becomes unavailable

3.	“S&F operation” indication is provided in SIB1

4.	RAN2 assumes that AS signalling is not needed to indicate the architecture option (Split MME or Full CN).

5.	RAN2 waits for further progress in SA2, if any, before working on RAN2 aspects of a CIoT-UP solution for SF operation

6.	RAN2 understands that the UE configured with a satellite ID list by MME is not prevented to camp on a satellite operating in normal IoT NTN mode (i.e. with feeder-link connection), and perform subsequent access and data/signalling communication with that satellite

7.	RAN2 understands the UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list.

With these indications in the last meeting (RAN2 #128), the User Equipment (UE) is made aware of the current operational mode, S&F, through the indication provided in the system information. UE can be aware that the feeder link will eventually become available, so some NAS procedures can be delayed until the feeder link is resumed.

For the opposite transition, it means that the feeder link becomes unavailable at some point; the UE can prepare a change from a normal/real-time mode to the S&F. Mode change operations can coincide with the stop serving time of a cell, t-service.

The UE needs preparation of a change from a normal/real-time mode to the S&F.

We can reuse the legacy t-service as the mode switching time. In the legacy mechanism, the t-service in SIB19 indicates when the current serving cell will stop the service in the covered area. The satellite gains or loses feeder link connectivity when the UE is close to the minimum elevation angle. One possible NW implementation is setting the t-service as a switching time from normal to S&F mode. In this case, the t-service indicates the remaining time of the current operating mode.

Legacy t-service can be used as mode switching time. It indicates the remaining time of the current operating mode.

UE can be informed via a new timer that the cell should stop serving.

A new release cause is not needed when the feeder link is unavailable.

The arrival of mobile terminating data prompts the network to page the UE using the legacy paging procedure. The core network transmits the mobile terminating data to the satellite, which receives and stores it in S&F mode if a feeder link is available. The satellite waits for the service link availability to page the UE, or the UE can initiate an RRC connection, after which the network sends mobile terminating data to the UE.

When the service link is available, the UE can initiate an RRC connection or be paged for the mobile termination data.

The network doesn’t need to page the UE when the service link is available if no mobile terminating data is received during feeder link availability. The UE doesn’t monitor the paging channel or initiate an RRC connection to save energy. However, the UE needs to be informed to save energy when the network has no mobile terminating data buffered.

Proposal 4: The UE needs to be informed to save power when the network has no mobile terminating data.

During the subsequent pass, the downlink non-access stratum data can be stored at the mobility management entity on the satellite serving the UE. In this scenario, the stored data will be lost if the UE cannot access the intended satellite.

The stored data will be lost if the UE cannot access the intended satellite.

The current access control mechanism may lead to the rejection of the UE’s RRC connection request due to load status; it is beneficial for the eNB to prioritise the UE’s access should it subsequently re-attempt to connect to the network, thereby preventing potential data loss and unnecessary energy consumption.

Proposal 5: It is beneficial for the eNB to prioritise the UE’s access should it subsequently re-attempt to connect to the network, thereby preventing potential data loss and unnecessary power consumption.

# 3	Conclusion

We made the following observations in the previous section:

- The UE needs preparation of a change from a normal/real-time mode to the S&F.

- When the service link is available, the UE can initiate an RRC connection or be paged for the mobile termination data.

The stored data will be lost if the UE cannot access the intended satellite.

We propose the following based on the discussion in the previous section:

- Legacy t-service can be used as mode switching time. It indicates the remaining time of the current operating mode.

UE can be informed via a new timer that the cell should stop serving.

A new release cause is not needed when the feeder link is unavailable.

The UE needs to be informed to save power when the network has no mobile terminating data.

It is beneficial for the eNB to prioritise the UE’s access should it subsequently re-attempt to connect to the network, thereby preventing potential data loss and unnecessary power consumption.

# 4	References

3GPP RAN2 #128 Meeting: R2-128 NR-NTN-IoT-NTN (Sergio)_ EOM

RP-243278 - WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

3GPP TR 38.864 V18.1.0 (2023-03) Study on network energy savings for NR (Release 18)

3GPP TR 38.811 V15.4.0 (2020-09) Study on New Radio (NR) to support non-terrestrial networks (Release 15)

3GPP TR 38.821 V16.2.0 (2023-03) Solutions for NR to support non-terrestrial networks (NTN)  (Release 16)

# 5	Annex – Power Saving

TR 38.864 [3] states that paging resources are grouped compactly, so the potential specification impact of improvements in paging transmission includes the paging reception procedure. This mechanism allows a base station node to page. To save energy, paging can be done in certain SSBs instead of all the SSBs within a terrestrial cell. For active DL transmission, the BS power consumption is provided by

- PDL=PstaticDL+PdynamicDL

where

PstaticDL is a static part of the power for the base station in active, which is not scaled based on reference configurations?

PdynamicDL is a dynamic part of the power for the active base station, which is scaled based on a reference configuration.

For active UL transmission, the BS power consumption is provided by

PUL=PstaticUL+PdynamicUL

In this section, a comparable power calculation is conducted in a non-terrestrial network to assess the energy savings of the paging channel in Proposals 4 and 5. The link level [4] and system level [5] parameters are utilised below. NTN deployment scenarios and related system parameters such as architecture, altitude, orbit were chosen based on the previous decision in 3GPP RAN2. The following table shows the radio frequency characteristics of the IoT device used in simulations:

| Characteristics | VSAT (Note 2) | Handheld | Other (Note 1) |
| --- | --- | --- | --- |
| Frequency band | Ka band(i.e. 30 GHz UL and 20 GHz DL) | S band (i.e. 2 GHz) | Ka band(i.e. 30 GHz UL and 20 GHz DL) |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter | (1, 1, 2) with omni-directional antenna element | Directional (M,N,P,Mg,Ng) = (TBD,TBD,2,1,1); (dV,dH) = (TBD, TBD)λ with directional antenna element (HPBW=65 deg) |
| Polarisation | circular | Linear: +/-45°X-pol | Linear: +/-45°X-pol |
| Rx Antenna gain | 39.7 dBi | 0 dBi per element | TBD dBi per element |
| Antenna temperature | 150 K | 290 K | TBD K |
| Noise figure | 1.2 dB | 7 dB | TBD dB |
| Tx transmit power | 2 W (33 dBm) | 200 mW (23 dBm) | [TBD W (TBD dBm)] |
| Tx antenna gain | 43.2 dBi | 0 dBi per element | TBD dBi per element |
| NOTE 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices. These values are provided for information. NOTE 2: VSAT terminal characteristics could be implemented with phased array antenna |  |  |  |

The handheld is used for the simulation. 5G system elements mapping in NTN architecture is shown at below.

| 5G elements - NTN elements mapping |  |  |  |
| --- | --- | --- | --- |
| NTN architecture options | NTN Terminal | Space or HAPS | NTN Gateway |
| A1: access network serving UEs via bentpipe satellite/aerial | UE | Remote Radio Head (Bent pipe relay of Uu radio interface signals) | gNB |
| A2: access network serving UEs with gNB on board satellite/aerial | UE | gNB or Relay Node functions | Router interfacing to Core network |
| A3: access network serving Relay Nodes via bent pipe satellite/aerial | Relay Node | Remote Radio Head (Bent pipe relay of Uu radio interface signals) | gNB |
| A4: access network serving Relay Nodes with gNB on board satellite/aerial | Relay Node | gNB or Relay Node functions | Router interfacing to Core network |

The following table shows the satellite parameters are considered for the system level simulator. LEO-600 is prioritised for the sake of simplicity.

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | S-band (i.e. 2 GHz) | 22 m | 2 m | 2 m |
| Satellite EIRP density |  | 59 dBW/MHz | 40 dBW/MHz | 34 dBW/MHz |
| Satellite Tx max Gain |  | 51 dBi | 30 dBi | 30 dBi |
| 3dB beamwidth |  | 0.4011 deg | 4.4127 deg | 4.4127 deg |
| Satellite beam diameter (Note 2) |  | 250 km | 90 km | 50 km |
| Equivalent satellite antenna aperture (Note 1) | Ka-band (i.e. 20 GHz for DL) | 5 m | 0.5 m | 0.5 m |
| Satellite EIRP density |  | 40 dBW/MHz | 10 dBW/MHz | 4 dBW/MHz |
| Satellite Tx max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| 3dB beamwidth |  | 0.1765 deg | 1.7647 deg | 1.7647 deg |
| Satellite beam diameter (Note 2) |  | 110 km | 40 km | 20 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | S-band (i.e. 2 GHz) | 22 m | 2 m | 2 m |
| G/T |  | 19 dB K-1 | 1.1 dB K-1 | 1.1 dB K-1 |
| Satellite Rx max Gain |  | 51 dBi | 30 dBi | 30 dBi |
| Equivalent satellite antenna aperture (Note1) | Ka-band (i.e. 30 GHz for UL) | 3.33 m | 0.33 m | 0.33 m |
| G/T |  | 28 dB K-1 | 13 dB K-1 | 13 dB K-1 |
| Satellite RX max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |  |

The beam layout parameters captured in the following table are adopted for single satellite simulations.

| Scenario | Scenario A | Scenario C2/D2 |
| --- | --- | --- |
| Carrier frequency | S-band: 2 GHz Ka-band: 20 GHz for DL | S-band: 2 GHz Ka-band: 20 GHz for DL |
| Adjacent beam spacing (ABS) on UV plane | S-band: Set 1: ABS = 0.0061 Set 2:ABS = 0.0111 Ka-band: Set 1: ABS = 0.0027 Set 2: ABS = 0.0067 | S-band: Set 1: ABS = 0.0668 Set 2: ABS = 0.1334 Ka-band: Set 1: ABS = 0.0267 Set 2: ABS = 0.0667 |
| Satellite location | Any position on the geostationary orbit | Any position on the LEO orbit |
| Central beam center elevation angle target | Baseline: 45 deg | Baseline: 90 deg |
| Central beam bore sight direction coordinates in UV plane | Baseline: (0.107,0) | Baseline: (0,0) |
| Gateway direction coordinates in UV plane | Baseline: Same as central beam bore sight direction coordinates in UV plane Note: Not needed for calibration |  |

The base station, gNB, energy savings via adapting paging transmission patterns based on the mobile terminating data is evaluated at below.

| Energy Saving | NTN Scenario | UE | Load Scenario | Energy Saving |
| --- | --- | --- | --- | --- |
| Enhanced paging by decreasing the number of paging when there is no mobile terminating data (50%) | LEO-600 | Handheld | Paging Load 3.6% | 12.30% |
| Enhanced paging by decreasing the number of paging when there is no mobile terminating data (50%) | LEO-600 | Handheld | Paging Load 2.0% | 8.20% |