3GPP TSG-RAN WG2 #129					      		 R2-2500933

Athens, Greece – February 17 - 21, 2025		                   Revision of R2-2410570

Agenda item:	8.13.2

Source: 	Kyocera

Title: 	Discovery and (re)selection under multihop relay

Document for:	Discussion

# Introduction

In this contribution, we consider the some of the key issues to address for relay discovery and (re)selection under U2N multihop relay.

# Discussion

During RAN2#128, the following agreements were reached on discovery and relay (re)selection:

Agreements:

A lower bound of Uu RSRP is not required for the UE to operate as an intermediate Relay UE.

If a UE can operate as a last relay UE according to the restrictions configured by the network, it does not operate as an intermediate relay UE towards the same cell for any remote UE (i.e., minimise number of hops when possible).  FFS if operation as an intermediate relay UE towards a different cell would be supported.

The network can configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE.  If the upper bound is not configured, there is no threshold, but this does not override the previous agreement.  FFS if there would be additional restrictions in the case of no upper bound.

The following cases are supported for L2 multihop relay:

- One last Relay UE can have two connections with one intermediate Relay UE and one Remote UE (the intermediate Relay UE and Remote UE are physically different UE).

- Two physically different Remote UE(s) can have each indirect path via the same intermediate Relay UE and the same last Relay UE.

FFS if the last relay UE can use the same L2ID for both of the connections in either case.

Cases with two indirect paths to the gNB for the same remote UE are not supported.

An Intermediate Relay UE can serve multiple multi-hop indirect paths of different Remote UEs.

If the intermediate Relay UE also is acting as a Remote UE, it cannot support different indirect paths to the gNB with same/different last/U2N/parent intermediate Relay UE(s) based on different PC5 unicast links.

## 2.1.	Discovery

Based on the discussions in RAN2#128, it was considered whether discovery announcement message can be forwarded at the intermediate Relay UE when the SD-RSRP or SL-RSRP is above a configured threshold.  However, there were also discussions on whether the thresholds are needed if the intermediate Relay UE is PC5 connected to its parent node (e.g., last Relay UE). Based on SA2’s LS response [1], it is now clear that for Model A discovery, the intermediate Relay UE needs to be PC5-S connected to its parent node, although it isn’t required for Model B discovery.

- Observation 	Candidate relay UEs in all the hops is assumed to be PC5-S connected prior to Model A discovery announcements.

We assume the reason for the need for the PC5 connection is to allow for better reliability in serving remote UEs, which isn’t different in principle to the need for requiring SD-RSRP or SL-RSRP to be above a configured threshold before discovery announcement.  We think the following options should be considered in RAN2 for whether the intermediate Relay UE is allowed to forward Model A discovery announcement.

- Only the PC5-S connection is needed as required by SA2.

- In addition to the PC5-S connection requirement, apply the AS threshold requirement with SD-RSRP/SL-RSRP.

- In addition to the PC5-S connection requirement, apply the AS threshold requirement as part of the PC5-RRC link monitoring with SD-RSRP/SL-RSRP.

With Option 1, we can simplify AS requirements for Model A discovery announcement by relying on the combination of DCR/DCA messages as a gauge for reachability.

With Option 2, the gNB has further control for the sidelink connection on top of PC5-S connection.

With Option 3, it is assumed that once the PC5-S is connected it is also assumed that PC5-RRC connection is established, so the reliability of the sidelink connection can be controlled through the existing PC5-RRC connection. Furthermore, to support service continuity in Scenarios C & D (i.e., path switch towards multihop relay indirect path) in Approach 1, it is required that the candidate relay UE(s) should already be in RRC Connected.

- Proposal 1	RAN2 should discuss whether additional AS threshold requirements are applicable for intermediate Relay UEs to announce discovery.

RAN2 has already agreed in the last meeting that:

The network can configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE.  If the upper bound is not configured, there is no threshold, but this does not override the previous agreement.  FFS if there would be additional restrictions in the case of no upper bound.

With respect to the FFS for the case of no upper bound. We assume if the NW has no restriction for where in coverage the intermediate relay UE can transmit discovery, then it’s reasonable to assume the NW would also not configure the last Relay UE with lower bound and/or upper bound thresholds.  However, it’s certainly possible that the threshold(s) for the last Relay UE is provided for the single-hop Relay UE from Rel-17. Under this circumstance, there’s the possibility that the intermediate Relay UE, without the upper bound threshold configured, would transmit discovery closer to the cell than even the last Relay UE, which is unreasonable.  Therefore, there should be a restriction for the intermediate Relay UE with no upper bound threshold configuration to use the upper bound threshold configured for the last Relay UE (if one is configured) as the default upper bound threshold value.

- Proposal 2	RAN2 should discuss whether additional AS threshold requirements are applicable for intermediate Relay UEs to announce discovery.

Another issue that requires further discussion is whether the intermediate Relay UE may operate in a cell different from the last Relay UE.

A lower bound of Uu RSRP is not required for the UE to operate as an intermediate Relay UE.

If a UE can operate as a last relay UE according to the restrictions configured by the network, it does not operate as an intermediate relay UE towards the same cell for any remote UE (i.e., minimise number of hops when possible).  FFS if operation as an intermediate relay UE towards a different cell would be supported.

In our view, there is no particular reason not to support this scenario. As some companies mentioned, there is the case when the intermediate relay camps in a cell that does not support relay operation, while it is still needed an additional hop to reach a remote UE that isn’t directly reachable by the last Relay UE.  However, even if both cells support relay operation, there’s no specific incentive to restrict intermediate Relay UE’s operation under this circumstance.  In case discovery resources are also configured in the intermediate Relay UE’s camped cell, the intermediate Relay UE should use the configured resources rather than the pre-configured resources for discovery transmissions.

- Proposal 3	Intermediate Relay UE’s operation in a cell different from the last Relay UE is allowed.

Currently it has already agreed that a candidate last relay UE for discovery can be in any RRC state, but it should also be decided whether the intermediate Relay UE can be in any RRC state for discovery operation, esp. when the intermediate Relay UE is operating in-coverage but in a different cell than that of the last Relay UE. If the intermediate relay UE is RRC connected to its cell while it is also PC5 connected to a last Relay UE on a different cell such a combination would result in complicated SL configurations between two cells to support a remote UE. Therefore, it’s preferable that the intermediate relay UE is in RRC Connected, it should always be connected to the same cell as the last Relay UE.

- Proposal 4	If an intermediate Relay UE is forwarding discovery announcement while in RRC connected, it must be RRC connected to the same cell as the last Relay UE.

## 2.2	Relay (re)selection

For relay selection and reselection, during the email discussion [2], many companies think some of the legacy rules for the remote UE to trigger selection and reselection as below:

[Relay (re)selection triggering conditions for Remote UE]

- If the Remote UE has no serving cell;

- If the Remote UE does not have a selected intermediate Relay UE;

- If the SL-RSRP of the currently selection intermediate Relay UE is below a configured threshold;

- If the SD-RSRP of the currently selected intermediate Relay UE is below a configured threshold;

- If the upper layer indicates not to use the currently selected intermediate Relay UE;

- If the upper layer of the selected intermediate Relay UE requests the release of the PC5-RRC connection;

- If the RLF is detected on the PC5-RRC connection with the current intermediate Relay UE;

- If the Remote UE receives a notification message from the intermediate Relay UE caused by one of the following:

h-1) if intermediate Relay UE detects PC5 RLF between intermediate Relay UE and last Relay UE (or serving intermediate Relay UE)

h-2) if intermediate Relay UE receives RRCReconfiguration message for HO (if the intermediate Relay UE is in RRC_CONNECTED)

h-3) if intermediate Relay UE performs cell reselection

h-4) if intermediate Relay UE fails Uu RRC connection establishment/Resume via last Relay UE

h-5) if intermediate Relay UE receives PC5-RRC connection release between intermediate Relay UE and last Relay UE (or serving intermediate Relay UE)

In particular, a), b), c), d), f) seem to be agreeable by most companies, while most companies also think the above conditions are not used by intermediate relay UEs to trigger relay reselection. So, there remains the issue of under what condition(s) will it be appropriate for the intermediate relay UE to perform relay reselection. It is also necessary to consider if and when the intermediate relay UE should send notification message to the remote UE as described in h) above. In our view, the intermediate relay UE’s trigger conditions for relay reselection should be discussed together with the notification message delivered to the remote UE.

The following options for trigger conditions and notification delivery should be considered:

- The intermediate relay UE initiates relay reselection using the same triggering conditions adopted for the remote UE.  The intermediate relay UE does not send notification message to the remote UE if a suitable candidate relay UE (e.g., last relay UE) is selected.

- The intermediate relay UE initiates relay reselection using the same triggering conditions adopted for the remote UE.  The intermediate relay UE sends the notification message (w/ failure condition) to the remote UE, including any candidate relay UE(s) along with measured quantities (e.g., SL-RSRP/SD-RSRP, number of hops).

- The intermediate relay UE sends a notification message to the remote UE and it’s up to the remote UE to perform relay reselection. The intermediate relay UE is allowed to initiate discovery transmissions after PC5 connection to the remote UE is released.

- The intermediate relay UE does not initiate relay reselection upon detecting a failure condition, but it sends a notification message to the remote UE.  If needed, the remote UE should be allowed to request the intermediate relay UE to perform relay reselection (e.g., if the remote UE cannot find candidate relay UEs on its own).

With Option 1, the intermediate relay UE may act as a remote UE in terms of relay reselection and if a candidate relay UE from the same cell meets the SL-RSRP/SD-RSRP threshold condition(s) and the upper layer criteria is satisfied, the path re-routing can be done seamlessly without remote UE involvement during the relay reselection process. The intermediate relay UE will send a notification message to the remote UE only if a suitable candidate relay UE cannot be selected.

With Option 2, the intermediate relay UE reports to the remote UE, via the notification message, candidate relay UE(s) that it found. Upon receiving the notification message, the remote UE is allowed to trigger relay reselection on its own and decide whether to select a new intermediate relay UE, a last relay UE or to stay with the existing intermediate relay UE.

With Option 3, the intermediate relay UE sends a notification message to the remote UE (w/ failure condition) which triggers the remote UE to perform relay reselection.  It is assumed that the intermediate relay UE is allowed to find a parent relay UE for discovery transmission once the PC5 connection with the remote UE is released.  As in the legacy procedure, the remote UE is allowed to select any of the available Intermediate relay UE that satisfies its criteria for relay reselection.

Option 4 is similar to Option 3, with the exception that the remote UE is allowed to request the intermediate relay UE to perform relay reselection if the remote UE cannot find a candidate relay UE meeting the necessary criteria.

Among the 4 options, Options 1, 2 and 4 have the benefit that intermediate relay UE is allowed to perform relay reselection, albeit under different triggering conditions.

- Proposal 5	Intermediate relay UEs should be allowed to perform relay reselection upon failure detection and whether it should send notification message to the remote UE.  It is FFS what triggers the intermediate relay UE to perform relay reselection.

# Conclusion

In this contribution, we addressed the issues that may arise with multihop relay discovery and relay (re)selection.  RAN2 is kindly asked to take into account the observation and proposals below:

- Observation 	Candidate relay UEs in all the hops is assumed to be PC5-S connected prior to Model A discovery announcements.

- Proposal 1	RAN2 should discuss whether additional AS threshold requirements are applicable for intermediate Relay UEs to announce discovery.

- Proposal 2	RAN2 should discuss whether additional AS threshold requirements are applicable for intermediate Relay UEs to announce discovery.

- Proposal 3	Intermediate Relay UE’s operation in a cell different from the last Relay UE is allowed.

- Proposal 4	If an intermediate Relay UE is forwarding discovery announcement while in RRC connected, it must be RRC connected to the same cell as the last Relay UE.

- Proposal 5	Intermediate relay UEs should be allowed to perform relay reselection upon failure detection and whether it should send notification message to the remote UE.  It is FFS what triggers the intermediate relay UE to perform relay reselection.

# References

- S2-2501334, “Reply LS on relay discovery announcement”, LGE

- R2-24xxxxx, “Report of [Post127][401][Relay] Multi-hop relay discovery and (re)selection” LGE