3GPP TSG-RAN WG2 #129					      		 R2-2500934

Athens, Greece – February 17 - 21, 2025		                   Revision of R2-2410569

Agenda item:	8.13.3

Source: 	Kyocera

Title: 	Control Plane under multihop L2 U2N relaying

Document for:	Discussion

# Introduction

In this contribution, we consider the some of the key issues for control plane under L2 U2N multihop relaying.

# Discussion

Based on the discussions in RAN2#128, the following agreements have been reached:

Agreements:

In one approach (“approach 1”) of U2N relays, each of the Intermediate Relay UEs must be in RRC_CONNECTED when the U2N remote UE is in RRC_CONNECTED.  Connection establishment in the U2N remote UE first requires that each Intermediate Relay UE which is in RRC_IDLE/RRC_INACTIVE first enters RRC_CONNECTED.  FFS whether connection establishment of an Intermediate Relay UE (other than the Last Relay UE) is captured in specification as connection establishment of a remote UE or a relay UE.  FFS if SRB0 would be forwarded before connection establishment of each intermediate relay UE.

The figure and description under P1 of R2-2410006 serves as a baseline connection establishment procedure for discussion for multi-hop U2N Relays if Approach 1 (all relay UEs must be in RRC_CONNECTED when the remote UE is in RRC_CONNECTED) is adopted.

In one approach (“approach 2”) of U2N relays, Intermediate Relay UEs (other than the Last Relay UE) can be in any RRC state when the U2N remote UE is in RRC_CONNECTED.

In approach 2, any intermediate relay UE which happens to be in RRC_CONNECTED towards the last relay UE’s serving gNB and is operating as a remote UE is assumed to obtain its relaying configuration (RLC channel configuration, SRAP configuration, etc.) in dedicated signalling from the last relay UE’s serving gNB via the U2N connection.

The figure and description under P4 of R2-2410006 serves as a baseline connection establishment procedure for discussion for multi-hop U2N Relays if Approach 2 (relays other than the Last Relay may/may not remain in RRC_IDLE/RRC_INACTIVE when the remote UE is in RRC_CONNECTED) is adopted.

For the baseline procedure, the PDB for each hop is provided by the network according to gNB implementation.   This does not preclude the possibility of assistance information from the UE(s).

If enhancements allow some intermediate relay UEs to remain in idle/inactive, the PDB split between the Uu hop and all remaining hops is performed by the network.  FFS how to split the PDB over each of the individual remaining hops.

In multi-hop, the U2N Remote UE acquires the SI of the cell of the Last Relay UE, which is forwarded via the Intermediate Relay UE(s).  FFS how to perform the forwarding and whether an intermediate relay UE can forward available SI directly (rather than retrieving it from the last relay UE).

Support a baseline procedure for the case in which the intermediate relay UEs for a remote UE all transition to RRC_CONNECTED (if not already there) when the remote UE goes to RRC_CONNECTED.

1. Support this case based on existing U2N framework, with all the UEs RRC_CONNECTED to the last relay UE’s serving cell.

2. Continue to discuss whether/how to support the case that intermediate relay UEs do not all move to RRC_CONNECTED when the remote UE triggers connection establishment

All agreements in this WI apply to both these cases unless otherwise specified.

Furthermore, a CP email discussion [1] has raised many issues which covers connection establishment, SIB and Paging handling, some of which are discussed below. However, other issues such as failure handling and connection reestablishment should also be addressed and also discussed here.

2.1.	Connection establishment

During the RAN2#128 meeting, the following agreement regarding RRC Setup Request is reached, including the highlighted FFS that should be considered.

In one approach (“approach 1”) of U2N relays, each of the Intermediate Relay UEs must be in RRC_CONNECTED when the U2N remote UE is in RRC_CONNECTED.  Connection establishment in the U2N remote UE first requires that each Intermediate Relay UE which is in RRC_IDLE/RRC_INACTIVE first enters RRC_CONNECTED.  FFS whether connection establishment of an Intermediate Relay UE (other than the Last Relay UE) is captured in specification as connection establishment of a remote UE or a relay UE.  FFS if SRB0 would be forwarded before connection establishment of each intermediate relay UE.

In our understanding the FFS issue is to consider whether the intermediate relay UE should wait for the last relay UE (in IDLE/INACTIVE) to transition to RRC CONNECTED before forwarding the remote UE’s RRC Setup Request (in addition to the intermediate relay UE’s own RRC Setup Request).  This means the Intermediate relay UE cannot forward the remote UE’s RRC Setup Request until it is RRC Connected to the gNB.  This introduces unnecessary delay and signalling overhead for all UEs to transition to RRC Connected, esp. with increased number of relay hops.  Additionally, the agreement above only applies to Approach 1. With Approach 2, the Intermediate relay UEs are not required to transition to RRC Connected while the remote UE still needs to be in RRC Connected.  Therefore, with Approach 2 the Intermediate Relay UE(s) can forward the remote UE’s RRC Setup Request without RRC Connection of its own which would result with lower latency in connection establishment compared to Approach 1. Figure 1 provides a flow diagram of the reduced latency in connection establishment for Approach 1.  In this example, the Intermediate relay UE forwards the remote UE’s RRC Setup Request along with its own RRC Setup Request in the same message (step 4). Note that it is still necessary for the Last relay UE to complete the RRC connection over the Uu link before forwarding the RRC Setup Requests from the remote UE and the Intermediate relay UE.

Figure 1: RRC Setup procedure with multihop relay UEs.

According to the baseline agreement, once the relay UEs are selected for the desired path towards the NW, the PC5 connections for all the sidelink hops must also be established.

- Proposal 1	The intermediate relay UE should be allowed to forward the remote UE’s RRC Setup Request in case while it is still in RRC IDLE/INACTIVE.

Currently, SA2 has updated the link modification procedure [2], whereby: “The link modification procedure is enhanced to inform the parent relay, e.g. another 5G ProSe Intermediate UE-to-Network Relay or the 5G ProSe UE-to-Network Relay, of the User Info of the child relay(s) or the Remote UE(s).” However, this information is not available to the gNB.  During the email discussion [1], one of the issues that was addressed as part of connection establishment is the issue of how the SidelinkUEInformation (SUI) is sent to the gNB to request for the dedicated configurations required to support the multi-hop relay operation with its PC5-RRC connected UE.  Majority of companies preferred that each of the relay UEs and remote UE send their own respective SUI to the gNB.  The baseline assumption is that the legacy SUI should be reused for multihop relay.  With legacy SUI sent by the first relay UE, the gNB can provide the dedicated configuration for the PC5 link between the remote UE and first relay UE; however, for the SUI from any additional intermediate relay UE(s) and the last relay UE, the gNB would need to be able to associate the all SUIs with the path for the remote UE. The individual SUI should be able to identify the remote UE as well as all the associated relay UEs.  Assuming each of the SUIs are sent by including the remote UE’s L2ID, the gNB should be able figure out all the associated relay UEs that forms the path for the remote UE to the gNB.  This would also be used for the gNB to configure a Local ID for all the associated UEs on the path.

Alternatively, if the last relay UE has information of all the relay UEs associated with the path to the remote UE (e.g., based on the link modification procedure), the last relay UE may provide the information to the gNB.

- Proposal 2	RAN2 should consider how the gNB should be informed of the associated relay UEs for the remote UE’s path to the gNB.

2.1.2	Failure condition handling

The failure conditions for multihop relay should follow the same principles and failure reporting as for the legacy U2N failure conditions.  Currently, the NotificationMessageSidelink message already allows for certain indications to be sent from the relay UE to the remote UE.  These include the cases for Uu RLF, relay UE HO, relay UE cell reselection, RRC failure, including establishment and resume failures. Also, for U2U relay, it was further added that indication of SL-RLF may also be indicated, which is particularly useful for the multihop relay UE scenario which wasn’t an issue with the legacy U2N relay scenario.  Additionally, the SidelinkUEInformationNR message allows the U2N relay UE to report the SL-SRLF to the gNB.

- Observation 	The existing indications in the failure NotificationMessageSidelink message and SidelinkUEInformationNR message can be largely reused for the multihop relay scenario.

For the multihop relay scenario, although there’s only one Uu interface, there are multiple PC5 hops, which depends on the number of relay UE in the indirect path.  Therefore, when the U2N relay UE detects SL-RLF or is notified of the SL-RLF from a PC5 connected intermediate relay UE, it should inform the gNB not only of a SL-RLF, but also which relay UE detected the SL-RLF.  This will prevent the gNB from releasing a relay UE that still has good connection to the gNB and continue to provide relay service to other remote UEs.

- Proposal 3	U2N relay UE should inform the gNB of the intermediate relay UE that detected the SL-RLF.

The same condition may also be considered for the indication sent to the remote UE.  In case the NotificationMessageSidelink message is sent to the remote UE, the relay UE should also include in the message the relay UE that was the cause for the indication which would be helpful for the remote UE to avoid selecting the relay UE during relay reselection.

- Proposal 4	The remote UE should be informed via the NotificationMessageSidelink message which relay UE is the cause for the indication.

2.1.2	RRC Connection Re-establishment

For multihop relay scenario, the baseline assumption should be similar to the legacy U2N case, i.e., upon the detection of SL-RLF, the U2N Remote UE may perform relay reselection and perform RRC reestablishment. However, with multihop relay, it is still undecided which UE should perform RRC reestablishment as point out in A1.6 in the email discussion [1]. Particularly, it should be decided whether the intermediate relay UE or the remote UE or both UEs should perform RRC reestablishment, since in Rel-17 it was already assumed that the U2N relay UE does not initiate RRC reestablishment.  To avoid complexity at the NW for handling “duplicated” RRC reestablishment requests, we should not allow multiple UEs to initiate RRC reestablishment simultaneously.  Then it should be considered if the intermediate relay UE or the remote UE should perform the RRC reestablishment.   We consider two options for which UE should perform RRC reestablishment due to Uu RLF or for the case when there are multiple intermediate Relay UEs, it also includes SL-RLF beyond the PC5 connection between the remote UE and the first relay UE.

- The remote UE will initiate RRC reestablishment.  The relay UEs should inform the remote UE of the failure via notification messages.

- The intermediate UE will initiate RRC reestablishment upon receiving notification message from the last relay UE due to Uu RLF. The intermediate relay UE does not send notification message to the remote UE until the reestablishment procedure is completed (either successful or failure to reestablish).

With Option a), the RRC reestablishment procedure will remain a responsibility of the remote UE, which should result in less specification impact.

With Option b), since the RRC reestablishment is initiated at the intermediate relay UE, it is likely closer to the cell and should have a better chance of successful reestablishment. Furthermore, in case multiple remote UEs are connected to the intermediate relay UE, the intermediate relay UE’s reestablishment would benefit all the PC5 connected remote UEs. In case the RRC reestablishment is unsuccessful, the intermediate relay UE should send a notification message to the remote UE, which would trigger the remote UE to perform relay reselection and RRC reestablishment.

- Proposal 5	RAN2 should decide whether the remote UE or the intermediate relay UE should initiate RRC reestablishment upon Uu RLF.

# Conclusion

In this contribution, we addressed the issues that may arise with multihop relay on the control plane.  RAN2 is kindly asked to take into account the observation and proposals below:

- Proposal 1	The intermediate relay UE should be allowed to forward the remote UE’s RRC Setup Request in case while it is still in RRC IDLE/INACTIVE.

- Proposal 2	RAN2 should consider how the gNB should be informed of the associated relay UEs for the remote UE’s path to the gNB.

- Observation 	The existing indications in the failure NotificationMessageSidelink message and SidelinkUEInformationNR message can be largely reused for the multihop relay scenario.

- Proposal 3	U2N relay UE should inform the gNB of the intermediate relay UE that detected the SL-RLF.

- Proposal 4	The remote UE should be informed via the NotificationMessageSidelink message which relay UE is the cause for the indication.

- Proposal 5	RAN2 should decide whether the remote UE or the intermediate relay UE should initiate RRC reestablishment upon Uu RLF.

# References

- R2-24xxxxx, “Report of [POST128][401][Relay] Control Plane Baseline Solution”, Interdigital

- TS 23.304, “Proximity based Services (ProSe) in the 5G System (5GS)”.