# 3GPP TSG-RAN2 Meeting #129	R2-2500954

# Athens, Greece, Feb 17-21, 2025

Agenda Item:	8.2.2

Work Item:	Ambient_IoT_Solutions

Source: 	Qualcomm Incorporated

Title: 	Ambient IoT Paging

Document for:	Discussion/Decision

# Introduction

RAN#106 approved WI for Ambient IoT in [1]. One of the objectives for RAN2 is listed as follows:

- Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes.

This document provides details about paging for Ambient IoT.

# Paging Message Design and Content

## Device identifier in paging message

The WID objective indicates the following:

Support the options that a paging message contains one identifier, and that a paging message contains no identifier.

RAN2 has previously discussed and agreed that ‘no identifier’ is equivalent to ‘paging all devices’ within the range of the reader. Now, some clarification is needed on what ‘one identifier’ means. It could mean one identifier which can address one unique device, and it could as well be a ‘group identifier’ or a ‘filter criteria’ which can address a group of devices. In our understanding, the intent of ‘one identifier' is to include such one group identifier or filter criteria.

A-IoT paging message containing one group identifier or filter criteria is within the scope of the WI.

Support A-IoT paging message that contains one ‘group’ identifier or ‘filter criteria’ in Rel-19.

## Msg1 resource indication and subsequent R2D/Paging message

RAN2 previously agreed in RAN2#126:

2. AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message).  FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion.

Above agreement was reached after discussion in RAN2 which indicates that there is no need of separate message to indicate resources for D2R response. RAN2 also agreed to support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages associated with the same request from the CN.

In our view, how this is achieved is up to the reader implementation and deployment. Furthermore, in terms of design and aiming at reducing complexity, we think the subsequent AIoT trigger/paging message would be no different from the initial trigger/paging message. Of course, what is actually signalled in the initial trigger vs the subsequent trigger can be different.

In terms of design, subsequent trigger/paging message is not different from initial trigger/paging message.

Design of initial trigger message is reused for subsequent trigger message, i.e. a separate message is not introduced for subsequent trigger/paging message.

There were also proposals to introduce new messages indicating access occasion announcement. We do not see a need of such new messages. In our view, the resource allocation including whether there is one or multiple resource opportunities should be indicated by the initial trigger/paging message.

The resource allocation including whether there is one or multiple resource opportunities should be indicated by the initial trigger/paging message.

One may argue introducing a new/separate type of message, such as a Query-like or QueryRep-like message, enables selecting a subset of devices for each round of communication. See for example below figure from [4].

Figure from [4]

However, in our view, with the possibility to optionally include the fields in the paging message itself (i.e. the initial Trigger Type 1 in above figure) such as group ID, filter criteria, resource allocation for msg1, it is possible for the reader to achieve the same behaviour without explicitly defining a new Query-like or QueryRep-like message(s) (such as Trigger type 2 or type 3). This is explained in our companion paper [5] in more detail, while a figure is shown below for quick reference.

Figure from [5]: A-IoT random access triggered by A-IoT paging

Similar behaviour like Query-like or QueryRep-like can be achieved by using a single A-IoT paging message format by defining optional fields for information such as group ID, filter criteria, resource allocation for msg1 etc.

Separate ‘access occasion announcement message’ (e.g. Query or QueryRep) is not needed.

## Indication about subsequent R2D message

RAN2#126 discussed the so-called ‘Msg4’. RAN2 agreed to not use the term ‘msg4’ for further discussion of the AIoT random access. Instead, it could be called subsequent R2D message after D2R transmission. Such message may be a failure indication for Msg3 transmission or could be the acknowledgement like the Msg4 of NR random access.

RAN2#128 agreed the following:

Whether we include further information in paging message to indicate service type/command type or to indicate that further subsequent messages are coming can be discussed in WI phase once we understand more the paging design and AS ID.

While some scenarios would need the subsequent R2D message (e.g. for acknowledging the reception of the AIoT device ID and/or AIoT upper layer data sent in the previous D2R message or for triggering the next round of AIoT re-access or AIoT data communication), in other scenarios there would be no such subsequent R2D message (e.g. Reader requesting information from temperature sensors and no intention to ACK/NACK successful reception).

It is beneficial for AIoT device to have correct expectation on potential future steps after transmitting a D2R communication in response to the initial trigger/AIoT paging. To facilitate the AIoT device actions, the initial trigger message can indicate to the AIoT device whether a subsequent R2D message would be present.

The initial trigger message indicates the presence and information carried in the subsequent R2D message after D2R transmission.

## RA type indication in the paging message

Since AIoT access is triggered by a Reader, the Reader is responsible for providing the access resource considering the different service types, i.e., inventory request or command message. Reader may have preference on which type of AIoT access to be triggered to respond to the service request.

For example, if Reader would like to only issue the ‘Write’ command to a small number of AIoT devices, a 2-step AIoT access can be selected for AIoT devices to feed back the simple confirmation after the command execution. In another example, Reader may indicate AIoT devices to perform so called ‘3-step’ AIoT access for the case that a large number of AIoT devices perform AIoT access by using the contention-based resource. Therefore, Reader should be able to indicate the type of AIoT access to AIoT devices in the trigger message.

Reader explicitly indicates the type of AIoT access (e.g., 2-step, 3-step) to AIoT devices in the initial trigger message (aka AIoT paging).

  - Indication of Device ID in D2R

Suppose the AIoT application has some idea about the AIoT devices in a given area (e.g. based on deployment information within a warehouse) but does not have most up to date inventory information. In such case, the application may want to initiate a procedure which involves inventorying all the devices that satisfy certain criteria (e.g. all devices which are temperature sensors and belonging to booth # X). One might see the benefit of having ability to send a single message in the R2D link to indicate to the sensors (satisfying the criteria) to reply with their temperature values in the D2R. In such scenario, the device ID may be absent in the D2R message, which means including the device ID in D2R response should be optional. When to include it and when to leave it out should be indicated by the Reader in the initial trigger.

However, there may be some challenges to this. Firstly, it depends on how many devices would satisfy the criteria. It would be difficult if the reader does not have a good idea without prior inventory. However, in some use cases based on deployment information, the reader might already know how many devices may possibly respond to the initial trigger. Depending on the number of AIoT devices that need to respond, the reader can estimate the resources needed for the D2R link, e.g. whether the devices have to go through contention based random access procedure or whether they can be provided with contention free resources. In case of contention-based RA, the amount of potential collisions also depends on the potential number of devices that get triggered.

In some scenarios, the device ID may be absent in the D2R message in response to the A-IoT paging. When to include it and when to leave it out should be indicated by the Reader in the A-IoT paging message.

The A-IoT Paging Message indicates whether AIoT Device ID needs to be included in the D2R  message.

## Multiple nearby readers and the reader selection

There may be more than one Reader deployed in a particular area in a deployment. The readers can be nearby. In such case, there can be A-IoT devices within the coverage of multiple readers. Without proper coordination between the readers, in such case, it is possible that both the Readers use the same/overlapping radio resources (in time and frequency). The Reader 1 may allocate a certain resource to AIoT Tag 1, whereas Reader 2 may allocate the same resource to AIoT Tag 2. When Tag1 which is within the coverage of both readers respond, Reader 2 may mistakenly assume that Tag2 has responded.

Figure 1 A-IoT devices within the coverage of multiple Readers.

When multiple readers are nearby, there is potential for R2D message intended for one reader interfering with other reader.

A properly defined coordination between the Readers would be beneficial for all topologies. However, to minimize the specification efforts especially for Topology 1 in this release, RAN2 can assume that the reader coordination and awareness of presence of other readers is left to deployment and network implementation unless other WGs are eager to specify it.

To minimize the specification efforts for Topology 1 in this release, RAN2 can assume that the reader coordination and awareness of presence of other readers is left to other WGs or NW deployment/implementation.

RAN2 assumes that the reader coordination and awareness of presence of other readers is left to other WGs or NW deployment/implementation.

However, even with such implementation-based coordination, one reader may not be aware that a particular device is within the coverage of another reader. And the potential confusion described above is still possible if the R2D message is not unambiguously attributable to the intended device by the Reader. Therefore, in case of dedicated communication, simply including a Random ID in the R2D message is neither beneficial, nor sufficient.

In case of dedicated communication, an R2D message from a device needs to be unambiguously attributable to the particular device by the Reader, and as such simply including Random ID in R2D message is neither beneficial nor sufficient.

In case of dedicated communication, an R2D message from a device shall be unambiguously attributable to the particular device by the Reader.

In terms of interaction on the NW side, each Reader that received the inventory request from AIOT Function (AIOTF) should trigger the inventory procedure to discover or trigger AIoT devices to perform AIoT access procedure. Considering this, RAN2#128 agreed the following:

- 2. Capture the scenario that different readers may page the same device for same service request.  We will discuss this in WI phase (if in scope of WI) and consider progress from all the WGs

In our view, the AIoTF can indicate the last known reader to initiate AIoT command towards the AIoT device(s). However, the last known reader may not be able to reach the AIoT device(s). In such case, the AIoTF may perform escalation techniques which are outside of RAN2 discussion scope. It should be noted that this should not mean a AIoT reader creates a cell.

From RAN2 point of view, RAN2 can assume that A-IoT paging for the same service from multiple Readers targeting the same Device(s) is avoided by Reader and NW implementation.

RAN2 can assume that A-IoT paging for the same service from multiple Readers targeting the same Device(s) can be avoided by Reader and NW implementation.

RAN2 assumes the AIoT Function (AIoTF) selects the appropriate reader to initiate AIoT service towards the AIoT device(s) (e.g. based on the last known reader).

RAN2 assumes that A-IoT paging for the same service from multiple Readers targeting the same Device(s) is avoided by Reader and NW implementation.

# Summary

Based on the discussion above, we have following observations:

Observation 1.	A-IoT paging message containing one group identifier or filter criteria is within the scope of the WI.

Observation 2.	In terms of design, subsequent trigger/paging message is not different from initial trigger/paging message.

Observation 3.	The resource allocation including whether there is one or multiple resource opportunities should be indicated by the initial trigger/paging message.

Observation 4.	Similar behaviour like Query-like or QueryRep-like can be achieved by using a single A-IoT paging message format by defining optional fields for information such as group ID, filter criteria, resource allocation for msg1 etc.

Observation 5.	In some scenarios, the device ID may be absent in the D2R message in response to the A-IoT paging. When to include it and when to leave it out should be indicated by the Reader in the A-IoT paging message.

Observation 6.	When multiple readers are nearby, there is potential for R2D message intended for one reader interfering with other reader.

Observation 7.	To minimize the specification efforts for Topology 1 in this release, RAN2 can assume that the reader coordination and awareness of presence of other readers is left to other WGs or NW deployment/implementation.

Observation 8.	In case of dedicated communication, an R2D message from a device needs to be unambiguously attributable to the particular device by the Reader, and as such simply including Random ID in R2D message is neither beneficial nor sufficient.

Observation 9.	RAN2 can assume that A-IoT paging for the same service from multiple Readers targeting the same Device(s) can be avoided by Reader and NW implementation.

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	Support A-IoT paging message that contains one ‘group’ identifier or ‘filter criteria’ in Rel-19.

Proposal 2:	Design of initial trigger message is reused for subsequent trigger message, i.e. a separate message is not introduced for subsequent trigger/paging message.

Proposal 3:	Separate ‘access occasion announcement message’ (e.g. Query or QueryRep) is not needed.

Proposal 4:	The initial trigger message indicates the presence and information carried in the subsequent R2D message after D2R transmission.

Proposal 5:	Reader explicitly indicates the type of AIoT access (e.g., 2-step, 3-step) to AIoT devices in the initial trigger message (aka AIoT paging).

Proposal 6:	The A-IoT Paging Message indicates whether AIoT Device ID needs to be included in the D2R  message.

Proposal 7:	RAN2 assumes that the reader coordination and awareness of presence of other readers is left to other WGs or NW deployment/implementation.

Proposal 8:	In case of dedicated communication, an R2D message from a device shall be unambiguously attributable to the particular device by the Reader.

Proposal 9:	RAN2 assumes the AIoT Function (AIoTF) selects the appropriate reader to initiate AIoT service towards the AIoT device(s) (e.g. based on the last known reader).

Proposal 10:	RAN2 assumes that A-IoT paging for the same service from multiple Readers targeting the same Device(s) is avoided by Reader and NW implementation.

# References

[1] RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, RAN#106, Dec 2024

[2] RP-240826, Study on solutions for Ambient IoT (Internet of Things) in NR, RAN#103, March 2024

[3] TR 38.848, Study on Ambient IoT (Internet of Things) in RAN, Release 18

[4] R2-2410351, A-IoT random access procedure, Huawei, RAN2#128, Nov 2024

[5] R2-2501126, Views on Random Access Aspects of Ambient IoT, Qualcomm Incorporated, RAN2#129, Feb 2025