3GPP TSG-RAN WG2 Meeting #129	R2-2500965

Athens, Greece, 17 - 21 February 2025

Agenda Item:	8.6.2

Source:	Huawei, HiSilicon

Title:	Inter-CU LTM

Document for: Discussion and Decision

# 1	Introduction

LTM (L1/L2 Triggered Mobility) was introduced in Rel-18 for the intra-CU scenario. In Rel-19 [1], the scope is extended to inter-CU to support low latency and low interruption cell switch in broader areas. In this contribution, we discuss some remaining issues on inter-CU LTM.

# 2	Discussion

## 2.1	MCG LTM in NR-DC

RAN2 agreed that MCG LTM cell switch can setup an SCG or modify the SCG, including PSCell change, as long as the SN is not changed. In Rel-18, upon MCG LTM cell switch, the target DU can be notified, but there is no notification of the SN. In Rel-19, as the SCG configuration can be changed, the SN needs to know which configuration to apply. One solution could be that the UE always performs reconfiguration with sync of the SCG, with different dedicated preambles for each candidate LTM configuration, but the SN would need to reserve more preambles for each UE, which is not convenient from a network perspective. Therefore, the MN should notify the SN of the target configuration.

Proposal 1: For Rel-19 MCG LTM, at LTM cell switch execution, the (target) MN indicates the target LTM candidate configurations ID to the SN.

In Rel-19 MCG LTM, if the MN key is changed, the SN key also needs to be changed, and it is derived by the target MN from the MN key. For MN key change, the UE derives the new MN key from the NH and from an NCC value, and derives the SN key from the MN key and the sk-counter, and the SN receives the SN key from the MN.

In Rel-15, the SN key is provided in the SgNB addition request, but for LTM, this message is likely to be used only at the preparation of an LTM candidate cell, not at LTM cell switch execution. As a notification from the MN to the SN is needed at LTM cell switch to indicate the candidate configuration ID, the same notification could include the SN key.

Depending on the exact scheme designed by SA3, it may be possible to provide the SN key earlier, so that operation on SN-terminated MCG bearers could start without waiting for the notification from the target MN, but this would require further specification work and the MN could send the notification early enough to the SN.

Proposal 2: For Rel-19 MCG LTM, if there are SN-terminated bearers in the target LTM candidate configuration, the target MN provides the SN key to the SN at LTM cell switch execution.

## 2.2	SCG LTM

### 2.2.1	SCG LTM SRB handling

For SRB3 handling in inter-CU SCG LTM, RAN2 made the following agreement in the previous meeting:

RAN2 #128 2. For SRBs in inter-CU SCG LTM, Rel-19 ID is used to determine whether PDCP re-establishment or PDCP SDU discard is performed for LTM execution for SRB3.

According to the agreements, when the Rel-19 ID is different for the source SCG and the target SCG, the UE should perform PDCP re-establishment for SRB3. When the Rel-19 ID is the same for the source SCG and the target SCG, we can interpret the agreement as PDCP SDU (and also PDU) discarding always needs to be performed. However, according to the Rel-18 intra-CU LTM principle, whether to perform PDCP SDU and PDU discarding is based on network configuration (discardOnPDCP), which means PDCP SDU and PDU discarding is not a mandatory behaviour for SRB.

Observation 1: In Rel-18 intra-CU LTM, whether to perform PDCP SDU and PDU discarding for SRB is based on network configuration (discardOnPDCP).

The Rel-18 principle can be applied to Rel-19 SCG LTM case, in which the network has flexibility to control PDCP behaviour.

Proposal 3: In Rel-19 SCG LTM, for SRB3, when the Rel-19 ID is the same for the source SCG and the target SCG, the UE follows discardOnPDCP to determine whether to perform PDCP SDU and PDU discarding. When the Rel-19 ID is different, the UE performs PDCP re-establishment.

### 2.2.2	Coexistence of intra-CU and inter-CU SCG LTM

In Rel-18 SCG (intra-CU) LTM, LTM is configured by the SN without MN involvement. In Rel-19 SCG LTM, LTM is configured by the MN and the candidate LTM configuration is an MN RRC message that includes the SCG configuration, possibly with other changes by the MN. Besides, in order to support the co-existence of intra-SN candidate cells and inter-SN candidate cells, RAN2 agreed that intra-SN LTM candidate configuration can be configured by the MN as well, unlike the Rel-18 SCG LTM configuration.

RAN2 #128 8. RAN2 to support intra-CU SCG LTM in MN RRC message (i.e. MN RRCReconfiguration message), in addition to SN RRC message.

When there are intra-SN LTM candidate cells and inter-SN LTM candidate cells, to simplify the LTM preparation and configuration procedure, we can re-use a Rel-18 S-CPAC principle i.e., when there are inter-SN and intra-SN candidate cells, it is the MN that configures all the candidate configurations.

Proposal 4: When there are inter-SN and intra-SN LTM candidate cells for SCG LTM, it is the MN that configures all the LTM candidates (with a MN RRC message).

### 2.2.3	SCG LTM UL grant handling

In inter-CU SCG LTM, when SCG LTM is triggered, the UE applies the LTM candidate configuration, performs RACH-less or RACH-based LTM cell switch to the target PSCell and sends an RRCReconfigurationComplete message to the MN. Then, the MN transfers the SN RRCReconfigurationComplete message to the SN.

In Rel-18, if SRB3 is configured, the UE will transmit the RRCReconfigurationComplete message to the SN directly. However, in Rel-19, even if SRB3 is configured, the UE still transmits the RRCReconfigurationComplete message via SRB1.

Observation 2: In Rel-18 intra-CU LTM, when SRB3 is configured, the UE sends the  RRCReconfigurationComplete message to the SN directly. However, in Rel-19 SCG LTM, the UE sends the RRCReconfigurationComplete to the MN, regardless of whether SRB3 is configured.

In Rel-19 SCG LTM, if the SCG LTM cell switch is RACH-less, the UE may not have uplink data or signalling to transmit to the SCG. If the network configures UL grant skipping, the SCG may not receive any UE uplink transmission and consider RACH-less cell switch to be failed. To avoid this problem, the network should not configure the UE to skip UL grants in RACH-less SCG LTM procedure, e.g., not configure enhancedSkipUplinkTxDynamic, enhancedSkipUplinkTxConfigured and skipUplinkTxDynamic.

Proposal 5: For inter-CU SCG LTM, the network ensures that the UE does not skip the UL grant for RACH-less cell switch, regardless of whether SRB3 is configured.

# 3	Conclusion

This contribution makes the following proposals:

MCG LTM in NR-DC

Proposal 1: For Rel-19 MCG LTM, at LTM cell switch execution, the (target) MN indicates the target LTM candidate configurations ID to the SN.

Proposal 2: For Rel-19 MCG LTM, if there are SN-terminated bearers in the target LTM candidate configuration, the target MN provides the SN key to the SN at LTM cell switch execution.

SCG LTM SRB handling

Proposal 3: In Rel-19 SCG LTM, for SRB3, when the Rel-19 ID is the same for the source SCG and the target SCG, the UE follows discardOnPDCP to determine whether to perform PDCP SDU and PDU discarding. When the Rel-19 ID is different, the UE performs PDCP re-establishment.

Coexistence of intra-CU and inter-CU SCG LTM

Proposal 4: When there are inter-SN and intra-SN LTM candidate cells for SCG LTM, it is the MN that configures all the LTM candidates (with a MN RRC message).

SCG LTM UL grant handling

Proposal 5: For inter-CU SCG LTM, the network ensures that the UE does not skip the UL grant for RACH-less cell switch, regardless of whether SRB3 is configured.

# 4	Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom.

- R2-2411133, [AT128][116][MOB] Inter-CU LTM (ZTE), ZTE Corporation.