3GPP TSG-RAN WG2 Meeting #129	R2-2500965

Athens, Greece, 17 - 21 February 2025

Agenda Item:	8.6.4

Source:	Huawei, HiSilicon

Title:	Intra-CU conditional LTM

Document for: Discussion and Decision

# Introduction

In RAN #105 meeting, the WID of Rel-19 NR mobility enhancements Phase 4 was updated. According to the updated WID [1], conditional LTM (C-LTM) should be specified for intra-CU case in Rel-19, and we should start with non-DC case discussion.

In RAN2 #127bis and #128 meetings, RAN2 has made some agreements. In this contribution, we discuss the RACH-less procedures for intra-CU C-LTM, event evaluation and the co-existence between network triggered LTM and C-LTM.

# 2	Discussion

## 2.1	RACH-less cell switch procedure

### 2.1.1	DG-based RACH-less cell switch

To achieve RACH-less cell switch, the UE needs a UL grant to send an UL message to the target cell. In Rel-18 network-triggered LTM, both CG-based and DG-based RACH-less cell switch are supported. In RAN2 #127bis meeting, RAN2 agreed to support CG-based RACH-less, but support of DG-based RACH-less is FFS.

RAN2 #127bis: 10. For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

In CG-based RACH-less, the candidate cell needs to provide CG resources to the UE in advance, and the UE may use it after a long time or even not use it at all because the execution condition is not satisfied. For some candidate cells, UL resources could be limited, so it is difficult for those cells to allocate CG resources for RACH-less LTM. Therefore, to achieve RACH-less cell switch, it is better to also support DG-based RACH-less cell switch.

Proposal 1: Support DG-based RACH-less for conditional LTM.

In DG-based RACH-less procedure, the source cell/DU needs to send a request message to the candidate cell/DU, so that the candidate cell/DU can begin DG scheduling. RAN2 agreed that periodic, semi-persistent, aperiodic and event triggered L1 reports can be used in conditional LTM. Therefore, the source cell/DU can know which cell the UE may switch to and which uplink and downlink beams the UE may use, so the source cell/DU can send a notification to the candidate cell/DU, with beam information. Then, the candidate cell/DU can try scheduling the UE using the indicated downlink beam and monitor/receive uplink transmission from the UE using the indicated uplink beam. The exact scheduling time can be up to network implementation.

Proposal 2: For DG-based RACH-less cell switch, the source DU can notify a candidate DU (via CU) that a UE may switch to a candidate cell of the candidate DU and provide uplink/downlink beam information (TCI state) that the UE will use.

When the condition for cell switch is met, the UE does not know whether the source cell/DU has already sent a notification to the target cell/DU for dynamic grant transmission. If the source cell/DU did not send any notification and the UE waits for DG from the target cell, the cell switch is likely to fail since the target cell/DU may not be able to successfully receive a SR from the UE due to beam mismatch. Then, after T304 has expired, the UE will initiate RRC re-establishment or fast recovery (if supported). Such a situation may cause a rather long interruption. To avoid this problem, the network should let the UE know whether a notification such as the one just described, was provided to the target cell/DU or not. If not, the UE can directly initiate RACH to the target cell.

Besides, to allow successful initial transmission to the candidate cell, the UE should use the beams that were indicated to the candidate cell by the source cell, so such information should also be included in the notification to the UE that the candidate cell will send DG.

Proposal 3: The source DU can indicate to the UE a (list of) candidate cell(s) that will send DG and monitor SR, and indicate the uplink/downlink beams (TCI states) to use for RACH-less LTM cell switch.

Proposal 4: When the condition for LTM cell switch to a candidate cell for which CG is not configured is met, the UE performs RACH-less LTM cell switch to that cell if the UE was notified that the cell will monitor SR and send DG, otherwise the UE initiates RA.

The above procedure for DG-based RACH-less LTM cell switch is shown in the figure below:

Figure1: one example of DG-based RACH-less LTM cell switch procedure

It is possible that the indicated TCI state from the source cell/DU is not good enough when later conditional LTM cell switch is triggered, if the UE sticks to this indicated beam to monitor DCI or to transmit first UL data, the RACH-less cell switch may fail. To avoid this, the UE could evaluate the quality of the RS associated with the indicated TCI state, and if the quality is below a threshold (or the RS does not meet the entering condition of the C-LTM event), the UE initiates RA when accessing the target cell.

If this method would reduce the usage of RACH-less LTM cell switch too much, the source DU could send multiple beams/TCI states to the target DU, so that the target DU could try different beam directions, and the source cell would provide the same list of beams/TCI state to the UE, and if one of the RSs associated with the beams/TCI state in that list, the UE can select it and execute RACH-less LTM cell switch.

Proposal 5: In conditional LTM, when LTM cell switch is triggered, if the target cell is configured for RACH-less LTM cell switch (with CG or DG), the UE has previously received TCI states and the quality of the RS associated with the indicated TCI states is above a threshold, the UE initiates RACH-less LTM cell switch. If the quality of the RS associated with the indicated TCI states is below the threshold,  the UE initiates RA when accessing the target cell.

Proposal 5a: Consider whether to provide several TCI states to the UE and to the target cell, so that the UE can select the TCI state associated to the highest quality RS in order to perform RACH-less LTM cell switch, and only perform RACH if none of the RS is above the threshold.

### 2.1.2	CG-based RACH-less cell switch

In the CG-based RACH-less LTM cell switch procedure, the CG occasions are associated with SSBs. If the target cell uses joint TCI mode, for a particular CG occasion, the target cell knows which UL beam to use to receive the UE UL message and also knows which DL beam to use to schedule the UE afterwards. If the target cell uses separate TCI mode, the network may not know the beam to use for DL transmissions since RAN2 agreed that the CG occasions are associated with UL TCI states when separate TCI mode is used. However, it would be possible for the UE to include in the first UL transmission an indication of a good DL beam, so that the network can further schedule the UE. There is a risk that this first UL transmission is not successfully decoded by the network and then the network cannot schedule a retransmission, but the network could configure automatic repetition in the CG so that the UE autonomously performs retransmissions if the network does not provide a further UL grant.

Proposal 6: For CG-based RACH-less cell switch, when the execution condition is met, the UE selects a suitable UL TCI state and performs RACH-less LTM cell switch using the CG. If separate TCI mode is used, the UE indicates a suitable DL TCI state in the initial UL transmission (details FFS)..

Proposal 7: When the condition for LTM cell switch to a candidate cell is met, if the candidate cell is not configured with CG and the UE did not receive uplink/downlink beam information for DG-based RACH-less LTM cell switch for the candidate cell, the UE initiates RA to the target cell.

### 2.1.3	TA delivery and maintenance

For PDCCH ordered early RACH, RAN2 agreed that it is the source cell that sends the TA to the UE via a new MAC CE.

RAN2 #128: 8. The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4. 9. The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM. 10. Candidate cell TA is maintained by a new timer.

In this new MAC CE, an absolute TA value (12 bits) should be included. There is no strong motivation to also support relative TA value, since the most common case should be initial TA acquisition for a candidate cell. In addition, the MAC CE should indicate for which candidate cell the TA can be applied.

Proposal 8: The new MAC CE for candidate cell TA deliver includes the absolute TA value (12 bits) and the candidate configuration ID (3 bits).

For serving cells, there are TA groups, with the same TA value used for several serving cells. The serving DU can see the UE uplink transmission and adjust the TA, and if it appears that for two cells that belong to the same TAG, the TA needs to be different, the serving DU can reconfigure the TAGs. As there can be up to 32 serving cells and the UE needs to constantly maintain and adjust TA for each serving cell, with TA commands that are sent constantly to update the TA according to received uplink transmissions. Having only up to 4 TAGs can reduce the signalling for TA commands and simplify the UE implementation.

For LTM, the TA is only sent following early RA, which is far less frequent, and there are at most 8 candidate cells. If the 8 candidate cells would be grouped into candidate cell TA groups (CTAGs) limited e.g., to 4 groups, if cells in different locations require a different TA, this only allows configuring candidate cells for 4 locations, which may require to regularly release some candidate cells to add new ones where the UE is moving, and partially defeat the benefit of LTM.

If there can be 8 CTAG, there is no such problem but there is no gain of having CTAGs. Instead of configuring CTAGs to the UE, it would be possible that the source DU sends two TA indications when the same TA value is valid for two candidate cells, and the UE can handle them as separate information. The signalling overhead remains rather small and there is no specification impact.

Proposal 9: If the serving cell knows that the TA obtained following early RA to a candidate cell is valid for another candidate cell, the serving cell can send one MAC CE for each candidate cell with the same TA value. No further optimization is needed.

After the UE has received the TA by the new MAC CE, the UE stores it, and uses a timer to determine its validity. If the TA validity timer is running when conditional LTM cell switch is triggered and the UE has the necessary beam information as discussed above, the UE performs RACH-less LTM cell switch. Otherwise, the UE initiates RA when accessing the target cell. There can be a similar timer at the source DU side to determine the TA validity. If the source DU determines that the TA is not valid but the UE might execute LTM cell switch to the target cell, the source DU can send a PDCCH order. Therefore, when the UE determines a TA is invalid, there is no need to notify the network.

Proposal 10 In PDCCH-ordered early RACH for conditional LTM, upon receiving the TA via the new MAC CE, the UE stores the receive TA value and starts a timer. If the timer expires, the UE deletes the stored TA value and does not perform any other action (the network can trigger another PDCCH order if useful.)

## 2.2	Event evaluation

RAN2 agreed that both L1 execution condition (LTM3-like, LTM5-like) and L3 execution condition (CondEventA3, CondEventA5) are supported.

RAN2 #127bis: 2. Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions. RAN2 #128: 1. The triggering condition of conditional LTM can be based on L3 measurement. 2. CondEventA3 and CondEventA5 conditions can be baseline for the conditional LTM execution.

For event-triggered L1 report, the TTT is evaluated per beam. This can be re-used for C-LTM L1 execution condition evaluation. For instance, when a conditional LTM3/LTM5-like event is configured for a candidate cell, the UE compares each configured RS of the candidate cell with the serving cell RS associated with current indicated TCI state. When the entering condition is met for the TTT duration, C-LTM cell switch is triggered.

For CHO, the execution condition evaluation is  per candidate cell. For conditional LTM, the same principle can be reused.

Proposal 11: For conditional LTM based on L1 measurements (LTM3/5-like event), the evaluation is per candidate beam. For conditional LTM based on L3 measurements (condA3/condA5 events), the evaluation is per candidate cell (i.e., with consolidation of multiple beams of the candidate cell).

## 2.3	Co-existence between network triggered LTM and C-LTM

Conditional LTM shares a lot of characteristics with Rel-18 LTM: L1 measurement reporting, early RA, early TCI state activation, LTM cell switch procedure, using candidate and possibly reference configuration. Apart from the reception of a MAC CE following early RA and the use of execution conditions for a MAC CE, the UE behaviour is the same and then the configuration of a candidate LTM cell should be exactly the same plus the execution conditions, as is already the case in the draft RRC CR.

For all the procedure that the UE performs before LTM cell switch, whether an LTM candidate is for conditional LTM or for Rel-18 LTM does make any difference, so the LTM configuration should

Proposal 12: The UE can be simultaneously configured with Rel-18 LTM candidate configurations and with LTM candidate configurations for conditional LTM.

As discussed above, when an LTM candidate includes execution condition, apart from the measurements for the execution condition and the reception of a TA value, all the UE behaviours associated with this LTM candidate are the same. Therefore, it should be possible for the UE to execute LTM cell switch to this LTM candidate if the network sends an LTM cell switch command MAC CE to the UE.

Proposal 13: Upon reception of an LTM cell switch command MAC CE, the UE performs LTM cell switch to the indicated candidate, regardless whether it is configured for Rel-18 LTM or for conditional LTM.

# 3	Conclusion

This contribution makes the following proposals:

DG based RACH-less

Proposal 1: Support DG-based RACH-less for conditional LTM.

Proposal 2: For DG-based RACH-less cell switch, the source DU can notify a candidate DU (via CU) that a UE may switch to a candidate cell of the candidate DU and provide uplink/downlink beam information (TCI state) that the UE will use.

Proposal 3: The source DU can indicate to the UE a (list of) candidate cell(s) that will send DG and monitor SR, and indicate the uplink/downlink beams (TCI states) to use for RACH-less LTM cell switch.

Proposal 4: When the condition for LTM cell switch to a candidate cell for which CG is not configured is met, the UE performs RACH-less LTM cell switch to that cell if the UE was notified that the cell will monitor SR and send DG, otherwise the UE initiates RA.

Proposal 5: In conditional LTM, when LTM cell switch is triggered, if the target cell is configured for RACH-less LTM cell switch (with CG or DG), the UE has previously received TCI states and the quality of the RS associated with the indicated TCI states is above a threshold, the UE initiates RACH-less LTM cell switch. If the quality of the RS associated with the indicated TCI states is below the threshold,  the UE initiates RA when accessing the target cell.

Proposal 5a: Consider whether to provide several TCI states to the UE and to the target cell, so that the UE can select the TCI state associated to the highest quality RS in order to perform RACH-less LTM cell switch, and only perform RACH if none of the RS is above the threshold.

CG based RACH-less

Proposal 6: For CG-based RACH-less cell switch, when the execution condition is met, the UE selects a suitable UL TCI state and performs RACH-less LTM cell switch using the CG. If separate TCI mode is used, the UE indicates a suitable DL TCI state in the initial UL transmission (details FFS)..

Proposal 7: When the condition for LTM cell switch to a candidate cell is met, if the candidate cell is not configured with CG and the UE did not receive uplink/downlink beam information for DG-based RACH-less LTM cell switch for the candidate cell, the UE initiates RA to the target cell.

TA delivery and maintenance

Proposal 8: The new MAC CE for candidate cell TA deliver includes the absolute TA value (12 bits) and the candidate configuration ID (3 bits).

Proposal 9: If the serving cell knows that the TA obtained following early RA to a candidate cell is valid for another candidate cell, the serving cell can send one MAC CE for each candidate cell with the same TA value. No further optimization is needed.

Proposal 10 In PDCCH-ordered early RACH for conditional LTM, upon receiving the TA via the new MAC CE, the UE stores the receive TA value and starts a timer. If the timer expires, the UE deletes the stored TA value and does not perform any other action (the network can trigger another PDCCH order if useful.)

Event evaluation

Proposal 11: For conditional LTM based on L1 measurements (LTM3/5-like event), the evaluation is per candidate beam. For conditional LTM based on L3 measurements (condA3/condA5 events), the evaluation is per candidate cell (i.e., with consolidation of multiple beams of the candidate cell).

Co-existence between LTM and C-LTM

Proposal 12: The UE can be simultaneously configured with Rel-18 LTM candidate configurations and with LTM candidate configurations for conditional LTM.

Proposal 13: Upon reception of an LTM cell switch command MAC CE, the UE performs LTM cell switch to the indicated candidate, regardless whether it is configured for Rel-18 LTM or for conditional LTM.

# 4	Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom.