3GPP TSG-RAN WG2 Meeting #129											        R2-2500980

Athens, 17-21 Feb. 2025

Agenda item:			8.9.2

Title:						Further considerations on S&F operations

Source:					Continental Automotive

Document for:		Discussion and Decision

# Introduction

The aim of this 3GPP Rel-19 WI is to enhance IoT-NTN with the following objectives [1]:

Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore:

o	Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2]

	At least specify necessary enhancements e.g. related to S1 protocol, especially to address
the feeder link switch over as needed [RAN3]

Note: Strive to minimise UE impact.

Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

During RAN2#128 [2] the following agreements were made:

Agreements:

1.	Time information can be broadcasted to indicate when the current satellite operation mode will transit from “S&F operation” mode to real-time/normal mode. RAN2 assumes that a R19 UE could use this information at least to delay some NAS procedures until the feeder link is resumed (FFS what we specify in RAN2 specs for this and if we differentiate the behaviour for R19 UEs supporting and not supporting S&F mode, also considering the relation to the Wait Timer). FFS which SIB is used (SIB1 or SIB31). FFS if absolute or relative time is used. FFS on the opposite transition (i.e. whether to indicate when the “S&F operation” will start)

2.	It is FFS whether we need to introduce a new release cause when the NW wants to release a R19 UE to RRC idle because the feeder link becomes unavailable

3.	“S&F operation” indication is provided in SIB1

4.	RAN2 assumes that AS signalling is not needed to indicate the architecture option (Split MME or Full CN).

5.	RAN2 waits for further progress in SA2, if any, before working on RAN2 aspects of a CIoT-UP solution for SF operation

6.	RAN2 understands that the UE configured with a satellite ID list by MME is not prevented to camp on a satellite operating in normal IoT NTN mode (i.e. with feeder-link connection), and perform subsequent access and data/signalling communication with that satellite

7.	RAN2 understands the UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list.

In this paper, we share further considerations on S&F operations.

# 2. Discussion

Impacts on UE power consumption and access control

It was agreed in RAN2#128 that the network will provide an “S&F operation” indication in system information block type 1 (SIB1). Further, it was agreed that the network can broadcast a time indication to indicate when the current satellite will transmit from “S&F operations” to real-time/normal mode. This broadcast “wait time” indication is considered beneficial for the UE, since it could delay some NAS procedures and idle mode tasks, and thus, save some energy. For example, the UE could delay intra-/inter-frequency measurements and cell (re-)selection based on the broadcast “wait time” indication.

Observation 1: The proposed “wait time” indication is considered beneficial for the UE, since it could delay some NAS procedures and idle mode tasks, and thus, save some energy.

However, while the proposed “wait time” indication could be beneficial with respect to UE power consumption in time periods where no feeder link or satellite is available, the broadcast “wait time” indication may also create a subsequent problem, particularly, in scenarios with a high UE density: All UEs will use the same “wait time” indication when trying to access the network, e.g., when the current satellite switches back to real-time/normal mode or when another satellite operating in real-time/normal mode becomes available. This can significantly increase the likelihood of random access collisions among UEs and lead to network congestion.

Observation 2: All UEs will use the same “wait time” indication when trying to access the network. This can significantly increase the likelihood of random access collisions among UEs and lead to network congestion.

Proposal 1: RAN2 should further discuss how UE energy savings can be realized while avoiding network congestion.

# Conclusions

The following observations and proposals have been made in this contribution:

Observation 1: The proposed “wait time” indication is considered beneficial for the UE, since it could delay some NAS procedures and idle mode tasks, and thus, save some energy.

Observation 2: All UEs will use the same “wait time” indication when trying to access the network. This can significantly increase the likelihood of random access collisions among UEs and lead to network congestion.

Proposal 1: RAN2 should further discuss how UE energy savings can be realized while avoiding network congestion.

# References

- RP-240776, Revised WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3 (revision of RP-234077).

- R2-2410913, Report from Break-out session on NR-NTN and IoT-NTN, Session Chair (ZTE Corporation), RAN2#128, Orlando, Nov 18th – 22nd, 2024.