3GPP TSG-RAN WG2 Meeting #129                                    	R2-2500981

Athens, Greece, Feb 17th – 21nd, 2025

Agenda item:	8.9.3

Source:	Skylo Technologies

Title:	Discussion on EDT Enhancements for IOT NTN

Document for:	Discussion and Decision

# 1 Introduction

In the last RAN2 meetings, RAN2 had discussed several general aspects on the CB-Msg3 and DSA mechanism and had achieved the following agreements [1]:

RAN2#128 Agreements: - Only system Information is used to provide cell-specific CB-Msg3 PUSCH resources (FFS if anything is needed in dedicated signalling for the TA validation parameters, if needed, for the case of 15kHz SCS NB-IoT and eMTC CE mode B) - Reuse the existing CE level selection procedure for CB-Msg3, at least for the initial selection. FFS whether we can reuse the same thresholds. FFS on the number of levels - The UE triggers CB-Msg3 only if the size of pending UL data is less than the configured maximum TBS (FFS if the maximum TBS is same or different for different CE levels) - There will be a RSRP threshold that determines whether CB-Msg3 can be used (if the RSRP is below such threshold, PRACH will have to be used) (FFS if we need a separate set of thresholds, including a different minimum threshold, in case CB-msg3 EDT is combined with OCC) - The number of replicas for DSA will be configured by the NW: 1 (SA), 2, 3, 4. The configuration of the number of replicas is CE-level specific - There will be a configurable time window for DSA CB-Msg3 occasion selection. FFS on the details (e.g. when the time window starts) - For SA case (single replica), after the end of all repetition of CB-Msg3 PUSCH transmission, UE starts a window for response reception taking UE-eNB RTT into account. FFS if we need to consider additional delay e.g. for the processing time - For DSA case, FFS if we only have one or multiple PDCCH monitoring window(s) (i.e. one window per each replica) for response reception. FFS when the window(s) is/are started (or restarted) and stopped. FFS on the window length. FFS if the UE needs to monitor only one RNTI or multiple RNTIs) - The UE stops the PDCCH monitoring window(s) once it receives a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4) - Assuming that there will be scenarios where it’s possible to receive a CB-msg4 before the UE transmits some replicas, a UE stops transmitting the remaining replicas if it has received a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4) - Within the configured time window, the UE shall select randomly different time domain occasions for transmitting different replicas. And for each time domain occasion, the UE shall select randomly a frequency domain resource. - RAN2 understands that, for DSA, once the eNB successfully decodes one of the multiple replicas, it may respond without waiting for the remaining replica(s) (FFS when the response window(s) is/are started)

The downlink carries a significant amount of broadcast messaging, including MIB, SIBs, PSS, and SSS. Combined with the high number of repetitions needed for reliable link closure in NTN, this creates a bottleneck for IoT NTN. Hence, to take complete advantage of all the uplink enhancements we need to ensure appropriate downlink enhancements are considered.

Based on the agreements as per the Work Item Description in [2] which states objectives for this work item to include

- Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete,

- Reducing the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction

In this contribution, we will further discuss the efficient delivery of msg4 (especially with multicast) and Early Data transmission enhancements for IoT-NTN.

# 2 Discussion

Figure 1: Multiplexed CB-MSG4

## 2.1 Multiplexing of CB-Msg4

- Enable the new CB-Msg4 to carry NPDSCH messages addressing different UEs simultaneously, improving network efficiency and reducing signaling overhead.

- Resource Efficiency and overhead: Many a times the Msg 4 content remains same for multiple UEs. By addressing a single Msg4 to multiple UEs, the network reduces PDSCH and signaling resource usage, enhancing overall capacity and accommodating more UEs within the same resource pool.

- Simplified Network Operations: With multiple UEs sharing Msg4, the network’s contention resolution process becomes more efficient, allowing simultaneous responses to several UEs, reducing the need for individual Msg4s.

Proposal 1: Enable the new CB-MSG4 to carry NPDSCH messages targeting multiple UEs simultaneously.

Often, the content of Msg4 is identical for multiple UEs. CB-MSG4 can include a single Msg4 transmission addressed to multiple UEs.

- CB-msg4 will need to support the ability to carry contention resolution identities for multiple UEs simultaneously.

Proposal 2: Multiplex multiple UE Contention Resolution Identities in CB-msg4.

- All UEs who sent CB-Msg3 should get a response in one of the CB-Msg4. eNB can have DL data for a few UEs while do not have DL data for the rest of them. Both of these should be able to share the same CB-Msg4. For the UEs for whom there is no DL Data, a simple NDPCCH DCI (ACK/NACK) is sent.

Proposal 3: The multiplexed CB-msg4 should have provision to send only DCI (L1 ACK/NACK) for UEs for whom there is no DL data transmission.

## 2.2 Contention Resolution Identity Used for CB-Msg4

- To optimize resource usage and the number of UEs which can be addressed in a single CB-Msg4, the size of UE identifiers used in contention resolution should be minimized.

Proposal 4: Reduce the size of UE identifiers for the new multiplexed CB-Msg4.

To reduce the size of Contention Resolution Identity (CRI) in Multiplexed CB-Msg4. We can consider using a Hash polynomial based mechanishm.

CRI Hash Computation

- Let’s denote the 48-bit S-TMSI bits by c47, c46, ..., c0, and the polynomial defined by these coefficients by C(x).

- The hashed S-TMSI is the remainder of x48 * C(x) divided by the generator polynomial G(x).

- The generator polynomial is given by

G(x) = x24 + x23 + x18 + x17 + x14 + x11 + x10 + x7 + x6 + x5 + x4 + x3 + x + 1

- Hence, we have conversion from 48-bit S-TMSI to 24 bit Contention Resolution ID now.

Proposal 5: Use a hash map to reduce the size of Contention Resolution identity (S-TMSI) used in CB-Msg4.

- The Multicast CRI indicated to the UE is a 24-bit hash of the original 48-bit CRI.

## 2.3 Structure of Multiplexed CB-Msg4

Following Multicast-CB-MSG4 structure can be used for UEs which have NAS payload and UEs which don’t have NAS payload. (REDC = RRCEarlyDataComplete-NB)

Figure 2: Multiplexed CB-MSG4 structure

- Contention Resolution Identity (CRIs) of the UEs which have NAS payload should be placed first in the Group CRI. And CRIs of the UEs which don’t have NAS payload should be placed at the end. SDU-1 belongs to UE with CRI-UE-1, SDU-2 belongs to UE with CRI-UE-2 and so on. REDC-common(i.e. SDU-(n+1)) belongs to UEs with CRI-UE-(n+1) to CIR-UE(n+m).

- RRC messages of UEs which have NAS payload should be placed first in the sequence of RRC messages. And One common RRC message should be placed at the end of the RRC message sequence for the UEs which don’t have NAS payload.

- Group-CRI is a concatenation of each UE’s CRI(CRI-UE). L field is included in the sub-header  for the Group-CRI as per the specification. CRI-UE is fixed length. Group-CRI is variable length and the length is a multiple of CRI-UE length.

Proposal 6: Contention Resolution Identity (CRIs) of the UEs which have NAS payload should be placed first in the Group CRI. And CRIs of the UEs which don’t have NAS payload should be placed at the end.

Proposal 7: RRC messages of UEs which have NAS payload should be placed first in the sequence of RRC messages.

Proposal 8:  One common RRC message should be placed at the end of the RRC message sequence for the UEs which don’t have NAS payload.

## 2.4 Connected Mode EDT

We also propose to extend both CB-msg 3 EDT being discussed in Rel 19 and CP-EDT released as part of Release 15 to be supported for both RRC connected as well as RRC Idle mode. EDT feature helps in saving resources even when the device is in an RRC-connected state. Hence, the feature should be extended to RRC Connected mode too.

Proposal 9: CB-EDT mechanism should be extended to RRC connected mode.

Proposal 10: Regular EDT should also be extended to RRC connected mode.

## 2.5 Multicast MSG4 for CP-EDT

We propose to enable multicast MSG4 for regular CP-EDT too. Existing CP-EDT can be used by millions of IOT devices for infrequent short data transmission. By supporting multicast response for Msg4 transmissions, DL resource consumption can be optimized.

Proposal 11: Multicast CB-MSG4 mechanism should be extended to regular EDT for RRCEarlyDataComplete.

Proposal 12: Multicast CB-MSG4 should be extended to connected mode EDT.

# 3 Conclusion

In this contribution, we have provided our understanding of CB-Msg3 procedure. And we made the following proposals:

Multiplexing of CB-Msg4

Proposal 1: Enable the new CB-MSG4 to carry NPDSCH messages targeting multiple UEs simultaneously.

Proposal 2: Multiplex multiple UE Contention Resolution Identities in CB-Msg4.

Proposal 3: The multiplexed CB-Msg4 should have provision to send only DCI (L1 ACK/NACK) for UEs for whom there is no DL data transmission.

Contention Resolution Used for CB-Msg4

Proposal 4: Reduce the size of UE identifiers for the new multiplexed CB-Msg4.

Proposal 5: Use a hash map to reduce the size of Contention Resolution identity (S-TMSI) used in CB-Msg4.

Structure of Multiplexed CB-Msg4

Proposal 6: Contention Resolution Identity (CRIs) of the UEs which have NAS payload should be placed first in the Group CRI. And CRIs of the UEs which don’t have NAS payload should be placed at the end.

Proposal 7: RRC messages of UEs which have NAS payload should be placed first in the sequence of RRC messages.

Proposal 8:  One common RRC message should be placed at the end of the RRC message sequence for the UEs which don’t have NAS payload.

Connected Mode EDT

Proposal 9: CB-EDT mechanism should be extended to RRC connected mode.

Proposal 10: Regular EDT should also be extended to RRC connected mode.

Multicast MSG4 for CP-EDT

Proposal 11: Multicast CB-MSG4 should be extended to connected mode EDT.

Proposal 12: Multicast CB-MSG4 mechanism should be extended to regular EDT for RRCEarlyDataComplete.

# 4 Reference

[1] R2-2410913. Report from Break-out session on NR-NTN and IoT-NTN

[2] RP-243278. Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3.

[3] R2-2410725, Discussion on EDT Enhancements for IOT NTN, Skylo Tech.

[4] R2-2410050, CB-Msg3 and Msg4 enhancements, Qualcomm Incorporated.

[5] R2-2409975, Uplink capacity enhancement in IoT NTN, Apple.