3GPP TSG-RAN WG2 Meeting #129	R2-2500993

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.4.2

Source:	Nokia

Title:	LP-WUS in IDLE and INACTIVE

WID/SID:	NR_LPWUS-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In RAN#102 meeting, a new work item (RP-240801) was agreed for Low Power Wake Up Signal and Receivers designs. These designs are to be primarily targeted at delay and power-sensitive, small form-factor devices, such as industrial sensors, controllers and wearables. Unlike previous power saving study items, the objectives for this study encompasses new signals and receiver architectures.

RAN discussed inter frequency LP-WUS scenarios and the following way forward was agreed in [1]:

- For idle/inactive mode, consider a standardized solution for intra-frequency case (e.g. solution 1, 2, and 3 in RP-242656).  For Rel-19, inter-frequency solutions (e.g. solution 4) will not be considered.

- Detailed discussions and final solution selection is left up to WGs.

In this contribution we focus on the following objective:

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

# 2	Discussion

RAN discussed the following scenarios described in [2]:

Our understanding of the solutions 1, 2, 3 is the following:

Solution 1:

LP-WUS UE prioritizes bands supporting LP-WUS in the cell (re)selection and in case LP-WUS cell is found all the procedures including system information reception, LP-WUS, paging, connection establishment/resume for paging response happens on the same band.

Solution 2:

LP-WUS UE prioritizes bands supporting LP-WUS in the cell (re)selection and in case LP-WUS cell is found system information reception, LP-WUS, paging happens on that band and then connection establishment/resume for paging response happens on another band.

Solution 3:

LP-WUS follows legacy cell (re)selection evaluation and NW can redirect with dedicated signaling the UE to the band where LP-WUS is supported.

|  | Overload on LR band | Limited LP-WUS usage | Specification impact |
| --- | --- | --- | --- |
| Solution 1 | No, LP-WUS UEs will (re)select cells on the bands where LP-WUS is supported. NW can avoid overload by setting proper cell reselection parameters for all the UEs and dedicated frequency priorities can be used to mitigate overload. | No | - Small specification impact - Cell (re)selection needs to be enhanced for LP-WUS UEs so that they can prioritize band supporting LP-WUS |
| Solution 2 | No, UE can be moved to another band for performing connection establishment after paging. | No | - Intermediate specification impact - cell (re)selection needs to be enhanced for LP-WUS UEs so that they can prioritize band supporting LP-WUS - Procedure for selecting cell after LP-WUS / paging. - There will be a delay after paging. Too long delays e.g. more than second may not be acceptable. Delay length can be asked from RAN4. |
| Solution 3 | Yes. Depends how effectively NW can redirect the UE to between bands using dedicated signaling. For this option the UE needs to request RRC connection/resume or the UE needs to be moved to connected in order dedicated signaling can be provided to the UE. | Yes, depends how effectively NW can redirect the UE to between bands | - No specification impact since dedicated frequency priorities are already supported. |

It would be beneficial that the UE camps on the cell supporting LP-WUS more often to save battery and considering the specification impact we propose the following

Proposal 1: Cell (re)selection rules are enhanced so that LP-WUS UE can prioritize bands supporting LP-WUS

Proposal 2: RAN2 to discuss whether to specify procedure for (re)selecting cell after LP-WUS / paging.

Small data transmission (SDT) was specified in R17 and R18. SDT is a procedure allowing data and/or signalling transmission while remaining in RRC_INACTIVE state (i.e. without transitioning to RRC_CONNECTED state). SDT can be initiated either by the UE in case of MO-SDT (Mobile Originated SDT) or by the network in case of MT-SDT (Mobile Terminated SDT). MO-SDT is initiated by the UE only if less than or equal to a configured amount of UL data awaits transmission across all radio bearers for which SDT is enabled, the DL RSRP is above a configured threshold, and a valid SDT resource is available. MT-SDT is initiated by the network with an indication to the UE in a paging message when DL data awaits transmission for radio bearers configured for SDT; based on the indication, the UE initiates the MT-SDT only if the DL RSRP is above a configured threshold. SDT was specified also for power saving purposes like LP-WUS and therefore it can be assumed that it may be beneficial to implement both SDT and LP-WUS features in the same device.

Proposal 3: LP-WUS UE may support MO-SDT and/or MT-SDT

Proposal 4: The UE can initiate MO-SDT while UE is monitoring LP-WUS

Proposal 5: NW can initiate MT-SDT while UE is monitoring LP-WUS

In scope of the IDLE/INACTIVE mode operation, the WID objective seeks to specify LP-WUS triggered paging monitoring. In strict sense this would seem to imply that LP-WUS triggering should result UE to monitor the associated PO. In Rel-18 different approaches were discussed how to determine the PO which UE should monitor, upon LP-WUS triggering:

- ‘Dynamic PO’; where LP-WUS trigger the UE to monitor a time occasion where PDCCH and PDSCH are monitored would appear dynamically, some predefined time after the LP-WUS

- ‘Closest PO’; where the LP-WUS trigger the UE to monitor PO that would be closest to the LP-WUS

- ‘Normal PO’; where LP-WUS would trigger the UE to monitor the same IDLE/Inactive mode PO that UE would monitor normally without LP-WUS

In any case network won’t be aware if UE is monitoring LP-WUS or, as a fall-back, the PO(s). This means that network would need to account both possible situations when transmitting the paging. From network perspective, the alternative procedures, ‘dynamic PO’ and ‘closest PO’ would mean that network would need in minimum transmit the paging in two alternative POs during the paging cycle, one associated to the LP-WUS and other in the normal PO. This would increase the paging overhead, doubling it. Based on that the following is proposed.

Proposal 6: UE monitors the legacy PEI and/or PO after receiving LP-WUS indicating wake-up and no changes to the paging occasion to be monitored are supported.

As a part of the WID objectives the need to determine entry and exit conditions for LP-WUS monitoring was identified. These procedures are mostly related to RAN2 and the objective (overall) is to be led by RAN2:

- For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

Now in scope of the LR based measurements, both entry/exit conditions would be relevant. For entry/exit condition, it could be expected that network can set a threshold for the UE in the system information, after which the LP-WUS should not be monitored and vice versa when LP-WUS should be monitored. Based on this the following is proposed.

The following baseline was agreed in the RAN2#126 meeting:

- Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. - Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS.

Based on the above baseline the UE behaviour is not clear i.e. it is not clear when the UE monitors LP-WUS or legacy PO. Deterministic and testable UE behaviour should always be targeted and therefore we propose the following:

Proposal 7: It is specified when the UE supporting LP-WUS shall monitor LP-WUS and when legacy PO.

Proposal 8: The UE supporting LP-WUS shall start LP-WUS monitoring when LP-WUS monitoring entry condition (better than threshold) is fulfilled.

Proposal 9: The UE supporting LP-WUS shall stop LP-WUS monitoring when LP-WUS monitoring exit condition (worse than threshold) is fulfilled. In this case the UE shall monitor legacy PO instead of LP-WUS.

A UE which has reselected to another cell than the last active cell may experience extended wake-up latency, since the network may initially attempt to wake-up the UE on the last active cell before escalating the wake-up to a broader area. Between each escalation, there will be a certain delay since the network first needs to conclude that the UE did not respond to the paging.

If the network escalates the wakeup messages to the complete RA (Registration Area) it may have a negative impact on the FAR (False Alarm Rate) as well as the overhead of radio resources consumed by the LP-WUS wakeup request.

To mitigate the above, the UE could assist the network with information about which cells it is mainly reselecting between, e.g. in the UEAssistanceInformation message, potentially divided in multiple sets of cells with different priorities. The network can then limit the initial wakeup request to the cells indicated by the UE or if there are multiple sets of cells, the network can perform the wakeup according to the priority of each set. If the UE moves outside the set of cells, then it shall not expect a wakeup message, and it will have to switch back to MR. The benefit for the network is that it does not have to escalate the wakeup message to the entire tracking area. The benefit for the UE is that it may be woken up immediately without escalation latency even if it has reselected to another cell, assuming that the reselection was done to one of the cells in the set with highest priority.

Proposal 10: The UE shall switch back to main radio if it has moved away from the indicated set of cells and last active cell.

Proposal 11: The UE may send updated assistance information, e.g. if it has moved to another area.

# 3	Conclusion

The following is proposed based on the discussion:

Proposal 1: Cell (re)selection rules are enhanced so that LP-WUS UE can prioritize bands supporting LP-WUS

Proposal 2: RAN2 to discuss whether to specify procedure for (re)selecting cell after LP-WUS / paging.

Proposal 3: LP-WUS UE may support MO-SDT and/or MT-SDT

Proposal 4: The UE can initiate MO-SDT while UE is monitoring LP-WUS

Proposal 5: NW can initiate MT-SDT while UE is monitoring LP-WUS

Proposal 6: UE monitors the legacy PEI and/or PO after receiving LP-WUS indicating wake-up and no changes to the paging occasion to be monitored are supported.

Proposal 7: It is specified when the UE supporting LP-WUS shall monitor LP-WUS and when legacy PO.

Proposal 8: The UE supporting LP-WUS shall start LP-WUS monitoring when LP-WUS monitoring entry condition (better than threshold) is fulfilled.

Proposal 9: The UE supporting LP-WUS shall stop LP-WUS monitoring when LP-WUS monitoring exit condition (worse than threshold) is fulfilled. In this case the UE shall monitor legacy PO instead of LP-WUS.

Proposal 10: The UE shall switch back to main radio if it has moved away from the indicated set of cells and last active cell.

Proposal 11: The UE may send updated assistance information, e.g. if it has moved to another area.

# 4	References

[1] RP-243266 Way forward on Rel-19 LP-WUS separate band issues RAN2 chair, Interdigital

[2] RP-242656 On LR and MR operating in different bands in release 19, Vodafone, Vivo, Huawei, HiSilicon