3GPP TSG-RAN WG2 Meeting #129	R2-2500994

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.4.3

Source:	Nokia

Title:	RRM measurement relaxation in RRC_IDLE/INACTIVE

WID/SID:	NR_LPWUS-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In RAN#102 meeting, a new work item (RP-240801) was agreed for Low Power Wake Up Signal and Receivers designs. These designs are to be primarily targeted at delay and power-sensitive, small form-factor devices, such as industrial sensors, controllers and wearables. Unlike previous power saving study items, the objectives for this study encompasses new signals and receiver architectures.

In this contribution we focus on the following objective:

- For IDLE/INACTIVE modes

  - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

# 2	Discussion

The following working assumption was done in RAN2#127bis meeting:

- Working assumption

For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’.

FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

As a part of the WID objectives the need to specify RRM relaxation. These procedures are mostly related to RAN2 and RAN4:

- For IDLE/INACTIVE modes - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

Now in scope of the LR based measurements, RRM relaxation of UE MR would be relevant. For RRM relaxation, it could be expected that network configure RRM relaxation condition in the system information. Based on this the following is proposed:

Proposal 1: NW can configure in the system information RRM relaxation conditions for IDLE/INACTIVE.

Proposal 2: The UE starts RRM relaxation when RRM relaxation entry condition is fulfilled, and the UE stops RRM relaxation when RRM relaxation exit condition is fulfilled.

Two different kinds of RRM relaxations can be considered i.e. 1. all serving and neighbour cell the measurements are omitted from the MR and 2. serving and/or neighbour cell measurements are relaxed.

Proposal 3: When serving cell quality measured by LR is better than threshold X, MR RRM measurements are omitted.

Proposal 4: When serving cell quality measured by LR is better than threshold Y, serving and neighbour cell RRM measurements are relaxed.

Of course, if MR measurements are relaxed, but not fully omitted, some joint evaluation of threshold based on measurements from both, MR and LR could be considered. It is not clear whether these would need to be accounted as joint quantity, or whether the evaluation could be done based on measurements from both, LR and MR.

Proposal 5: RRM relaxation entry and exit condition can include both LR and MR measurement evaluations. FFS whether LR and MR measurements are somehow combined or evaluated independently.

Evidently both exit and entry condition to be considered would relate to the coverage, partly of the cell and more importantly to the LP-WUS coverage. For entry/exit condition, it would seem that most relevant measurement quantity would be such that allows some coverage/range-based threshold to be set. For MR based measurements, it would seem likely that existing measurement quantities suffice as they cover both absolute and relative quantities.

Proposal 6: RRM relaxation entry and exit condition is related to LP-SS and/or SSB quality.

RAN2 received an LS [1] from RAN1 where it was indicated the UE may not support LP-WUS reception on all the bands supported by the UE. When LP-WUS is deployed it can be assumed that not all the cells will support LP-WUS. In some NW implementations LP-WUS support can be introduced in some specific cells or frequency(s). For the UE supporting LP-WUS it would be more saving friendly to camp on the cell which is supporting LP-WUS. It can be assumed that from system perspective it would be beneficial if the UE always camps and starts the access on the best cell of the frequency to avoid interference. Different cell (re)selection enhancements could be studied for enabling that the UE camps on the cell supporting LP-WUS more often, possibly considering adjusting or applying different priorities based on the LP-WUS support/coverage.

Proposal 7: UE supporting LP-WUS prioritizes frequencies supporting LP-WUS in the inter-frequency cell (re)selection procedure.

LP-WUS is intended to be used for the benefit of UE power saving. One approach for UE power saving is to offload some or all current measurements performed by the MR to the LR. At least in the rel-18, only serving cell measurements are performed by LR. There are currently various procedures, such as EMR (aka idle/inactive measurements) and cell reselection measurements, which aim to assist both network and UE to perform fast data transmission (e.g. with fast Scell activation) upon connection setup/resume.

The rel-18 work on fast CA/DC setup also expanded the idle-mode procedures to cover cell reselection measurements.  By using LP-WUS, the benefits of these features would be diminishing as there would be no measurements reports to provide to the network, since some or all measurements can be offloaded to LR and LR may not be able perform neighbour cell measurements. In some case even all MR measurements can be omitted when LP-WUS is used.

Proposal 8: Discuss what is the intended UE behaviour while MR relaxation is applied, and UE is configured with IDLE/INACTIVE measurements

# 3	Conclusion

The following is proposed based on the discussion:

Proposal 1: NW can configure in the system information RRM relaxation conditions for IDLE/INACTIVE.

Proposal 2: The UE starts RRM relaxation when RRM relaxation entry condition is fulfilled, and the UE stops RRM relaxation when RRM relaxation exit condition is fulfilled.

Proposal 3: When serving cell quality measured by LR is better than threshold X, MR RRM measurements are omitted.

Proposal 4: When serving cell quality measured by LR is better than threshold Y, serving and neighbour cell RRM measurements are relaxed.

Proposal 5: RRM relaxation entry and exit condition can include both LR and MR measurement evaluations. FFS whether LR and MR measurements are somehow combined or evaluated independently.

Proposal 6: RRM relaxation entry and exit condition is related to LP-SS and/or SSB quality.

Proposal 7: UE supporting LP-WUS prioritizes frequencies supporting LP-WUS in the inter-frequency cell (re)selection procedure.

Proposal 8: Discuss what is the intended UE behaviour while MR relaxation is applied, and UE is configured with IDLE/INACTIVE measurements

# 4	References

- R2-2407921 LS on LP-WUS operation in IDLE/INACTIVE mode

# 5	Annex

The following was agreed in RAN2#126 meeting:

### 8.4.3 RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE - For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell. - RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise. - RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

The following was agreed in RAN2#127 meeting:

### 8.4.3 RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE - RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs. - For serving cell measurement offloading (i.e., there is no serving cell measurement by MR): - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. - The exit condition is based on the LR measurement results.

The following was agreed in RAN2#127bis meeting:

### 8.4.3 RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE - Working assumption For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

The following was agreed in RAN2#128 meeting:

8.4.3 RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

The entry condition for serving cell RRM relaxation is at least ‘if serving cell quality measured by MR is higher than relaxation threshold, e.g. RSRP and/or RSRQ’. FFS if LR measurement is needed.

FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements

FFS if the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation.