3GPP TSG-RAN WG2 Meeting #129	R2-2500995

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.18

Source:	Nokia, InterDigital

Title:	CORESET 0 restriction for DL BWP for SDT

WID/SID:	TEI19

Document for:	Discussion and Decision

# 1	Introduction

The following discussion took place in RAN2#128 meeting:

R2-2410542	Quality Indication in Msg3 for SDT	Ericsson	discussion	Rel-19	TEI19

Observation 1	MT-SDT operation over initial BWP restricts DL MT-SDT transmissions to the bandwidth of CORESET 0 in RRC_INACTIVE state.

Observation 2	MT-SDT BWP restriction with no DL quality feedback results in smaller, more conservative TB allocations in the downlink at low MCS values. For larger data volumes, smaller TBS translate to a higher number of subsequent transmissions.

Proposal 1	RAN2 addresses the BWP restriction in MT-SDT by introducing a Channel Quality Indication (CQI) in Msg3 or MsgA.

Option 1: Remove the CORESET 0 restriction for DL BWP for SDT from/after first DL MT-SDT data

Option 2: Introduce Channel Quality Indication (CQI) in Message 3 or MsgA

-	ZTE has sympathy on the problem statement but not sure about the solution, we should just remove the restriction.   The CQI solution is not addressing the problem identified.

-	Qualcomm thinks that removing the BWP restriction is not a good choice.   If we don’t reports CQI in msg3 it will impact RAN4 as well.

-	LG thinks that the BWP restriction is the root cause and we should remove it. Nokia agrees.

-	Sony explains that a similar issues was identified in MBS and we agreed to expand coreset 0, introduce CFR.  We can reuse that solution.

- RAN2 will address this issue.  FFS the exact solution

- Noted

In this contribution we propose way forward for addressing the issue.

# 2	Discussion

For first DL MT-SDT data, the frequency domain resource assignment is provided in TS 38.212 [3] as follows.

3GPP TS 38.212 18.4.0

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by TC-RNTI: - Identifier for DCI formats - 1 bit - The value of this bit field is always set to 1, indicating a DL DCI format - Frequency domain resource assignment - bits - is the size of CORESET 0

The same restriction is applicable for subsequent transmissions as well. In TS 38.212, this is provided as follows.

3GPP TS 38.212 18.4.0 – Paragraph 7.3.1.2.1

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: - Identifier for DCI formats - 1 bits - The value of this bit field is always set to 1, indicating a DL DCI format - Frequency domain resource assignment - bits where is given by Clause 7.3.1.0

3GPP TS 38.212 18.4.0 – Paragraph 7.3.1.0

Determine DCI format 1_0 monitored in a common search space according to clause 7.3.1.2.1 where is given by - the size of CORESET 0 if CORESET 0 is configured for the cell; and - the size of initial DL bandwidth part if CORESET 0 is not configured for the cell.

According to TS 38.331 [2], the UE applies this restriction until after the reception of RRCSetup/RRCResume/RRCReestablishment when it switches to Connected mode.

The following options were discussed for solving the problem:

Option 1: Remove the CORESET 0 restriction for DL BWP for SDT from/after first DL MT-SDT data

This option involves some RAN1 specification changes and some RAN2 changes in 38.331. This option solves the identified problem.

Option 2: Introduce Channel Quality Indication (CQI) in Message 3 or MsgA

This option would require new CQI measurements and reporting procedure in MSG3 and MSGA impacting RAN1 and RAN2 specifications. It can be assumed that there would be large impact in specifying new measurements and reporting procedure. In addition, it is not clear to us how this would solve the identified problem. Therefore, we propose:

Proposal 1: Remove the CORESET 0 restriction for DL BWP for SDT

Proposal 2: Inform RAN1 about the RAN2 agreement.

# 3	Conclusion

The following is proposed based on the discussion:

Proposal 1: Remove the CORESET 0 restriction for DL BWP for SDT

Proposal 2: Inform RAN1 about the RAN2 agreement.

# 4	References

[1] R2-2410542 Quality Indication in Msg3 for SDT, Ericsson