3GPP TSG-RAN WG2 Meeting #129	R2-2500997

Athens, Greece,17 – 21 February 2025

Agenda item:	8.1.2.2

Source:	Nokia

Title:	LCM for UE-side Beam Management

WID/SID:	NR_AIML_air-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In this paper, we focus on the issues related to functionality-based LCM for UE-side AI/ML beam management. First, we review the RAN1 agreement regarding the LS sent out by RAN2.We discuss features concerning applicability reporting options proposed by RAN1 along with the aspects common to both. Then, we discuss the content for reporting applicable configurations. Next, we discuss the remaining issues concerning applicable configurations through RRC state transitions in the same cell, and performance monitoring configuration.

## 2	Discussion

## 2.1 RAN1 LS

RAN 1 made the following agreements related to the Questions from RAN 2 in RAN1#119 [1]:

Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-Report Config for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3 Agreement For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability. Conclusion For the CSI-ReportConfig for inference configuration provided in Step 5, - aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete. - periodic CSI Report is considered as activated after RRCReconfigurationComplete. - Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig - Any specification impact is a separate discussion

The following sections will discuss the options provided by RAN1 for applicability determination: A) the provision of a full inference configuration and B) the provision of one or more sets of inference-related parameters.

### 2.1.1	RAN2 impact of A) at Step 3

In A), the UE is provided with one or more full inference configurations. A full inference configuration contains all the information required by the UE to enable an AI/ML model and provide inference reports to the network. Therefore, RAN1 agreed that if a UE reports a full inference configuration candidate as applicable, that it should be considered activated. If the reportConfigType of the applicable CSI-ReportConfig is periodic, then the CSI-ReportConfig shall be considered to be activated and reporting. Otherwise, for semiPersistentOnPUCCH, semiPersistentOnPUSCH, and aperiodic, an activation command using a DCI or MAC-CE is necessary to configure the UE to activate reporting. That is, after a full inference configuration is reported to be applicable, legacy behaviour is assumed.

Observation 1: According to RAN1, a CSI-ReportConfig, configured with reportConfigType equal to periodic, is considered to be activated immediately after the UE reports through RRCReconfigurationComplete that it is applicable.

Observation 2: According to RAN1, a CSI-ReportConfig, configured with reportConfigType equal to semiPersistentOnPUCCH, semiPersistentOnPUSCH, or aperiodic, is activated using an activation command from the NW after the UE reports through RRCReconfigurationComplete that it is applicable.

Full inference configurations for beam prediction use the same CSI-ReportConfig IE as those configured for non-AI/ML CSI measurement reporting, so they can be placed in the same location as CSI measurement reporting configurations for non-AI/ML measurement reporting. That is, the full inference configurations can be configured in csi-ReportConfigToAddModList in CSI-MeasConfig.

Proposal 1: Full inference configurations for beam prediction are configured in csi-ReportConfigToAddModList under CSI-MeasConfig

According to RAN1, a full inference configuration, i.e., a full CSI-ReportConfig, reported as inapplicable in RRCReconfigurationComplete, should not be considered activated or activatable. And a full inference configuration, i.e., a full CSI-ReportConfig, reported as applicable in RRCReconfigurationComplete should be considered activated if reportConfigType is equal to periodic, and otherwise activatable through an activation command or trigger.

Additionally, the RAN1 agreement stated that “applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported.” This means that a DCI or MAC-CE command is required to activate and deactivate applicable CSI-ReportConfigs with reportConfigType equal to aperiodic or semi-persistent, and they can only be activated if they were reported as being applicable.

Proposal 2: Full inference configurations, i.e., full CSI-ReportConfigs for beam prediction, which have not been reported as applicable through RRCReconfigurationComplete shall not produce inference reports if reportConfigType is equal to periodic and shall not be activated for other reportConfigTypes.

The procedure for activating a CSI-ReportConfig containing a full inference configuration is shown in Figure 2.1.1-1.

Figure 2.1.1-1: Behaviour of CSI-ReportConfig with Full Inference Configuration

When the UE reports that a CSI-ReportConfig corresponding to a beam prediction configuration is inapplicable, it is up to the NW to take further action. To deactivate reporting of a CSI-ReportConfig with reportConfigType equal to periodic, the NW can use csi-ReportConfigToRemove to remove the configuration and stop reporting. To deactivate CSI-ReportConfig with reportConfigType not equal to periodic, the NW can use csi-ReportConfigToRemove as in the case for periodic reporting, or it can deactivate reporting using a DCI or MAC-CE command, for semiPersistentOnPUSCH and semiPersistentOnPUCCH, respectively. Aperiodic reporting deactivates automatically after the report is transmitted.

Observation 3: The only mechanism available to deactivate the reporting of a previously applicable, and now inapplicable, CSI-ReportConfig with reportConfigType equal to periodic is to remove the configuration.

RAN1 did not mention UAI in their agreement regarding the automatic activation of applicable periodic CSI-ReportConfigs reported through RRCReconfigurationComplete. Therefore, it is our assumption that in response to receiving UAI with an update to the applicability of a full inference configuration, the UE would take no autonomous action, but rather wait for instructions from the NW, e.g., to deactivate or remove a full inference configuration.

The agreements from RAN1 regarding activation are fully compatible with previous RAN2 agreements. However, we need to address the deactivation behaviour. Previously in RAN2#128 that “when a functionality becomes non-applicable the UE doesn’t autonomously deactivate.

Observation 4: RAN1 excluded UAI from their agreement that a CSI-ReportConfig, configured with reportConfigType equal to periodic, is considered to be activated immediately after the UE reports through RRCReconfigurationComplete that it is applicable.

Proposal 3: RAN2 assumes that UAI will be used to provide updates to the applicability of a CSI-ReportConfig, configured with any reportConfigType, whether from applicable to inapplicable or from inapplicable to applicable. Any actions considering the provided UAI on applicability or inapplicability is up to NW and how the UE is configured.

According to above RAN2#128 agreement that “when a functionality becomes non-applicable the UE doesn’t autonomously deactivate, the NW is expected to deactivate an activated functionality when it receives a report from UE that it is inapplicable. However, if a full inference configuration with reportConfigType=’periodic’ is provided in Step 3, the UE can report the inapplicability of a periodic beam prediction inference reporting configuration using UAI. Although the UE cannot deactivate the inference reporting autonomously, inapplicability may be reported using UAI while periodic beam prediction reporting is activated and reporting. This introduces a potential ambiguity because UE does not know what should be reported since the configuration was reported as inapplicable (See Figure 2.1.1-2).

Figure 2.1.1-2. Ambiguity of UE behaviour after an active periodic inference configuration becomes inapplicable

Observation 5: If a full inference configuration is provided in Step 3 and reportConfigType=’periodic’, the UE can report the inapplicability of a periodic beam prediction inference reporting configuration using UAI  Although the UE cannot deactivate the inference reporting autonomously, inapplicability may be reported using UAI while periodic beam prediction reporting is activated and reporting, introducing a potential ambiguity as to what should be reported since the configuration was reported as inapplicable.

One approach to resolve the above issue is to define a new UE reporting behaviour. Instead of reporting Set A beam predictions, the UE would report Set B beam measurements after the UE reports the inapplicability of a beam prediction reporting configuration. This avoids wasting configured air interface resources, but also supports performance monitoring at the NW (e.g., in case of Type-1 Option 1).

Proposal 4: To resolve ambiguities of the behaviour of the UE in the case when the UE is configured with a beam prediction configuration and its configuration becomes inapplicable during inference reporting, the UE shall suspend the transmission of Set A beam predictions and report Set B measurements corresponding to the newly inapplicable, activated beam prediction inference configuration until the UE is reconfigured by the NW.

### 2.1.2	RAN2 impact of B) at Step 3

In B), the UE is provided one or more sets of inference-related parameters to be evaluated for applicability. These sets of inference-related parameters are not sufficient on their own to configure the UE for inference, but rather require that the gNB further configures the UE after determining applicability. Unlike with A), this type of applicability determination information is completely separate from the eventual configuration.

Because the inference-related parameters proposed by RAN1 can be found in CSI-ReportConfig, we propose that the inference-related parameters are provided in CSI-ReportConfigs. Because the UE’s behaviour is different depending on whether it is provided a full inference configuration (A)) or a set of inference-related parameters, a flag should be included to indicate that a CSI-ReportConfig corresponds to inference-related parameters instead of a full inference configuration. With the flag differentiating the A) from B), the inference-related parameters can also be placed in csi-ReportConfigToAddModList in CSI-MeasConfig.

Observation 6: The inference-related parameters suggested by RAN1, including any that will be added for AI/ML, will be present in CSI-ReportConfig.

Proposal 5: CSI-ReportConfig is used to transmit inference-related parameters for beam prediction.

Proposal 6: CSI-ReportConfigs corresponding to sets of inference-related parameters are configured in csi-ReportConfigToAddModList in CSI-MeasConfig.

Proposal 7: Include an indication in CSI-ReportConfig indicating that the configuration corresponds to a set of inference-related parameters.

In B), the UE is provided one or more sets of inference-related parameters which can serve as the basis for the NW to configure the UE with an eventual full inference configuration. It is expected that the NW will be able to determine a full inference configuration for any inference-related parameters reported as applicable.

Figure 2.1.2-1 describes the behaviour of B), wherein the UE is provided inference-related parameters for applicability determination, the UE provides the NW with which sets inference-related parameters are applicable or inapplicable, the NW determines and provides a full inference configuration to the UE, and the UE activates the full inference configuration.

Figure 2.1.2-1: Behaviour of Option B with inference-related parameters

The full inference configuration provided in Step 5 will be treated the same way as if a full inference configuration were provided in Step 3, as in A). That is, the UE shall report whether the full inference configuration is applicable or inapplicable in the RRCReconfigurationComplete message in Step 6 acknowledging the configuration and follow the behaviours defined for A) in the previous section, e.g.., immediately activating a CSI-ReportConfig with reportConfigType equal to periodic.

Proposal 8: The full inference configuration provided in Step 5 after the determination of applicability of a set of inference-related parameters shall be treated the same way as a full inference configuration provided in Step 3. That is, the UE shall report applicability in the RRCReconfigurationComplete in Step 6, and if the full inference configuration configured with reportConfigType equal to periodic, and the configuration is applicable, the UE shall start producing reports immediately.

If a set of inference-related parameters becomes inapplicable and the UE provides an update to the gNB via UAI, the gNB would reconfigure the UE with a new applicable AI/ML-enabled configuration or a legacy configuration, removing, or deactivating, in the case of non-periodic configurations, inapplicable configurations from CSI-MeasConfig.

Proposal 9: If a set of inference-related parameters becomes in applicable and the UE provides an update to the gNB via UAI, it is up to gNB whether to reconfigure the UE and deactivate the inapplicable configuration.

### 2.1.3	Aspects Common to A) and B)

Whether applicability reporting is configured using Option A and B, there are some common aspects to consider.

RAN1 agreed that a full inference configuration consists of a CSI-ReportConfig. The following are excerpts, with some fields excluded, from the ASN.1 definitions of CSI-ReportConfig, CSI-MeasConfig, and the relationships between the IEs storing configurations for CSI RS resources from TS38.331. Only the ToAddMod structures are shown for brevity.

CSI-ReportConfig is configured with resourcesForChannelMeasurement, which is a pointer to a CSI-ResourceConfig. In RAN1#119, it was agreed that Set A and Set B resource configurations could be configured using CSI-ResourceConfigs [1].

CSI-ReportConfig ::=                SEQUENCE {

reportConfigId                          CSI-ReportConfigId,

resourcesForChannelMeasurement          CSI-ResourceConfigId,

CSI-ResourceConfig can contain a sequence of NZP-CSI-RS-ResourceSetIds pointing to NZP-CSI-RS-ResourceSets.

CSI-ResourceConfig ::=      SEQUENCE {

csi-ResourceConfigId        CSI-ResourceConfigId,

csi-RS-ResourceSetList      CHOICE {

nzp-CSI-RS-SSB              SEQUENCE {

nzp-CSI-RS-ResourceSetList  SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig))

OF NZP-CSI-RS-ResourceSetId OPTIONAL, -- Need R

NZP-CSI-RS-ResourceSet contains a sequence of NZP-CSI-RS-ResourceIds pointing to NZP-CSI-RS-Resources.

NZP-CSI-RS-ResourceSet ::=          SEQUENCE {

nzp-CSI-ResourceSetId               NZP-CSI-RS-ResourceSetId,

nzp-CSI-RS-Resources                SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF

NZP-CSI-RS-ResourceId,

And finally, the elements of each sequence of CSI-ResourceConfig, NZP-CSI-RS-ResourceSet, and NZP-CSI-RS-Resource are configured in CSI-MeasConfig.

CSI-MeasConfig ::=                  SEQUENCE {

nzp-CSI-RS-ResourceToAddModList     SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF

NZP-CSI-RS-Resource   OPTIONAL, -- Need N

nzp-CSI-RS-ResourceSetToAddModList  SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF

NZP-CSI-RS-ResourceSet OPTIONAL, -- Need N

csi-ResourceConfigToAddModList      SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF

CSI-ResourceConfig   OPTIONAL, -- Need N

csi-ReportConfigToAddModList        SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF

CSI-ReportConfig  OPTIONAL, -- Need N

Whether a UE is configured with a full inference configuration or with inference-related parameters to evaluate applicability, CSI-MeasConfig is the most logical place to configure the CSI resources. The CSI resources configured in CSI-MeasConfig can be used during the applicability evaluation process and during inference for active configurations.

Proposal 10: For the purpose of applicability determination and inference configuration, the same CSI resource configurations under CSI-MeasConfig, i.e., NZP-CSI-Resource, NZP-CSI-ResourceSet, and CSI-ResourceConfig, can be used.

It was previously proposed to include a flag in CSI-ReportConfig to differentiate inference-related parameters from full inference configurations. We think it could be useful for the flag to indicate either that the configuration is a full inference configuration or a set of inference-related parameters. That is, a flag could be introduced to indicate that the configuration is subject to applicability determination and also whether it represents a full inference configuration or a set of inference-related parameters.

Observation 7: It may be useful to indicate whether a CSI-ReportConfig contains a full inference configuration or a set of inference-related parameters and whether it is subject to the applicability determination procedure.

Proposal 11: Include a field in CSI-ReportConfig which could be, e.g., Optional and Enumerated. When present, the field indicates that the configuration is subject to the applicability configuration procedure. The field can store one of two values: full configuration and partial configuration.

## 2.2	Content of Applicability Report

Irrespective of whether a full inference configuration (A)) or a set of inference-related parameters (B)) is provided in Step 3, UAI is used by the UE to signal updates to previously signalled applicability information.

In RAN2#128, it was left “FFS whether the UE explicitly reports an inapplicable functionality when there is a change of applicability. Verify this aligns with RAN1 configuration design.” Based on current discussion, it seems that the applicability report will be fairly simple and small, that is, consisting of identifiers and applicability indications.

Additionally, we find it could be useful for the UE to be able to report reasons for inapplicability such as changes of prediction accuracy or evaluation of UE internal conditions. For instance, the performance of an inference configuration may change as the UE moves through the cell. Using the additional information, the NW can more easily configure and activate inference configurations with better performance. The ongoing discussion in RAN1 about performance monitoring metrics (e.g., beam prediction accuracy or RSRP prediction accuracy) may also be used to decide which metrics to include in the applicability reporting.

Observation 8: It may be useful for the UE to provide the NW with a reason for inapplicability or expected performance information of inapplicable and/or applicable configurations.

To support the provision of reasons for inapplicability or expected performance information, it must be possible to explicitly signal inapplicability.

Proposal 12: Support the explicit reporting of inapplicability.

Proposal 13: Support reporting specific reasons for applicability and inapplicability alongside the applicability indication such as changes of prediction accuracy level.

When reporting applicability, because we have proposed that inference-related parameters and full configurations are provided as CSI-ReportConfigs in csi-ReportConfigToAddMod in CSI-MeasConfig, the applicability of either type of CSI-ReportConfig should be reported the same way. That is, the applicability report should include a list of CSI-ReportConfigIds and their corresponding applicability. In this way, full inference configurations (A)) and sets of inference-related parameters (B)) can coexist in the specification and in implementation.

Proposal 14: Support combined reporting of applicability of full inference configurations, A), and of inference-related parameters, B), where full configurations and inference-related parameters are configured in the same csi-ReportConfigToAddMod/RemoveList, sharing the same set of CSI-ReportConfigIds. Each CSI-ReportConfigId used in applicability reporting will be reported as applicable or inapplicable in the report.

Lastly, the behaviour for initial configuration has been provided, but it isn’t clear what the UE should do when a full inference configuration or a set of inference-related parameters for beam prediction is modified through the ToAddMod structure, which would be used for A), and possibly for B). The simplest approach would be to treat a modification to a configuration the same way as the addition. That is, anytime a full inference configuration or set of inference-related parameters is modified, the applicability determination procedure should be triggered.

Proposal 15: The modification of a full inference configuration or set of inference-related parameters triggers an applicability determination and reporting procedure.

## 2.3	Fallback

Fallback to non-AIML means de-configuring one AI/ML beam prediction configuration and activating a non-AI/ML beam prediction configuration. The purpose of fallback is to mitigate performance losses by reverting to a non-AI/ML method for beam management or beam measurement reporting. However, the NW may fall into an unnecessary ping-pong between non-AI/ML and AI/ML-enabled beam prediction configurations, which could lead to signalling overhead to activate or deactivate an AI/ML configuration on one hand and imposing unnecessary activation delays on the other hand.

Because the performance of the AI/ML beam prediction configuration was satisfactory prior to the decision to fall back, it could be reasonably assumed that the AI/ML configuration will perform well once again.

Observation 9: Because the performance of an AI/ML beam prediction configuration could have been satisfactory prior to the decision to fall back, it could be reasonably assumed that the AI/ML configuration will perform well once again.

Therefore, monitoring a beam prediction configuration during fallback could be useful to help the NW determine when to exit fallback and resume the use of the beam prediction configuration to make beam prediction decisions. During this type of fallback, the beam prediction reporting configuration would still be active, but it could be configured with a lower reporting frequency.

Proposal 16: Discuss whether during fallback if the UE can be configured to report inference outputs, e.g., at a lower periodicity, of a configuration which performed poorly after initially performing well to aid the NW in exiting fallback.

## 2.4	Performance Monitoring

### 2.4.1	Overview

During the Release 18 SI, two types were introduced for performance monitoring of UE-side functionalities.

Type 1, Option 1, enables the NW to make LCM decisions based on performance monitoring metrics calculated at the NW. Type 1, Option 2, also known as UE-assisted performance monitoring, enables the NW to make LCM decisions based on performance monitoring metrics calculated and reported by the UE to the NW. Type 1 performance monitoring, including Option 1 and Option 2, may require signalling enhancements such that the UE can be configured to report additional measurements for use by the NW in calculating a performance metric or such that the UE can be configured to calculate and report a performance metric to the NW.

Type 2 performance monitoring enables the UE to make LCM decisions based on performance monitoring metrics calculated by the UE. Type 2 does not require any RAN2-related signalling enhancements as it is mainly applicable to model-level performance monitoring.

In RAN1#118 [2], four alternatives were agreed as performance monitoring metric for further study. As such, any discussions on the specific signaling or triggering of additional measurement reporting or KPI calculation in RAN2 should be discussed after RAN1 progresses on deciding the performance monitoring metric.

Observation 10: Performance monitoring configuration is controlled by the NW for Type 1 Option 1 and Option 2. If   Option 2 is configured, the metrics to be reported need to be specified. Performance monitoring metrics calculation for Option 1 is up to NW implementation.

### 2.4.2	Type 1 Performance Monitoring

In every case of Type 1 performance monitoring, an AI/ML configuration is activated, and additional information is used to determine the quality of the performance of the AI/ML configuration.

UE is configured with L1-RSRP measurements for Inference by the network. This configuration includes measurements required as inputs to the beam prediction configuration, e.g., measurements of Set B for beam prediction Case-1 or measurements of Set A within the observation window for beam prediction Case-2. The beam prediction inference, e.g., L1-RSRP and/or RS index, are reported to the network.

If UE is configured with Type-1 Option-1, then UE is also configured with L1-RSRP measurements for monitoring, that are required for network -based KPI calculation. These measurements are also reported by the UE. After receiving these monitoring measurement reports, network can perform KPI calculation based on the monitoring measurement reports and predicted measurement reports. Then, the network uses the output of the network -based KPI calculation to make LCM decisions.

On the other hand, if UE is configured with Type-1 Option-2, then UE is configured with L1-RSRP measurements for monitoring, which are used for UE-based KPI calculation. UE-based KPIs are calculated and reported to the network and network makes LCM decisions based on these reports.

Observation 11: Based on discussions on candidates for KPI calculation for AI/ML beam prediction configuration performance monitoring in RAN1, for all Type 1 performance monitoring options, the outputs of the activated beam prediction configuration are used by the NW (Option 1) or by the UE (Option 2) for KPI calculation.

Observation 12: Based on discussions on candidates for KPI calculation for AI/ML beam prediction configuration performance monitoring in RAN1, for Type 1 Option 1, additional L1-RSRP measurements are reported to the NW alongside the configuration outputs, and for Type 1 Option 2, performance monitoring KPIs are reported to the NW alongside the configuration outputs.

Proposal 17: RAN2 only to support signalling enhancements for Type 1 (Option 1 and 2) of an activated AI/ML beam prediction configuration through reporting, e.g., additional L1-RSRP measurements or performance monitoring KPIs for both beam prediction-Case1 and beam prediction-Case2.

# 3	Conclusion

This document has made the following observations:

Observation 1: According to RAN1, a CSI-ReportConfig, configured with reportConfigType equal to periodic, is considered to be activated immediately after the UE reports through RRCReconfigurationComplete that it is applicable.

Observation 2: According to RAN1, a CSI-ReportConfig, configured with reportConfigType equal to semiPersistentOnPUCCH, semiPersistentOnPUSCH, or aperiodic, is activated using an activation command from the NW after the UE reports through RRCReconfigurationComplete that it is applicable.

Observation 3: The only mechanism available to deactivate the reporting of a previously applicable, and now inapplicable, CSI-ReportConfig with reportConfigType equal to periodic is to remove the configuration.

Observation 4: RAN1 excluded UAI from their agreement that a CSI-ReportConfig, configured with reportConfigType equal to periodic, is considered to be activated immediately after the UE reports through RRCReconfigurationComplete that it is applicable.

Observation 5: If a full inference configuration is provided in Step 3 and reportConfigType=’periodic’, the UE can report the inapplicability of a periodic beam prediction inference reporting configuration using UAI  Although the UE cannot deactivate the inference reporting autonomously, inapplicability may be reported using UAI while periodic beam prediction reporting is activated and reporting, introducing a potential ambiguity as to what should be reported since the configuration was reported as inapplicable.

Observation 6: The inference-related parameters suggested by RAN1, including any that will be added for AI/ML, will be present in CSI-ReportConfig.

Observation 7: It may be useful to indicate whether a CSI-ReportConfig contains a full inference configuration or a set of inference-related parameters and whether it is subject to the applicability determination procedure.

Observation 8: It may be useful for the UE to provide the NW with a reason for inapplicability or expected performance information of inapplicable and/or applicable configurations.

Observation 9: Because the performance of an AI/ML beam prediction configuration could have been satisfactory prior to the decision to fall back, it could be reasonably assumed that the AI/ML configuration will perform well once again.

Observation 10: Performance monitoring configuration is controlled by the NW for Type 1 Option 1 and Option 2. If   Option 2 is configured, the metrics to be reported need to be specified. Performance monitoring metrics calculation for Option 1 is up to NW implementation.

Observation 11: Based on discussions on candidates for KPI calculation for AI/ML beam prediction configuration performance monitoring in RAN1, for all Type 1 performance monitoring options, the outputs of the activated beam prediction configuration are used by the NW (Option 1) or by the UE (Option 2) for KPI calculation.

Observation 12: Based on discussions on candidates for KPI calculation for AI/ML beam prediction configuration performance monitoring in RAN1, for Type 1 Option 1, additional L1-RSRP measurements are reported to the NW alongside the configuration outputs, and for Type 1 Option 2, performance monitoring KPIs are reported to the NW alongside the configuration outputs.

And the following proposals:

Proposal 1: Full inference configurations for beam prediction are configured in csi-ReportConfigToAddModList under CSI-MeasConfig

Proposal 2: Full inference configurations, i.e., full CSI-ReportConfigs for beam prediction, which have not been reported as applicable through RRCReconfigurationComplete shall not produce inference reports if reportConfigType is equal to periodic and shall not be activated for other reportConfigTypes.

Proposal 3: RAN2 assumes that UAI will be used to provide updates to the applicability of a CSI-ReportConfig, configured with any reportConfigType, whether from applicable to inapplicable or from inapplicable to applicable. Any actions considering the provided UAI on applicability or inapplicability is up to NW and how the UE is configured.

Proposal 4: To resolve ambiguities of the behaviour of the UE in the case when the UE is configured with a beam prediction configuration and its configuration becomes inapplicable during inference reporting, the UE shall suspend the transmission of Set A beam predictions and report Set B measurements corresponding to the newly inapplicable, activated beam prediction inference configuration until the UE is reconfigured by the NW.

Proposal 5: CSI-ReportConfig is used to transmit inference-related parameters for beam prediction.

Proposal 6: CSI-ReportConfigs corresponding to sets of inference-related parameters are configured in csi-ReportConfigToAddModList in CSI-MeasConfig.

Proposal 7: Include an indication in CSI-ReportConfig indicating that the configuration corresponds to a set of inference-related parameters.

Proposal 8: The full inference configuration provided in Step 5 after the determination of applicability of a set of inference-related parameters shall be treated the same way as a full inference configuration provided in Step 3. That is, the UE shall report applicability in the RRCReconfigurationComplete in Step 6, and if the full inference configuration configured with reportConfigType equal to periodic, and the configuration is applicable, the UE shall start producing reports immediately.

Proposal 9: If a set of inference-related parameters becomes in applicable and the UE provides an update to the gNB via UAI, it is up to gNB whether to reconfigure the UE and deactivate the inapplicable configuration.

Proposal 10: For the purpose of applicability determination and inference configuration, the same CSI resource configurations under CSI-MeasConfig, i.e., NZP-CSI-Resource, NZP-CSI-ResourceSet, and CSI-ResourceConfig, can be used.

Proposal 11: Include a field in CSI-ReportConfig which could be, e.g., Optional and Enumerated. When present, the field indicates that the configuration is subject to the applicability configuration procedure. The field can store one of two values: full configuration and partial configuration.

Proposal 12: Support the explicit reporting of inapplicability.

Proposal 13: Support reporting specific reasons for applicability and inapplicability alongside the applicability indication such as changes of prediction accuracy level.

Proposal 14: Support combined reporting of applicability of full inference configurations, A), and of inference-related parameters, B), where full configurations and inference-related parameters are configured in the same csi-ReportConfigToAddMod/RemoveList, sharing the same set of CSI-ReportConfigIds. Each CSI-ReportConfigId used in applicability reporting will be reported as applicable or inapplicable in the report.

Proposal 15: The modification of a full inference configuration or set of inference-related parameters triggers an applicability determination and reporting procedure.

Proposal 16: Discuss whether during fallback if the UE can be configured to report inference outputs, e.g., at a lower periodicity, of a configuration which performed poorly after initially performing well to aid the NW in exiting fallback.

Proposal 17: RAN2 only to support signalling enhancements for Type 1 (Option 1 and 2) of an activated AI/ML beam prediction configuration through reporting, e.g., additional L1-RSRP measurements or performance monitoring KPIs for both beam prediction-Case1 and beam prediction-Case2.

# 4	References

[1] R1-2410898, “Reply LS on applicable functionality reporting for beam management UE-sided model”, Orlando, Florida, RAN1, November 2024

[2] R1-2407554, “FL summary #5 for AI/ML in beam prediction, 3GGP TSG RAN WG1 #118 – August 19th – August 23rd”, August 2024