3GPP TSG-RAN WG2 Meeting #129	R2-2500998

Athens, Greece, 17 – 21 February 2025

Agenda item:	8.1.5

Source:	Nokia

Title:	Discussion on RAN1 LS on Dataset and Parameter Transfer (R1-2410922/R2-2500015)

WID/SID:	FS_NR_AIML_air - Release 19

Document for:	Discussion and Decision

# 1	Introduction

RAN1 sent an LS to RAN2 in R1-2410922/R2-2500015 to request an analysis on the feasibility of dataset sharing, encoder parameter sharing, and encoder parameter + dataset sharing over the air or by using other approaches.

# 2	Discussion

It is our understanding that the purpose of dataset and/or encoder parameter sharing is to aid in the training of UE-side encoder models by providing training data or an exemplary model, in the form of parameters. We also understand that at least in Release 19, it is not expected that UEs will train their own models, but rather that servers controlled by the UE vendors and/or chipset vendors will be responsible for model training. Given that models will be trained offline, and according to the LS that models will be trained or updated infrequently, we see no reason to burden the air interface with dataset or model parameter transfer.

## 2.1	Implication and Usefulness of Using Air Interface

Given that UE-side AI/ML models will be trained offline at a server for training UE-side AI/ML models, the destination for the datasets and parameter sets described in the RAN1 LS is the server for training UE-side AI/ML models. Therefore, the use of the air interface to transfer datasets and parameter sets for use in AI/ML model training implies that UEs would be used as relays between the NW and the server for training UE-side AI/ML models. While it is technically feasible to use the UEs as relays, it isn’t worth the standardization effort since the vast majority of UEs would not be required to act as a relay for this data, since each unique set of data only needs to arrive at the server once.

Observation 1: To transfer datasets and parameter sets for use in training UE-side models over the air when UE-side models are to be trained offline at a server implies that UEs would act as relays for the data transfer.

Additionally, because the data would first be transmitted to the UE prior to being transmitted from the UE to the server for training UE-side AI/ML models, the air resource utilization would be at least double that of transmitting the data through the core network or OAM to the server.

Observation 2: Using UEs as a relay between the NW and the server for training UE-side AI/ML models would require air interface resources equal to twice the size of the datasets and/or parameter sets being transmitted: once over the downlink to provide the data to the UE, and once over the uplink for the UE to provide the same data to a server.

We suggest sending an LS to SA2 and SA5, who are working on data collection aspects in the core network and the OAM, respectively, since those WGs could assist in methods for transferring datasets, which may reside in a network function (NF) or in OAM, such as in the trace collection entity (TCE), directly from the NW to the server for training UE-side models without involving the UE.

Proposal 1: Respond to RAN1, including SA2 and SA5 as recipients in the To line suggesting that RAN1 works with SA2 and/or SA5 to determine how to transfer datasets and parameter sets from the NW to a server for training UE-side models.

A draft LS has been provided in Section 4.

# 3	Conclusion

In this TDoc, the following observations were made:

Observation 1: To transfer datasets and parameter sets for use in training UE-side models over the air when UE-side models are to be trained offline at a server implies that UEs would act as relays for the data transfer.

Observation 2: Using UEs as a relay between the NW and the server for training UE-side AI/ML models would require air interface resources equal to twice the size of the datasets and/or parameter sets being transmitted: once over the downlink to provide the data to the UE, and once over the uplink for the UE to provide the same data to a server.

And the following proposals were made:

Proposal 1: Respond to RAN1, including SA2 and SA5 as recipients in the To line suggesting that RAN1 works with SA2 and/or SA5 to determine how to transfer datasets and parameter sets from the NW to a server for training UE-side models.

## 4	LS Reply to RAN1

3GPP TSG-RAN WG2 Meeting #129	R2-250xxxx

Athens, Greece, 17 – 21 February 2025

Title:	[DRAFT] LS Reply to RAN1 LS on signalling feasibility of dataset and parameter sharing

Response to:	R1-2410922/R2-2500015

Release:	Release 19

Work Item:	FS_NR_AIML_air

Source:	Nokia [TSG RAN WG2]

To:	RAN1, SA2, SA5

Cc:	SA3

Contact Person:

#### Name:	Jerediah Fevold

E-mail Address:	jerediah.fevold@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 would like to thank RAN1 for sending LS on signalling feasibility of dataset and parameter sharing.

We have evaluated the three options for transferring NW-side model parameters and datasets and have come to the same conclusion on each. Since UE-side AI/ML models will be trained offline at a “UE-side” server, or a server for training UE-side AI/ML models, the UEs do not need to be involved in the transfer of NW-side model parameters and datasets. These data could be transferred from the NW entity storing the data to the UE-side server without involving the UE.

Using the air interface would require the use of UEs as relays for the data and would result in a duplicate transmission of the data: first from the NW to the UE, and then from the UE to the UE-side server.

Therefore, RAN2 recommends asking SA2, since they are working on UE-side data collection using the core network, and SA5, since they are working on UE-side data collection using OAM, about solutions that require no involvement of the UE.

2. Actions:

To: RAN1.

ACTION: 	RAN2 respectfully asks RAN1 to consider our recommendation.

To: SA2, SA5.

ACTION: 	RAN2 respectfully asks SA2 and SA5 to reply RAN1 about solutions that require no involvement of the UE.

3. Date of Next TSG-RAN WG2 Meetings:

RAN2#129-bis	from 2025-04-07	to 2025-04-11		China, CN

RAN2#130	from 2025-05-19	to 2025-05-23		Malta, MT

RAN2#131			from 2025-08-25	to 2025-08-29		India (TBC), IN