3GPP TSG-RAN WG2 Meeting #129	 R2-2501001

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.6.4

Source:	KDDI Corporation

Title:	Discussion on Conditional Intra-CU LTM

WID:       	NR_Mob_Ph4-Core

Tdoc Type:	Discussion

Document for: 	Discussion and decision

- Introduction

In RAN #105, the Rel-19 WID on NR mobility enhancements Phase 4 was updated [1], where the objectives relevant to conditional LTM (CLTM) are described below:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

In RAN2#127bis, the following progress was made:[2]

Agreements on C-LTM

- Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions.

- Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions.

- Source cell and each candidate cell provides its own execution condition for conditional LTM.

- It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used.

- RACH-less Conditional intra-CU LTM is supported.

- RACH based conditional intra-CU LTM is supported.

- UE based TA measurement mechanism is supported for conditional intra-CU LTM.

- PDCCH ordered early TA acquisition is supported for conditional LTM.

- Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM.

- For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

- The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

In RAN2#128, the following progress was made:[3]

Agreements on C-LTM:

- The triggering condition of conditional LTM can be based on L3 measurement.

- CondEventA3 and CondEventA5 conditions can be baseline for the conditional LTM execution.

- The L1 execution condition of a candidate cell is associated to only one triggering event.

- For L3 execution condition, it may consist of one or two triggering condition(s). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met. Only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell.

- To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition:

- The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell.

- The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell.

- The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH.

- For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration.

- The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4.

- The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

- Candidate cell TA is maintained by a new timer.

- For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

In this contribution, we will discuss open issues form RAN2#128 and share our views on CLTM for “Specify support of conditional Intra-CU LTM” described in the WID objective above.

- Discussion

2.1 Beam(s) used for C-LTM event evaluation

In RAN2#128, two proposals were discussed regarding how and when UE evaluate the execution condition for candidate beam and triggers the LTM cell switch procedure but not reached to make decision below:[3]

9. Beam(s) used for C-LTM event evaluation R2-2409595 Discussion on Conditional Intra-CU LTM CATT discussion Rel-19 NR_Mob_Ph4-Core Proposal 13 (modified): UE evaluates the execution condition for candidate beam and triggers the LTM cell switch procedure when at least one beam fulfills the associated execution condition. R2-2409617 Discussion on conditional LTM LG Electronics Inc. discussion Rel-19 NR_Mob_Ph4-Core Proposal 11: RAN2 to consider at least one of the following approaches for event evaluation based on multiple beams: - Approach1) UE derives a consolidated quality of multiple beams of candidate cell based on a method similar to the cell quality derivation as specified in 5.5.3.3a of 38.331 and use the consolidated result for evaluation CLTM execution condition. - Approach2) Network configures the minimum number of good candidate cell beams for CLTM event (event 3/4/5), say N. If N is configured for Event CLTM, and UE considers the CLTM execution condition to be met if at least N beams of the candidate cell satisfy the CLTM condition respectively. The value range of N is {1, 2, 3, 4}. Discussion: [IDC]: If L3 measurement is used for C-LTM, do we still need anything new? [Apple]: For C-LTM, consideration of one beam should be enough. [Qualcomm]: For L3 measurement, approach 1. For L1 measurement, per beam makes sense. [CMCC]: For L1 measurement, approach 2 is preferred. [MediaTek]: Prefer CATT proposal. Will revisit it next meeting.

If event evaluation relies on a single beam, the effectiveness of the cell switch may decrease if the quality of that beam is poor. By considering the integrated quality of multiple beams, the overall communication quality is likely to improve, leading to increased communication stability. More detail analyzation is below.

| Option | Pros | Cons |
| --- | --- | --- |
| One single beam | Simplicity: The approach of triggering the LTM cell switch when at least one beam meets the execution condition simplifies the decision-making process for the UE. Quick Response: Allows for faster switching between cells | Quality Dependence: If the selected beam is of poor quality, it can lead to reduced effectiveness of the cell switch, potentially affecting the overall communication quality and user experience. Limited Evaluation: Relying on a single beam may not account for variability in the communication environment, making it less robust against changes in signal quality. |
| Multiple beams | Improved Stability: By evaluating multiple beams, this approach increases the likelihood of maintaining a reliable connection, as it considers integrated quality instead of depending on a single beam. Flexibility: The network can configure the minimum number of beams, allowing adaptability to different network conditions and performance requirements. | Complexity: Additional complexity in evaluating multiple beams can make the implementation more challenging and may result in slower decision-making compared to simpler approaches. Configuration Overhead: Requires ongoing configuration of the network to set appropriate thresholds and the minimum number of beams, which may add to operational overhead. |

Proposal 1: For L3 measurements, Approach 1. UE derives a consolidated quality of multiple beams of candidate cell based on a method similar to the cell quality derivation as specified in 5.5.3.3a of 38.331 and use the consolidated result for evaluation CLTM execution condition.

Proposal 2: For L1 measurements, Approach 2. Network configures the minimum number of good candidate cell beams for CLTM event (event 3/4/5), say N. If N is configured for Event CLTM, and UE considers the CLTM execution condition to be met if at least N beams of the candidate cell satisfy the CLTM condition respectively.

2.2 Beam selection in RACH-less LTM with CG

UE Empowerment: By allowing the UE to choose the configured grant, UE can select the optimal beam based on specific communication environments or conditions. This enables the UE to adapt its choices to the current network situation, which can vary widely.

Quality Assurance through Network Settings: Additionally, by setting RSRP thresholds in the network, the choice can be ensured to be based on quality beams that meet certain conditions. This means that even though the UE is making the selection, it is doing so from a pool of beams that are guaranteed to satisfy minimum quality conditions, thereby improving overall communication reliability.

Adaptive Beam Selection: This includes selecting beams based on either the best communication quality or the first reference signal meets the specified threshold. This flexibility allows the UE to maintain optimal communication performance by quickly adapting to changing conditions within the network.

Overall, the autonomy of the UE in beam selection with quality control measures provided by the network aims to enhance the efficiency and reliability of the RACH-less conditional LTM process.

Proposal 3: In the case of CG-based RACH-less conditional LTM, the configured grant is selected by the UE based on the quality of the relevant beams. The beams associated with the configured grant are selected from either the reference signal with the best communication quality or the first reference signal found that meets a specified quality (threshold) or higher.

- Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1: For L3 measurements, Approach 1. UE derives a consolidated quality of multiple beams of candidate cell based on a method similar to the cell quality derivation as specified in 5.5.3.3a of 38.331 and use the consolidated result for evaluation CLTM execution condition.

Proposal 2: For L1 measurements, Approach 2. Network configures the minimum number of good candidate cell beams for CLTM event (event 3/4/5), say N. If N is configured for Event CLTM, and UE considers the CLTM execution condition to be met if at least N beams of the candidate cell satisfy the CLTM condition respectively.

Proposal 3: In the case of CG-based RACH-less conditional LTM, the configured grant is selected by the UE based on the quality of the relevant beams. The beams associated with the configured grant are selected from either the reference signal with the best communication quality or the first reference signal found that meets a specified quality (threshold) or higher.

- References

[1] RP-2402356 Revised Work Item: NR mobility enhancements Phase 4 (Apple Inc, China Telecom)

[2] RAN2 #127bis Report from session on LTE V2X and NR SL (Vice Chairman: Samsung)

[3] RAN2 #128 Report from session on LTE V2X and NR SL (Vice Chairman: Samsung)