3GPP TSG-RAN WG2 Meeting #129	R2-2501060

Athens, Greece, Feb 17th – 21st, 2025

Agenda Item:	8.7.5

Source:	Huawei, HiSilicon

Title:	Discussion on RLC AM enhancements

Document for: Discussion and Decision

# 1	Introduction

In the Rel-19 XR WID approved in RAN#103 [1], one of the objectives is about the RLC re-transmission enhancements for RLC AM. In this contribution, we will provide our views on unnecessary retransmission avoidance and timely RLC retransmission.

# 2	Discussion on avoiding unnecessary transmission

In RAN2#127-bis [2], the combined approach was agreed to avoid unnecessary RLC AM retransmission for the outdated RLC SDU (segment), in which the Tx RLC entity stops retransmitting the outdated data and the Rx RLC entity stops waiting for the outdated data based on a new timer. It is important for the Tx RLC and Rx RLC entities to know how to determine the outdated data respectively.

## 2.1	Determination of the outdated data at Rx RLC entity

In order to determine the outdated RLC SDUs at the Rx RLC entity, a new timer will be introduced at the Rx RLC entity, as confirmed in RAN2#128 [3] below:

- A new RLC timer at the Rx is introduced to determine obsolete RLC SDUs. The timer starts when the gap is detected at RLC layer.

In such approach, the Rx RLC entity can stop receiving the RLC SDU (segment) based on a new timer, which can be maintained per RLC SDU or per Rx RLC entity. To achieve this, we believe that at least two possible timer based options have been observed:

- 2.1.1 Option 1 Timer maintained for each RLC SDU

When there is an RLC SN gap, e.g. RX_Next_Highest > RX_Next + 1, the Rx RLC entity can start a Timer for each missing RLC SDU.  Upon the expiry of the Timer, the Rx RLC entity considers the associated missing RLC SDU as outdated and updates the receiving window and the Status Report accordingly.

Figure 1 gives an example for new timer maintained for each RLC SDU. In Figure 1, the RLC SDUs labelled with SN = 1,2,3 and 4 are transmitted, where the RLC SDUs of SN = 1, 2 and 3 are not successfully received. As a consequence, the resultant RX_Next_Highest > RX_Next + 1 allows the Rx RLC entity to identify the missing RLC SDUs, hence starting their timers, respectively. Moreover, after receiving the RLC Status Report, the Tx RLC entity performs retransmission of RLC SDUs of SN = 1, 2 and 3 together with initial transmission of RLC SDUs of SN = 5 and 6 to the Rx RLC entity, where the RLC SDUs of SN = 2 and 6 are successfully received, the RLC SDUs of SN = 1 and 3 are still received incompletely, and the RLC SDU of SN = 5 is missing. Then, the Rx RLC entity stops the timer of RLC SDU of SN = 2, and starts the timer of RLC SDU of SN = 5. After a while, the timer of RLC SDUs with SN = 1 and 3 expire, the Rx RLC entity stops waiting for RLC SDUs of SN = 1 and 3, and updates the variable RX_Next accordingly.

Figure 1 Timer maintained per RLC SDU for the Rx initiated RLC AM retransmission enhancement

- 2.1.2 Option 2 Timer maintained for the Rx RLC entity

When there is an RLC SN gap, e.g. RX_Next_Highest > RX_Next + 1, the Rx RLC entity can start a Timer.  Upon the expiry of the Timer, the Rx RLC entity considers the missing RLC SDU(s) whose SN falls within RX_Next and RX_Discard_Trigger as outdated and updates the receiving window and the Status Report accordingly, where RX_Discard_Trigger can be the variable to record the SN triggering the Timer.

Figure 2 illustrates an example for a single new timer maintained for the Rx RLC entity. Instead of maintaining a timer for each RLC SDU, a timer similar to t-reassembly, referred as t_Discard below, can be maintained for the Rx RLC entity. Meanwhile, a new variable RX_Discard_Trigger can be introduced at the Rx RLC entity to determine the outdated RLC SDUs. When the timer t_Discard expires, the Rx RLC entity will consider the unsuccessfully received RLC SDUs with SN between RX_Next and RX_Discard_Trigger as outdated, and then updates RX_Next and RX_ Discard_Trigger accordingly.

For example, the RLC SDUs with SN = 1,2,3 and 4 are transmitted, where the RLC SDUs of SN = 1, 2 and 3 are not successfully received. The resultant SN gap drives the Rx RLC entity to start the new timer t_Discard and meanwhile to set the new variable RX_Discard_Trigger to RX_Next_Highest, i.e. SN = 5, as shown in Figure 2 (a).  Moreover, after receiving the RLC Status Report, the Tx RLC entity performs retransmission of RLC SDUs of SN = 1, 2 and 3 together with initial transmission of RLC SDUs of SN = 5 and 6 to the Rx RLC entity, where the RLC SDUs of SN = 2 and 6 are successfully received, and the RLC SDUs of SN = 1, 3 and 5 are unsuccessfully received, as shown in Figure 2(b). After a while, as the timer t_Discard  expires, the Rx RLC entity considers the RLC SDUs with SN between RX_Next and RX_Discard_Trigger, i.e. SN = 1 and 3, as outdated, and hence stops receiving their RLC SDUs. As a consequence depicted in Figure 2(c), the Rx RLC entity updates RX_Next to SN = 5. Since an SN gap still remains, i.e. RX_Next_Highest > RX_Next + 1, the timer t_Discard will be started again, and RX_Discard_Trigger is set to SN = 7.



(a)



(b)



(c)

Figure 2 Timer maintained per entity for the Rx initiated RLC AM retransmission enhancement

It is evident that Option 1 provides a more accurate discarding in comparison to Option 2, at the expense of more processing power utilization. As there can be hundreds of PDUs within a PDU set, the processing cost for maintaining these timers will be significant. However, we believe that considering the unaffordable processing cost at the Rx RLC entity, the benefit of such precise discarding offered by Option 1 is marginal. Since the PDUs within a PDU set are expected to be transmitted within the same PSDB, a single timer with maximum running time larger than the PSDB can avoid inaccurate discarding. Moreover, as the Tx RLC entity can stop retransmitting the outdated RLC SDUs, there will be no capacity loss even if the Rx RLC entity keeps sending NACK for the RLC SDUs whose remaining time for transmission has run out. To elaborate more with Figure 2 (b), compared to Option 1 where a new time for RLC SDU 6 in can be started as soon as the missing SDU is detected, in Option 2 the RX_Discard_Trigger can only be updated upon its expiry, which may introduce additional latency in discarding. However, as the Tx RLC entity can stop the retransmission when the RLC SDU is outdated, the delayed timer in Option 2 brings no capacity loss. Based on the analysis, the details of the procedure for Option 2 can be found in Appendix A.

Proposal 1:	RAN2 specifies a new timer per Rx RLC entity to determine the obsolete data, when there is an RLC SN gap, as shown in Appendix A.

- 2.1.3 Rx RLC entity operations upon new timer expiry

We believe that at least two procedures should be performed at the Rx RLC entity when the new timer expires:

- Discard the RLC SDU segment with SN falling within the range RX_Next <= SN < RX_Discard_Trigger;

- Update RX_Next to the SN of the first RLC SDU with SN >= RX_Discard_Trigger for which RLC SDU is not abandoned and all bytes have been received.

Proposal 2:	When the new timer expires, the Rx RLC entity should discard the outdated RLC SDU(s) and update the reassembly window accordingly, i.e. set RX_Next to the SN of the first RLC SDU with SN >= RX_Discard_Trigger for which RLC SDU is not abandoned and all bytes have been received, as detailed in Appendix A.

Furthermore, another issue is on the reuse of the Status Report by the following RAN2 agreement:

- Rx informs Tx side about the abandoned SDUs, as a baseline we assume existing SR can be reused unless issues are identified

With the introduction of a new timer and the agreement above on the transmission of Status Report from the Rx RLC entity to the Tx RLC entity, we propose the following on the triggering of RLC Status Report. The details can be found in TP in Appendix A.

Proposal 3: When the new timer expires, RLC Status Report is triggered, as shown in Appendix A.

## 2.2	Determination of the outdated data at Tx RLC entity

- 2.2.1 discardTimer based Tx RLC entity

For the sake of avoiding resource waste, RAN2#127-bis [2] agreed to allow the Tx RLC entity to stop the retransmission of the outdated RLC SDU, as shown below:

- RAN2 will adopt a “combined” approach for avoiding unnecessary RLC retransmissions, i.e. - TX side stops transmissions of an outdated SDU

The outdated RLC SDU becomes less useful and should be discarded, and we believe that the existing PDCP discardTimer can be applied to determine which RLC SDUs are outdated. First of all, the discardTimer is configured for each PDCP SDU and hence is able to precisely record the remaining time for each associated RLC SDU. Moreover, the expiry of discardTimer usually indicates that the data has reached its maximum transmission delay budget and it should be discarded for the sake of avoiding resource waste. Therefore, the Tx RLC entity can consider an RLC SDU as outdated upon the expiry of its associated discardTimer .

Proposal 4: The Tx RLC entity considers an RLC SDU as outdated upon the expiry of its associated discardTimer.

- 2.2.2 Discarding of Important XR Data

In the combined approach, the Tx RLC entity has the knowledge of the outdated RLC SDU based on their remaining time and hence determines whether to stop the retransmission. However, according to TS 26.522[4], the RLC SDU should not be randomly discarded, as given below:

In general, whenever the RAN needs to discard packets (e.g., under congestion situations), it is better to discard packets of lower importance rather than discarding packets randomly. If a discarded packet is critical for the media stream, the QoE may be severely degraded. For this reason, the PDU Set Importance (PSI) field can be used to mark PDU Sets with their importance level. The PSI field can then be used by the RAN to discard PDU sets. The PSI field can then be used by the RAN to discard PDU sets. In case of congestion, PDU Sets with higher PSI values are more likely to be discarded.

In other words, we can conclude that some frames can significantly contribute to the decoding of their successive frames that have dependency on them. In such case, their successive frames cannot be successfully decoded and displayed even if they are transmitted correctly. Since the new timer at Rx RLC entity starts later than the time point of actual transmission and it usually is configured with sufficient value for avoiding mistaken discarding, there remain chances for retransmission of the high importance RLC SDU, which is capable of securing the reliability for the entire XR traffic. Therefore, we believe that it is beneficial that the Tx RLC entity keeps the retransmission of the higher PSI RLC SDU (segment) regardless of its remaining time, until its associated acknowledgement is received.  The specification impact of TP is provided in Appendix B.

Observation 1: Discarding critical packets for the media stream will severely degrade the QoE.

Proposal 5: Tx RLC entity can determine whether to stop retransmitting an outdated RLC SDU dependent on its importance, e.g. stopping retransmission of lower importance RLC SDU, as given in Appendix B.

# 3. Discussion on timely RLC retransmission enhancement

An agreement on analysing solutions to ensure timely RLC retransmission has been reached in RAN2#128 [3] meeting, as given below:

- Timely RLC retransmission solution covers both autonomous retransmission and polling enhancement and NW can configure either or both of them.

## 3.1	Enhancement on autonomous retransmission

Autonomous retransmission is to offer the Tx RLC entity a sort of capability to autonomously retransmit an RLC SDU (segment) even if the corresponding NACK is not received. From our analysis, we believe there can be two potential solutions to achieve the autonomous retransmission.

Option 1: An RLC SDU (segment) that has been submitted to the MAC entity can be considered for autonomous retransmission when the corresponding remaining time of the RLC SDU becomes below a threshold before the associated feedback is received, as shown in Figure 3. To be more specific, the remaining time can be counted upon the reception of the RLC SDU, e.g. based on the PDB or the discardTimer.

Figure 3. Retransmitting SDU#1 when its remaining time below a threshold and no associated feedback is received

Option 2: An RLC SDU (segment) can be considered for autonomous RLC retransmission when the associated feedback is not received at Tx RLC entity since its last transmission, as illustrated in Figure 4. In such scenario, a timer can be introduced and started once an RLC SDU (segment) is transmitted.



Figure 4. Retransmitting SDU#1 when no any associated feedback is received within a certain period

The major concern imposed by the autonomous retransmission is the network capacity loss. In Option 1, the reliability of the RLC SDU (segment) can be highly ensured, at expense of tremendous capacity loss. For example, due to the uncertainty of uplink resources allocation, the hundreds of RLC SDUs within a PDU Set may be transmitted in various time points, and the unacknowledged late-transmitted RLC SDUs may be experiencing HARQ procedures. However, it is highly likely that retransmitting these RLC SDUs brings no benefit for the users` experience but inevitably deteriorates the system`s capacity. For example, assuming that the threshold for autonomous retransmission is 10ms and an RLC SDU is transmitted right before its remaining time reaching the threshold, e.g. at remaining time of 11ms, there is only 1ms for the Tx RLC entity to send Status Report or for the network to schedule the HARQ retransmission.

Observation 2: Autonomous retransmission relaying on the threshold introduces inevitable capacity loss, especially for the RLC SDUs transmitted right before the remaining time reaching the threshold.

By contrast, Option 2 mitigates the capacity impact by allowing the RLC SDU to experience a complete HARQ procedure. To be more specific, the autonomous retransmission can be triggered upon the expiry of the timer, which avoids the potential redundant retransmission between the HARQ and ARQ. Although there may be little chance for autonomously retransmitting the late-transmitted RLC SDU, with the aid of the BSR or DSR, the network has the knowledge of the remaining data volume and the remaining time, and hence can allocate more robust resource, e.g. low-order MCS, to guarantee the robustness. The specific TP is provided in Appendix C. Moreover, Option 2 also allows for the time for the Status Report reception at the Tx RLC entity, further avoiding the unnecessary autonomous retransmission for the positively acknowledged RLC SDUs.

Observation 3: The capacity loss imposed by the autonomous retransmission can be mitigated if it allows for time for HARQ procedure and Status Report reception for each transmitted RLC SDU.

Proposal 6: Autonomous retransmission timer for an RLC SDU should be started when RLC SDU is submitted to lower layers for initial transmission.

Proposal 7: Upon the expiry of the autonomous retransmission timer, the associated RLC SDU can be considered for retransmission by Tx RLC entity.

## 3.2 Enhancement on polling triggering

The polling based method tends to reuse the existing Status Report based RLC retransmission mechanism, but applies a more proactive mechanism to request a frequent Status Report. One potential enhancement is to consider the remaining time of the data as an additional factor for polling triggering. For example, when there is a delay-critical RLC SDU, or when the data volume of the delay-critical SDUs exceeds a threshold, the polling can be triggered. Upon the reception of the polling, the Rx RLC entity can trigger Status Report. We believe that there can be two options to achieve the remaining time based polling enhancement.

Option 1: An RLC SDU that has been submitted to the MAC entity and for which the associated feedback is not received can trigger the polling, when its associated remaining time becomes below a threshold, as given in Figure 5.

Figure 5 triggering polling when remaining time of SDU#1 becomes below the polling threshold and no associated feedback is received

Option 2: When the data volume or the number of delay-critical RLC SDUs for which the associated feedback is not received reaches a threshold, the polling can be triggered. Option 2 also needs to define a threshold to determining the delay-sensitive RLC SDU, as depicted in Figure 6.

In comparison to Option 1 that only needs to define a polling threshold, Option 2 requires to introduce an additional threshold to further determine whether the data volume or the number of delay-sensitive RLC SDUs reaches the threshold, incurring more specification workload. Additionally, accumulating the delay- sensitive RLC SDU to reach the threshold further introduces latency and hence delay the polling triggering. Therefore, we believe that simple factors for triggering polling can be the remaining time of an unacknowledged RLC SDU below a polling threshold. To be more specific, when the shortest remaining time of a PDCP SDU among the PDCP SDUs buffered in the RLC transmitting entity that have been transmitted and have not been positively acknowledged drops below the polling threshold, polling should be triggered.

Figure 6 Triggering polling when number of delay-sensitive & unacknowledged RLC SDUs is below a threshold

Proposal 8: RAN2 supports to trigger polling when the shortest remaining time of a PDCP SDU among the PDCP SDUs buffered in the RLC transmitting entity that have been transmitted and have not been positively acknowledged drops below a configured threshold.

Moreover, different from the legacy polling triggering method that determines whether to include a poll in an AMD PDU upon the notification transmission opportunity, there may be a case that no available RLC SDU can be transmitted when the remaining time becomes below the threshold, e.g. both the transmission and retransmission buffer are empty. As this is relatively similar to the behaviour of t-PollRetransmit expiry, we believe that the similar operation can be reused for reference. For example, when a poll is triggered based on the remaining time, and if there is no available RLC SDU to be transmitted, the Tx RLC entity can consider a specific RLC SDU for retransmission to include the poll. Finally, a TP is provided in the Appendix D.

Proposal 9: If there is no available RLC SDU to convey the poll when a poll is triggered based on the remaining time, a specific RLC SDU can be considered for retransmission, as given in Appendix D.

# 4. Conclusion

This contribution makes the following proposals:

Avoiding unnecessary transmission

Observation 1: Discarding critical packets for the media stream will severely degrade the QoE.

Proposal 1:	RAN2 specifies a new timer per Rx RLC entity to determine the obsolete data, when there is an RLC SN gap, as shown in Appendix A.

Proposal 2:	When the new timer expires, the Rx RLC entity should discard the outdated RLC SDU(s) and update the reassembly window accordingly, ie.eg. set RX_Next to the SN of the first RLC SDU with SN >= RX_Discard_Trigger for which RLC SDU is not abandoned and all bytes have been received, as detailed in Appendix A.

Proposal 3:  When the new timer expires, RLC Status Report is triggered, as shown in Appendix A.

Proposal 4: The Tx RLC entity considers an RLC SDU as outdated upon the expiry of its associated discardTimer.

Proposal 5: Tx RLC entity can determine whether to stop retransmitting an outdated RLC SDU dependent on its importance, e.g. stopping retransmission of lower importance RLC SDU, as given in Appendix B.

Timely RLC retransmission enhancement

Observation 2: Autonomous retransmission relaying on the threshold introduces inevitable capacity loss, especially for the RLC SDUs transmitted right before the remaining time reaching the threshold.

Observation 3: The capacity loss imposed by the autonomous retransmission can be mitigated if it allows for time for HARQ procedure and Status Report reception for each transmitted RLC SDU.

Proposal 6: Autonomous retransmission timer for an RLC SDU should be started when RLC SDU is submitted to lower layers for initial transmission.

Proposal 7: Upon the expiry of the autonomous retransmission timer, the associated RLC SDU can be considered for retransmission by Tx RLC entity.

Proposal 8: RAN2 supports to trigger polling when the shortest remaining time of a PDCP SDU among the PDCP SDUs buffered in the RLC transmitting entity that have been transmitted and have not been positively acknowledged drops below a configured threshold.

Proposal 9: If there is no available RLC SDU to convey the poll when a poll is triggered based on the remaining time, a specific RLC SDU can be considered for retransmission, as given in Appendix D.

# 5. Reference

[1] RP-240791 Revised WID on XR (eXtended Reality) for NR Phase 3, 3GPP TSG RAN Meeting #103

[2] Chair note of RAN2#127-bis, https://www.3gpp.org/ftp/tsg_ran/WG2_RL2/TSGR2_127bis/Inbox/Chair_Notes/R2_127bis%20R18%20MBS%2C%20R18%20QoE%20and%20R19%20XR%20session%20notes%20(Dawid)_EOM.docx

[3] Chair note of RAN2#128, https://www.3gpp.org/ftp/tsg_ran/WG2_RL2/TSGR2_128/Inbox/Chair_Notes/R2_128%20R18%20MBS%2C%20R18%20QoE%20and%20R19%20XR%20session%20notes%20(Dawid)_EOM.docx

[4] 3GPP TS 26.522

- Appendix A

## 5.2	Data transfer procedures

### 5.2.3	AM data transfer

#### 5.2.3.2	Receive operations

##### 5.2.3.2.1	General

The receiving side of an AM RLC entity shall maintain a receiving window according to the state variable RX_Next as follows:

- a SN falls within the receiving window if RX_Next <= SN < RX_Next + AM_Window_Size;

- a SN falls outside of the receiving window otherwise.

When receiving an AMD PDU from lower layer, the receiving side of an AM RLC entity shall:

- either discard the received AMD PDU or place it in the reception buffer (see clause 5.2.3.2.2);

- if the received AMD PDU was placed in the reception buffer:

  - update state variables, reassemble and deliver RLC SDUs to upper layer and start/stop t-Reassembly as needed (see clause 5.2.3.2.3);

  - if t-Discard is configured:

    - start/stop t-Discard as needed (see clause 5.2.3.2.3).

When t-Reassembly expires, the receiving side of an AM RLC entity shall:

- update state variables and start t-Reassembly as needed (see clause 5.2.3.2.4).

When t-Discard expires, the receiving side of an AM RLC entity shall:

- update state variables and start t-Discard as needed (see clause 5.2.3.2.X).

##### 5.2.3.2.3	Actions when an AMD PDU is placed in the reception buffer

When an AMD PDU with SN = x is placed in the reception buffer, the receiving side of an AM RLC entity shall:

- if x >= RX_Next_Highest:

  - update RX_Next_Highest to x+ 1.

- if all bytes of the RLC SDU with SN = x are received:

  - reassemble the RLC SDU from AMD PDU(s) with SN = x, remove RLC headers when doing so and deliver the reassembled RLC SDU to upper layer;

  - if x = RX_Highest_Status:

    - update RX_Highest_Status to the SN of the first RLC SDU with SN > current RX_Highest_Status for which not all bytes have been received.

  - if x = RX_Next:

    - update RX_Next to the SN of the first RLC SDU with SN > current RX_Next for which not all bytes have been received.

- if t-Reassembly is running:

  - if RX_Next_Status_Trigger = RX_Next; or

  - if RX_Next_Status_Trigger = RX_Next + 1 and there is no missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU; or

  - if RX_Next_Status_Trigger falls outside of the receiving window and RX_Next_Status_Trigger is not equal to RX_Next + AM_Window_Size:

    - stop and reset t-Reassembly.

- if t-Reassembly is not running (includes the case t-Reassembly is stopped due to actions above):

  - if RX_Next_Highest> RX_Next +1; or

  - if RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU:

    - start t-Reassembly;

    - set RX_Next_Status_Trigger to RX_Next_Highest.

- if t-Discard is running:

  - if RX_Discard_Trigger = RX_Next; or

  - if RX_Discard_Trigger = RX_Next + 1 and there is no missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU; or

  - if RX_Discard_Trigger falls outside of the receiving window and RX_Discard_Trigger is not equal to RX_Next + AM_Window_Size:

    - stop and reset t-Discard.

- if t-Discard is not running (includes the case t-Discard is stopped due to actions above):

  - if RX_Next_Highest > RX_Next +1; or

  - if RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU:

    - start t-Discard;

    - set RX_Discard_Trigger to RX_Next_Highest.

- 5.2.3.2.X	Actions when t-Discard expires

When t-Discard expires, the receiving side of an AM RLC entity shall:

- update RX_Next to the SN of the first RLC SDU with SN >= RX_Discard_Trigger for which not all bytes have been received;

- discard all segments with SN < updated RX_Next;

- if RX_Next_Highest > RX_Next +1; or

- if RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU:

  - start t-Discard;

  - set RX_Discard_Trigger to RX_Next_Highest.

## 5.3	ARQ procedures

### 5.3.1	General

ARQ procedures are only performed by an AM RLC entity.

### 5.3.4	Status reporting

An AM RLC entity sends STATUS PDUs to its peer AM RLC entity in order to provide positive and/or negative acknowledgements of RLC SDUs (or portions of them).

Triggers to initiate STATUS reporting include:

- Polling from its peer AM RLC entity:

  - When an AMD PDU with SN = x and the P field set to "1" is received from lower layer, the receiving side of an AM RLC entity shall:

    - if the AMD PDU is to be discarded as specified in clause 5.2.3.2.2; or

    - if x < RX_Highest_Status or x >= RX_Next + AM_Window_Size:

      - trigger a STATUS report.

    - else:

      - delay triggering the STATUS report until x < RX_Highest_Status or x >= RX_Next + AM_Window_Size.

NOTE 1:	This ensures that the RLC Status report is transmitted after HARQ reordering.

- Detection of reception failure of an AMD PDU

  - The receiving side of an AM RLC entity shall trigger a STATUS report when either t-Reassembly or t-Discard expires.

NOTE 2:	The expiry of t-Reassembly triggers both RX_Highest_Status to be updated and a STATUS report to be triggered, but the STATUS report shall be triggered after RX_Highest_Status is updated.

## 7.1	State variables

The receiving side of each AM RLC entity shall maintain the following state variables:

a) RX_Next – Receive state variable

This state variable holds the value of the SN following the last in-sequence completely received RLC SDU, and it serves as the lower edge of the receiving window. It is initially set to 0, and is updated whenever the AM RLC entity receives an RLC SDU with SN = RX_Next.

b) RX_Next_Status_Trigger – t-Reassembly state variable

This state variable holds the value of the SN following the SN of the RLC SDU which triggered t-Reassembly.

c) RX_Highest_Status – Maximum STATUS transmit state variable

This state variable holds the highest possible value of the SN which can be indicated by "ACK_SN" when a STATUS PDU needs to be constructed. It is initially set to 0.

d) RX_Next_Highest – Highest received state variable

This state variable holds the value of the SN following the SN of the RLC SDU with the highest SN among received RLC SDUs. It is initially set to 0.

e) RX_Discard_Trigger – t-Discard state variable

This state variable holds the value of the SN following the SN of the RLC SDU which triggered t-Discard.

## 7.3	Timers

The following timers are configured by TS 38.331 [5]:

a) t-PollRetransmit

This timer is used by the transmitting side of an AM RLC entity in order to retransmit a poll (see clause 5.3.3).

b) t-Reassembly

This timer is used by the receiving side of an AM RLC entity and receiving UM RLC entity in order to detect loss of RLC PDUs at lower layer (see clauses 5.2.2.2 and 5.2.3.2). If t-Reassembly is running, t-Reassembly shall not be started additionally, i.e. only one t-Reassembly per RLC entity is running at a given time.

c) t-StatusProhibit

This timer is used by the receiving side of an AM RLC entity in order to prohibit transmission of a STATUS PDU (see clause 5.3.4).

d) t-Discard

This timer is used by the receiving side of an AM RLC entity in order to stop receiving the obsolete RLC PDUs (see clause 5.2.3.2). If t-Discard is running, t-Discard shall not be started additionally, i.e. only one t-Discard per RLC entity is running at a given time.

- Appendix B

## 5.3	ARQ procedures

### 5.3.1	General

ARQ procedures are only performed by an AM RLC entity.

### 5.3.2	Retransmission

The transmitting side of an AM RLC entity can receive a negative acknowledgement (notification of reception failure by its peer AM RLC entity) for an RLC SDU or an RLC SDU segment by the following:

- STATUS PDU from its peer AM RLC entity.

When receiving a negative acknowledgement for an RLC SDU or an RLC SDU segment by a STATUS PDU from its peer AM RLC entity, the transmitting side of the AM RLC entity shall:

- if the SN of the corresponding RLC SDU falls within the range TX_Next_Ack <= SN < = the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer,

- if stopRxTxObsoleteSDU is not configured; or

- if stopRxTxObsoleteSDU is configured and psi-BasedStopObsoleteSDU is not configured, and the corresponding RLC SDU has not been indicated as discard from upper layer; or

- if psi-BasedStopObsoleteSDU is configured and the corresponding RLC SDU belongs to a high importance PDU set; or

- if psi-BasedStopObsoleteSDU is configured and the corresponding RLC SDU belongs to a low importance PDU set and has not been indicated as discard from upper layer:

      - consider the RLC SDU or the RLC SDU segment for which a negative acknowledgement was received for retransmission.

NOTE:	Identification of PSI of a PDU Set and determination of low importance PDU Set are left up to UE implementation.

## 7.4	Configurable parameters

The following parameters are configured by TS 38.331 [5]:

a) maxRetxThreshold

This parameter is used by the transmitting side of each AM RLC entity to limit the number of retransmissions corresponding to an RLC SDU, including its segments (see clause 5.3.2).

b) pollPDU

This parameter is used by the transmitting side of each AM RLC entity to trigger a poll for every pollPDU PDUs (see clause 5.3.3).

c) pollByte

This parameter is used by the transmitting side of each AM RLC entity to trigger a poll for every pollByte bytes (see clause 5.3.3).

d) stopRxTxObsoleteSDU

This parameter is used by the transmitting side of each AM RLC to stop the retransmission for an RLC SDU which has been indicated as discard from upper layer. (see clause 5.3.2).

e) psi-BasedStopObsoleteSDU

This parameter is used by the transmitting side of each AM RLC to stop the retransmission for an RLC SDU which belong to the low importance PDU set and has been indicated as discard from upper layer. (see clause 5.3.2).

- Appendix C

## 5.2	Data transfer procedures

### 5.2.3	AM data transfer

#### 5.2.3.1	Transmit operations

##### 5.2.3.1.1	General

The transmitting side of an AM RLC entity shall prioritize transmission of RLC control PDUs over AMD PDUs. The transmitting side of an AM RLC entity shall prioritize transmission of AMD PDUs containing previously transmitted RLC SDUs or RLC SDU segments over transmission of AMD PDUs containing not previously transmitted RLC SDUs or RLC SDU segments.

The transmitting side of an AM RLC entity shall maintain a transmitting window according to the state variable TX_Next_Ack as follows:

-	a SN falls within the transmitting window if TX_Next_Ack <= SN < TX_Next_Ack + AM_Window_Size;

-	a SN falls outside of the transmitting window otherwise.

The transmitting side of an AM RLC entity shall not submit to lower layer any AMD PDU whose SN falls outside of the transmitting window.

For each RLC SDU received from the upper layer, the AM RLC entity shall:

-	associate a SN with the RLC SDU equal to TX_Next and construct an AMD PDU by setting the SN of the AMD PDU to TX_Next;

-	increment TX_Next by one.

When submitting an AMD PDU that contains a segment of an RLC SDU, to lower layer, the transmitting side of an AM RLC entity shall:

-	set the SN of the AMD PDU to the SN of the corresponding RLC SDU;

- if the AMD PDU includes the last byte of an RLC SDU:

  - start the autonomousRetxTimer for the corresponding RLC SDU, if configured.

The transmitting side of an AM RLC entity can receive a positive acknowledgement (confirmation of successful reception by its peer AM RLC entity) for an RLC SDU by the following:

-	STATUS PDU from its peer AM RLC entity.

When receiving a positive acknowledgement for an RLC SDU with SN = x, the transmitting side of an AM RLC entity shall:

-	send an indication to the upper layers of successful delivery of the RLC SDU;

-	set TX_Next_Ack equal to the SN of the RLC SDU with the smallest SN, whose SN falls within the range TX_Next_Ack <= SN <= TX_Next and for which a positive acknowledgment has not been received yet;

- if the autonomousRetxTimer for the RLC SDU is running:

  - stop the autonomousRetxTimer for the RLC SDU.

## 5.3	ARQ procedures

### 5.3.1	General

ARQ procedures are only performed by an AM RLC entity.

### 5.3.2	Retransmission

The transmitting side of an AM RLC entity can receive a negative acknowledgement (notification of reception failure by its peer AM RLC entity) for an RLC SDU or an RLC SDU segment by the following:

        - STATUS PDU from its peer AM RLC entity.

When receiving a negative acknowledgement for an RLC SDU or an RLC SDU segment by a STATUS PDU from its peer AM RLC entity, the transmitting side of the AM RLC entity shall:

- if the SN of the corresponding RLC SDU falls within the range TX_Next_Ack <= SN < = the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer:

  - consider the RLC SDU or the RLC SDU segment for which a negative acknowledgement was received for retransmission,

- if a negative acknowledgement is for the RLC SDU and the autonomousRetxTimer for the corresponding RLC SDU is running:

  - stop the autonomousRetxTimer for the RLC SDU.

When the autonomousRetxTimer for an RLC SDU expires, the transmitting side of the AM RLC entity shall:

- if the SN of the corresponding RLC SDU falls within the range TX_Next_Ack <= SN < = the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer:

  - consider the corresponding RLC SDU or the RLC SDU segment which is neither positively nor negatively acknowledged for retransmission.

When an RLC SDU or an RLC SDU segment is considered for retransmission, the transmitting side of the AM RLC entity shall:

- if the RLC SDU or RLC SDU segment is considered for retransmission for the first time:

  - set the RETX_COUNT associated with the RLC SDU to zero.

- else, if it (the RLC SDU or the RLC SDU segment that is considered for retransmission) is not pending for retransmission already and the RETX_COUNT associated with the RLC SDU has not been incremented due to another negative acknowledgment in the same STATUS PDU:

  - increment the RETX_COUNT.

- if RETX_COUNT = maxRetxThreshold:

  - indicate to upper layers that max retransmission has been reached.

## 5.4	SDU discard procedures

When indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity or the transmitting UM RLC entity shall discard the indicated RLC SDU and stop the autonomousRetxTimer for the RLC SDU if it is running, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. The transmitting side of an AM RLC entity shall not introduce an RLC SN gap when discarding an RLC SDU.

## 7.3	Timers

The following timers are configured by TS 38.331 [5]:

a) t-PollRetransmit

This timer is used by the transmitting side of an AM RLC entity in order to retransmit a poll (see clause 5.3.3).

b) t-Reassembly

This timer is used by the receiving side of an AM RLC entity and receiving UM RLC entity in order to detect loss of RLC PDUs at lower layer (see clauses 5.2.2.2 and 5.2.3.2). If t-Reassembly is running, t-Reassembly shall not be started additionally, i.e. only one t-Reassembly per RLC entity is running at a given time.

c) t-StatusProhibit

This timer is used by the receiving side of an AM RLC entity in order to prohibit transmission of a STATUS PDU (see clause 5.3.4).

d) autonomousRetxTimer

This parameter is used by the transmitting side of each AM RLC entity to consider the retransmission for the associated RLC SDU or the associated RLC SDU segment when its associated timer expires (see clause 5.3.2).

- Appendix D

## 5.3	ARQ procedures

### 5.3.1	General

ARQ procedures are only performed by an AM RLC entity.

### 5.3.3	Polling

#### 5.3.3.1	General

An AM RLC entity can poll its peer AM RLC entity in order to trigger STATUS reporting at the peer AM RLC entity.

#### 5.3.3.2	Transmission of a AMD PDU

Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission such that the AMD PDU contains either a not previously transmitted RLC SDU or an RLC SDU segment containing not previously transmitted byte segment, the transmitting side of an AM RLC entity shall:

-	increment PDU_WITHOUT_POLL by one;

-	increment BYTE_WITHOUT_POLL by every new byte of Data field element that it maps to the Data field of the AMD PDU;

-	if PDU_WITHOUT_POLL >= pollPDU; or

-	if BYTE_WITHOUT_POLL >= pollByte:

-	include a poll in the AMD PDU as described below.

Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission, the transmitting side of an AM RLC entity shall:

-	if both the transmission buffer and the retransmission buffer becomes empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) after the transmission of the AMD PDU; or

-	if no new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling);

-	include a poll in the AMD PDU as described below.

NOTE:	Empty RLC buffer (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) should not lead to unnecessary polling when data awaits in the upper layer. Details are left up to UE implementation.

If PollingThreshold is configured, the transmitting side of an AM RLC entity shall:

- If the smallest remaining time of the running PDCP discardTimers among all the PDCP SDUs buffered in the transmitting side of the AM RLC entity that have been submitted to the lower layers and have not been positively acknowledged becomes below PollingThreshold

  - if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements); or

  - if no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to window stalling)

- consider the RLC SDU with the highest SN among the RLC SDUs submitted to lower layer for retransmission; or

- consider any RLC SDU which has not been positively acknowledged for retransmission.

  - include a poll in an AMD PDU as described below.

To include a poll in an AMD PDU, the transmitting side of an AM RLC entity shall:

-	set the P field of the AMD PDU to "1";

-	set PDU_WITHOUT_POLL to 0;

-	set BYTE_WITHOUT_POLL to 0.

Upon submission of an AMD PDU including a poll to lower layer, the transmitting side of an AM RLC entity shall:

-	set POLL_SN to the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer;

-	if t-PollRetransmit is not running:

-	start t-PollRetransmit.

-	else:

-	restart t-PollRetransmit.

#### 5.3.3.X Poll triggering

If PollingThreshold is configured, the transmitting side of an AM RLC entity shall:

- If the smallest remaining time of the running PDCP discardTimers among all the PDCP SDUs buffered in the transmitting side of the AM RLC entity that have been submitted to the lower layers and have not been positively acknowledged becomes below PollingThreshold

  - if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements); or

  - if no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to window stalling)

- consider the RLC SDU with the highest SN among the RLC SDUs submitted to lower layer for retransmission; or

- consider any RLC SDU which has not been positively acknowledged for retransmission.

  - include a poll in an AMD PDU as described in clause 5.3.3.2.

Editor’s note: The new poll triggering mechanism can be added as a new clause 5.3.3.X or as a paragraph in 5.3.3.2, either of which works well.

## 7.4	Configurable parameters

The following parameters are configured by TS 38.331 [5]:

a) maxRetxThreshold

This parameter is used by the transmitting side of each AM RLC entity to limit the number of retransmissions corresponding to an RLC SDU, including its segments (see clause 5.3.2).

b) pollPDU

This parameter is used by the transmitting side of each AM RLC entity to trigger a poll for every pollPDU PDUs (see clause 5.3.3).

c) pollByte

This parameter is used by the transmitting side of each AM RLC entity to trigger a poll for every pollByte bytes (see clause 5.3.3).

d) PollingThreshold

This parameter is used by the transmitting side of each AM RLC entity to trigger a poll for the RLC SDUs with remaining time below the threshold (see clause 5.3.3).