3GPP TSG RAN WG2 #129			R2-2501061

Athens, Greece, Feburaly 17th – 21st, 2025

Source:	NTT DOCOMO, INC.

Title:	Discussion on on-demand SIB1

Agenda Item:	8.5.3

Document for: 	Discussion and Decision

- Introduction

At the RAN#105 meeting, the new work item of “Enhancement of network energy savings for NR” was agreed [1]. In this contribution, we provide our views on on-demand SIB1 for idle/inactive mode UEs.

Agreements on OD-SIB1@RAN2#127

Scenarios:

- No consensus for RAN1 case 3 in RAN2.

OD-SIB1 Validity:

- We can rely on legacy UE behaviour to ensure UE has valid SIB1 for the cell (i.e. UE reacquire SIB1 whenever (re)selecting cell).

- Further UE keeps SIB1 updated while on cell via regular SI modification procedure (confirmation of earlier RAN2 agreement).

WUS configuration of NES cell from NES cell:

- Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

MSG3 based OD-SIB1 REQ:

- Msg 3 based OD-SI procedure is not supported for on-demand SIB1 request in case 2 (for requesting to the NES Cell).

Access restriction for legacy UEs:

- Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded.

- Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB.

- Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

- Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

Impacts to RRC connected UEs:

- RAN2 understands the NW can avoid impact on legacy RRC connected UE and R19 RRC connected UE due to on-demand SIB1 (e.g. NES cell with OD-SIB1 is measured by legacy RRC_CONNECTED UE and can be configured as its PSCell/SCells/target cell).

OD-SIB1 SI conclusion:

- RAN2 conclude that on-demand SIB1 is feasible from RAN2 perspective and recommend normative work of case 2 for on-demand SIB1.

Agreements on OD-SIB1@RAN2#127bis

- We will inherit all agreements made during SI phase to WI phase.

- A NES cell can include neighbouring NES cell’s WUS configuration.

- NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration).

- In on-demand SIB1 procedure, the UE considers RACH failure when PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1.

- The MAC layer will indicate the RACH failure for SI request to upper layers. FFS: after that the UE upper layer will consider the cell as barred.

- The legacy UE behaviour can be reused upon on-demand SIB1 acquisition failure, i.e., the NES UE should follow the intraFreqReselection in MIB of NES cell.

- A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

- If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

- Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced.

- NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities).

- RAN2 will not start the discussion on the issue when the UL WUS configuration update in Cell A is not synchronised with the UL WUS configuration update in the NES cell, unless we’re asked to do that by other WG, e.g. RAN3.

Agreements on OD-SIB1@RAN2#128

1.	NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing K_SSB < 24 for FR1 and K_SSB < 12 for FR2, will acquire SIB1 as in legacy.

2.	New NES-specific reselection priority parameters for NES UEs are defined for the purpose of prioritiz-ing/deprioritizing a NES frequency.

3.	Introduce new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs enable proper reselection behaviour of legacy and NES UEs.

4.	Reuse legacy cell reselection criterion as trigger condition of OD-SIB1 acquisition. No need to specify other conditions (e.g. a new RSRP threshold).

5.	The existing 1-second rule in the cell reselection criteria is still applied to the triggering condition of UL WUS transmission.

6.	The UE considers the cell as barred after MAC indicates max number of preamble transmission for the OD-SIB1 request.

7.	If MSG2 (ACK) is received, but UE fails to receive SIB1 then the UE may consider this cell as barred, no spec impact.

8.	A UE bars the NES/SIB1 less cell and/or excludes it as a candidate for reselection since the UE had no cor-responding UL WUS configuration, the UE would treat this cell as if cell status is “not barred” and consider it as candidate for cell reselection once it has received a UL-WUS configuration to request SIB1 for this cell.

9.	UE considers WUS configuration only valid in directly succeeding cell reselection from cell where UE ac-quired WUS configuration. FFS on SI validity area rather than cell level.

10.	Working assumption: UL-WUS configuration in RRC release is not supported.

- Discussion

2.1	Behavior on NES cell

Before a UE requests SIB1 to a cell, the UE needs to check if SIB1 is currently being broadcasted or provided on demand, as agreed in the previous meeting [2]. This agreement should be clarified at least for the following two issues: (issue 1) whether legacy UE can use a NES cell where SIB1 is broadcasted (Note: a NES cell means a cell for which UL WUS configuration is available in this context) and (issue 2) how a UE identifies if the cell transmits SIB1 in broadcast mode or on-demand mode.

On issue (1), We would like to note that, regardless of whether legacy UEs can camp on the NES cell with broadcasted SIB1, it should be up to gNB implementation to switch NES cell (including the case where SIB1 is on-demand or broadcasted) to legacy/normal cell that legacy UEs can camp on. In our understanding, such operation can be implemented in a spec transparent manner, and spec never prevent such operation.

Proposal 1:

- Clarify whether or not legacy UE can use (camp on) a cell with SIB1 currently being broadcasted for which SIB1 request (UL WUS) configuration is available.

  - Note: Regardless of the above discussion, it should be up to gNB implementation to switch NES cell (including the case where SIB1 is on-demand or broadcasted) to legacy/normal cell that legacy UEs can camp on.

On top of broadcast of SIB1 on NES cell, we understand that a cell for which SIB1 request configuration is available, can periodically broadcast any other SI. From operators’ perspective, NW should have flexibility to broadcast other SI as well according to situation (e.g., considering the amount of request to SI)

Proposal 2:

- A cell for which SIB1 request (UL WUS) configuration is available can periodically broadcast any other SI message(s) in addition to SIB1.

On issue (2), according to RAN1 agreement, on-demand SIB1 is expected to be transmitted within a certain time window after being requested. In other words, there will also be a duration when SIB1 in on-demand mode would not be transmitted, e.g., outside a configured time window for monitoring on-demand SIB1. Therefore, checking if SIB1 is transmitted in a certain duration (like listen before talk) can be a possible way for UE to identify if SIB1 is on-demand mode or broadcast mode.

Another way could be to introduce an explicit signaling via PBCH/MIB or SIB1. It is now discussed in RAN1 whether kssb in PBCH/MIB on NES can be used as NES cell identification. The NES cell identification can be a possible indication in order for a UE to determine whether to transmit UL WUS after the UE obtains UL WUS configuration. Whether to use NES cell identification for this purpose seems to depend on whether legacy UEs can camp on a NES cell with broadcasted SIB1 (mentioned in proposal1) and kssb in PBCH/MIB is used to bar legacy UEs. It is also workable to introduce new RRC parameters (e.g., “broadcasting” or “notBroadcasting”) in SIB1, similar to the legacy parameters for other SI(s).

Proposal 3:

- For identifying whether SIB1 is currently being broadcasted or provided on demand on the NES cell, the following mechanism should be considered:

  - Alt1. New/repurposed parameters in PBCH/MIB

    - E.g., NES cell identification discussed in RAN1

  - Alt2. New parameters in SIB1

    - E.g., “broadcasting” or “notBroadcasting”

  - Alt3. UE check if SIB1 is periodically transmitted in some duration (like LBT)

Regarding the monitoring timeline of SIB1 PDCCH, the following agreement was made in the previous RAN1 meeting.

Agreement@RAN1#119 The reference time point to determine the window starting time for on-demand SIB1 is based on the RAR window for UL WUS wherein UE successfully received a RAR. - FFS: Use starting or ending slot of the RAR window as reference time

Although the start or end slot of the RAR will be used to determine the start time for OD-SIB1, in the former case a UE would be expected to receive SIB1 PDCCH first and msg2 second. We believe that the higher layer procedure of OD-SIB1 request should be defined by reusing the legacy on-demand SI procedure, which means that the higher layer at the UE will acquire OD-SIB1 just after receiving an indication to indicate msg2 reception from the lower layer.

Proposal 4:

- From RAN2 perspective, a UE capable of requesting OD-SIB1 should start to acquire OD-SIB1 requested by the UE upon reception of RAR in response to its UL WUS TX, following the legacy OD-SI procedure.

The concerns for the impact by on-demand SIB1 operation on UEs in RRC-connected mode was raised in the previous meetings and also discussed in the last RAN plenary meeting.

Proposal 8: Support RRC_CONNECTED UEs to trigger on-demand SIB1 acquisition by enhancing the legacy DedicatedSIBRequest or a new RRC message. Wait one cycle of RAN discussion (i.e. to see whether WID is updated or not to handle RRC connected UEs)

As to whether UEs in RRC connected mode can trigger OD-SIB1 or not, we believe that no enhancement is required for UEs to request OD-SIB1. As RAN2 already agreed that NW can avoid any impact of OD-SIB1 on legacy UEs in RRC connected mode and allow them to access without any specification enhancements, the same handling should be sufficient for R19 OD-SIB1 UEs, e.g., NW ensures that the UE has a valid version of SIB1 via dedicated SIB1 delivery and/or the UE is configured with an active BWP without common search space.

Proposal 5:

- Not support spec. enhancement to enable RRC-connected UEs to request OD-SIB1.

Another remaining issue is whether OD-SIB1 request can be applied to the RRC re-establishment procedure. In the current spec., after a UE detects radio link failure, the UE has to perform cell selection, including SIB1 acquisition for the target cell to which the UE attempts RRC re-establishment. We think such mechanism is not necessary because of the following two reasons, (1) NES cell should be used only when it is strong enough to wake up the cell (i.e., cell re-selection criteria are met, as agreed) and (2) the whole procedure, UL WUS TX and random access for RRC re-establishment, requires twice the RA procedure, which increases unnecessary latency and complexity to UE. It is better to perform RRC re-establishment to cells other than NES cell (e.g., cell A) in terms of both UE and NW power savings.

Proposal 6:

- Prefer not to support OD-SIB1 request when UEs perform RRC re-establishment.

2.2	UL WUS configuration

On the FFS part of “SI validity area rather than cell level” [3], we support WUS configurations provided by new SIB to be applicable among some areas.

According to the agreements, UL WUS configurations available in NES cell#1 can be provided by new SIB from any cell A or other NES cells (including that NES cell#1 can update the WUS configurations that is already provided by another cell). And in our understanding, one new SIB signaling from one cell can contain multiple WUS configurations corresponding to multiple NES cells. Based on the design of WUS configurations, it is expected that one “WUS configuration set”, comprising multiple WUS configurations that can trigger each corresponding NES cell, can be common across some area formed by several NES cells and cell As.

In our understanding, in the legacy SI framework, a UE checks if the stored SI is valid or not by comparing the parameters of “area score / area ID” and “value tag” in the stored SI and the ones provided by SIB1. We think, basically, the legacy SIB validity rule would work even for UL WUS configuration provided by new SIB, i.e., WUS configurations are available among cells whose area is aligned with one of cell A providing the WUS configurations in new SIB, so that NW and UE do not exchange WUS configurations so frequently.

Proposal 7:

- Support to follow the legacy SI validity area for UL WUS configurations provided by new SIB, i.e.,

  - UE determines that UL WUS configurations are valid in the area as long as the area ID is aligned with one associated with the new SIB providing the UL WUS configurations, when configured.

- Conclusion

In this contribution, we provided the following observations and proposals of on-demand SIB1 operation for idle/inactive mode UEs network energy saving.

Proposal 1:

- Clarify whether or not legacy UE can use (camp on) a cell with SIB1 currently being broadcasted for which SIB1 request (UL WUS) configuration is available.

  - Note: Regardless of the above discussion, it should be up to gNB implementation to switch NES cell (including the case where SIB1 is on-demand or broadcasted) to legacy/normal cell that legacy UEs can camp on.

Proposal 2:

- A cell for which SIB1 request (UL WUS) configuration is available can periodically broadcast any other SI message(s) in addition to SIB1.

Proposal 3:

- For identifying whether SIB1 is currently being broadcasted or provided on demand on the NES cell, the following mechanism should be considered:

  - Alt1. New/repurposed parameters in PBCH/MIB

    - E.g., NES cell identification discussed in RAN1

  - Alt2. New parameters in SIB1

    - E.g., “broadcasting” or “notBroadcasting”

  - Alt3. UE check if SIB1 is periodically transmitted in some duration (like LBT)

Proposal 4:

- From RAN2 perspective, a UE capable of requesting OD-SIB1 should start to acquire OD-SIB1 requested by the UE upon reception of RAR in response to its UL WUS TX, following the legacy OD-SI procedure.

Proposal 5:

- Not support spec. enhancement to enable RRC-connected UEs to request OD-SIB1.

Proposal 6:

- Prefer not to support OD-SIB1 request when UEs perform RRC re-establishment.

Proposal 7:

- Support to follow the legacy SI validity area for UL WUS configurations provided by new SIB, i.e.,

  - UE determines that UL WUS configurations are valid in the area as long as the area ID is aligned with one associated with the new SIB providing the UL WUS configurations, when configured.

References

- RP-242354, Revised WID: Enhancements of network energy savings for NR, Ericsson

- 3GPP, RAN2#127bis meeting, Chairman’s notes

- 3GPP, RAN2#128 meeting, Chairman’s notes