3GPP TSG RAN WG2 #129			R2-2501062

Athens, Greece, Feburaly 17th – 21st, 2025

Source:	NTT DOCOMO, INC.

Title:	Discussion on adaptation of common signal and channel

Agenda Item:	8.5.4

Document for: 	Discussion and Decision

- Introduction

At the RAN#102 meeting, the new work item of “Enhancement of network energy savings for NR” was agreed [1]. In this contribution, we provide our views on adaptation of common signal and channel.

- Discussion

For NES purpose, we discuss the candidates for adaptations and applicable scenarios for each common signal/channel. First, we’d like to clarify how to obtain NES gain by adaptation over the legacy operation. One approach is to introduce new properties/parameters to reduce occasion/duration in time domain at gNB. The other approach is to introduce more efficient/dynamic mechanism to change property of common signal/channel following a change in condition/situation (e.g., when configured PRACH capacity is lacking, NW decides to provide additional PRACH resources more dynamically by less overhead signaling to satisfy the situation). Any techniques in this objective should be evaluated from these perspectives for NES operation.

Observation 1:

- For adaptation of common signal/channel in NES operation, the following two approaches should be considered.

  - Introduce new properties/parameters to reduce the gNB TX/RX occasion/duration in time domain.

  - Introduce more efficient/dynamic mechanism to change properties of common signals/channels.

2.1	SSB adaptation

Indication mechanism

On the applicable scenario for SSB adaptation, RAN1 could reach conclusion that SSB adaptation to be supported only on SCell as shown following agreements [2][3][4].

Agreement@RAN1#118 For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’) Conclusion@RAN1#118bis There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19 Conclusion@RAN1#119 There is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell (connected mode)

Although RAN1 has not decided SSB adaptation mechanism yet, we believe that some new signaling methods (e.g., new MAC CE) to adapt SSB properties which does not require reconfiguration with sync should be supported so that NW has an efficient way over the legacy update of SSB properties. Considering the situation in RAN1 where introduction of new MAC CE to indicate OD-SSB transmission is agreed which might notify UEs of index of OD-SSB configuration provided by RRC signalling in advance, similar mechanism should be a baseline for SSB adaptation. Another reason to unify signalling method is that on-demand SSB (especially in case2) and periodic SSB adaptation would be equivalent in many situations. We would not like to introduce duplicated mechanism to achieve the same benefit.

Proposal 1:

- For SSB adaptation on SCell, multiple SSB properties should be provided via RRC configuration.

  - SSB properties include, at least, SSB burst periodicity.

Observation 2:

- For on-demand SSB on SCell, multiple SSB properties are provided via RRC configuration.

Proposal 2:

- Support unified MAC CE design to indicate either of following by informing an index of corresponding SSB configuration

  - Switch of periodic SSB properties (as SSB adaptation in objective3)

  - Activation or deactivation of on-demand SSB (as on-demand SSB in objective1)

Impact on L3 measurement

There was the discussion on whether to extend the current L3 measurement framework by introducing SSB adaptation. We think that some new mechanisms for L3 measurement to follow dynamic change in SSB properties should be supported.

For SSB adaptation, the target of SSB adaptation which was agreed is only SSB burst periodicity, and adaptation of SSB index is under discussion in RAN1.

The legacy L3 measurement configuration can configure the periodicity from the range between 5ms and 160ms and the SSB indexes to be measured as measurement object, which may or may not be aligned with the actual SSB burst periodicity and SSB index completely. This means which SSB occasions to be measured are determined by a semi-static manner.

If we consider when NW determines to adapt SSB (and what property of SSB), one criterion should be that UE’s speed and channel condition in a given cell. For example, if a UE is in relatively high speed, the UE should be provided with adaptive SSB whose periodicity is relatively shorter to follow the channel fluctuation so that NW can obtain accurate measurement result from the UE in a timely manner.

In this situation, it is preferable that all indicated adaptive SSB should be L3-measured for accurate and timely evaluation of the SCell strength. If the update of measurement object requires the legacy scheme, e.g., RRC Reconfiguration signaling, there seems to be less benefit by introducing dynamic adaptation of SSB, because anyway gNB has to send RRC signaling for applying adaptive SSB TX and corresponding measurement. Even using the legacy scheme, it is indeed possible to set two measurement configs where one corresponds to short SSB and the other is longer SSB in one frequency, while this is an inefficient way from UE measurement budget and energy consumption perspective, so this should be avoided.

Observation 3:

- It is expected that NW would determine whether and what property to adapt SSB for faster / more accurate L3 measurement according to UE’s condition (e.g., UE’s speed).

Proposal 3:

- Introduce a mechanism that measurement object, if this includes time and frequency of adaptive SSB, is dynamically changed according to the adaptation of SSB.

2.2	PRACH adaptation

Applicable scenario

We believe that Rel-19 additional PRACH is to improve RACH capacity and/or RACH latency for better communication performance during NES operation (while gNB is infrequent wake-up to monitor legacy PRACH resources by configuring less ROs or longer periodicity).

The applicable case would be that NW may configure smaller number of ROs and/or longer RACH periodicity as a default for NES operation by reducing RO monitoring. Generally, NW may be able to expect the rough number of IDLE UEs, while cannot expect the probability of traffic occurrence for each of IDLE UEs. Configuring many ROs for those IDLE UEs will cause inefficient NW energy usage due to unused ROs, but if only a limited number of ROs is configured while traffic occurs for many idle UEs, a lot of PRACH transmissions will occur within a cell, that exceeds with the RO capacity. In order to address this, additional PRACH resources can be indicated to new UEs which try to access a NES cell.

Based on the above analysis, we consider applicable use-case and scenario for adaptation PRACH. The legacy specification supports several cases where additional PRACH configuration can be configured as follows:

- sPcell or scell

- Contention based RACH or contention free based RACH

- 4-step RACH or 2-step RACH

- Feature combination, e.g.,

  - Redcap / eRedCap

  - Small data transmission

  - NSAG

  - msg3 / msg1 repetition

- Other use-cases, e.g., IAB

From our understanding, at least CFRA should be out of scope of the PRACH adaptation. PRACH resources for CFRA are generally provided for RRC-connected UEs by dedicated configuration so that each UE has its own resources. We do not see any motivation to change more dynamically these resources.

Proposal 4:

- Not support PRACH adaptation in Rel-19 for CFRA.

Configuration/indication of additional PRACH

RAN1 agreed that additional PRACH resources are semi-statically provided, and DCI is used to indicate its availability. Around discussion of signaling, some companies mention that additional PRACH resources should be available only via semi-static signaling, i.e., without DCI. However, we rather believe DCI-based provision should be supported. This is because, there seems no introduction of new parameters applied to additional PRACH to generate more confined patterns, which means additional PRACH finally turn out to be almost same as legacy PRACH resources. In this sense, if additional PRACH resources are provided by semi-static signaling, we can say the legacy SIB1 can provide almost all PRACH patterns that is combination of legacy and additional. We cannot see any motivation to support what the legacy framework already supports, so additional PRACH resources should be activated only via DCI.

Proposal 5:

- For PRACH adaptation mechanism, prefer not to support additional PRACH resources available only via semi-static signalling (i.e., SIB1), i.e., additional PRACH resources are only available when DCI is indicated.

2.3	Paging adaptation

Firstly, we consider the legacy paging configurations and necessary extension for further NES gain.

UE acquires paging configuration from SIB1 and determines paging frames (PF), paging occasions (PO) and paging monitoring occasions (PMO) as follows.

- SFN for the PF is determined by:

  - (SFN + PF_offset) mod T = (T div N)*(UE_ID mod N)

- Index (i_s), indicating the index of the PO is determined by:

  - i_s = floor (UE_ID/N) mod Ns

- Configurations

  - Default or UE specific DRX cycle (T={32, 64, 128, 256 [radio frame]} or extended ={256, 512, 2014 [radio frame]} )

  - N = {T, T/2, T/4, T/8, T/16}

  - Ns: number of paging occasions for a PF ({4,2,1})

  - UE_ID: 5G-S-TMSI mod 1024 or 5G-S-TMSI mod 4096 for eDRX

For example, in case of T = 128 rf and N = T/16 (for confining PF/PO within a paging cycle as possible), there would be multiple PFs (at most 8) every 160 [ms] within 1280 [ms] for different UEs. Within a PF, the number of PO and time resource position of paging PDCCH (paging monitoring occasion) comprising each PO can be configured by NW. Although reducing the paging cycle (or UE’s DRX cycle) can result in fewer paging frames within a paging cycle, it is not the preferable method for UE energy saving. Additionally, NW may still need to wake up frequently due to the short paging cycle after all.

Agreements on adaptation@RAN2#127bis

Agreements on paging adaptation

1.	Select option-b as baseline for R19 NES paging enhancement.

2.	R2 should aim at signaling overhead minimization (e.g., Ns=8 and FFS for other larger values)

3.	Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration.

4.	Legacy UE is not barred.

5.	Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration.

Therefore, with the legacy spec., there is a case where NW has to wake up and transmit paging multiple times within a paging cycle for UE using the current specification, which inhibits NES operation. In order to address the issue, RAN2 agreed that extension of “N”, “Ns” and “firstPDCCH-MonitoringOccasionOfPO” would be supported with minimizing signalling overhead [5].

For “N”, we prefer as minimized value of N as possible at least for each DRX cycle so that NW would wake-up once per each DRX cycle. In this sense, value of N should be extended to {T/32, T/64, T/128, T/256} for as confined paging occasion as possible.

Regarding “Ns” determining the number of paging occasions within each paging frame, if we consider extending “Ns” to fully compensate the extension of N, the value of Ns should be extended to {8, 16, 32, 64}. With the extended “Ns”, the payload size of “firstPDCCH-MonitoringOccsionsOfPO” is increased, for example, it exceeds 1000bits for Ns=64 and N= with 30kHz SCS. From NES perspective for confined paging occasions, we believe that all Ns should not necessarily be supported even though all values of N are supported because there seem to be few UEs in NES cell with paging adaptation. We think Ns equals to 8 and 16 can be acceptable considering the payload size.

Proposal 6:

- For paging adaptation,

  - support extension of N to {T/32, T/64, T/128, T/256}

  - support extension of Ns to at least 8, 16

- Conclusion

In this contribution, we provided the following observations and proposals of adaptation of common signal and channel for network energy saving.

Observation 1:

- For adaptation of common signal/channel in NES operation, the following two approaches should be considered.

  - Introduce new properties/parameters to reduce the gNB TX/RX occasion/duration in time domain.

  - Introduce more efficient/dynamic mechanism to change properties of common signals/channels.

Proposal 1:

- For SSB adaptation on SCell, multiple SSB properties should be provided via RRC configuration.

  - SSB properties include, at least, SSB burst periodicity.

Observation 2:

- For on-demand SSB on SCell, multiple SSB properties are provided via RRC configuration.

Proposal 2:

- Support unified MAC CE design to indicate either of following by informing an index of corresponding SSB configuration

  - Switch of periodic SSB properties (as SSB adaptation in objective3)

  - Activation or deactivation of on-demand SSB (as on-demand SSB in objective1)

Observation 3:

- It is expected that NW would determine whether and what property to adapt SSB for faster / more accurate L3 measurement according to UE’s condition (e.g., UE’s speed).

Proposal 3:

- Introduce a mechanism that measurement object, if this includes time and frequency of adaptive SSB, is dynamically changed according to the adaptation of SSB.

Proposal 4:

- Not support PRACH adaptation in Rel-19 for CFRA.

Proposal 5:

- For PRACH adaptation mechanism, prefer not to support additional PRACH resources available only via semi-static signalling (i.e., SIB1), i.e., additional PRACH resources are only available when DCI is indicated.

Proposal 6:

- For paging adaptation,

  - support extension of N to {T/32, T/64, T/128, T/256}

  - support extension of Ns to at least 8, 16

References

- RP-234065, New WID: Enhancements of network energy savings for NR, RAN Meeting #102, December 11th-15th, 2023.

- 3GPP, RAN1#118 meeting, Chairman’s notes

- 3GPP, RAN1#118bis meeting, Chairman’s notes

- 3GPP, RAN1#119 meeting, Chairman’s notes

- 3GPP, RAN2#127bis meeting, Chairman’s notes