3GPP TSG-RAN WG2 Meeting #129	R2-2501071

Athens, Greece, Feb 17th – Feb 21st, 2025

Agenda item:	8.3.2.2

Source:	Qualcomm Incorporated

Title:	Simulation Results for Model Generalization

WID/SID:	FS_NR_AIML_Mob – Release 19

Document for:	Discussion and Decision

# Introduction

In the last RAN2 meeting (RAN2 #128 [1]), the simulation framework, assumptions, and parameters for Model Generalization for RRM measurement prediction were discussed. As captured in the Chair notes for the meeting [1], it was agreed to study Model Generalization performance for the following cases:

- Baseline: The AI/ML model is trained using the dataset with Configuration #B and tested using the dataset with Configuration #B​

- Generalization Case #1 (GC#1): The AI/ML model is trained using the dataset with Configuration #A but tested using the dataset with Configuration #B​

- Generalization Case #2 (GC#2): The AI/ML model is trained using mixed datasets with both configurations and tested using the dataset with Configuration #B

It was agreed to prioritize generalization over UE speeds, with datasets in which mobile UEs move with speeds of 30 km/h, 60 km/h, and 90 km/h datasets considered. Tables 1 and 2 summarize the agreed UE speed simulation combinations for FR1 and FR2 generalization study, respectively.

Table 1: Agreed simulation combinations for FR1 Model Generalization study

|  | UE speeds in Training dataset |  |  | UE speeds in Inference dataset |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 30 km/h | 60 km/h | 90 km/h | 30 km/h | 60 km/h | 90 km/h |
| Baseline | Yes |  |  | Yes |  |  |
| GC #1 |  | Yes |  | Yes |  |  |
| GC #1 |  |  | Yes | Yes |  |  |
| GC #2 | Yes | Yes | Yes | Yes |  |  |
| Baseline |  | Yes |  |  | Yes |  |
| GC #1 | Yes |  |  |  | Yes |  |
| GC #1 |  |  | Yes |  | Yes |  |
| GC #2 | Yes | Yes | Yes |  | Yes |  |
| Baseline |  |  | Yes |  |  | Yes |
| GC #1 | Yes |  |  |  |  | Yes |
| GC #1 |  | Yes |  |  |  | Yes |
| GC #2 | Yes | Yes | Yes |  |  | Yes |

Table 2: Agreed simulation combinations for FR2 Model Generalization study

|  | UE speeds in Training dataset |  |  | UE speeds in Inference dataset |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 60 km/h | 90 km/h | 120 km/h | 60 km/h | 90 km/h | 120 km/h |
| Baseline | Yes |  |  | Yes |  |  |
| GC #1 |  | Yes |  | Yes |  |  |
| GC #1 |  |  | Yes | Yes |  |  |
| GC #2 | Yes | Yes | Yes | Yes |  |  |
| Baseline |  | Yes |  |  | Yes |  |
| GC #1 | Yes |  |  |  | Yes |  |
| GC #1 |  |  | Yes |  | Yes |  |
| GC #2 | Yes | Yes | Yes |  | Yes |  |
| Baseline |  |  | Yes |  |  | Yes |
| GC #1 | Yes |  |  |  |  | Yes |
| GC #1 |  | Yes |  |  |  | Yes |
| GC #2 | Yes | Yes | Yes |  |  | Yes |

In this contribution, we provide simulation evaluations for a subset of the above Generalization Case #1 simulation combinations for FR1 60 km/h and 90 km/h datasets. In addition, we provide evaluations for Generalization Case #2 in which the AI/ML model is trained using mixed 60 km/h and 90 km/h datasets. Table 3 summarizes the UE speed simulation combinations discussed in this contribution.

Table 3: Discussed simulation combinations for FR1 Model Generalization study

|  | UE speeds in Training dataset |  | UE speeds in Inference dataset |  |
| --- | --- | --- | --- | --- |
|  | 60 km/h | 90 km/h | 60 km/h | 90 km/h |
| Baseline | Yes |  | Yes |  |
| GC #1 |  | Yes | Yes |  |
| GC #2 | Yes | Yes | Yes |  |
| Baseline |  | Yes |  | Yes |
| GC #1 | Yes |  |  | Yes |
| GC #2 | Yes | Yes |  | Yes |

# Simulation assumptions

In this section, we discuss assumptions applicable to both FR1 and FR2 data generation simulations, detailing the methodology for UE trajectory generation, soft line-of-sight (LoS) updates, and shadow fading updates.

## UE trajectory generation formulation

This section details the chosen model for generating random trajectories for mobile UEs in data generation simulations. The chosen model is based on Option 2 in TR 38.343 [2], Section 6.3.1, with modifications based on Option 4 in [2] and a minor modification for extended mobility.

A UE has initial position (x0,y0, z0), constant forward velocity s, and initial azimuthal velocity angle ϕv,0. Temporal granularity tgran is defined as the smallest unit of time considered in random trajectory generation and mean temporal step size tstep is defined as the mean travel time for walk steps along the random trajectory. tstep is chosen such that it will be a whole-number multiple of the temporal granularity:

tstep=Nsubstepstgran, where

- Nsubsteps∈Z+ is the mean number of substeps per walk step.

- Maximum walk time tmax≫tstep is likewise chose to be a whole-number multiple of tgran.

- Distance granularity dgran is computed as dgran=stgran.

For the purposes of random trajectory generation for urban macro and urban micro simulations, a geometric sector SNb is defined as a geometric region consisting of the intersection of two regions: the first region consists of one third of the hexagonal cell surrounding a gNB and symmetric with respect to the gNB’s azimuthal orientation ϕNb, and the second region consists of the area outside a circle with radius dNb centered on the gNB. Figure 1 provides an example of a geometric sector.

Figure 1 Geometric sector defined for UE mobility constraints

The overall valid UE trajectory region Svalid is the geometric region in which a UE trajectory may be generated and consists of the union of all geometric sectors corresponding to each gNB in the simulation:

Svalid=i=1NNbSi

Where NNb is the number of gNBs in the simulation. Figure 2 visualizes Svalid for a 2-tier UMa simulation with ISD 200.

Figure 2 Geometric sectors for every gNB in a 2-tier Urban Macro simulation. Each geometric sector is labelled with the MAC ID of its corresponding gNB. The valid UE trajectory region Svalid consists of the union of all sectors.

Given the above setup, the following algorithm is used to generate a random trajectory for a particular UE:

- Initialize total sub-step counter m=0.

- For walk step k=1,2,…:

  - Draw number of sub-steps for the kth walk step Nsubsteps,k from a geometric distribution with probability of success 1Nsubsteps.

  - Draw azimuthal velocity angle delta for the kth walk step Δϕv,k from a uniform distribution in the range [-45°, 45°].

  - Compute sub-step velocity angle delta: Δϕv,k,substep=Δϕv,kNsubsteps,k

  - For each sub-step n=1, …,Nsubsteps,k:

    - Increment total sub-step counter m=m+1. If the UE’s total travel time thus far mtgran is greater than tmax, terminate the walk. Otherwise, continue.

    - Update the UE’s sub-step velocity angle: ϕv,m=ϕv,m-1+Δϕv,k,substep.

    - Compute potential updated UE position: (xm,ym,zm)=xm-1+dgrancosϕv,m,ym-1+dgransinϕv,m,z0.

If xm,ym,zm  is outside the valid UE trajectory region Svalid, terminate the walk. Otherwise, update the UE’s position.

Figure 3 shows an example set of UE trajectories generating using the above algorithm.

Figure 3 Example random trajectories for 20 UEs within a 2-Tier UMa model

## Channel modelling: soft LoS

This section details the approach used to update line-of-sight (LoS) states for mobile UEs in data generation simulations. Discussion of the motivation for adopting this soft LoS model has been provided in [3].

In the soft LoS model provided in TR 38.901 [4], Section 7.6.3.3, each gNB-UE link maintains a soft LoS state LOSsoft∈[0, 1], calculated as

LOSsoftG,d=12+1πarctan20λG+Fd, where

- G is a spatially consistent Gaussian R.V. with correlation distance dcor according to 38.901 Table 7.6.3.1-2,

- λ is the wavelength of the center frequency fc (in meters), and

- Fd=2erf-12PrLOSd-1.

To apply this soft LoS state, for each link, both LoS and NLoS channel coefficients HLOS and HNLOS are generated per the 38.901 Fast Fading Model (Section 7.5, Steps 2-12), and the overall channel is calculated per 38.901 Equation 7.6-18 as

HLOSsoft=HLOSLOSsoft+HNLOS1-LOSsoft2.

Although TR 38.901 provides correlation distances for LoS random variables, a specific correlation function is not provided. To correlate the Gaussian random variables G used in soft LoS state generation in Option B, we use exponential correlation:

RLOSΔd=exp-Δddcor, where

- Δd is the distance between any two points in the xy-plane, and

- dcor is the correlation distance provided in Table 7.6.3.1-2.

As discussed in [3], this exponential correlation function is chosen to be consistent with the exponential autocorrelation function provided for shadow fading in 38.901, Section 7.4.4, as well as with the spatial correlation procedure provided in WINNER II [5], Section 3.3.1. We also note that the distribution statistics of the Gaussian random variables G are not specified in TR 38.901; for simplicity, we choose G to be standard normal Gaussians.

We assume that exponential correlation is applied on a per-gNB basis; that is, the Gaussians used to generate the soft LoS states for all UE-gNB links associated with a particular gNB are correlated per RLOSΔd, and there is no inter-gNB LoS correlation. We also assume that the generated LoS states for given xy-locations are fixed in time, meaning UEs will only experience LoS state transitions as a result of mobility. With these assumptions, the following procedure is used to calculate soft LoS states for mobile UEs:

- For each gNB in the simulation n=1,2,…,NNB:

  - Establish a grid of location points Dn that covers the area in which mobile UE trajectories will be constrained.

  - Over Dn generate a grid Gn of standard normal Gaussian realizations at each grid point correlated in distance by RLOSΔd=exp-Δddcor.

  - Using the soft LoS state procedure outlined in TR 38.901, Section 7.6.3.3, generate a grid of soft LoS realizations Ln= LOSsoft(Gn, Dn) from Gn and PrLOSd for the given scenario.

- During the channel update for each mobile UE i=1,2,…,NUE:

  - For each gNB in the simulation n=1,2,…,NNB, interpolate the soft LoS state for the UE-gNB link from Ln given the mobile UE’s xy-location.

Figure 4 shows an example soft LoS grid for an FR2 UMi simulation with the gNB located at (0, 0).

Figure 4 Example Soft LoS grid (UMi, fc = 30 GHz)

To demonstrate the impact of soft LoS on the generated channels for mobile UEs, Figure 5 provides a comparison between simulation results for a mobile UE with and without soft LoS updates enabled. In the simulation with LoS updates disabled (orange line), the mobile UE has a constant LoS = 1 state with its serving gNB, while in the simulation with LoS updates enabled (blue line), the LoS state is updated as a function of the UE’s position. As the mobile UE enters NLoS regions, the maximum RSRP across all beam pair links in the simulation with LoS updates tends to drop significantly compared to the maximum RSRP for the simulation where the LoS state remains 1.

Figure 5 Simulation comparison for a mobile UE with and without soft LoS updates enabled. Top: maximum RSRP for mobile UE with and without soft LoS updates. Bottom: LoS state for mobile UE with and without LoS updates. A value of 0 corresponds to a completely NLoS link, while a value of 1 corresponds to a completely LoS link.

## Channel modelling: shadow fading

This section describes the model used for updating shadow fading values for mobile UEs in data generation simulations. Similarly to soft LoS state updates, we use a grid-based approach to update shadow fading values in a spatially consistent manner. Discussion of the motivation for adopting this approach has been provided in [3].

As with soft LoS values, we assume that shadow fading values are fixed in time for given xy-locations and independently calculated on a per-gNB basis. We also assume that a gNB’s shadow fading values are autocorrelated exponentially as per TR 38.901 [4], Section 7.4.4, with autocorrelation function

RΔx=exp-Δxdcor, where

- Δx is the 2D-distance between any two locations in the simulation, and

- dcor is the environment-specific correlation distance provided in TR 38.901, Table 7.5-6.

With these assumptions, the following procedure is used to update shadow fading values for mobile UEs:

- For each gNB in the simulation n=1,2,…,NNB:

  - Establish a grid of location points Dn that covers the area in which mobile UE trajectories will be constrained.

  - On top of Dn generate a grid of correlated shadow fading values SFn~N0, σsf, where σsf is the scenario/LoS-state-specific shadow fading standard deviation provided in TR 38.901, Table 7.5-6, and the R.V.s at each grid point are correlated in distance by RLOSΔd=exp-Δddcor .

- During the channel update for each mobile UE i=1,2,…,NUE:

  - For each gNB in the simulation n=1,2,…,NNB, interpolate the LoS and NLoS shadow fading values for the UE-gNB link from SFn given the mobile UE’s xy-location and UE-gNB link LoS.

Figure 6 shows an example pair of LoS and NLoS shadow fading grids for a UMi simulation.

Figure 6 Example LoS (left) and NLoS (right) shadow fading grids, UMi

## Channel modelling: disabling wraparound

To facilitate modelling of inter-site UE mobility, we disable wraparound for UMa and UMi models. Figure 7 demonstrates the impact of disabling wraparound on the channel modelling parameters on the arrival and departure LoS cluster angles between UEs in the bottom sector and gNBs in the upper-left sector of a 2-tier model. Disabling wraparound ensures that the channels between mobile UEs and distant gNBs do not experience instantaneous changes as the UEs draw nearer.

Figure 7 Effect of disabling wraparound on LoS cluster angles between UEs and gNBs in separate sectors. Left: when wraparound is enabled, the UE LoS arrival angles (blue arrows) point toward a “wrapped” version of gNB 40 (not shown). Right: when wraparound is disabled, the UE LoS arrival angles point toward the non-wrapped version of gNB 40.

## Derivation of cell-level metrics

For a given UE-gNB link, given measured RSRPs across all Tx-Rx beam pairs and polarizations, the cell-level metric for RRM prediction is derived from Tx-Rx beam pair RSRPs via the following procedure:

- RSRPs are summed across Rx polarizations, and the maximum RSRP is taken across Tx polarizations.

- For k gNBs, the cell level metric Mn is calculated as

Mn=m1,n,m2,n,…,mk,n, where

- mi,n=1Nbj=1Nbri,j,n,

- ri,j,n is the RSRP of the jth top SSB for the ith sector at measurement instance n.

By taking the maximum RSRP across Rx beams, we assume a genie Rx beam selection algorithm that provides an upper-bound approximation for a realizable Rx beam selection algorithm.

The number of top beams averaged for this metric Nb is chosen to be Nb=3 for FR2 simulations and Nb=1 for FR1 simulations.

The resulting metrics are filtered in time via the Layer 3 filtering procedure outlined in 3GPP TS-38.331 [6], Section 5.5.3.2:

F0=M0

Fn=1-aFn-1+aMn, where

- Fn is the filtered cell metric result at measurement instance n,

- a=12k4, where k is the filter coefficient for the corresponding measurement quantity.

For the simulations discussed in this document, a filtering coefficient of k=4 is used to filter cell-level metrics. Figure 8 shows the effect of L3 filtering on calculated cell-level metrics for a given UE trajectory and 3 co-located gNBs.

Figure 8 Cell-level metrics with and without L3 filtering, k=4

# Simulation evaluation for FR1 intra-frequency, inter-cell temporal domain Case B model generalization

In this section, we discuss the details of the simulation assumptions, parameters, and methodology for FR1 intra-frequency, temporal domain Case B model generalization, and discuss the simulation results.

## Trajectory generation

For FR1 intra-frequency, temporal domain Case B model generalization dataset generation, we use the trajectory generation formulation outlined in Section 2.1 of this document. Table 4 lists the parameters used in this case.

Table 4: UE trajectory generation parameters for FR1 intra-frequency, intra-cell temporal Case B model generalization

| Parameter | Value |
| --- | --- |
| tstep | 1 s |
| tgran | 0.01 s |
| tmax | 160 s |
| s | 60, 90 km/h |
| dNb | 35 m |
| ISD | 500 m |

For this dataset, we consider inter-cell mobility, meaning UE trajectories span all 21 geometric sectors corresponding to each gNB in the 2-tier model. Figure 9 visualizes several example random trajectories for this inter-site mobility case.

Figure 9 Example random trajectories, FR1 intra-frequency, inter-cell temporal domain Case B

## Soft LoS and shadow fading grids

When modeling inter-site mobility, for each set of 3 co-located gNBs a unique soft LoS grid is used to model line-of-sight states for mobile UEs, totaling 7 grids for the entire dataset. Similarly, 7 pairs of LoS/NLoS shadow fading grids are used to model the shadow fading states between the mobile UEs and each set of co-located gNBs. Figure 10 shows the soft LoS grid used for this dataset corresponding to the set of co-located gNBs at (0, 0), and Figure 11 shows the corresponding shadow fading grids.

Figure 10 soft LoS grids for FR1 model generalization. Left: grid for gNBs located at (0, 0) m. Right: grid for gNBs located at (0, 500) m.

Figure 11 Shadow fading grids for FR1 model generalization studies. Top: LoS (left) and NLoS (right) shadow fading grids for gNBs located at (0, 0) m. Bottom: LoS (left) and NLoS (right) shadow fading grids for gNBs located at (0, 500) m.

## Summary of simulation assumptions

Table 5 summarizes FR2 simulation assumptions and parameters.

Table 5: Simulation assumptions for FR2 intra-frequency, intra-cell, temporal domain case A

| Parameter | Value |
| --- | --- |
| Frequency Range | FR1 @ 4 GHz; SCS: 30 kHz |
| Deployment | 500m ISD, 7 site, 3 sectors/cells per site |
| Channel model | UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. Fast fading is considered for all the RRM sub-cases. Do not consider Oxygen absorption (7.6.1), Time-varying Doppler shift (7.6.6), Explicit ground reflection model (7.6.8), and blockage (7.6.4), for channel modelling (38.901). Soft LoS and Shadow Fading aspects of Channel modelling are discussed in the sections above. |
| System BW | 20 MHz |
| UE Speed | 60 km/h, 90 km/h |
| UE distribution | 100% outdoor. UEs are randomly dropped within a cell with uniform distribution. |
| BS Antenna Configuration | 1 TX antenna per panel 1 panel per sector |
| BS Antenna radiation pattern | 3-sector antenna radiation pattern, 8 dBi |
| UE Antenna Configuration | 1 RX antenna per panel 2 panels |
| UE Antenna radiation pattern | Omni-direction |
| BS Tx Power | 44dBm |
| Maximum UE Tx Power | 23dbm |
| BS receiver Noise Figure | 5db |
| UE receiver Noise Figure | 9dB |
| Inter site distance | 500m |
| BS Antenna height | 25m |
| UE Antenna height | 1.5m |
| Spatial consistency | Procedure A in TR38.901 |
| UE trajectory model | Based on Option 2 in TR 38.343 [2], Section 6.3.1, with modifications based on Option 4 and a minor modification for extended mobility. Refer to section 2.1.1 for a detailed discussion of the trajectory model. |
| L1 sampling period | 40ms |
| Prediction windows | 400ms |

## Simulation dataset evaluation

For FR1 intra-frequency, temporal domain Case B model generalization, we have generated two datasets, one in which mobile UEs travel at 60 km/h, and one in which UEs travel at 90 km/h. Table 6 provides statistics for the trajectories within the datasets; Figure 12 shows CDFs of the trajectory lengths.

Table 6: FR1 model generalization dataset statistics

| Parameter | Dataset @60 km/h | Dataset @90 km/h |
| --- | --- | --- |
| # of trajectories (total) | 2436 | 2854 |
| Total data time (hr.) | 29.8 | 21.48 |
| Training/test split | 1948/488 | 2283/571 |
| Mean trajectory length (s) | 44.1 | 27.1 |
| Median trajectory length (s) | 29.72 | 19.6 |
| Min. trajectory length (s) | 0.04 | 0.04 |
| Max. trajectory length (s) | 159.96 | 159.96 |

Figure 12 Empirical CDFs of FR1 model generalization dataset trajectory lengths. Left: 60 km/h dataset. Right: 90 km/h dataset.

To evaluate the generated dataset for FR1 model generalization, in Figure 13 we plot the cell level metrics for each sector as a function of UE position across the dataset. These plots demonstrate the impacts of UE-gNB distance, gNB panel azimuthal pointing angle, and soft LoS state on the cell-level metric values.

Figure 13 Cell-level metrics for each cell as functions of UE position, FR1 intra-frequency, inter-cell temporal case B dataset. Left: cell metric data for gNB 22. Right: cell metric data for gNB 25.

## AI/ML model and methodology

For the RRM cell-level metric Case B prediction problem, we consider a repeating measurement pattern in which cell-level metrics are measured at x contiguous time instances followed by y contiguous time instances at which no measurements are taken, referred to as an “MxPy” measurement pattern. For given values of x and y, the corresponding measurement reduction rate can be calculated as yx+y.

Given this measurement pattern, the prediction problem can be stated as follows: given past cell metric measurements taken at measured beam management cycles, predict cell level metrics for the next y prediction cycles. Figure 14 visualizes cell-level metric prediction within an MxPy beam measurement pattern framework.

Figure 14 Illustration of Case B prediction within an MxPy measurement pattern framework

In this setting, the observation window (OW) can be defined as the period of time from the present stretching into the past containing all measurements used by the predictor, and the prediction window (PW) can be defined as the period of time containing the next y predicted beam management cycles.

Our AI/ML predictors are encoder-only transformer models. We compare AI/ML prediction results to prediction results from a baseline sample-and-hold prediction scheme, where the latest measured cell-level metrics are used as predictions for future metric values. We consider a prediction window length of 400ms.

For model generalization studies, we trained three models: one on a 60 km/h training dataset, one on the 90 km/h training dataset, and one on both 60 and 90 km/h training datasets. We compare performance of these models on 60 and 90 km/h test datasets.

To evaluate predictor performance, we consider the following metrics:

- Average L3 cell level RSRP difference (dB): the mean absolute error in dB between the predicted and measured cell-metric values.

- Average L3 cell level RSRP difference at prediction window edge (dB): the mean absolute error in dB between the predicted and measured cell-metric values for the final predicted instance in the prediction window (the prediction window “edge”).

- Top cell prediction accuracy across prediction window (%): the average prediction rate accuracy for the cell with the top RSRP across all predicted instances in the prediction window.

- Top cell prediction accuracy at prediction window edge (%): the average prediction rate accuracy for the cell with the top RSP for the final predicted instance in the prediction window (the prediction window “edge”).

## Simulation results

The following table summarizes simulation results for FR1 intra-cell, intra-frequency, temporal Case B prediction.

Table 7: Simulation results for FR1 intra-frequency, intra-cell, temporal domain Case B

| Report parameters |  | Qualcomm |  |  |
| --- | --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option | Option 2 in TR 38.343 [2], Section 6.3.1, with modifications based on Option 4 and a minor modification for extended mobility |  |  |
|  | UE trajectory boundary processing option | Trajectory termination |  |  |
|  | UE speed (30,60,90,120 Km/h) | 60 km/h, 90 km/h |  |  |
|  | Inter-frequency correlation assumption in general (yes or no) | N/A |  |  |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no) | N/A |  |  |
|  | Whether soft LoS is modeled or not | Yes |  |  |
|  | Spatial consistency option (A or B) | A |  |  |
|  | Number of TX beams | 1 |  |  |
|  | Number of RX beams | 1 |  |  |
|  | Measurement reduction rate | 90% |  |  |
|  | Observation window | <=1280 ms |  |  |
|  | Prediction window (Note3) | 400 ms |  |  |
|  | Any other parameters | N/A |  |  |
| Data Size (Number of Samples) | Training/validity | 1588/390 (60 km/h), 1826/457 (90 km/h) |  |  |
|  | Testing | 488 (60 km/h), 571 (90 km/h) |  |  |
| AI/ML model input/output | Model input | Up to 32 sets of past measured L3-filtered cell-level metric values |  |  |
|  | Model output | Prediction for future L3-filtered cell-level metric values |  |  |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | Transformer |  |  |
|  | Model complexity in a number of parameters (K) | 1279K |  |  |
|  | Model complexity in model size (e.g. Mbyte) | 9.8 Mb |  |  |
| Metrics | Average L3 cell level RSRP difference across prediction window (dB) | Predictor | Inference on 60 km/h | Inference on 90 km/h |
|  |  | AI/ML trained on 60 km/h dataset | 2.373 | 2.723 |
|  |  | AI/ML trained on 90 km/h dataset | 2.448 | 2.776 |
|  |  | AI/ML trained on 60 km/h dataset and 90 km/h dataset | 2.377 | 2.712 |
|  |  | Baseline (sample-and-hold) | 2.210 | 2.604 |
|  | Average L3 cell level RSRP difference at prediction window edge (dB) | Predictor | Inference on 60 km/h | Inference on 90 km/h |
|  |  | AI/ML trained on 60 km/h dataset | 3.201 | 3.724 |
|  |  | AI/ML trained on 90 km/h dataset | 3.273 | 3.763 |
|  |  | AI/ML trained on 60 km/h dataset and 90 km/h dataset | 3.223 | 3.721 |
|  |  | Baseline (sample-and-hold) | 3.112 | 3.708 |
|  | Top cell prediction accuracy across prediction window (%) | Predictor | Inference on 60 km/h | Inference on 90 km/h |
|  |  | AI/ML trained on 60 km/h dataset | 81.871 | 79.274 |
|  |  | AI/ML trained on 90 km/h dataset | 81.688 | 78.941 |
|  |  | AI/ML trained on 60 km/h dataset and 90 km/h dataset | 82.028 | 79.385 |
|  |  | Baseline (sample-and-hold) | 82.397 | 79.445 |
|  | Top cell prediction accuracy at prediction window edge (%) | Predictor | Inference on 60 km/h | Inference on 90 km/h |
|  |  | AI/ML trained on 60 km/h dataset | 76.639 | 73.002 |
|  |  | AI/ML trained on 90 km/h dataset | 76.498 | 72.899 |
|  |  | AI/ML trained on 60 km/h dataset and 90 km/h dataset | 76.691 | 73.202 |
|  |  | Baseline (sample-and-hold) | 76.734 | 72.768 |

Figure 14 shows the top cell prediction accuracy for each predictor, both across the prediction window and at the prediction window edge, for the 60 km/h and 90 km/h test datasets. Similarly, Figure 15 shows the top average L3 cell-level RSRP difference for each predictor, both across the prediction window and at the prediction window edge, for the 60 km/h and 90 km/h test datasets.

We note that, as with the 3-cell FR1 intra-frequency temporal cell metric prediction Case B results reported in Section 4.6 of [7], the sample-and-hold baseline is competitive with the AI/ML predictors in terms of top cell prediction accuracy, with all predictors demonstrating high accuracy for a 400-ms observation window. We also note that all predictors display similar prediction performance for both the 60 km/h and 90 km/h datasets, demonstrating that an AI/ML cell-metric predictor trained on a certain UE speed is has similar capability in predicting cell metrics for UEs at a different speed as a predictor that has been specifically trained for UEs at that different speed. Finally, we note that across the board, prediction performance for 90-km/h UEs is worse than that for 60-km/h UEs, demonstrating that cell-metric prediction becomes increasingly difficult with increasing UE speed.

Figure 14 top cell prediction accuracies for FR1 model generalization

Figure 15 L3-cell level RSRP differences for FR1 model generalization

Observation 1: In the case of FR1 intra-frequency, inter-cell temporal domain Case B prediction, sample-and-hold predictors are competitive with the AI/ML predictors in terms of top cell prediction accuracy and L3 cell-level RSRP difference metrics.

Observation 2: In the case of FR1 intra-frequency, inter-cell temporal domain Case B prediction, an AI/ML cell-metric predictor trained on a dataset with UEs moving at 60 km/h has similar prediction performance for a 90 km/h test dataset as a cell-metric predictor trained on a dataset with UEs moving at 90 km/h (and vice versa), demonstrating an ability for cell-metric predictors to generalize across UE speeds.

Observation 3: In the case of FR1 intra-frequency, inter-cell temporal domain Case B prediction, performance for 90-km/h UEs is worse than that for 60-km/h UEs, demonstrating that cell-metric prediction becomes increasingly difficult with increasing UE speed.

# Conclusions

Observation 1: In the case of FR1 intra-frequency, inter-cell temporal domain Case B prediction, sample-and-hold predictors are competitive with the AI/ML predictors in terms of top cell prediction accuracy and L3 cell-level RSRP difference metrics.

Observation 2: In the case of FR1 intra-frequency, inter-cell temporal domain Case B prediction, an AI/ML cell-metric predictor trained on a dataset with UEs moving at 60 km/h has similar prediction performance for a 90 km/h test dataset as a cell-metric predictor trained on a dataset with UEs moving at 90 km/h (and vice versa), demonstrating an ability for cell-metric predictors to generalize across UE speeds.

Observation 3: In the case of FR1 intra-frequency, inter-cell temporal domain Case B prediction, performance for 90-km/h UEs is worse than that for 60-km/h UEs, demonstrating that cell-metric prediction becomes increasingly difficult with increasing UE speed.

# References

[1] Chair notes, RAN2 #128 meeting.

[2] “3GPP TR 38.843 V18.0.0: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”, 3GPP, January 2024, https://portal.3gpp.org/desktopmodules/Specifications/SpecificationDetails.aspx?specificationId=3983 (accessed May 7, 2024).

[3] “3GPP RAN2 2405205: Considerations for mobile UE trajectory generation and channel modelling for simulation evaluation”, 3GPP, May 2024, 3gpp.org.

[4] “3GPP TR 38.901 V17.0.0: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on channel model for frequencies from 0.5 to 100 GHz (Release 17).”  3GPP RAN Plenary, January 2024, 3gpp.org.

[5] IST-WINNER II Deliverable 1.1.2 v.1.2, "WINNER II Channel Models", IST-WINNER2, Tech. Rep., 2007

[6] “3GPP TR 38.331 V18.2.0: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Radio Resource Control (RRC) protocol specification (Release 18).” 3GPP RAN Plenary, June 2024, 3gpp.org.

[7] “3GPP RAN2 2410774: Simulation Results for RRM Measurement Prediction”, 3GPP, November 2024, 3gpp.org.