3GPP TSG-RAN WG2#129		      					R2-2501073

Athens, Greece, February 17 – 21, 2025

Agenda item:	8.6.4

Source: 	Kyocera

Title: 	Initial consideration of coexistence scenarios for C-LTM

Document for:	Discussion

Introduction

RAN2#127bis agreed to the following regarding the L1 execution conditions [1].

2. Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions.

The following is the RAN#128 agreement on the execution conditions [2] :

Agreements on C-LTM 2. CondEventA3 and CondEventA5 conditions can be baseline for the conditional LTM execution. 3. The L1 execution condition of a candidate cell is associated to only one triggering event. 4. For L3 execution condition, it may consist of one or two triggering condition(s). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met. Only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell. 5. To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition: - The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell. - The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell. 11. For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

In this contribution, various coexistence scenarios identified, and approaches are discussed.

Discussion

Coexistence of C-LTM and LTM

In the revised WID [3], the support for the coexistence of C-LTM and LTM is still unclear. On the other hand, the draft agenda indicates that companies are encouraged to consider whether to support the coexistence of C-LTM and LTM, and the additional specification impacts, if any. So, it would be discussed whether any additional impact is foreseen for each phase of (C-)LTM.

Regarding the preparation phase, according to the current RRC running CR [6], the L1/L3 condition for C-LTM is configured within the existing LTM Config as an extension. So, the coexistence of C-LTM and LTM can be achievable with little or no specification impact.

LTM-Config-r18 ::=   SEQUENCE {

[…]

ltm-CandidateToReleaseList-r18        SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-CandidateId-r18    OPTIONAL,   -- Need N

ltm-CandidateToAddModList-r18         SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-Candidate-r18      OPTIONAL,   -- Need N

[…]

}

LTM-Candidate-r18 ::=     SEQUENCE {

ltm-CandidateId-r18                            LTM-CandidateId-r18,

[…]

...,

[…]}

[…]

CLTM-ExecutionConditions-r19 ::= SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF ExecutionConditions-r19

ExecutionConditions-r19 ::= SEQUENCE {

ltm-CandidateId-r19                            LTM-CandidateId-r18,

executionCondition-r19                         CHOICE {

l1-Conditions-r19                              LTM-CSI-ReportConfigId-r18,

l3-Conditions-r19                              SEQUENCE (SIZE (1..2)) OF MeasId

}                                                                                                    OPTIONAL,   -- Need R

Editor’s Note: FFS whether L1 and L3 execution conditions can be configured simultaneously or not.

...

}

Since the same LTM Config is reused, the same LTM candidate IDs in each LTM Candidate are applicable regardless of whether the LTM Candidate contains the L1/L3 condition IE. Therefore, the NW can indicate the LTM candidate ID of C-LTM candidate cell by LTM Cell Switch Command MAC CE (LTM CSC MAC CE) without any change (including the MAC CE format and the corresponding UE behaviour.) So, even with Rel-19 LTM Config IE, the NW-controlled LTM execution (i.e., Rel-18 LTM) is triggered by the legacy LTM CSC MAC CE.

RAN2 should agree that Rel-19 C-LTM and Rel-18 LTM configurations can coexist within the existing LTM Config, as in the current RRC running CR.

RAN2 should agree that the existing LTM Cell Switch Command MAC CE can be reused as is, if the NW indicates the UE to execute the LTM candidate which is configured with the L1/L3 condition for C-LTM.

If Proposal 2 is agreeable, the NW may use the LTM CSC MAC CE to indicate to the UE to access the LTM candidate cell under the C-LTM event evaluation for the same candidate cell (Scenario #A) or a different candidate cell (Scenario #B). For CHO, if the UE receives a legacy Handover command while evaluating CHO conditions, it will abandon the CHO evaluation and proceed with the use of the latest handover configuration received.

LTM Cell Switch Command MAC CE may initiate Rel-18 LTM execution towards the LTM candidate cell under C-LTM event evaluation.

On top of Observation 1, it’s further discussed what the UE behaviour is, if LTM CSC MAC CE is received and the C-LTM event condition is fulfilled at the same time. In that case, there are two scenarios:

- Scenario #A (Figure 1 Upper): The cell where the execution condition of C-LTM is fulfilled is the same as the cell indicated by LTM CSC MAC CE

  - Execution condition is fulfilled for LTM candidate ID =1

  - LTM CSC MAC CE indicates LTM candidate ID =1

- Scenario #B (Figure 1 Lower): The cell where the execution condition of C-LTM is fulfilled is different from the cell indicated by LTM CSC MAC CE.

  - Trigger execution condition is fulfilled LTM candidate ID =1

  - LTM CSC MAC CE indicates LTM candidate ID =2 (regardless of whether to be configured with Rel-18 LTM with CSC MAC CE or Rel-19 C-LTM)

Figure 1 Two scenarios in which C-LTM and LTM coexist

In Scenario #A, no issue is observed since the same LTM candidate cell is indicated by LTM CSC MAC CE and fulfils the execution condition, i.e., it’s sufficient for the UE to just access the LTM candidate cell. If the LTM candidate cell of C-LTM is not configured to send measurement reports, Scenario #A doesn’t need to be considered.

On the other hand, in Scenario #B, the LTM candidate cell indicated by LTM CSC MAC CE is different from the LTM candidate cell that fulfils the execution condition. So, the UE may be uncertain about which LTM candidate cell to perform the LTM execution. If the selection of LTM candidate cell is left up to UE implementation, it may cause the “handover to wrong cell” as captured in [7], [8] which may be avoided if the UE follows the CSC MAC CE indicated by the NW since the NW has more knowledge of mobility control in general. Therefore, the UE should access to the LTM candidate cell indicated by LTM CSC MAC CE from the NW.

RAN2 should agree that the LTM execution triggered by LTM Cell Switch Command MAC CE is prioritized over the one triggered by C-LTM execution condition.

If Proposal 3 is agreeable, the UE behaviour to prioritize LTM CSC MAC CE should be further considered. For example, in C-LTM, two types of execution conditions, the L1 execution condition and the L3 execution condition, have been agreed upon [1], [2]. Therefore, there are two scenarios to consider:

- Scenario #B-1: L1 execution condition is fulfilled simultaneously with reception of LTM CSC MAC CE

- Scenario #B-2: L3 execution condition is fulfilled simultaneously with reception of LTM CSC MAC CE

For Scenario #B-1, L1 execution condition evaluation is at the MAC layer [2], so LTM CSC MAC CE is preferred. In the case of scenario #B-2, however, L3 execution condition evaluation is at RRC layer　[2], which is different from the layer receiving LTM CSC MAC CE. Therefore, it is necessary to coordinate between layers.

If Proposal 3 is agreeable, RAN2 should further discuss how RRC layer handles if L3 execution condition is fulfilled when MAC sublayer receives LTM Cell Switch Command MAC CE simultaneously.

Coexistence of L1 event and L3 event

RAN2#127bis agreed to the L1 execution conditions (i.e., Event LTM3/5-like) [1] and RAN2#128 agreed that L3 execution conditions (i.e., CondEventA3/5) can be baseline for the C-LTM execution [2].

According to the current RRC running CR [6], the NW cannot configure L1 and L3 execution conditions simultaneously for the same LTM candidate ID. Though, there is no restriction that L1 execution condition is configured to an LTM candidate ID while L3 execution condition is configured to a different LTM candidate ID, i.e., the L1/L3 evaluation conditions can coexist within the LTM Config. So, in the following scenario, the NW may configure the UE with the L1 and L3 execution conditions for different LTM candidate IDs, respectively:

- A low-frequency C-LTM candidate cell is configured with L3 execution condition.

  - The result of L3 measurement is higher reliability and stability than those of L1 measurement, so the L3 execution condition is configured.

- A high-frequency C-LTM candidate cell is configured with L1 execution condition.

  - The radio environment fluctuates significantly, making it difficult for L3 measurement to keep up, so the L1 execution condition is configured.

RAN2 should agree on the coexistence of L1/L3 conditions, i.e., a candidate cell with L1 condition and a different candidate cell with L3 condition simultaneously.

According to the TS38.331 [4] below, CHO has specifications for cases where multiple execution conditions are triggered simultaneously, and it is up to UE implementation which candidate cell to perform handover to.

5.3.5.13.5	Conditional reconfiguration execution

The UE shall:

1> if more than one pair of triggered PCell and associated triggered PSCell exist:

2> select one of the triggered PCell(s) and the associated triggered PSCell(s) as the selected cells for

conditional reconfiguration execution;

[…]

NOTE:	If multiple NR cells are triggered in conditional reconfiguration execution, it is up to UE implementation which one to select, e.g. the UE considers beams and beam quality to select one of the triggered cells for execution.

Therefore, it’s also possible that multiple execution conditions may be triggered simultaneously in C-LTM. If Proposal 5 is agreeable, there are the following three scenarios (Figure 2):

- Scenario 1: L1 execution conditions are triggered simultaneously in different C-LTM candidate cells.

- Scenario 2: L3 execution conditions are triggered simultaneously in different C-LTM candidate cells.

- Scenario 3: L1 execution conditions and L3 execution conditions are triggered simultaneously in different C-LTM candidate cells.

Figure 2 Scenarios where multiple execution conditions are triggered

In scenarios 1 and 2, as in the case of CHO, the execution conditions are triggered in the same layer. So, it’s possible the layer evaluating the event can compare both radio qualities from different candidate cells and should select the candidate cell with better radio condition. As another option, it may be up to UE implementation which C-LTM candidate cell to execute the LTM.

In scenario 3, however, execution conditions are triggered at different layers. Since there has been no discussion about multiple execution conditions being triggered at different layers, it should be considered as a new scenario.

In C-LTM, it is necessary to consider the case where L1 and L3 execution conditions are fulfilled at the same time for different C-LTM candidate cells.

In the case of scenario 3, there are three types of UE behaviour as below:

- Type 1: The UE accesses the C-LTM candidate cell triggering the L1 execution condition.

- Type 2: The UE accesses the C-LTM candidate cell triggering the L3 execution condition.

- Type 3: It is up to UE implementation to select the C-LTM candidate cells.

- According to the TS38.300 [5] below, after driving beam quality at the physical layer (i.e., L1 measurement), cell quality is derived from multiple beams at the RRC level (i.e., L3 measurement). Therefore, compared to the L1 execution condition (i.e., Type 1), the L3 execution condition (i.e., Type 2), which uses the results of the L3 measurement, is considered to have higher reliability and stability. Consequently, when the execution conditions of L1 and L3 are simultaneously fulfilled, the UE should prioritize accessing the C-LTM candidate cell that fulfills the execution condition of L3 (Type 2) over L1 (Type 1).

- Filtering takes place at two different levels: at the physical layer to derive beam quality and then at RRC level to derive cell quality from multiple beams.

- On the other hand, it is also possible to consider Type 3, where the selection of the C-LTM candidate cell is up to UE implementation, as in scenarios 1 and 2.

RAN2 should discuss whether the UE accesses the C-LTM candidate cell triggering the L3 execution condition or selects the C-LTM candidate cell based on UE implementation.

Coexistence of C-LTM and Conditional Reconfigurations (CHO/CPC/CPA)

According to the revised WID [3], C-LTM in DC is not supported so far. Therefore, there is no need to consider the coexistence of C-LTM, CPC and CPA. However, C-LTM and CHO may be configured simultaneously. In this case, as in 2.2, multiple execution conditions may be triggered simultaneously, and the following scenario is considered.

- Scenario i: C-LTM L1 execution conditions and CHO execution conditions are triggered simultaneously in different C-LTM candidate cells.

- Scenario ii: C-LTM L3 execution conditions and CHO execution conditions are triggered simultaneously in different C-LTM candidate cells.

If it is worth discussing the case of simultaneous triggering of different C-LTM execution conditions (i.e., L1 and L3 execution conditions), then the case of simultaneous triggering of CHO and C-LTM execution conditions should also be discussed.

Conclusion

In this contribution, the issues for supporting conditional intra-CU LTM are discussed. RAN2 is kindly asked to take into account the proposals below:

Proposal 1	RAN2 should agree that Rel-19 C-LTM and Rel-18 LTM configurations can coexist within the existing LTM Config, as in the current RRC running CR.

Proposal 2	RAN2 should agree that the existing LTM Cell Switch Command MAC CE can be reused as is, if the NW indicates the UE to execute the LTM candidate which is configured with the L1/L3 condition for C-LTM.

Observation 1	LTM Cell Switch Command MAC CE may initiate Rel-18 LTM execution towards the LTM candidate cell under C-LTM event evaluation.

Proposal 3	RAN2 should agree that the LTM execution triggered by LTM Cell Switch Command MAC CE is prioritized over the one triggered by C-LTM execution condition.

Proposal 4	If Proposal 3 is agreeable, RAN2 should further discuss how RRC layer handles if L3 execution condition is fulfilled when MAC sublayer receives LTM Cell Switch Command MAC CE simultaneously.

Proposal 5	RAN2 should agree on the coexistence of L1/L3 conditions, i.e., a candidate cell with L1 condition and a different candidate cell with L3 condition simultaneously.

Observation 2	In C-LTM, it is necessary to consider the case where L1 and L3 execution conditions are fulfilled at the same time for different C-LTM candidate cells.

Proposal 6	RAN2 should discuss whether the UE accesses the C-LTM candidate cell triggering the L3 execution condition or selects the C-LTM candidate cell based on UE implementation.

Observation 3	If it is worth discussing the case of simultaneous triggering of different C-LTM execution conditions (i.e., L1 and L3 execution conditions), then the case of simultaneous triggering of CHO and C-LTM execution conditions should also be discussed.

References

- R2-2409211 “Report from session on V2X/SL, R18/19 MOB, and R19 NES”, Vice Chairman (Samsung), RAN2#127bis

- R2-2410911 “Report from session on R16/17/18 V2X/SL, R18/19 MOB and R19 NES”, Vice Chairman (Samsung), RAN2#128

- RP-242356 “Revised Work Item: NR mobility enhancements Phase 4”, Apple Inc, China Telecom, RAN#105

- TS38.331, “Radio Resource Control (RRC) protocol specification”, 3GPP

- TS38.300, “NR and NG-RAN Overall Description; Stage 2”, 3GPP

- R2-24xxxxx “Rapporteur RRC for inter-CU and conditional LTM”, Ericsson, RAN2#129

- TR36.902, “Self-configuring and self-optimizing network (SON) use cases and solutions”, 3GPP

- TS36.399, “E-UTRA and E-UTRAN Overall description”, 3GPP