3GPP TSG-RAN WG2#129		      					R2-2501074

Athens, Greece, February 17 – 21, 2025

Agenda item:	8.6.4

Source: 	Kyocera

Title: 	Discussion on TA validity timer for C-LTM

Document for:	Discussion

Introduction

RAN2#128 agreed to the following regarding the early sync phase [1]:

Agreements on C-LTM 7. For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration. 8. The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4. 9. The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM. 10. Candidate cell TA is maintained by a new timer. 11. For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

In this contribution, a new timer handling is discussed.

Discussion

New timer for TA value of C-LTM candidate cell

In Rel-18 LTM, the serving gNB manages the TA validity acquired in the early sync phase for each LTM candidate cell and always sends the valid TA value in Cell Switch Command MAC CE. The UE applies the TA value and performs the LTM execution to the target cell, at the same time [2].

For Rel-19 Conditional LTM (C-LTM), however, the TA value acquired in the early sync phase is indicated via a new MAC CE which is different from Rel-18 Cell Switch Command MAC CE. So, there is a time difference between the reception of the new MAC CE with the TA value at the UE and the occurrence of the C-LTM execution. In this sense, it’s straightforward that the TA value indicated by the new MAC CE needs to be stored by the UE. In addition, different LTM candidate cells have different TA values, so the UE should store the association between the multiple TA values and each LTM candidate cell.

RAN2 should agree that the UE stores the multiple TA values associated with each LTM candidate cell.

After the early sync phase, the UE needs to check whether the TA value it stores is valid before the RACH-less C-LTM execution. For this purpose, it was agreed that “Candidate cell TA is maintained by a new timer” in the last meeting [1]. However, RAN2 has not discussed any details of the new validity timer and corresponding UE behaviours.

Before looking at the new validity timer for LTM candidate cell’s TA, it’s summarized that the current timing alignment timer (TAT) for the serving cell’s TA management. According to TS38.321 [3], the UE uses the TAT to manage the validity of the TA value for UL transmissions to the serving cell. Upon the TAT expiry, the UE MAC proceeds to flush HARQ buffer, indicate RRC to remove the PUCCH/SRS and cancel already received downlink assignment/uplink grants. The TAT is configured per Timing Advance Group (TAG) associated with multiple serving cells.

-	timeAlignmentTimer (per TAG) which controls how long the MAC entity considers the Serving Cells to the associated TAG to be uplink time aligned for the TAG;

In C-LTM, the UE uses the new timer to manage the validity of the TA value for the C-LTM candidate cell, which was obtained in the early sync phase, whereby it should be noted that the new timer works for the C-LTM candidate cell, not the serving cell like the existing TAT. Considering different early sync acquisitions for different C-LTM candidate cells don’t happen at the same time, the multiple validity timers should be able to be started for different C-LTM candidate cells, while the existing TAT which is configured per TAG. Note that it’s FFS whether the configuration of timer value(s) is common for all individual timers or separately configured per C-LTM candidate cells.

RAN2 should agree that the new validity timer is configured per C-LTM candidate cell.

Besides, it would be considered what the UE behaviour upon the new validity timer expiry. In the existing TAT, the UE performs to stop UL/DL transfer upon timer expiry. On the other hand, upon the new validity timer expiry, the UE does not need to stop UL/DL transfer towards the serving cell (i.e., the source cell) but should consider the corresponding TA value as invalid and/or discard the stored TA value.

RAN2 should agree that upon the new timer expiry, the UE considers the stored TA value, which is associated with the timer, as invalid and/or discards the stored TA value.

According to the current MAC running CR [4], after a C-LTM cell switch is triggered, the TA value corresponding to the C-LTM candidate cell (i.e., the target cell) on which the LTM cell switch was executed is applied to the PTAG, and the existing TAT associated with the PTAG is started or restarted.

1> when a conditional LTM cell switch procedure is triggered for a LTM candidate cell as in clause 5.y.2, and the ltm-Candidate-TimeAlignmentTimer associated with the LTM candidate cell is running: 2> apply the stored the TA value associated with the LTM target cell for the PTAG as specified in clause 6.1.3.75; 2> start or restart the timeAlignmentTimer associated with the PTAG as specified in clause 6.1.3.75.

However, it’s still unclear how to handle the new validity timers associated with other TA values for other C-LTM candidate cells, when the C-LTM cell switch is executed.

In legacy Conditional Handover, the TA value is always acquired upon the CHO execution, so no TA value is stored for CHO candidate cells in advance. It’s also noted that CHO does not support the subsequent CHO execution.

In Rel-18 LTM, the subsequent LTM execution is introduced, whereby the multiple LTM executions are performed without RRC Reconfiguration. According to TS38.300 [2], the steps of the early synchronization, LTM cell switch execution, and LTM cell switch completion can be performed multiple times for the subsequent LTM, using the LTM candidate configuration(s) provided in LTM preparation phase. It should be noted that the early sync phase is not mandated to be performed for the subsequent LTM and the TA values for LTM candidate cells are managed by the network. So, for more efficient subsequent LTM, it’s possible as a gNB implementation that the TA value acquired before an LTM cell switch is indicated to the UE for the next LTM cell switch (i.e., the TA value is not acquired in the current cell but in the previous serving cell; or something like an “early sync-less RACH-less subsequent LTM”.)

With regards to the subsequent C-LTM, it’s natural that the same principle as Rel-18 is applied, i.e., the subsequent C-LTM is performed without the LTM preparation phase, i.e., the UE maintains LTM candidate configurations. In addition, as a smart implementation in Rel-18 LTM as above, the early sync phase is not essential even for the RACH-less subsequent C-LTM.

So, it is better to maintain the TA values for other LTM candidate cells as long as it is valid, even after the C-LTM to a candidate cell is executed. In order to manage the validity of the TA values after the C-LTM cell switch, the new validity timer should not be stopped after the LTM cell switch as well. With this mechanism, the UE can perform RACH-less C-LTM even if the early sync phase does not always happen after the C-LTM cell switches.

Furthermore, if all the TA values are discarded every time after the C-LTM cell switch, and if the execution condition is fulfilled for the subsequent C-LTM before the UE obtains the TA value in early sync phase, the RACH-based C-LTM is needed to be executed, which is inefficient. The RACH-based C-LTM may be reduced if the UE keeps storing the TA values and the corresponding validity timers running.

RAN2 should agree that the UE can keep the new validity timers running and storing the TA values even after C-LTM cell switch, in order to support more efficient Subsequent C-LTM.

Conclusion

In this contribution, the issues for supporting conditional intra-CU LTM are discussed. RAN2 is kindly asked to take into account the proposals below:

Proposal 1	RAN2 should agree that the UE stores the multiple TA values associated with each LTM candidate cell.

Proposal 2	RAN2 should agree that the new validity timer is configured per C-LTM candidate cell.

Proposal 3	RAN2 should agree that upon the new timer expiry, the UE considers the stored TA value, which is associated with the timer, as invalid and/or discards the stored TA value.

Proposal 4	RAN2 should agree that the UE can keep the new validity timers running and storing the TA values even after C-LTM cell switch, in order to support more efficient Subsequent C-LTM.

References

- R2-2410911 “Report from session on R16/17/18 V2X/SL, R18/19 MOB and R19 NES”, Vice Chairman (Samsung), RAN2#128

- TS38.300, “NR and NG-RAN Overall Description; Stage 2”, 3GPP

- TS38.321, “Medium Access Control (MAC) protocol specification”, 3GPP

- R2-25000xx “Running MAC CR for enhanced mobility Ph4”, vivo, RAN2#129