3GPP TSG-RAN WG2 Meeting #129	R2-2501078

Athens, Greece, Feburary 17th – 21th, 2025

Agenda Item:	8. 18

Source:	SK Telecom, LG Uplus

Title:	Discussion on packet priority based access control

Document for:	Discussion and Decision

1	Introduction

In a congestion situation (e.g., subway station during rush hour), the network users experience poor quality of experience (QoE) due to frequent RRC connection failures. The RRC connection failures are derived from the large number of simultaneous RRC connection attempts, the significant portion of which are originated from the UEs transmitting packets with little impact on user QoE. From the perspective of ensuring QoE, it is crucial to prioritize RRC connections for packets generated by the users (e.g., browsing, gaming, or messaging) over those triggered without user interaction (e.g., syncing data, sending notifications, or uploading cloud data).

Access control mechanisms, which aim to resolve RRC connection congestions, have evolved in various aspects, such as Access Class Barring (ACB), Service Specific Access Control (SSAC), Application specific Congestion control for Data Communication (ACDC) and Unified Access Control (UAC) introduced in Rel-15 [1],[2]. However, these solutions exhibit a fundamental limitation—they all lack the ability to distinguish the packet priority based on whether the packets have little impact on user QoE or not.

In this contribution, we provide the motivations for defining the priority of the packets originated from the UEs as well as limiting the RRC Connection attempts from the UEs transmitting low-priority packets.

2	Discussion

2.1	RRC Connection Success Rate in Busy Hour

In a congestion situation (e.g., subway station during rush hour), the users experience poor quality of experience (QoE) due to frequent RRC connection failures. The RRC connection failures are derived from the large number of simultaneous RRC connection attempts.

[Figure 1. The variants of RRC connection success rate observed at the live network]

- The RRC connection failures are derived from the large number of simultaneous RRC connection attempts.

Since the RRC connection patterns of when a user actively generates packets and when there is no user interaction differ from each other, it is possible to distinguish whether the RRC connection attempts originate with or without user interaction. It is captured from the live network results that the significant portion of RRC connection attempts is originated from the UEs transmitting packets without user involvement. This implies that packets with little impact on user QoE account for a significant portion of the available RRC connections.

[Figure 2. Comparison of RRC connection patterns of when a user actively generates packets and when there is no user interaction]

- While the RRC connection patterns resulting from the packets when there is no user interaction show the periodic switch between idle and connected modes, those from when a user actively generates packets exhibit random behavior.

[Table 1. The comparison of the ratio of RRC connection attempts between passive and active packets]

| Time | 5G/LTE | Ratio of RRC Connection Attempt |  |
| --- | --- | --- | --- |
|  |  | Without User Interaction | With User Interaction |
| Idle Hour (4 AM) | 5G | 56% | 44% |
|  | LTE | 63% | 37% |
| Busy Hour (18 PM) | 5G | 28% | 72% |
|  | LTE | 37% | 63% |

- The packets with little impact on user QoE account for a significant portion of the available RRC connections.

2.2	Access Control Mechanisms

Access Class Barring is a basic access control mechanism employed in wireless networks to regulate UE access attempts during RRC congestion. This mechanism relies on barring parameters, such as barring time and barring factor. When a device attempts to send MO data or signaling, the RRC layer determines whether the access request is permissible based on these parameters.

Service-Specific Access Control is an access control technique designed to manage access for specific services, such as multimedia telephony voice and video. Unlike ACB, which primarily operates at the RRC layer, SSAC involves the IP Multimedia Subsystem (IMS) layer. When a device initiates an MMTEL voice or video call, the IMS layer references barring information from the RRC layer to make access decisions. SSAC facilitates targeted access control for specific services, ensuring prioritized management at the network congestion.

Application-Specific Congestion Control for Data Communication extends the scope of access control to the application layer, allowing for more granular control based on application categories. Devices are assigned specific ACDC categories, and the base station provides corresponding barring parameters. ACDC enables network operators to manage access based on the characteristics of different applications, offering a mechanism to control traffic with greater specificity.

Unified Access Control consolidates multiple access control mechanisms, including ACB, SSAC, and ACDC, into a unified framework standardized for 5G networks. UAC utilizes both standardized and operator-defined categories for access decisions, combining the functionalities of individual mechanisms to achieve consistency and flexibility. Similar to ACDC, UAC operates across application and RRC layers, while also accounting for exceptions such as emergency calls, incoming calls, or access requests from devices with specific identities.

However, these solutions introduced above exhibit a fundamental limitation— they all lack the ability to distinguish the packet priority based on whether the packets have little impact on user QoE or not.

- The current access control mechanisms all lack the ability to distinguish the packet priority based on whether the packets have little impact on user QoE or not.

2.3	Potential Objective

From the perspective of ensuring QoE, it is crucial to prioritize RRC connections for packets generated by the users (e.g., browsing, gaming, or messaging) over those triggered without user interaction (e.g., syncing data, sending notifications, or uploading cloud data). UEs can determine the priority of packets, for example, based on the user’s intent at the moment of attempting an RRC connection, such as whether the packet is initiated by the user or generated without the direct user interaction.

The gNB can transmit a message through specific SIB (e.g., SIB x), which imposes constraints on RRC connection attempts for low-priority packets using a probability and timer-based restriction. When a low-priority packet is generated in a UE, the connection attempt should proceed only if a randomly generated value meets the predefined probability threshold. If the condition is not met, the UE must wait for a specific duration before generating another random value to reattempt the connection. Conversely, for non-low-priority packets, the UE should establish an RRC connection without any restrictions. Note that the probability and timer-based restriction is the same approach that the UAC-based access barring exploits with the corresponding access control parameters [4].

[Figure 3. The proposed usage of SIB signaling for packet priority based access control]

Define a low priority packet that has no impact on the user QoE

        - UEs can determine the priority of packets, for example, based on the user’s intent at the moment of attempting an RRC connection, such as whether the packet is initiated by the user or generated without the direct user interaction.

Constrain RRC connection attempts from low priority packets

        - The gNB can transmit a message through specific SIB (e.g., SIB x), which imposes constraints on RRC connection attempts from low-priority packets using probability and timer-based restrictions.

3 Conclusion

Based on the discussion in the previous sections we propose the following:

- The RRC connection failures are derived from the large number of simultaneous RRC connection attempts.

- While the RRC connection patterns resulting from the packets when there is no user interaction show the periodic switch between idle and connected modes, those from when a user actively generates packets exhibit random behavior.

- The packets with little impact on user QoE account for a significant portion of the available RRC connections.

- The current access control mechanisms all lack the ability to distinguish the packet priority based on whether the packets have little impact on user QoE or not.

Based on the discussion in the previous sections we propose the following:

- Define a low priority packet that has no impact on the user QoE

Constrain RRC connection attempts from low priority packets

References

3GPP TS 22.011 V19.5.0 (2024-12): “Service accessibility”.

3GPP TS 24.501 V19.0.0 (2024-09): “Non-Access-Stratum (NAS) protocol for 5G System (5GS) Stage 3”.

3GPP TS 23.502 V19.2.0 (2024-12): “Procedures for the 5G System (5GS) Stage 2”.

3GPP TS 38.331 V18.3.0 (2024-09): “Radio Resource Control (RRC) protocol specification”.