3GPP TSG-RAN WG2#129	     					      R2-2501079

Athens, Greece, February 17 – 21, 2025

Agenda item:	8.1.2.2

Source: 	Kyocera

Title: 	Considerations on LCM for UE-sided model in Beam Management use case

Document for:	Discussion

Introduction

In this contribution, the remaining issues regarding LCM of the UE-sided model for the Beam Management use case, Applicable Functionality, the reason of Applicability changing and UAI prohibit timer are discussed.

Discussion

The Remaining Issues

Applicable Functionality

This section examines how the Applicable Functionality is defined. The RAN1 Reply LS [1] includes the following description of the Applicable Functionality.

Reply LS from RAN1 (R1-2410898) […] - The concept/terminology “functionality” of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters […] - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction

Based on the above LS from RAN1, our understanding is that the applicability report is based on A) and B).

The AssociatedID in A) linked to CSI-ReportConfig(s) is considered appropriate and an obvious conclusion is to apply the AssociatedID as the Applicable Functionality. However, it should be noted that RAN1 treats this as a working assumption.

RAN2 should adopt the AssociatedID in A) as the Applicable Functionality, taking into account that RAN1 has defined this as a Working Assumption, and treat it as a Working Assumption.

In the case of B), Applicable Functionality is not necessarily associated with the AssociatedID since it involves signalling the UE with some related parameters. These parameters may or may not include the associated ID, and can potentially be classified into the following three cases:

  - AssociatedID

  - AssociatedID, Related information

  - Related information

Related Information：

- Set A related information

- Set B related information

- Report content related information

- For BM-Case 2

  - Time instances related information for measurements

  - Time instances related information for prediction

Focus 1: Whether to Use IDs
The first focus is whether "Related Information" should be assigned an ID or used as is. The following criteria should be considered when deciding on ID usage:

- Reusability: If the information is shared among multiple UEs, assigning an ID can reduce the amount of information required for transmission.

- Scalability: Evaluate whether assigning an ID facilitates the addition of new information or accommodates specification changes.

- Transmission Efficiency: Quantitatively assess how much signal overhead can be reduced by assigning an ID.

- Uniqueness: If the information is specific to a particular UE (e.g., Set A related information), assigning an ID may not improve transmission efficiency and might offer limited value.

Focus 2: Who Should Assign the IDs?
The next focus is determining which Working Group (WG) should assign the IDs for this information. For example:

- When it should be requested from RAN1:

  - When the information is handled at the physical layer (PHY) (e.g., CSI measurement-related information).

- When it should be handled by RAN2:

  - When the information is handled at higher layers (RRC) (e.g., information in applicability reports).

- Handling of ambiguous information:

  - For information related to both the PHY and RRC layers, it is necessary to discuss and clarify which WG should take the lead.

Based on the above, the following is proposed:

RAN2 should discuss whether IDs related to information regarding B) (e.g., Set A/B related information, time-related information, etc.) should be assigned, considering criteria such as reusability, scalability, and transmission efficiency.

RAN2 should clarify which group (RAN1 or RAN2) is responsible for assigning IDs related to information regarding B), based on the division of roles between the physical layer (PHY) and higher layers (RRC).

RAN2 should discuss how to handle ambiguous information (e.g., functionality-related information).

RAN2 should discuss which working group (WG) should handle ambiguous information, related to both PHY and RRC.

Applicable Functionality Reporting

The Reason for Applicability Changing

In RAN2#128, RAN2 reached the following agreement on how the not-applicable functionality is reported [2].

Agreements (RAN2#128) FFS whether the UE reports explicitly “non-applicable” functionality when there is a change of applicability.

During the RAN2#128 meeting, the discussion on whether the UE should explicitly or implicitly report not-applicable functionality to the NW resulted in an FFS, as opinions were divided, with some companies supporting explicit reporting and others supporting implicit reporting. At the same time, there was a discussion on whether it is necessary to include the reason when a functionality changes. Here, we will examine whether it is necessary to provide a reason when applicable functionality becomes not-applicable.

1. Positive Impacts of Providing Reason Information

Providing reason information enables the network side to clearly understand why the applicability of a functionality has changed on the UE side. This allows for specific actions such as the following:

A) Occurrence of Hardware Limitations on the UE Side (e.g., computational resources, power)
For example, if the UE is constrained by computational resources or memory, providing reason information enables the network to dynamically assess the situation and optimize its response. This applies to scenarios where the network's actions are not predetermined and can be adjusted based on specific resource limitations, such as selectively disabling AI/ML functions.

B) Matching or Not-Matching of Conditions
For example, if the UE no longer meets certain conditions, such as the NW-side additional condition, the network can decide to apply alternative functionalities. By obtaining reason information, the network can be notified of specific reasons.

C) Compatible or Incompatible Conditions
For example, if the condition required by the UE does not match that of the network, providing reason information enables the network to identify the mismatched parameters and quickly adjust the condition as needed.

2. Cases Where Reason Information Is Unnecessary

On the other hand, the following scenarios are considered cases when reason information is unnecessary:

1) When hardware limitations on the UE side are detected, and the network cannot apply AI/ML functionality due to a predetermined response, reason information becomes unnecessary. This applies to scenarios where the network's actions are fixed and cannot be adjusted based on the specific reason certain hardware limitation.

2) When conditions are found to be incompatible, the UE cannot use the corresponding functionality regardless of the reason. Therefore, providing reason information is considered unnecessary in such cases.

3) Determination Based on Inference Results and Monitoring

- Through monitoring, the network may be able to estimate changes in the UE's applicability status. For example, if the UE fails to meet signal strength or throughput requirements, the network can infer the reasons from monitoring data. If the monitoring results are sufficient, it may not be necessary to explicitly have the UE transmit the reason information.

- Furthermore, if such a reason is included in inference results, the network might identify the causes that led the UE to a not-applicable state and make decisions to apply alternative functionalities. Therefore, including the reason information as part of functionality changes isn’t needed.

3. RAN1 Reply LS and Its Impact

As discussed in Section 2.1.1 of this document, the Applicable Functionality can be linked to the AssociatedID, and consequently, it may also be connected to CSIReportConfig and various types of Related Information. These can be considered as representing the conditions for applicability and not-applicability.
Therefore, if non-conformance to these conditions determines applicability or not-applicability, transmitting reason information becomes unnecessary.

Note that the above discussion pertains to changes from Applicable to Not-applicable. However, changes from Not-applicable to Applicable can be considered in the same manner.

Overall, it seems that there is no clear justification for introducing reason information at this stage. Therefore:

RAN2 should agree that that providing reason is not required when reporting changes in functionality.

Methods of Explicit or Implicit Expression

Regarding methods of expressing Explicit or Implicit, it is possible to effectively convey both meanings, for example, by using a bitmap. Through work in Stage-3, it is important to ultimately determine the specific method of expression and finalize the appropriate specifications, as the method can carry both implicit and explicit meanings.

The determination of whether explicit or implicit methods should be used can be addressed in the Stage-3 phase.

UAI Prohibit Timer

In RAN2#127-bis, RAN2 reached the following agreement on how the not-applicable functionality is reported [3].

Agreements (RAN2#127-bis) UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality. We should aim to align the design on how the applicable functionality are signaled. FFS on the applicability reporting content.

To date, functions related to UAI (e.g., DelayBudgetReport, OverheatingAssistance, etc.) have been equipped with a Prohibit Timer, which has been shown to effectively suppress unnecessary reporting over short periods and reduce network load. This proposal considers the possibility of applying a similar Prohibit Timer to Applicable Functionality Reporting.

The advantages and disadvantages of introducing a Prohibit Timer is described as follows:

Advantages and Disadvantages of the Prohibit Timer

- Advantages:

By limiting the consecutive transmission of UAI over a short period, the Prohibit Timer can reduce network load and enable efficient signal transmission.

Applying a mechanism already adopted in other functionalities to AI/ML functionalities can standardize system behavior and reduce operational complexity.

- Disadvantages:

In scenarios where rapid response to environmental or conditional changes is required, the introduction of the Prohibit Timer may cause notification delays, potentially affecting the applicability of models or the adaptability of the system.

If state transitions are infrequent, the effectiveness of introducing the Timer may be limited, posing a risk of reduced flexibility.

When fine granularity is required, multiple Timers may need to be configured, raising concerns about increased operational complexity.

It may be difficult for the network to determine the appropriate value for the Prohibit timer. This process is not always straightforward, especially when some of the factors influencing the decision are dependent on the UE.

Based on the above pros and cons, it can be understood that the main disadvantage of introducing the Prohibit Timer is its inability to handle frequent changes between applicable and not-applicable states. However, the legacy UAI Prohibit Timer allows the Timer value to be set within a range of, for example, DelayBudgetReportingProhibitTimer: 0, 0.4, 0.8, 1.6, 3, 6, 12, 30 seconds, enabling appropriate values to be selected even in scenarios where responsiveness is required. Based on this, it is considered that there are no significant issues associated with the introduction of the Prohibit Timer. Therefore, the introduction of the Prohibit Timer is recommended.

Next, consider the granularity of the Prohibit Timers.

Granularity and Responsiveness of the Prohibit Timer

The feasibility of introducing a Prohibit Timer in Applicable Functionality Reporting heavily depends on the granularity of the Timer and the requirements for responsiveness.

- Coarse Granularity (e.g., Use Case level, Sub-Use Case level):
In this case, the Timer is set for larger functional units, resulting in lower requirements for responsiveness. The introduction of a Prohibit Timer is expected to be effective.

- Fine Granularity (e.g., applicable functionality level, such as report configuration level, resource configuration level, or resource set level):
In this case, frequent state changes may occur, and responsiveness could become critical. In such scenarios, it would be necessary to set the Timer value with smaller increments.

Discussion on the Granularity of Applicable Functionality in RAN1

As discussed in Section 2.1.1 of this document, the granularity of Applicable Functionality is based on the following:

- AssociatedID

- AssociatedID, Related information

- Related information

In this sense, small changes in Applicable Functionality can lead to the need for reporting applicable functionality which may be mapped to Fine Granularity.

However, if multiple Timers are used, for example, dividing them at the granularity of the AssociatedID may improve responsiveness, but it is likely to increase the complexity of the specifications and add overhead.

That stated, setting Timers at the use case level could result in too coarse a granularity, making control difficult. Therefore, defining Timers at the sub-use case level is a possible approach, but it is suggested that this should be addressed in Stage-3.

RAN2 should introduce the Prohibit Timer related to changes in applicable functionality as OPTIONAL, similar to the legacy UAI configuration.

The number of Timers and the detailed configuration of Timer values should be determined in Stage-3, with the assumption that flexible Timer value settings will be used in scenarios where responsiveness is required.

Conclusion

In this contribution, we discuss the issues in terms of LCM for UE-sided model in Beam Management use case. RAN2 is kindly asked to take into account the proposals below:

Proposal 1	RAN2 should adopt the AssociatedID in A) as the Applicable Functionality, taking into account that RAN1 has defined this as a Working Assumption, and treat it as a Working Assumption.

Proposal 2	RAN2 should discuss whether IDs related to information regarding B) (e.g., Set A/B related information, time-related information, etc.) should be assigned, considering criteria such as reusability, scalability, and transmission efficiency.

Proposal 3	RAN2 should clarify which group (RAN1 or RAN2) is responsible for assigning IDs related to information regarding B), based on the division of roles between the physical layer (PHY) and higher layers (RRC).

Proposal 4	RAN2 should discuss how to handle ambiguous information (e.g., functionality-related information).

Proposal 5	RAN2 should discuss which working group (WG) should handle ambiguous information, related to both PHY and RRC.

Proposal 6	RAN2 should agree that that providing reason is not required when reporting changes in functionality.

Proposal 7	The determination of whether explicit or implicit methods should be used can be addressed in the Stage-3 phase.

Proposal 8	RAN2 should introduce the Prohibit Timer related to changes in applicable functionality as OPTIONAL, similar to the legacy UAI configuration.

Proposal 9	The number of Timers and the detailed configuration of Timer values should be determined in Stage-3, with the assumption that flexible Timer value settings will be used in scenarios where responsiveness is required.

References

- “Reply LS on applicable functionality reporting for beam management UE-sided model” (Samsung) (R1-2410898)

- “RAN2 Chair’s Notes”, (InterDigital), RAN2#128

- “RAN2 Chair’s Notes”, (InterDigital), RAN2#127-bis