3GPP TSG RAN WG2 #129		R2-2501103

Athens, GR, Feb 17th – 21st, 2025

Title:	LCM for UE-sided model for BM

Agenda item:	8.1.2.2

Source:	Google

Document for:	Discussion and Decision

# 1 Introduction

In RAN2#128 meeting, there were some agreements and FFSs for UE-sided model for BM case.

Agreements

- When a functionality configured by the network to be reported via UAI, becomes from non-applicable to applicable, the UE can reports it to the network.   FFS detailed design

- When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.

- FFS whether the UE reports explicitly “non-applicable” functionality when there is a change of applicability.   Verify this aligns with RAN1 configuration design

- Applicable functionality reporting at handover is supported with the same RRC procedure that will be specified within a cell, as a baseline, i.e. the NW-side additional conditions and/or the inference configuration related to the target gNB are transmitted by the target gNB as part of the HO command, and the UE in response transmits the applicability report (either in RRCReconfigurationComplete or in UAI) to the target gNB after completing the handover.

- Source cell UAI (as is) can be sent from source cell to target cell using existing signaling.   No further optimizations will be considered in RAN2 related to UAI.

- For BM use case for UE-side model, data collection related configuration(s) (e.g., measurement resources configuration) and associated ID(s) can be included in training data collection configuration.

- For data collection configuration UE-side model training, the UE can send a request for data collection.   FFS what the request contains.

- The network can provide the data collection configuration (at any point in time), with or without UE request.

- The following methods for network control of the initiation and configuration for data collection:

- The network can decide when to start/stop the data collection and send configuration.

- The network can configure whether UE is allowed to initiate request for data collection.

- FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration

In RAN2#127bis meeting, there were some agreements and FFSs for UE-sided model for BM case.

Agreements for BM

- UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality.   We should aim to align the design on how the applicable functionality are signaled.   FFS on the applicability reporting content.

In this paper, we further discuss the remaining FFSs and give related considerations.

# 2 Discussion

## Reporting of Supported functionality information

According to the previous progresses, RAN2 has made the following signalling procedure on applicable functionality reporting for beam management:

Figure 1 Applicable functionality reporting

As shown in figure 1, the UE will report the supported functionalities by UECapablityInformation message in step 2.

Based on the LS [1] from RAN1, there are some conclusions on supported functionalities.

In RAN1’s discussion of RAN 2 terminologies on beam management,

- The concept/terminology “functionality” of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-enabled Features/FGs

- The concept/terminology “functionality” of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters

- The Activated functionalities may be enabled based on CSI framework.

Therefore, the meaning and the granularity of “functionality” for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same.

To enable the identification of the supported functionalities between UE and NW, it is an intuitive idea to introduce the functionality identification (ID) for each supported functionality. As shown in the LS [1], the granularity of supported functionalities may be feature/FGs.

Proposal 1: UE reports functionality IDs of supported functionalities, i.e., in the granularity of AI/ML-enabled feature/FGs, in UECapablityInformation message.

## Applicable Functionality Content and Reporting

### Content of Applicable Functionality Information

The reporting of applicable functionality information can assist the NW to identify the one(s) which can be chosen for activation. If the functionality ID is defined and introduced, it is straightforward to explicitly report the functionality ID of the applicable functionality, as previously reported in the supported functionalities.

As mentioned in the above section, the UE initially reports the supported functionalities, e.g., functionality ID, to the NW. Subsequently, it reports the applicable functionality information. The alternative solution may be to report implicit information for the applicable functionalities. For the implicit information, it can be mapped to the supported functionality list and finally used to identify the actual functionality information. Technically speaking, for the implicit reporting scheme, there may be less signalling overhead. For example, a bitmap can be reported as the applicable functionality information. For each bit in the bitmap, it corresponds to a specific functionality in the supported functionality reported in step 2. With the above knowledge, the NW can still deduce the explicit functionality ID of the applicable functionality information.

Proposal 2: For the content of applicable functionality reporting, the following two options can be considered:

- Opt 1: explicit functionality ID of the applicable functionality

- Opt 2: implicit information (e.g., bitmap) mapped to the supported functionality ID

### Reporting of applicable functionality information

The agreements on inference configuration in [1] are illustrated below:

Agreement

- In Step 3, following configurations are provided from NW to UE:

  - UE is allowed to do UAI reporting via OtherConfig,

  - The applicability report is based on A) and/or B)

    - It is up to RAN 2 to design the container

    - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied)

      - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3

    - B) One set or multiple sets of inference related parameters for applicability report only (not for inference)

      - It is up to RAN2 to design the container.

      - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g.,

        - the associated ID

          - Note: this doesn’t imply the associated ID is mandatory

        - Set A related information

        - Set B related information

        - Report content related information

        - For BM-Case 2,

          - Time instances related information for measurements

          - Time instances related information for prediction

Based on the information provided in LS [1], it is known that the NW may provide to the UE A) one or more of CSI-ReportConfig for inference configuration and/or B) one set or multiple sets of inference related parameters. The NW may also provide the associated ID to the UE. Based on the configurations received from the NW in step 3, the UE determines the applicable functionalities. When reporting these functionalities, the UE should categorize the information according to the associated CSI-ReportConfig or set of inference-related parameters.

Proposal 3: The applicable functionalities are reported, categorized by the associated CSI-ReportConfig or set of inference-related parameters.

In RAN2#127bis meeting, it was agreed that UE can support the applicable functionality reporting in both RRCReconfigurationComplete and UAI messages.

Agreements for BM

- UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality.   We should aim to align the design on how the applicable functionality are signaled.   FFS on the applicability reporting content.

If the UE receives NW-side associated ID for applicable functionality determination, e.g., in RRC Reconfiguration message, the UE can report applicable functionality information via RRC Reconfiguration Complete or UAI message. The combination of the two messages for applicable functionality reporting still need more detailed designs.

In solution 1, this can depend on UE implementation to select which message to report the applicable functionality information. If there is no applicable functionality provided in RRC Reconfiguration Complete message, one case is that the UE performs applicable functionality determination but fails to find any applicable one. The other case may be that the UE hasn’t started applicable functionality determination. Therefore, additional information may be provided for the NW to understand the case in the RRC Reconfiguration Complete message.

In another solution 2, we can follow the principle of measurement gap reporting scheme. The NW can indicate explicit information for UE to firstly report the applicable functionality information via RRC Reconfiguration Complete message. With the explicit information, the UE will definitely perform the applicability checking and decide applicable functionality before the content determination of RRC Reconfiguration Complete message. If there is at least one applicable functionality available, the UE will report the applicable functionality information to the NW in RRC Reconfiguration Complete message. If the UE doesn’t report any applicable functionality in RRC Reconfiguration Complete message, the NW will realize that no applicable functionality is found at UE side.

Proposal 4: Reporting applicable functionality in RRC Reconfiguration Complete or UAI message can be

- Opt 1: up to UE implementation

  - If the RRCReconfigurationComplete message contains no applicable functionalities, the UE sends an additional indication to the NW that no functionalities are available or that applicability checking was not performed.

- Opt 2: based on explicit information from NW (similar as measurement gap reporting scheme)

  - The explicit information indicates the UE to firstly report via RRC Reconfiguration Complete message (following the principle of measurement gap scheme).

### Reporting of updated applicable functionality information

In RAN2#128 meeting, RAN2 discussed UE behaviours when detecting an activated applicaple functionality becoming non-applicable. The following agreements were achieved below:

Agreements

- When a functionality configured by the network to be reported via UAI, becomes from non-applicable to applicable, the UE can reports it to the network.   FFS detailed design=>UE autonomously deactivates it based on indication from NW in advance.

- When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.

As previously mentioned, when an activated applicable functionality becomes non-applicable, the UE reports this to the NW. The NW is expected to deactivate the active functionality. However, this process introduces at least one L3 round-trip time (RTT) of latency before the UE receives the deactivation command. During this delay, the UE continues operating with the (now non-applicable) activated functionality for inference, leading to degraded inference accuracy compared to traditional, non-AI measurements.

To mitigate the latency and improve the system performance, the NW can pre-emptively send a deactivation indication before receiving the non-applicable report. If the deactivation indication is present, UE can deactivate the activated functionality when it becomes non-applicable. Subsequently, the UE reports the non-applicable information or the deactivation information of the activated functionality to the NW. If the deactivation indication is absent, the UE reports the non-applicable functionality to the NW and waits for the NW to deactivate the functionality via a subsequent response message.

Proposal 5: The NW can send a deactivation indication to the UE prior to receiving a non-applicability report.  The UE's behaviour depends on this indication:

- Deactivation Indication Present: The UE deactivates the previously activated functionality when it becomes non-applicable and informs the NW of the deactivation.

- Deactivation Indication Absent: The UE sends a non-applicability report and expects the NW to deactivate the functionality in a subsequent response.

### Other procedures for applicable functionality reporting

To enable the flexible request and reporting for the applicable functionality, the NW can configure the request in the RRC Resume and Reestablishment messages. Correspondingly, the UE can indicate the applicable functionality information in the RRC Resume/Reestablishment Complete messages respectively.

Proposal 6: RRC Resume and Reestablishment procedures can be used for request and reporting of the applicable functionality information.

## UE-side Training Data Collection

In RAN2#128 meeting, there were some agreements and FFSs for data collection for UE-side model:

Agreements

- For BM use case for UE-side model, data collection related configuration(s) (e.g., measurement resources configuration) and associated ID(s) can be included in training data collection configuration.

- For data collection configuration UE-side model training, the UE can send a request for data collection.   FFS what the request contains.

- The network can provide the data collection configuration (at any point in time), with or without UE request.

- The following methods for network control of the initiation and configuration for data collection:

- The network can decide when to start/stop the data collection and send configuration.

- The network can configure whether UE is allowed to initiate request for data collection.

- FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration

As shown in the agreements, both reactive (with UE request) and proactive (without UE request) schemes are agreed for NW to provide data collection configuration.

### Reactive scheme: data collection configuration wo UE request

In reactive approach, the UE can initiate the data collection request. Based on UE capability, the UE may support model training for some use case(s), features or feature groups. Correspondingly, the UE only wants to perform data collection for the supported model(s) training. When the UE initiates the request for data collection, it is straightforward to include its preferred data collection configuration(s). The preferred data collection configuration can include the use case/feature/feature group related information. The NW can provide related data collection configuration for the UE to collect data for the reported use case/feature/FG. In addition, if the UE has more knowledge of the model to be trained, e.g., NW-side associated ID, set A related information, it can include it in the request message.

Proposal 7: The data collection request includes the preference for data collection configuration for the model to be trained, e.g., the use case/feature/FG information, NW-side associated ID or set A related information.

Upon receiving the data collection request, the NW can configue the relevant configuration(s) and associated ID(s) in the training data collection configuration. To guarantee that the NW can support requested data collection configurations, one option is for the NW to send data collection support information to the UE. This support information specifies which data collection types can be requested. When initiating a data collection request, the UE must consider this data collection support information.

Proposal 8: The NW configures data collection support information before the UE initiates any data collection requests.

When receiving the data collection request, the NW may decide not to send related training data collection configuration to the UE. To avoid frequent data collection request from the UE, the prohibit timer similar as onDemandSIB-RequestProhibitTimer should be considered. The prohibit timer can be provided to the UE in system information or dedicated RRC message.

Proposal 9: A prohibit timer is introduced to prevent the UE from immediately re-initiating a data collection request.

### Proactive scheme: data collection configuration wo UE request

In the proactive scheme, the UE reports the UE capability of the data collection configuration or preferred data collection configuration(s) to the NW. The UE may not initiate the data collection request. When the NW can support the related data collection configuration and has enough resources, the NW can proactively provide the related data collection configuration to the UE based on UE capability.

Proposal 10: The UE provides data collection related capability to the NW and the NW provides the data collection configuration without UE request.

### Failure Indication

Based on the previous solutions, the UE receives the data collection configuration from NW for UE-side data collection and the NW concurrently provides the measurement resources. When the UE is unable to perform data collection based on received configuration, it is essential for the UE to notify the NW. The notification allows the NW to cease providing unnecessary measurement resources. This can conserve network resources.

Moreover, the UE can provide additional information, e.g., specifying the reason for its inability to perform data collection. The potential reasons can include abnormal UE states (e.g., low power, low memory or overheating) or inconsistency of the NW configurations (e.g., NW-side associated ID).

Proposal 11: The UE sends an indication to the NW, specifying the reason (e.g., UE inability or inconsistency of NW configuration) when it is unable to perform the configured data collection.

# 3 Conclusion

In this contribution, we discussed the potential solutions on the remaining FFSs and gave more considerations on the applicable functionality reporting schemes. The related observation and proposals are summarized below:

Supported functionality information:

Proposal 1: UE reports functionality IDs of supported functionalities, i.e., in the granularity of AI/ML-enabled feature/FGs, in UECapablityInformation message.

Applicable functionality content and reporting:

Proposal 2: For the content of applicable functionality reporting, the following two options can be considered:

- Opt 1: explicit functionality ID of the applicable functionality

- Opt 2: implicit information (e.g., bitmap) mapped to the supported functionality ID

Proposal 3: The applicable functionalities are reported, categorized by the associated CSI-ReportConfig or set of inference-related parameters.

Proposal 4: Reporting applicable functionality in RRC Reconfiguration Complete or UAI message can be

- Opt 1: up to UE implementation

  - If the RRCReconfigurationComplete message contains no applicable functionalities, the UE sends an additional indication to the NW that no functionalities are available or that applicability checking was not performed.

- Opt 2: based on explicit information from NW (similar as measurement gap reporting scheme)

  - The explicit information indicates the UE to firstly report via RRC Reconfiguration Complete message.

Proposal 5: The NW can send a deactivation indication to the UE prior to receiving a non-applicability report.  The UE's behaviour depends on this indication:

- Deactivation Indication Present: The UE deactivates the previously activated functionality when it becomes non-applicable and informs the NW of the deactivation.

- Deactivation Indication Absent: The UE sends a non-applicability report and expects the NW to deactivate the functionality in a subsequent response.

Proposal 6: RRC Resume and Reestablishment procedures can be used for request and reporting of the applicable functionality information.

UE-side data collection:

Reactive approach with UE request:

Proposal 7: The data collection request includes the preference for data collection configuration for the model to be trained, e.g., the use case/feature/FG information, NW-side associated ID or set A related information.

Proposal 8: The NW configures data collection support information before the UE initiates any data collection requests.

Proposal 9: A prohibit timer is introduced to prevent the UE from immediately re-initiating a data collection request.

Proactive approach without UE request:

Proposal 10: The UE provides data collection related capability to the NW and the NW provides the data collection configuration without UE request.

Proposal 11: The UE sends an indication to the NW, specifying the reason (e.g., UE inability or inconsistency of NW configuration) when it is unable to perform the configured data collection.

# Reference

[1]. R1-2410898, Reply LS on applicable functionality reporting for beam management UE-sided model