3GPP RAN WG2 Meeting #129	R2-2501122

Athens, Greece, February 17th – 21st 2025

Agenda Item:	8.1.2.2

Source:	InterDigital

Title:	LCM for UE-sided model for Beam Management use case

Document for:	Discussion, Decision

# Introduction

A RAN2 objective for AI/ML for Air Interface Work Item [1] is to design a general framework for one-sided AI/ML, including the signaling and protocol aspects of Life Cycle Management (LCM) to enable functionality selection, activation, deactivation, switching and fallback. In RAN2#127 [2] the following procedure for reporting applicable functionalities was agreed:

Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - (Between “Step 3” and “Step 4”) UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration). Step 5: 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not. The applicable functionality is initially activated by receiving its configuration when it is provided in Step 5. FFS on initial activation of applicable functionality if configuration of supported functionality is agreed to be provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation.

This contribution discusses the above procedure for applicable functionality determination and reporting, specifically remaining details on the applicable functionality reporting procedure and clarifications based on conclusions provided in an LS response from RAN1 [3].

# Discussion

## RAN1 response LS on applicable functionality reporting

In RAN2#127 an LS was sent to RAN1 [4] for clarification on several aspects of the general LCM procedure. A response LS [3] was recently received and is discussed below.

### Details of inference configuration

In Step 3, UE receives several configuration(s) to evaluate the applicability of a supported AIML functionality. One of these is the inference configuration of the supported AIML functionality, however configuration details are currently FFS:

- FFS on configuration (e.g. inference configuration) of supported functionalities.

From [3], UE receives one or more CSI-ReportConfig for inference configuration in either Step 3 or in Step 5. RAN2 can confirm that the inference configuration for AIML BM is provided via CSI-ReportConfig, however It is noted however that additional IEs specific to AIML/other Stage 3 details may be further considered.

Proposal 1:	Confirm CSI-ReportConfig is used for inference configuration.

### Applicable functionality reporting

UE determines and reports the applicable functionalities in Step 4. UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. Other configuration(s) that may be considered by the UE when determining applicability are currently FFS:

- FFS whether other configuration can considered by UE (e.g. inference configuration).

- As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration).

From [3], two options are provided: A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied); and/or B) One set or multiple sets of inference related parameters for applicability report only (not for inference). It is noted that these configuration(s) are considered in addition to the NW-side additional conditions (if provided), UE-side additional conditions and model availability in device.

Proposal 2:	Confirm UE decides applicable functionalities based on one or more CSI-ReportConfig and/or set(s) of inference related parameters (in addition to NW-side additional conditions (if provided), UE-side additional conditions and model availability in device).

Based on current LCM framework, part of the configurations provided by the NW to the UE for applicability reporting may include the NW-side additional conditions. It is however currently optional, and FFS how the UE will evaluate funcitonality applicability in their absence:

- FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

The inclusion of Associated ID (i.e., NW-side additional conditions) may be configured in CSI framework as a RAN1 working assumption currently. It may also be included within the set of inference related parameters, however again it’s inclusion is not considered mandatory.

Although this is currently a working assumption and may be subject to change based on RAN1 agreement, RAN2 may assume that if the Associated ID (i.e., NW-side additional conditions) are not included within the received configuration(s), it is not needed to evaluate the applcaibility of the supported functionality and the remaining information provided within CSI-ReportConfig or the set of inference related parameters is sufficient.

Proposal 3:	RAN2 assumes if NW-side additional condition is not provided in Step 3, UE can still determine applicable functionalities from other information within CSI-ReportConfig and/or set(s) of inference-related parameters.

### Functionality (de)activation

The UE may receive CSI-ReportConfig for inference configuration within Step 3 to support determination of functionality applicability, however the initial state of this configuration (e.g., whether it is considered deactivated and only used for applicability determination vs. being considered active and immediately available for inference) is currently FFS in RAN2:

- FFS the initial activation state.

- FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3.

For CSI-ReportConfig provided in Step 3, RAN1 has concluded that:

- Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW (e.g., via L1/L2 signalling) after the applicability report; and

- Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete.

And for CSI-ReportConfig provided in Step 5, RAN1 has concluded that:

- Aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete; and

- Periodic CSI Report is considered activated after RRCReconfigurationComplete.

In all above cases, CSI-ReportConfig for inference configuration is considered deactivated upon reception either until transmission of RRCReconfigurationComplete or reception of L1/L2 activation/triggering (i.e., in response to an applicability report). It is further noted that this understanding also revises the previous agreement of “The applicable functionality is initially activated by receiving its configuration when it is provided in Step 5”.

Proposal 4:	Confirm CSI-ReportConfig for inference configuration is initially deactivated upon reception and actived only after RRCReconfigurationComplete (e.g., in the following step) and/or reception of L1/L2 signaling (e.g., in response to applicability report).

The above RAN1 conclusions also address the following FFS (at least for the case of aperiodic and semi-persistence CSI report for inference):

- FFS on additional L1/L2 signaling for activation/deactivation.

Which is also explicitely confirmed via response to Q10: “RAN1 understands that L1 and MAC signalling can be used for aperiodic CSI Report and semi-persistent CSI report.”

Proposal 5:	Confirm L1/L2 signaling can be used for triggering aperiodic CSI report for inference and/or for (de)activation of semi-persistent CSI report for inference.

Finally, it is currently FFS in RAN2 whether multiple applicable functionalities (i.e., CSI-ReportConfig for inference configuration) can be activated at the same time:

- FFS if multiple applicable functionalities can be activated at the same time.

Which is clearly addressed by the following RAN1 agreement, also included within the LS response [3]:

Agreement For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability.

RAN2 can therefore confirm multiple applicable functionalities (i.e., CSI-ReportConfig for inference configuration) can be active/activated at the same time, resolving the above FFS.

Proposal 6:	Confirm multiple applicable functionalities (i.e., CSI-ReportConfig for inference configuration) can be active at the same time.

### Other aspects

The following FFSs are either still under discussion or considered as a working assumption in RAN1:

- Network may provide NW-side additional condition.  FFS on the RRC signalling and whether it is mandatory or optional.

- FFS on the content of configuration.

RAN2 may therefore postpone discussion on these topics until further progress in RAN1.

Proposal 7:	Postpone RAN2 discussion on functionality granularity, Associated ID, content of inference configuration, and content and optionality of NW-side conditions until further progress in RAN1.

With the above confirmations and conclusions, RAN2 should update the currently agreed LCM framework for BM to align with RAN1 agreements. For example, Step 3 may be updated as follows:

“Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) One or more of CSI-ReportConfig for inference configuration and/or one set or multiple sets of inference related parameters for applicability report only (not for inference) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - (Between “Step 3” and “Step 4”) UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. and one or more CSI-ReportConfig for inference and/or one or multiple sets of inference related parameters (if provided) FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

Proposal 8:	Update LCM steps for AIML BM to align with RAN1 conclusions.

## Applicability reporting

A high-level framework for applicability reporting was agreed in RAN2#127 [2] and further refined in RAN2#127bis [5] and RAN2#128 [6] based on the following agreements:

RAN2#127bis: - UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality. We should aim to align the design on how the applicable functionality are signalled. FFS on the applicability reporting content. - UE can report to the network when an applicable AI functionality becomes non-applicable. FFS how this is signaled (e.g. explicitly/implicitly). Consider different scenarios, whether it is regarding an active functionality) RAN2#128: - When a functionality configured by the network to be reported via UAI, becomes from non-applicable to applicable, the UE can reports it to the network. - FFS detailed design - FFS whether the UE reports explicitly “non-applicable” functionality when there is a change of applicability.

Regarding the content of the applicability report, in addition to reporting a functionality as “applicable”, it is FFS whether the network may determine which functionalities are “non-applicable” implicitly (e.g., if a supported functionality is not included in the applicable functionality report) or explicitly (e.g., a UE may explicitly report which functionality(ies) are not applicable).

As part of the LS response, RAN1 has included the following agreement related to applicability reporting [3]:

- In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete.

Regarding the yellow highlighed part of the agreement, this implies that the UE would report the applicability status for every CSI-ReportConfig and/or inference related parameters received in Step 3, including those which are non-applicable. This would mean that for at least the initial applicability reporting in Step 4, the UE  explicitely reports either “applicable” or “non-applicable” for each CSI-ReportConfig and/or sets of inference related parameters configured in Step 3.

Proposal 9:	For initial applicability report in Step 4, UE explicitely reports either “applicable” or “non-applicable” for each CSI-ReportConfig and/or sets of inference related parameters configured in Step 3.

RAN1 has also left as FFS whether and what other information is to be included with the applicability report (reference in blue highlight). A functionality may be non-applicable for many reasons outside of network control, such as model unavailability, model not applicable/trained for current UE side additional conditions (e.g., model not trained for the current UE speed), etc. However, if a functionality is not applicable due to missing information from the network (e.g., inference configuration), this may be easily corrected (e.g., via a UE request/response procedure, or via proactive reporting etc.). In addition to reporting a function as “applicable” or “non-applicable”, the UE should indicate the reason for non-applicability (e.g., model not available, not trained for current UE conditions, missing information, UE/NW side conditions etc.).

Proposal 10:	UE indicates the reason an AI/ML functionality is non-applicable.

In case a previously reported functionality changes from “applicable” to “non-applicable”, the UE may simply report the particular AI/ML functionality which is no longer applicable as “non-applicable”. The alternative (i.e., using the same approach as the initial applicability reporting) requires the UE re-send the entire list of applicable functionalities, which may cause significant overhead especially if functionality applicability changes dynamically (e.g., due to changing UE or NW side additional conditions).

To minimize reporting overhead in the case of applicable functionality change, a UE should therefore be able to explicitly report which CSI-ReportConfig/sets of parameters for inference configuration have changed applicability e.g., applicable to non-applicable and vice versa

Proposal 11:	When reporting change of applicability (e.g., applicable to non-applicable and vice versa), UE only indicates which CSI-ReportConfig/sets of parameters for inference configuration which have changed applicability.

# Conclusion

In this contribution the following proposals are made regarding LCM for beam management use case:

Proposal 1:	Confirm CSI-ReportConfig is used for inference configuration.

Proposal 2:	Confirm UE decides applicable functionalities based on one or more CSI-ReportConfig and/or set(s) of inference related parameters (in addition to NW-side additional conditions (if provided), UE-side additional conditions and model availability in device).

Proposal 3:	RAN2 assumes if NW-side additional condition is not provided in Step 3, UE can still determine applicable functionalities from other information within CSI-ReportConfig and/or set(s) of inference-related parameters.

Proposal 4:	Confirm CSI-ReportConfig for inference configuration is initially deactivated upon reception and actived only after RRCReconfigurationComplete (e.g., in the following step) and/or reception of L1/L2 signaling (e.g., in response to applicability report).

Proposal 5:	Confirm L1/L2 signaling can be used for triggering aperiodic CSI report for inference and/or for (de)activation of semi-persistent CSI report for inference.

Proposal 6:	Confirm multiple applicable functionalities (i.e., CSI-ReportConfig for inference configuration) can be active at the same time.

Proposal 7:	Postpone RAN2 discussion on functionality granularity, Associated ID, content of inference configuration, and content and optionality of NW-side conditions until further progress in RAN1.

Proposal 8:	Update LCM steps for AIML BM to align with RAN1 conclusions.

Proposal 9:	For initial applicability report in Step 4, UE explicitely reports either “applicable” or “non-applicable” for each CSI-ReportConfig and/or sets of inference related parameters configured in Step 3.

Proposal 10:	UE indicates the reason an AI/ML functionality is non-applicable.

Proposal 11:	When reporting change of applicability (e.g., applicable to non-applicable and vice versa), UE only indicates which CSI-ReportConfig/set of parameters for inference configuration which have changed applicability.

# References

RP-243244 – WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface – Qualcomm

R2-2409210 - RAN2#127 chair notes

R2-2500011 – Reply LS on applicable functionality reporting for beam management UE-sided model – Samsung

R2-2407848 LS on applicable functionality reporting for beam management UE-sided model – Intel

R2-2409502 - RAN2#127bis chair notes

Draft-RAN2_128_Meeting_Report