3GPP RAN WG2 Meeting #129	R2-2501123

Athens, Greece, February 17th – 21st 2025

Agenda Item:	8.1.2.3

Source:	InterDigital

Title:	LCM for Positioning use case

Document for:	Discussion, Decision

# Introduction

A RAN2 objective for AI/ML for Air Interface Work Item [1] is to design a general framework for one-sided AI/ML, including the signaling and protocol aspects of Life Cycle Management (LCM) to enable functionality selection, activation, deactivation, switching and fallback. In RAN2#127bis [2] the following procedure for reporting applicable functionalities was agreed for Case 1 AIML positioning:

1: The following procedures for LCM for UE sided model for AI positioning case 1 is the baseline: Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message. Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side. Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition). Step 4: UE reports the applicable functionality to the LMF by the LPP provide capabilities message. Step 5: The LMF requests the inferred location information using the LPP request location information message. Step 6: UE reports the inferred location using LPP provide location information message. 2: Whether the inference configuration is provided in step 3 or/and step 5 is FFS (to be revised based on RAN1 progress). 3: Whether network side additional condition is needed and what it contains is FFS (to be revised based on RAN1 progress). 4: FFS whether LMF controls the UE sending unsolicited LPP provide capabilities (i.e. whether step4 is sent reactively or proactively). FFS the signalling details. 5: RAN2 will decide whether AI positioning will be a new method after further details from RAN1 are received.

With further agreements in RAN2#128 [3]:

1: For POS Case 1, RAN2 confirm that the existing unsolicited UE capability report mechanism in LPP can support UE to report the applicable functionality in both “proactive” and “reactive” as a baseline. - Proactive case: When the applicability change, UE can send an unsolicited LPP ProvideCapabilities message to LMF . - Reactive case: If the applicability changes based on the configuration in LPP ProvideAssistanceData message in step 3, UE can send an unsolicited LPP ProvideCapabilities message to LMF. Configuration details are FFS 2 As a baseline, If the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AIML based positioning, and reply with LPP Providelocationinformation message with error cause. FFS if other fallback options are considered

After RAN2#128, a POST meeting email discussion [4] has been triggered to resolve FFSs from the above and to determine (if any) questions to be sent to RAN1 for additional clarification.

This contribution discusses aspects of the above procedure for applicable functionality determination and reporting, specifically remaining details on the applicable functionality reporting procedure not addressed by POST meeting email discussion [4]

# Discussion

## Functionality (de)activation and fallback

RAN2 has the following understanding on terminologies for AIML LCM:

- Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling)

- Applicable functionalities refer to functionalities that the UE is ready to apply for inference

- Activated functionalities refer to functionalities already enabled for performing inference

Based on existing agreements from RAN2#127bis, UE uses the LPP ProvideCapabilities message to indicate supported functionalities to the LMF in Step 2 and applicable functionalities in Step 4. When the UE considers a functionality as “activated” is not currently defined in AIML Positioning use case.

Upon indication of applicable functionalities, the LMF can request the inferred location information using the LPP RequestLocationInformation message in Step 5 and UE reports the inferred location using LPP ProvideLocationInformation message in Step 6. A functionality should therefore be considered as “activated” upon receiving a request for AIML positioning from LMF in Step 5.

Proposal 1: 	An AIML positioning functionality is considered “actived” once UE receives an LPP RequestLocationInformation from the LMF requesting inferred location information.

Notifying the LMF that a previously reported applicable functionality has become non-applicable is important to ensure functionality deactivation and appropriate reconfiguration (e.g., to legacy non-AIMl positioning method) to support the continuation of accurate positioning. In RAN2#127bis [2] and RAN2#128 [3], the following agreements were reached for the BM use case regarding the reporting a change of applicability:

- UE can report to the network when an applicable AI functionality becomes non-applicable. FFS how this is signaled (e.g. explicitly/implicitly). Consider different scenarios, whether it is regarding an active functionality)

- When a functionality configured by the network to be reported via UAI, becomes from non-applicable to applicable, the UE can reports it to the network.

In RAN2#128 it was agreed that the existing unsolicited UE capability report mechanism in LPP can support initial “proactive” and “reactive” applicable functionality reporting as a baseline. The “proactive” mechanism can also be used to notify the LMF upon detection of a change in applicability (i.e., a previously reported functionality is no longer applicable) instead of UAI.

Proposal 2: 	When an AIML positioning functionality becomes non-applicable, UE sends an explicit indication to the LMF via an unsolicited LPP ProvideCapabilities message.

Upon reception of the non-applicability message, in the BM use case the network is expected to deactivate the non-applicable functionality to avoid continued inference (which will most likely yield poor results). The same behaviour should be expected from the LMF upon reception of a non-applicability indication.

Proposal 3: 	LMF is expected to deactivate actived functionality when it receives non-applicability indication from UE (i.e., UE doesn’t autonomously deactivate).

## Fallback

In RAN2#128 the following it was agreed regarding UE behaviour when it cannont fulfill AIML-based positioning due to functionality non-applicability:

- If the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AIML based positioning, and reply with LPP ProvideLocationInformation message with error cause.  FFS if other fallback options are considered.

If for any reason UE cannot perform the AIML based positioning upon LMF location estimation request, it is currently under discussion whether there should be a fallback option apart from the error reporting. Based on [4], views are currently divergent with those supportive of introducing fallback mechanisms primarily noting the latency benefits in e.g., UE autonomous switching to a non-AIML positioning method.

However, if Proposals 2 and 3 above are agreed, the LMF should be notified quickly of any change in applicability via a proactive unsolicited UE capability report. The error scenario then becomes that the UE detects a change in applicability and notifies the LMF, but receives a location estimation request before corrective action can be taken. As the time between these messages should be relatively short, this may be considered a corner case and should not violate any LCS client requirements which require a fallback method as mentioned by proponents in [4].

Furthermore, the LMF can configure multiple positioning methods simultaneously with existing specification. If additional redundancy is needed, the LMF can configure both an AIML-based and non-AIML based positioning method. Upon reception of an LPP ProvideLocationInformation message with error cause for the AIML based positioning request, the LMF can then request a location estimate from the non-AIML positioning method. Therefore, it is suggested that for this release, additional fallback options (e.g., UE autonomous switching) are not considered.

Proposal 4: 	Other fallback options (e.g., UE-autonomous fallback) are not considered within this release.

## SA2 LS on LMF-based AIML Positioning for Case 2b

SA2 is defining LMF-based AIML positioning in R19 which covers Case 2b. An LS was sent in [6] to determine the progress in RAN1/2, as well as to determine any future work plans on data types and procedures defined for Case 2b. However, RAN2 agreements so far are almost exclusively focused on Case 1.

From the WID [1], Case 2b is considered 2nd priority:

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

Considering work in RAN2 is likely reliant on RAN1 progress and and it seems unlikely RAN1 will discuss 2nd priority cases at all considering the limited time remaining in the release, it is suggested that RAN2 respond that no future work is anticipated on Case 2b in this release.

Proposal 5a: 	Respond to SA2 LS indicating that Case 2b is considered 2nd priority in RAN, and RAN2 does not anticipate any future work in this release.

An accompanying LS response to SA2 can be found in a companion contribution in R2-2501124 [6]

Proposal 5b: 	Agree to the reply LS in R2-2501124

# Conclusion

In this contribution the following proposals are made regarding LCM for Positioning use case:

Proposal 1: 	An AIML positioning functionality is considered “actived” once UE receives an LPP RequestLocationInformation from the LMF requesting inferred location information.

Proposal 2: 	When an AIML positioning functionality becomes non-applicable, UE sends an explicit indication to the LMF via an unsolicited LPP ProvideCapabilities message.

Proposal 3: 	LMF is expected to deactivate actived functionality when it receives non-applicability indication from UE (i.e., UE doesn’t autonomously deactivate).

Proposal 4: 	Other fallback options (e.g., UE-autonomous fallback) are not considered within this release.

Proposal 5a: 	Respond to SA2 LS indicating that Case 2b is considered 2nd priority in RAN, and RAN2 does not anticipate any future work in this release.

Proposal 5b: 	Agree to the reply LS in R2-2501124.

# References

RP-243244 – WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface – Qualcomm

R2-2409502 – RAN2#127bis chair notes

Draft-RAN2_128_Meeting_Report

R2-250xxxx – Report of [Post128][026][AIML] LCM Procedure for Positioning Case 1 – Ericsson

S2-2501133 – LS on LMF-based AIML Positioning for Case 2b – vivo

R2-2501124 – [DRAFT] Reply LS on LMF-based AIML Positining for Case 2b – InterDigital