3GPP TSG-RAN2 Meeting #129	R2-2501126

Athens, Greece, Feb. 17th - 21st, 2025

Agenda item:	8.2.3

Source:	Qualcomm Incorporated

Title:	Views on Random Access Aspects of Ambient IoT

WID/SID:	Ambient_IoT_solutions – Release 19

Document for:	Discussion and Decision

# Introduction

The Rel-19 Ambient IoT WI was approved in [1] and the RAN2 objectives were described as follows:

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

In this paper, we will discuss the random-access aspects of Ambient IoT (A-IoT).

# Discussion

## Contention-Based A-IoT Random Access

In Rel-19 A-IoT SI phase, three candidate solutions for contention-based A-IoT random access were studied and captured in TR [2], namely Solution 1 (A-IoT Msg1 without data), Solution 2 (A-IoT Msg1 with data) and 3 (unified solution, A-IoT Msg1 optionally includes data).

The solution 1 is A-IoT Msg1 without data which is a 3-step A-IoT random access, and solution 2 is A-IoT Msg1 with data which is a 2-step A-IoT random access. The solution 3 is a unified solution supporting both solution 1 and 2 that A-IoT Msg1 optionally includes data.

The A-IoT Msg2 is used for contention resolution and the content of A-IoT Msg2 is the same in all of the three solutions, i.e., the reader responds with the successfully received random ID.

According to the analysis in TR [2], 3-step A-IoT random access may support relatively large coverage for A-IoT Msg1 than 2-step A-IoT random access due to the smaller data size of A-IoT Msg1 in the 3-step solution. 2-step A-IoT random access is obviously much more efficient to complete the random access in case of low probability of A-IoT Msg1 collision. The unified solution, i.e., solution 3, can cover both advantages of the solutions 1 and 2. In different environments, it is up to the Reader to decide which type of A-IoT random access to select to adapt to the various service requirements. This won’t add much complexity to the A-IoT device implementation as the decision to choose is on the Reader side. Thus, for contention-based A-IoT random access, the solution 3 is preferred.

Proposal 1: For contention-based A-IoT random access, the solution 3 (i.e., A-IoT Msg1 optionally includes data) is supported.

To support solution 3, Reader may explicitly indicate the type of A-IoT random access in the initial trigger message (a.k.a. A-IoT paging). Alternatively, the ‘indication’ could be based on the grant size (i.e., TB size) from the Reader. If the grant size for Msg1 is large enough to carry the device ID and/or upper layer data, it implies that 2-step A-IoT random access is allowed. Otherwise, only random ID is included in A-IoT Msg1, i.e., 3-step is selected.

Observation 1: The indication for the type of A-IoT random access could be implicit or explicit.

In our view, explicit indication is simpler for the device. Thus we propose, for contention-based A-IoT random access, Reader indicates the type of A-IoT random access (e.g., 2-step or 3-step) to A-IoT devices in the initial trigger message (a.k.a. A-IoT paging).

Proposal 2: Reader indicates the type of A-IoT random access (e.g., 2-step or 3-step) to A-IoT devices in the initial trigger message (a.k.a. A-IoT paging).

For contention-based A-IoT access, the A-IoT device determines to randomly select one access resource/occasion configured from the initial A-IoT paging message. Alternatively, if the energy status of A-IoT device is not good, or the A-IoT device is not ready for responding to the A-IoT paging, the A-IoT device may select the other access/occasion from a subsequent A-IoT paging message with the same service request.

Proposal 3: The A-IoT device randomly select the access resource from either the first A-IoT paging message or a subsequent A-IoT paging message corresponding to the same service request.

## Contention-Free A-IoT Random Access

For contention-free A-IoT access, the scheduled A-IoT device should be configured with dedicated D2R resource. From A-IoT device perspective, if the resource is dedicatedly configured or the resource configured can be one-to-one mapped to the device, the A-IoT device selects the indicated D2R resource and performs contention-free A-IoT access. When A-IoT paging indicates a device specific identifier and the paging message includes an A-IoT access resource (i.e., D2R resource) for that device, contention-free A-IoT access is performed.

Proposal 4: When A-IoT paging includes a device specific identifier and provides the A-IoT access resource for that A-IoT device, contention-free A-IoT access is performed.

In contention-free A-IoT access, the device ID and/or A-IoT upper layer data is sent in the first message via the D2R by using the dedicated D2R resource. The A-IoT device should use the identifier indicated in A-IoT paging as a response to the Reader which sends this paging message. The random ID is neither needed nor sufficient in the contention-free case. If random ID is used in contention-free, it will cause ambiguity between two Readers which may provide the same D2R resource to different A-IoT devices in the same vicinity (see our companion paper in [3] for detailed illustration). The “unification” between contention-free and contention-based A-IoT access is therefore neither beneficial nor necessary.

Proposal 5: In contention-free A-IoT access, the device ID and/or upper layer data is sent in the first D2R message. The random ID in Msg1 is neither needed nor sufficient for contention-free A-IoT access.

## Failure/Success Indication and A-IoT Re-access

In the A-IoT SI phase, it is supported for reader to use the optional explicit R2D failure/success feedback indication to determine the re-access. For the explicit R2D failure indication, it can be used to address the transmission failure of the A-IoT Msg3 which contains the device ID and/or any other A-IoT upper layer data. Since the A-IoT Msg3 is scheduled by A-IoT Msg2 including the random ID, one simple way is that Reader retransmits the same A-IoT Msg2 including the random ID again, and then the A-IoT device with this random ID may consider the previous A-IoT Msg3 was unsuccessfully sent and will simply follow the retransmitted A-IoT Msg2 to retransmit A-IoT Msg3. So, the R2D failure indication of A-IoT Msg3 is the retransmission of A-IoT Msg2 including the random ID.

Proposal 6: The R2D failure indication of A-IoT Msg3 is indicated by the retransmission of A-IoT Msg2 including the previously echoed random ID.

In different scenarios, the A-IoT Msg2 may echo the received random ID or may assign AS ID which is pending on the AS ID discussion. The A-IoT device will retransmit the A-IoT Msg3 if A-IoT Msg2 including the random ID or AS ID is received again.

Proposal 7: The A-IoT device retransmits the A-IoT Msg3 if the A-IoT Msg2 including the random ID or AS ID is received again.

Considering that the explicit R2D failure/success feedback indication is supported, without reception of R2D failure does not imply A-IoT access has successfully completed. For example, there are two A-IoT devices selecting the same access resource and occasionally generating the same random ID, and meanwhile Reader echoes the random ID but Reader has no idea on two A-IoT devices using the same resource. Eventually, only one A-IoT device’s device ID can be successfully received by Reader. But there is no failure indication to inform the other A-IoT device. In that sense, an explicit success indication is necessary to indicate the completion of the A-IoT random access. Without reception of this success indication, it implies that A-IoT device does not complete the A-IoT random access and should consider to re-access in the subsequent A-IoT paging message.

Proposal 8: A R2D success indication is sent to indicate the completion of the A-IoT random access.

In the A-IoT SI phase, it is supported that the reader can send subsequent A-IoT paging messages that are associated with the same service request. The duplicated response from A-IoT devices for the same service request should be avoided. Thus, for the subsequent paging message with the same service request, and if no success indication is received before, the A-IoT device should perform re-access in the subsequent A-IoT paging message.

Proposal 9: If the subsequent A-IoT paging with the same service request is received and no R2D success indication is received before, A-IoT device should perform re-access.

## Summary

Figure 1 gives an example of the A-IoT random access triggered by A-IoT paging.

- When A-IoT device receives initial A-IoT paging message, the A-IoT device randomly selects one access resource/occasion configured by the initial A-IoT paging message and sends A-IoT Msg1. It is also possible that the A-IoT device may select the other access/occasion from a subsequent A-IoT paging message corresponding to the same service request due to low energy status or device not ready to respond to the initial paging message (proposal 3).

- After A-IoT Msg3 transmission, R2D failure indication may carry the retransmitted A-IoT Msg2 which includes the previously echoed random ID to trigger the (same) A-IoT device to re-send the same D2R message, i.e., A-IoT Msg3 (proposals 6 and 7).

- Reader may inform the A-IoT device that inventory is complete by a success indication (proposal 8).

- If A-IoT device does not receive any success indication but receives a subsequent A-IoT paging message corresponding to the same service request, A-IoT device should preform re-access (starting from A-IoT Msg1) by using the newly assigned access resource/occasion (proposal 9).

- The A-IoT devices which have not responded to the initial A-IoT paging can select from the access resource/occasions assigned by subsequent A-IoT paging to perform A-IoT random access.

- Reader may use the subsequent A-IoT paging message to command the successfully-inventoried A-IoT device to perform A-IoT data communication (see our companion paper in [4]).

Figure 1 A-IoT random access triggered by A-IoT paging

# Conclusion

We have the following observations, and we’d recommend RAN2 to discuss and adopt the following proposals:

Observation 1: The indication for the type of A-IoT random access could be implicit or explicit.

Proposal 1: For contention-based A-IoT random access, the solution 3 (i.e., A-IoT Msg1 optionally includes data) is supported.

Proposal 2: Reader indicates the type of A-IoT random access (e.g., 2-step or 3-step) to A-IoT devices in the initial trigger message (a.k.a. A-IoT paging).

Proposal 3: The A-IoT device randomly select the access resource from either the first A-IoT paging message or a subsequent A-IoT paging message corresponding to the same service request.

Proposal 4: When A-IoT paging includes a device specific identifier and provides the A-IoT access resource for that A-IoT device, contention-free A-IoT access is performed.

Proposal 5: In contention-free A-IoT access, the device ID and/or upper layer data is sent in the first D2R message. The random ID in Msg1 is neither needed nor sufficient for contention-free A-IoT access.

Proposal 6: The R2D failure indication of A-IoT Msg3 is indicated by the retransmission of A-IoT Msg2 including the previously echoed random ID.

Proposal 7: The A-IoT device retransmits the A-IoT Msg3 if the A-IoT Msg2 including the random ID or AS ID is received again.

Proposal 8: A R2D success indication is sent to indicate the completion of the A-IoT random access.

Proposal 9: If the subsequent A-IoT paging with the same service request is received and no R2D success indication is received before, A-IoT device should perform re-access.

# References

- RP-243326, Solutions for Ambient IoT (Internet of Things) in NR.

- 38.769 v19.0.0, Study on solutions for ambient IoT (Internet of Things), 3GPP.

- R2-2500954, Ambient IoT Paging, Qualcomm Incorporated, RAN2#129.

- R2-2501127, Data Transmission and Other General Aspects of Ambient IoT, Qualcomm Incorporated, RAN2#129.