3GPP TSG-RAN2 Meeting #129	R2-2501127

Athens, Greece, Feb. 17th - 21st, 2025

Agenda item:	8.2.4

Source:	Qualcomm Incorporated

Title:	Data Transmission and Other General Aspects of Ambient IoT

WID/SID:	Ambient_IoT_solutions – Release 19

Document for:	Discussion and Decision

# Introduction

The Rel-19 Ambient IoT WI was approved in [1] and the RAN2 objectives were described as follows:

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

In this paper, we will discuss the data communication and other aspects of Ambient IoT (A-IoT).

# Discussion

The traffic types of DO-DTT and DT with the focus on indoor inventory and command should be considered. The service requirements on inventory procedure and command procedure should be supported from RAN perspective.

In inventory procedure, the A-IoT application in core network would like to discover the A-IoT devices who are nearby the Reader(s) and sends the inventory request to Reader(s). The inventory request message may include filter criteria, and other upper layer information. The Reader that receives the inventory request performs the inventory procedure. The Reader forwards the inventory request to the A-IoT devices to be inventoried. The inventory request from Reader could be the group signaling which are sent to all A-IoT devices who have harvested enough energy and are able to receive.

In RAN2#125bis meeting, it has been agreed that the A-IoT access procedure is triggered by the Reader and Reader may provide the information that the A-IoT devices need to respond for the A-IoT access trigger. The A-IoT devices who are triggered by the inventory request will perform the A-IoT access towards the Reader in response to the received inventory request.

Figure 1 Ambient IoT inventory

In command procedure, the A-IoT application issues a Command Request including command container, (a list of) A-IoT device identifier(s) that the command is destined or filter criteria that identify the A-IoT devices that are supposed to act upon the command. Therefore, the trigger message sent from Reader may include the device information or filter criteria for triggering A-IoT devices to perform A-IoT access. The trigger message may also include the command container if A-IoT application provides commands to Reader in the command procedure.

## Control plane functionalities

During the SI phase, it was agreed that based on the targeted lower complexity and cost of A-IoT devices, traditional Control Plane functions like RRC connection management, RRM L3 measurement reporting, ASN.1 encoding/decoding, legacy periodical SIB/MIB, legacy paging message, legacy RRC layer, legacy DRX, tracking area update from the device etc. will not be supported by A-IoT.

RAN2#128 agreed:

=> RAN2 assumes that there is no other L2 AS layer (i.e. A-IoT MAC layer only).   There is no CP/UP protocol stack differentiation on A-IoT interfaces.

However, RAN2 still has to conclude on some of the remaining CP aspects.

### A-IoT Device Registration, Subscription and CN Awareness

Due to the target on very low complexity, cost as well as power consumption, and given that potentially a very large number of A-IoT devices may be present in a given vicinity but perform only bursty data transmissions always triggered by the Reader, one of the fundamental architectural differences between A-IoT and existing 3GPP technologies may be how the devices are exposed to the Core Network functions.

In our understanding, SA2 and SA3 would need to discuss and conclude on this. From RAN2 point of view, we think requiring such A-IoT devices to support legacy UE-registration-like procedures would be very ambitious not only because of the complexity of the procedure and protocol, but also due to the number of communication steps required. The A-IoT devices will not have enough energy source or energy storage to perform multiple steps just for the purpose of registration. Additionally, having a ‘subscription’ similar to legacy UE subscriptions per A-IoT device may be a high burden for the core network to manage.

The A-IoT device may harvest energy, accumulate it up to the required minimum amount of energy for communication, perform the communication steps and may fully deplete its energy. If the A-IoT device runs out of energy after some communication steps, it may not be able to store any configurations, i.e. it will likely ‘forget’ everything. In other words, it is plausible that every communication session starts as if the device has just been powered up from the power off state. Then, it would not make sense to have the A-IoT device perform the legacy registration procedures every time it needs to communicate.

Legacy UE-registration-like procedure is not supported by A-IoT.

### NAS functions

NAS control protocol in NR performs the functions listed in TS 23.501 such as registration, authentication, mobility management, security control etc. Supporting all these legacy NAS functionalities by the A-IoT devices may be infeasible especially given their complexity, cost, processing power and energy/power constraints.

In our view, the functionalities like security and authentication for A-IoT communication should be provided by A-IoT function (AIoTF) using a separate protocol. We understand such details are being discussed in SA2.

Legacy NAS functions are not supported by A-IoT devices.

RAN2 assumes new A-IoT upper layer protocol (e.g. A-IoT-NAS) between A-IoT device and AIoTF will be introduced by SA2.

## User plane functionalities

In the A-IoT SI phase, it was agreed that legacy NR SDAP/PDCP/RLC protocol layers are not supported for A-IoT. In the WI scope [1], only MAC layer is included. For A-IoT MAC, not all legacy NR MAC functionalities are needed.

### Logical and transport channels

The NR logical channels are used to carry the control signaling and UP data. The A-IoT data can be carried in A-IoT MAC PDU. In A-IoT MAC, the upper layer signaling and A-IoT upper layer data can be mapped into one single logical channel and all MAC PDUs belonging to this one logical channel are concatenated in the single transport block to be delivered to the physical layer for transmission.

The concept of logical channels as in NR MAC is supported for A-IoT MAC.

Since RAN1 has agreed to support two physical layer channels, i.e., PRDCH and PDRCH, at least two transport channels mapping to physical channels should be supported in A-IoT MAC. Similar to NR DL-SCH and UL-SCH, A-IoT should support transport channels RD-SCH and DR-SCH.

At least two transport channels, i.e., RD-SCH and DR-SCH are supported for A-IoT.

### A-IoT Data Communication and Segmentation

When the contention-based A-IoT random access is successfully completed and contention is resolved, the device ID and/or A-IoT upper layer data is sent to Reader based on the service request in the A-IoT paging message. The A-IoT data communication is triggered by paging message for the specific A-IoT device. The target A-IoT device should transmit A-IoT upper layer data based on the upper layer command in the A-IoT paging message.

The A-IoT data transmission is triggered by A-IoT paging message including the upper layer command and identity of the target A-IoT device(s).

According to the WI scope [1], segmentation is supported at least in D2R. In the SI phase, RAN2 has studied a simplified segmentation solution in the A-IoT MAC. The candidate solution for segmentation was agreed to mainly focus on D2R and can be further considered for R2D. The sequence number, the segment number and the number of segments will not be supported for simplicity. The A-IoT device can send an indication to inform Reader whether the data is segmented or whether the transmission data is the last segment. The details of the indication, e.g., the size of this indication (one or two bits) and the corresponding details can be further discussed.

If the segment indication is one-bit, the first value is used to indicate that the data is segmented, and the second value is shared for the indication of the last segment and the complete packet. If the last segment is missing, the receiver side may aggregate the received segments with a different complete packet which may cause failure. For another example, assume there are two segments. If the first segment is missing, the receiver wouldn’t know there is a segment, and the indication on the last segment would be interpreted as a complete packet which also causes failure. Thus, one-bit indication is not good enough.

For two-bit segment indication, the Segmentation Info (SI) field in NR RLC PDU can be a candidate solution. It can indicate whether the A-IoT upper layer data is a complete packet, or the first, middle, or last segment of a complete packet. See subclause 6.2.3.4 in TS 38.322 for details.

To support segmentation, two-bit segment indication is supported. The Segmentation Info (SI) field in NR RLC PDU can be as a baseline solution.

With the segment indication, Reader schedules additional resource for the subsequent segments’ transmission. Reader is responsible for recording the received segments and estimates the remaining data size of the target A-IoT device. If the rough D2R message size could be provided by CN or AIoTF, Reader considers that as the assistance information to determine the TB size for subsequent segments.

When A-IoT device segments the A-IoT upper layer data, the size of segment can be determined by the TB size from the Reader. One possible way is that the size of segment is always equal to the TB size. If the rest of the upper layer data in A-IoT application buffer is smaller than the scheduled TB size, padding bit(s) may be needed. Thus, the A-IoT device segments and transmits the A-IoT upper layer data based on the scheduled TB size from the Reader addressed to the device ID.

A-IoT device segments and transmits the A-IoT upper layer data based on the scheduled TB size from the Reader addressed to the device ID.

It should be noted that ARQ/HARQ are already excluded during the SI phase with the understanding that A-IoT is not targeted to be a lossless communication. It is indeed some segments may be lost in the bad D2R link condition, and segment aggregation in Reader may fail. If Reader cannot aggregate the segments, it may simply inform the CN or AIoTF the data communication is not successful in the same manner as the failure handling of a complete A-IoT data. The packet error can be handled by CN or application function which is aligned with the general principle of A-IoT design.

Not support ACK/NACK based segment retransmission. It is up to Reader or AIoTF implementation to handle the failed A-IoT segment or data.

### AS ID

RAN2 agreed to assume that AS ID is used at least for the purpose of D2R scheduling and R2D reception, and the AS ID should be a short AS layer ID rather than the full upper layer device ID. In Rel-19 A-IoT SI phase, there are three options captured in TR [2]. Option 1 is to reuse the random ID, Option 2 is to let Reader assign the AS ID, and Option 3 is to let Reader to decide whether to reuse the random ID as the AS ID or assign a new AS ID.

If random ID is sent in A-IoT Msg1 (e.g., in contention-based A-IoT random access) and random ID can be echoed in A-IoT Msg2 as contention resolution, Reader can identify the A-IoT device. The random ID could be used as AS ID if Reader would like to do so. Reader and A-IoT device can use the random ID as AS ID for the subsequent D2R scheduling and R2D reception.

If random ID is not sent in A-IoT Msg1, the A-IoT device anyway need to send the device ID to Reader in the inventory. Reader can identify the A-IoT device after successful reception of device ID. Reader may assign a new AS ID or simply use the partial device ID as the AS ID. Using the partial device ID does not require A-IoT device to store or maintain any additional ID assigned from Reader.

In that sense, Option 3 is preferred. It is up to Reader to decide whether to reuse the random ID (if sent in the first D2R message) as the AS ID or to assign a new AS ID. The AS ID is assigned in a R2D message after Reader has identified the A-IoT device.

It is up to Reader to decide whether to reuse the random ID (if sent in the first D2R message) as the AS ID or to assign a new AS ID.

The AS ID is assigned in a R2D message after Reader has identified the A-IoT device.

# Protocol stack

Based on the above discussion on the needed functionalities, in our view the overall AS protocol stack may look something like below. This would be applicable to both A-IoT CP and UP (i.e. no differentiation between CP and UP protocol stacks as already agreed during SI phase).

Figure 2 A-IoT protocol stack

# Conclusion

We have the following observations, and we’d recommend RAN2 to discuss and adopt the following proposals:

Proposal 1:	Legacy UE-registration-like procedure is not supported by A-IoT.

Proposal 2:	Legacy NAS functions are not supported by A-IoT devices.

Proposal 3:	RAN2 assumes new A-IoT upper layer protocol (e.g. A-IoT-NAS) between A-IoT device and AIoTF will be introduced by SA2.

Proposal 4:	The concept of logical channels as in NR MAC is supported for A-IoT MAC.

Proposal 5:	At least two transport channels, i.e., RD-SCH and DR-SCH are supported for A-IoT.

Proposal 6:	The A-IoT data transmission is triggered by A-IoT paging message including the upper layer command and identity of the target A-IoT device(s).

Proposal 7:	To support segmentation, two-bit segment indication is supported. The Segmentation Info (SI) field in NR RLC PDU can be as a baseline solution.

Proposal 8:	A-IoT device segments and transmits the A-IoT upper layer data based on the scheduled TB size from the Reader addressed to the device ID.

Proposal 9:	Not support ACK/NACK based segment retransmission. It is up to Reader or AIoTF implementation to handle the failed A-IoT segment or data.

Proposal 10:	It is up to Reader to decide whether to reuse the random ID (if sent in the first D2R message) as the AS ID or to assign a new AS ID.

Proposal 11:	The AS ID is assigned in a R2D message after Reader has identified the A-IoT device.

# References

- RP-243326, Solutions for Ambient IoT (Internet of Things) in NR.

- 38.769 v19.0.0, Study on solutions for ambient IoT (Internet of Things), 3GPP.