3GPP TSG-RAN WG2 Meeting #128                                    R2-2501128

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item	: 8.2.3 (Ambient_IoT_solutions)

Source	: LG Electronics Inc.

Title	: Discussion on random access aspects for Ambient IoT

Document for	: Discussion and Decision

Introduction

In this contribution, we show our views on the random access procedure for A-IOT.

Discussion

  - Data transmission in Msg1

During the A-IOT SI phase, RAN2 discussed how to support the contention-based random access in A-IOT, and the following options are captured in the TR 38.769.

- Solution 1: A-IoT Msg1 without data - A-IoT Msg1: When the A-IoT device identifies the start of its own access occasion, it sends one 16-bit random ID generated by the A-IoT device to the reader. - A-IoT Msg2: The reader responds with the successfully received random ID. If the A-IoT device receives the A-IoT Msg2 including a random ID, which is the same as the previously transmitted one in A-IoT Msg1, it considers the contention resolution as successful. - Solution 2: A-IoT Msg1 with data - A-IoT Msg1: When the A-IoT device identifies the start of its own access occasion, it sends the A-IoT Msg1 including the upper layer data, which can be the device ID and/or any other upper layer data, in addition to one 16-bit random ID generated by the A-IoT device to the reader. - A-IoT Msg2: The reader may respond with the successfully received random ID. If the A-IoT device receives the A-IoT Msg2 including a random ID, which is the same as the previously transmitted one in A-IoT Msg1, it considers the contention resolution as successful. If the A-IoT Msg2 is not received by the device, the re-access is not autonomously performed i.e., the re-access is always controlled by the reader. - Solution 3: A-IoT Msg1 optionally includes data (unified solution supporting both Solution 1 and 2) - A-IoT Msg1: When the A-IoT device identifies the start of its own access occasion, it sends one 16-bit random ID generated by the A-IoT device to the reader. In addition, it is controlled by the reader on whether to also include the upper layer data, which can be the device ID and/or any other upper layer data. - A-IoT Msg2: The reader responds with the successfully received random ID. If the A-IoT device receives the A-IoT Msg2 including a random ID, which is the same as the previously transmitted one in A-IoT Msg1, it considers the contention resolution as successful.

As captured above, the difference between solution 1 and solution 2 is whether to transmit the data or A-IOT device ID in Msg1 or not. Since the A-IOT device is a passive device in the DT or DO-DTT scenario, the A-IOT device does not generate the data before the reception of the inventory message or command message. That is, the A-IOT device does not have the data to be transmitted in Msg1.

Then, RAN2 need to discuss whether to transmit the A-IOT device ID in Msg1. In our view, it would be better to transmit the A-IOT device ID in Msg3 because the contention resolution may not be successfully completed due to the collision of Msg1. In this case, if the contention resolution is not successfully completed, the transmission of the A-IOT device ID in Msg1 is useless. In addition, since the transmission delay of the A-IOT device ID is not a big problem, we do not see the motivation to transmit the A-IOT device ID in Msg1. Thus, we think that there is no reason to transmit the data/A-IOT device ID in Msg1.

Proposal 1. In contention-based random access, the A-IOT device transmits only random number in Msg1.

  - Consideration of FDMA

In WID, it is stated that the multiplexing/multiple access in R2D is by only TDMA, and in D2R is by only TDMA and FDMA. It means that since there is only one R2D frequency, the A-IOT device needs to receive the list of frequency information for D2R via the A-IOT paging message as proposed in R2-2500088. Based on this, the following scenario can be considered.

During SI phase, RAN2 agreed that the A-IOT device randomly selects an access occasion to reduce the collision probability. We think that the same principle can be applied to the frequency selection. For instance, if the A-IOT device receives an A-IOT paging message containing the number of access occasion and list of the frequency information for D2R, the A-IOT device randomly selects the access occasion and the frequency information to reduce the collision probability.

Proposal 2. The A-IOT device randomly selects an access occasion and the frequency among the list of the frequency information in A-IOT paging message.

After selection of the access occasion and the frequency, the A-IOT device transmits the Msg1 on the selected frequency. Then, the A-IOT device should receive the Msg2 containing the random number transmitted on Msg1 to complete the contention resolution. However, from the A-IOT reader point of view, the A-IOT reader should transmit the multiple random numbers to complete the contention resolution for multiple A-IOT devices if the FDMA is supported. Thus, RAN2 need to discuss how to indicate the multiple random numbers in Msg2.

In our view, the simple way is to map the random number and frequency index in Msg2. For this, the frequency index should be indicated in the A-IOT paging message. The following shows an example for the Msg2 format.

Example of Msg2 format

Proposal 3. In Msg2, the multiple random number is mapped with the frequency index included in the A-IOT paging message.

  - Re-access procedure

    - Reception failure for success/failure feedback indication

During SI phase, it was agreed that the A-IOT device does not autonomously perform the re-access procedure and the re-access procedure is controlled by the A-IOT reader. In addition, it was agreed that the R2D failure/success feedback indication applies to the Msg3 and the following D2R data after the random-access procedure for re-access purpose.

However, even though the success/failure feedback indication is introduced, RAN2 need to discuss what is a A-IOT device behaviour when the success/failure feedback indication is not received after transmitting the D2R response message. In this case, since the A-IOT device does not know whether the D2R response message is successfully transmitted or not, the A-IOT device is hard to determine whether to perform the re-access procedure or not. The following figure shows the cases when the A-IOT device does not receive the success/failure feedback indication.

For above cases, two options can be considered as follows.

- Option 1. The absent of the success/failure feedback indication is considered as successful transmission of D2R message, and the A-IOT device does not need to perform the re-access procedure.

- Option 2. The absent of the success/failure feedback message is considered as not successful transmission of D2R message, and the A-IOT device needs to perform the re-access procedure.

The option 1 requires that the A-IOT reader should transmit the dedicated A-IOT paging message for an A-IOT device if the A-IOT reader considers that all procedures are not successfully completed for the A-IOT device due to reception failure of the D2R message.

The option 2 may cause the congestion during the re-access procedure by unnecessarily performing the random access procedure even if the A-IOT device already successfully performs all procedures.

In our view, from the service perspective, both options are workable. However, the option 2 has an impact on the performance of the re-access procedure for all A-IOT devices requiring the re-access procedure, we slightly prefer to go option 1.

Proposal 4. The A-IOT device considers that the D2R transmission is successfully transmitted if there is no success/failure feedback indication.

    - How to indicate the re-access procedure

During SI phase, how to support the re-access procedure was discussed. In our understanding, the main purpose of the re-access procedure is to perform the random access procedure for the A-IOT devices that have failed the random access procedure. Thus, as agreed before, an A-IOT device may perform the re-access procedure when the subsequent A-IOT paging message is received if the A-IOT device does not successfully complete the random access procedure in the previous A-IOT paging round. The following figure show an example for the re-access procedure.

Observation 1. The A-IOT paging message is used for initial access and re-access purpose.

In TR 38.769, there is an option to extend the access occasions within one paging round when the A-IOT reader wants to extend the access occasions. We think that there are two motivations for the extension of the access occasion within one paging round. The first motivation is that if collision ratio is high, the A-IOT reader extends the access occasions to relax the collision. The second motivation is that the re-access procedure is performed within the extended access occasions. The following figure shows an example for extension of the access occasions within one paging round.

However, the A-IOT paging message can cover both motivations. For relaxation of the collision, if the A-IOT reader determines that the collision ratio is high, the A-IOT reader can terminate the current paging round and the A-IOT reader indicates new paging round by indicating the large number of the access occasions.

In addition, as agreed before, the A-IOT paging message is used to indicate the re-access procedure for both A-IOT devices that the random access failure happens and the A-IOT devices that the previous A-IOT paging message is missed. However, the indication for extension of access occasions indicates the re-access procedure only for the A-IOT devices that the random access failure happens.

In summary, we think that the A-IOT paging message can cover the both motivations of the indication for extension of access occasions. Thus, the additional indication for the same purposes is not needed.

Proposal 5. The additional indication is not introduced for extension of the access occasions within a paging round.

Conclusion

Based on the above discussion, we made following proposals and observations.

Proposal 1. In contention-based random access, the A-IOT device transmits only random number in Msg1.

Proposal 2. The A-IOT device randomly selects an access occasion and the frequency among the list of the frequency information in A-IOT paging message.

Proposal 3. In Msg2, the multiple random number is mapped with the frequency index included in the A-IOT paging message.

Proposal 4. The A-IOT device considers that the D2R transmission is successfully transmitted if there is no success/failure feedback indication.

Observation 1. The A-IOT paging message is used for initial access and re-access purpose.

Proposal 5. The additional indication is not introduced for extension of the access occasions within a paging round.