3GPP TSG-RAN2 Meeting #129	R2-2501129

Athens, Greece, Feb. 17th - 21st, 2025

Agenda item:	8.11.2

Source:	Qualcomm Incorporated

Title:	Views on random access for SBFD

WID/SID:	NR_duplex_evo-Core – Release 19

Document for:	Discussion and Decision

# Introduction

RAN plenary #102 approved the work item on NR duplex operation, Sub-Band Full Duplex (SBFD) and the latest WID is in [1]. The objective of the work item is to specify SBFD operation at the gNB side within a TDD carrier. One of the objectives is to specify SBFD operation to support random access in SBFD symbols for UE in RRC-connected UE while studying the benefit of extending the random-access operation in SBFD symbols to UE(s) in RRC idle and inactive state.

In RAN2 #127, the following agreements were made regarding random access for SBFD operation.

Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events. RAN2 assume RACH configuration for SBFD via SIB and/or dedicated RRC signalling is supported. Detailed signalling FFS. RAN2 to strive for a common SBFD CBRA framework independent of RRC state. FFS whether/how early indication is used during a SBFD RA procedure. RAN2 focus on 4-step RACH for SBFD RA, FFS on 2-step if needed. Cell-specific SBFD time/frequency configuration is provided by SIB1 (or via dedicated signalling to covey cell specific configuration). FFS on UE specific dedicated RRC configuration if needed, pending on RAN1 progress.

In RAN2 #127bis, the following agreements were made regarding random access for SBFD operation.

RAN2 understand that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact. From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework. Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network. For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first. The following two RACH configuration options are considered for SBFD based random access: - Option 1: Use one single RACH configuration based on the existing parameters of the single RACH configuration. Can extend the existing parameters if needed. - Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration For RACH configuration Option 2, RAN2 needs to specify RRC signalling for the new SBFD based RACH configuration with a new set of parameters. The RACH configuration for SBFD is transmitted via SIB1. FFS dedicated RRC signalling detail. FFS whether NW can provide both configurations.

RAN2 #128 made several agreements related to random access for SBFD operation.

CFRA The RO type is indicated by NW for CFRA. FFS on signaling (can FFS for the SI request case if needed). CBRA FFS on the following options Option 1 - Upon initiation of RACH procedure for a SBFD-aware UE, network provides the indication on the prioritization of the additional ROs over legacy RO. If there is no such indication from the NW, FFS on the following mechanism - UE select legacy RO or SBFD RO based on SSB RSRP, or - UE select the legacy RO, or - UE select the SBFD RO, or - Other metrics than SSB RSRP. Option 2 UE select legacy RO or SBFD RO based on SSB RSRP if such condition is configured, and if not configured, then UE can prioritize one type of the ROs, FFS which one. RACH configuration Only one RACH configuration option (i.e., either RACH configuration Option 1 with Alt 1-1 or RACH configuration Option 2) is supported in a cell.

In this contribution, we would like to discuss the configurations of random access resource in SBFD symbols for UE in RRC_CONNECTED and RRC_IDLE/INACTIVE.

# Discussion

There were already several basic agreements regarding supporting SBFD random access operation in RAN1.

Agreement Confirm the working assumption: Working assumption: For SBFD aware UEs in RRC CONNECTED state, support CBRA and CFRA in SBFD symbols. Agreement For SBFD aware UEs in RRC CONNECTED state, support Type-1 random access procedure (4-step RACH) in SBFD symbols. - FFS Type-2 random access procedure (2-step RACH)

The working assumption to support both CBRA and CFRA in SBFD symbols were confirmed in RAN1. RAN1 also agreed to support Type-1 random access (4-step RACH) in SBFD symbols and will further discuss Type-2 random access (2-step RACH) after finalizing the design of 4-step RACH. RAN2 #127 further agreed to focus on the 4-step RACH for SBFD RACH, FFS on 2-step if needed.

Observation 1: For the RRC_CONNECTED UE, RAN1 has agreed to support both CBRA and CFRA in SBFD symbols.

Observation 2: Both RAN2 and RAN1 has agreed to at least support 4-step RACH and FFS on 2-step RACH.

Several benefits can be observed if RACH procedure can use SBFD resource, such as shorter access latency, PRACH collision reduction and PRACH coverage improvement. In our view, all the existing RACH triggering events can be supported by random access in SBFD symbols. One may think that SI request may be the exception. However, we don’t see a reason to exclude it, since it has the same procedure if network configures the SBFD resource for SI request.

Proposal 1: RAN2 confirms the working assumption that random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

Regarding the RACH configuration for SBFD operation, there are two options. One option is single RACH configuration (i.e., RACH configuration Option 1 with Alt 1-1 defined in RAN1) by reusing the legacy RACH configuration with some modification, and another option is to have two separate RACH configurations (i.e., RACH configuration Option 2) including one legacy RACH configuration and one additional RACH configuration to configure RACH occasions (ROs) within UL subband in SBFD symbols.

The single RACH configuration would enable random access for SBFD operation for both SBFD-aware UE and legacy UE without extra signaling overhead. The separate RACH configurations for SBFD operation may enable more flexible configuration on the PRACH format, PRACH time/frequency resource, and so on.

RAN2 #128 has agreed that only one RACH configuration option is supported in a cell. RAN1 #118 agreed that UE is not required to support both options.

For the SBFD-aware UEs in RRC_CONNECTED state, UE indicates whether supporting RACH configuration Option 1 or Option 2 via UE capabilities. It is up to network to configure which RACH configuration Option in the serving cell. If network configures RACH configuration Option 2 but UE only supports RACH configuration Option 1, the network configuration does not match UE capability, and UE has to use the legacy RACH resource to perform RACH. Only if UE also supports RACH configuration Option 2 which matches network configuration, UE can use SBFD RACH resource.

Proposal 2: If SBFD-aware UE supports the RACH configuration for SBFD according to UE capability and network configures the RACH resource in SBFD symbols matching the UE capability, UE can use SBFD RACH resource.

The discussions on two options of SBFD RACH configuration in RAN1 are still ongoing. RAN2 can wait for RAN1 further inputs on the detailed SFD RACH parameters.

Proposal 3: RAN2 waits for RAN1 further inputs on the detailed RACH configuration parameters for SBFD.

In RAN2 #127bis, it has been agreed that ‘Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network.’

RAN2 #128 agreed to further study two options on the type of RO selection in the RACH initiation phase. The main differences of two options are whether allow network providing an indication on the prioritization between the additional ROs and legacy RO. Based on the discussions in RAN2 #128, in option 1, if network can provide the indication, SBFD-aware UE should follow the network indication, and while in option 2, network is not able to provide such indication.

In our understanding, network may have preference on which type of RO for SBFD-aware UE to perform random access. The network preference could be based on the collected data on the usage of RACH resource such as the collision probability or access loading. Network may prioritize SBFD-aware UE to utilize the additional-ROs instead of legacy-ROs to achieve better load balancing between legacy-ROs and additional-ROs and to further improve the PRACH capacity and reduce the collision probability. Some time network may prefer the SBFD-aware UE to use legacy-ROs if the legacy-RO is less congested, and the large inter-UE CLI is detected in the cell edge. Such preference indication is optional for network to configure. It is up to network to configure which type of RO that SBFD-aware UE is allowed to initiate RACH, and UE should follow the network indication.

Proposal 4: Upon initiation of RACH procedure for a SBFD-aware UE, if network provides the indication on the prioritization between the legacy ROs and additional ROs, UE selects the type of ROs based on network indication.

Proposal 5: The indication on the prioritization of the legacy ROs and additional ROs is optional for network to configure via RRC signaling.

For RACH configuration Option 1, the main purpose or motivation is to introduce additional-ROs to improve RACH capacity and reduce PRACH collision probability in legacy-ROs or reduce the random-access latency.

If SBFD-aware UE is capable to support PRACH transmission in the additional-ROs, it would be beneficial if UE selects the additional-ROs in the SBFD symbols with the high priority. By using additional-ROs to transmit PRACH is also beneficial for early indication of the UE SBFD-awareness. Thus, for RACH configuration Option 1, it is good that UE has priority to select additional-ROs in the SBFD symbols.

For RACH configuration Option 2, the additional PRACH resource like additional SBFD-ROs or different type of preamble supporting SBFD operation are separately configured. The main motivation for RACH configuration Option 2 is to improve the PRACH cell coverage, e.g. by introducing a long preamble sequence. Then, the design criteria for the selection of using the additional PRACH configuration should enable to extend the cell coverage for cell edges UEs. This can be achieved by comparing the RSRP of the SSB to a threshold (e.g. the criteria can be RSRP < threshold to select the additional RO). However, this will prohibit cell-centre SBFD-aware UE from early indication of the SBFD-aware based on msg1 preamble transmission in the additional-ROs. Also, it may create inter-UE CLI, due to higher transmission power by the cell edge UEs.

Observation 3: For PRACH configuration Option 2, prioritization of the additional-RO for cell-edge SBFD-aware UE, UE will improve the PRACH cell-coverage. However, it may create inter-UE CLI and will restrict cell centre SBFD-aware UEs from early indication based on msg1 PRACH transmission in additional-RO.

Observation 4: It is not suitable simply using the DL RSRP threshold as criteria for the RO type selection.

Thus, a unified solution should be design for both RACH configuration Optional 1 and Option 2, i.e., RO selection be based on the prioritization of the additional-ROs over legacy-ROs, if no indication is received from network.

Proposal 6: Upon initiation of RACH procedure for a SBFD-aware UE, if no indication is received from network, the initial attempt in the RACH procedure is determined based on the prioritization of the additional-ROs over legacy-ROs.

Regarding PRACH transmission repetition, RAN1 #118 made the following agreement regarding the PRACH transmission with preamble repetitions.

Agreement For SBFD aware UEs, at least support the following: - PRACH transmission with preamble repetitions within additional-ROs (not across additional-Ros and legacy ROs) Note: SBFD aware UEs support PRACH transmission with preamble repetitions only within legacy-ROs (not across additional-ROs and legacy ROs) as in legacy releases.

RAN1 #118bis has conclusion.

Conclusion PRACH transmission with preamble repetitions across additional-ROs and legacy-ROs is not supported.

RAN1 #119 made follow agreements.

Agreement Confirm the following working assumption Working Assumption For a PRACH transmission with Npreamblerep preamble repetition within additional-ROs, support reusing current specification rules for the determination of the set of Npreamblerep valid additional-ROs and the time period of the set(s) of Npreamblerep valid additional-ROs. - the time period of the set(s) of Npreamblerep valid additional-ROs is determined separately from the time period for legacy-ROs

Agreement For RACH configuration Option 2, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 and msg1-RepetitionNum for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs. - msg1-RepetitionNum is configured in RACH-ConfigCommon in current specification, i.e., for RACH configuration Option 2, msg1-RepetitionNum can already be separately configured for legacy-ROs and additional-ROs. Agreement For RACH configuration Option 1, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs. - Same msg1-RepetitionNum is used for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.

According to RAN1 agreements, it is supported that PRACH transmission with preamble repetitions configured within the additional-ROs. However, it is not supported that PRACH transmission with preamble repetitions configured across additional-ROs and legacy-ROs.

For the RO type selection in the RACH initiation phase, if preamble repetition is also configured within additional-ROs and/or legacy-ROs. One alternative is that the ROs configured with the preamble repetition is prioritized. If both ROs are configured with preamble repetition, the additional-ROs are prioritized. The other alternative is no matter ROs configured with preamble repetition, it does not impact the RO type selection.

In our view, further consideration on the configuration of PRACH transmission with preamble repetitions in the RO type selection may make the RO type selection procedure complex. For RACH configuration Option 1, it is possible that network configures the preamble repetitions in both types of ROs (i.e., additional-ROs and legacy ROs. However, for RACH configuration Option 2, since RACH configurations are separate, it is unclear whether it is possible preamble repetitions can be configured only one type of ROs or both types of ROs. If only legacy-ROs are configured with preamble repetitions, how to select RO should be further studied. Thus, we prefer to have a simple solution, i.e., whether ROs configured with preamble repetition or not does not impact the RO type selection.

Proposal 7: Whether ROs configured with the preamble repetition or not does not impact the RO type selection.

Another issue is about the RACH resource selection in MAC specification, i.e., when to perform RO type selection between additional-ROs and legacy-ROs. One factor is that should the RO type selection be before the selection of 2-step vs. 4-step RACH or after. Noted that it is still pending on RAN1 discussion on whether 2-step RACH supports the SBFD operation. If RO type selection is before the selection of 2-step vs. 4-step RACH and if 2-step RACH does not support SBFD operation, UE cannot fallback to legacy-ROs of 2-step RACH. Thus, considering minimum MAC specification impact, RACH type selection between additional-ROs and legacy-ROs should be after 2-step vs. 4-step RACH selection.

Proposal 8: RACH type selection between additional-ROs and legacy-ROs should be after 2-step vs. 4-step RACH selection.

In case of PRACH attempt failures, UE may perform PRACH re-attempt. In RAN2 #127bis, it has been agreed that ‘after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.’

If network indicates the SBFD-aware UE select the legacy RACH occasions first, and after UE keeps attempting on the legacy RO with a number of times and if it is still failed, it means the legacy RACH occasions resource may be congested and overload. UE should be allowed to switch to the SBFD RACH occasions for RACH re-attempt.

Proposal 9: For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in legacy RACH occasions, SBFD-aware UE is allowed to switch to SBFD RACH occasions for RACH re-attempt.

It is important for gNB to know the UE SBFD-aware capability in early stage of the random-access procedure to leverage the SBFD operation. The SBFD-aware indication is useful to inform the UE SBFD capability at the earliest time during RACH, e.g., network may schedule the resource for Msg3 in the SBFD resource in Msg2 if network can be aware the SBFD capable UE performing the RACH.

If network has configured the RACH occasions on SBFD symbols and UE also supports it, the SBFD-aware UE sends the Msg1 in the SBFD RACH occasions as the SBFD-aware indication to network. So, by using the SBFD RACH occasions can be already an implicit way as the SBFD-aware indication.

If the SBFD-aware UE selects the legacy RACH occasion, the additional SBFD-aware indication is needed. If Msg1 based SBFD-aware indication is not supported, Msg3 based SBFD-aware indication should be supported from RAN2 perspective.

Proposal 10: If Msg1 based SBFD-aware indication is not supported, Msg3 based SBFD-aware indication should be supported from RAN2 perspective.

In RACH configuration Option 2, when there are legacy-RO in legacy PRACH configuration and additional-RO in the SBFD PRACH configuration that are time overlapping and have same start symbol index in the uplink slot. In such case, if legacy UE selects the legacy RO and SBFD-aware UE selects the additional-RO, the computation from the existing RA-RNTI formulation will be the same. There is ambiguity on the reception of RA response. To resolve this issue, RAN2 to study either update the RA-RNTI formulation or rely on gNB implementation to avoid overlapping of legacy-RO and additional-ROs in the configuration

Proposal 11: RAN2 to study the RA-RNTI collision issue for RACH configuration Option 2.

# Conclusion

We have the following observations, and we’d recommend RAN2 to discuss and adopt the following proposals:

Observation 1: For the RRC_CONNECTED UE, RAN1 has agreed to support both CBRA and CFRA in SBFD symbols.

Observation 2: Both RAN2 and RAN1 has agreed to at least support 4-step RACH and FFS on 2-step RACH.

Observation 3: For PRACH configuration Option 2, prioritization of the additional-RO for cell-edge SBFD-aware UE, UE will improve the PRACH cell-coverage. However, it may create inter-UE CLI and will restrict cell centre SBFD-aware UEs from early indication based on msg1 PRACH transmission in additional-RO.

Observation 4: It is not suitable simply using the DL RSRP threshold as criteria for the RO type selection.

Proposal 1: RAN2 confirms the working assumption that random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

Proposal 2: If SBFD-aware UE supports the RACH configuration for SBFD according to UE capability and network configures the RACH resource in SBFD symbols matching the UE capability, UE can use SBFD RACH resource.

Proposal 3: RAN2 waits for RAN1 further inputs on the detailed RACH configuration parameters for SBFD.

Proposal 4: Upon initiation of RACH procedure for a SBFD-aware UE, if network provides the indication on the prioritization between the legacy ROs and additional ROs, UE selects the type of ROs based on network indication.

Proposal 5: The indication on the prioritization of the legacy ROs and additional ROs is optional for network to configure via RRC signaling.

Proposal 6: Upon initiation of RACH procedure for a SBFD-aware UE, if no indication is received from network, the initial attempt in the RACH procedure is determined based on the prioritization of the additional-ROs over legacy-ROs.

Proposal 7: Whether ROs configured with the preamble repetition or not does not impact the RO type selection.

Proposal 8: RACH type selection between additional-ROs and legacy-ROs should be after 2-step vs. 4-step RACH selection.

Proposal 9: For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in legacy RACH occasions, SBFD-aware UE is allowed to switch to SBFD RACH occasions for RACH re-attempt.

Proposal 10: If Msg1 based SBFD-aware indication is not supported, Msg3 based SBFD-aware indication should be supported from RAN2 perspective.

Proposal 11: RAN2 to study the RA-RNTI collision issue for RACH configuration Option 2.

# References

- RP‑241614, Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei