3GPP TSG-RAN WG2 Meeting #129	R2-2501131

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.4.3

Source:	InterDigital, Ericsson, Nokia, Sony, Vodafone

Title:	Discussion on neighboring cell measurement with LR

Document for:	Discussion

# 1. Introduction

In RAN2#127 meeting, the following agreement related MR (Main Radio) and LR (LP-WUS wake-up Receiver were made:

Serving cell measurment offloading (i.e., there is no serving cell measurement by MR) For serving cell measurement offloading (i.e., there is no serving cell measurement by MR): - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. - The exit condition is based on the LR measurement results.

In RAN2#127bis meeting, the following working assumption was made:

RRM relaxation for serving and neighbor cell For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

# 2. Discussion

2.1 Background

2.1.1 MR offloading and relaxation with LR

RAN2 agreed in RAN2#127 that when an entry condition (e.g., fully offloading) of serving cell measurement is satisfied, the UE performs the serving cell measurement with LR (i.e., no serving cell measurement with MR). Also, RAN2 made a working assumption in RAN2#127bis that if MR serving cell measurement is not available, neighboring cell measurement relaxation can be determined based on the LR measurement results.

2.1.2 Intra-frequency operation with MR and LR

According to the way forward in RANP#106 [1], the Rel-19 idle/inactive UE considers intra-frequency case that MR and LR operate in the same frequency/band. Based on the results, RAN2 needs to focus only intra-frqeunecy case in Rel-19.

2.2 The necessity of neighboring cell measurement with LR

2.2.1. MR RRM relaxation with cell coverage

Figure 1. Urban environmentsnradio condition [2]

According to [2], in urban environments the radio conditions may also change rapidly and dramatically, e.g. when turning around a corner. This implies that LP-WUS usage may be limited to a small part of the cell. In case of multi-beam operation the cell coverage may also be beam dependent, i.e. beams of the same cell provide different quality. This may be another reason why intra-frequency cell reselection is triggered. As a result, the UE is required to perform intra-frequency measurements in most of the cell and these intra-frequency measurements cannot be relaxed.

For this reason it is beneficial to discuss neighbor cell measurements performed by OFDM-based LR, i.e. the LR also performs intra-frequency measurements and ranking and the MR is only woken up when cell reselection is initiated.

Observation 1: In urban enviroments, MR RRM relaxation of neighboring cell measurements cannot be operated (e.g., triggered reselection) due to urban radio conditions.

2.2.2. MR RRM relexation with idle/inactive configuration

#### 5.2.4.9 Relaxed measurement ##### 5.2.4.9.0 Relaxed measurement rules When the UE is required to perform measurements of intra-frequency cells or NR inter-frequency cells or inter-RAT frequency cells according to the measurement rules in clause 5.2.4.2: - if lowMobilityEvaluation is configured and cellEdgeEvaluation is not configured; and - if the UE has performed normal intra-frequency, NR inter-frequency, or inter-RAT frequency measurements for at least TSearchDeltaP after (re-)selecting a new cell; and - if the relaxed measurement criterion in clause 5.2.4.9.1 is fulfilled for a period of TSearchDeltaP: - the UE may choose to perform relaxed measurements for intra-frequency cells, NR inter-frequency cells or inter-RAT frequency cells according to relaxation methods in clauses 4.2.2.9, 4.2.2.10, 4.2.2.11, 4.2C.2.7 and 4.2C.2.8 in TS 38.133 [8]; - if cellEdgeEvaluation is configured and lowMobilityEvaluation is not configured; and - if the relaxed measurement criterion in clause 5.2.4.9.2 is fulfilled: - the UE may choose to perform relaxed measurements for intra-frequency cells according to relaxation methods in clauses 4.2.2.9 and 4.2C.2.7 in TS 38.133 [8]; … NOTE 1: It is up to UE implementation when to start performing relaxed measurements in RRC Idle/Inactive if multiple methods are configured. NOTE 2: It is up to UE implementation which relaxation method to perform based on the "allowed" cases as specified in TS 38.133 [8] for RRC Idle/Inactive if multiple methods are configured. The above relaxed measurements and no measurement are not applicable for frequencies that are included in VarMeasIdleConfig, if configured and for which the UE supports dual connectivity or carrier aggregation between those frequencies and the frequency of the current serving cell.

According to current IDLE specification [7], MR RRM relaxation of neighboring cell measurement is not allowed when frequencies are configured with idle/inactive configuration (as highlighted in yellow). Given the situation, when the serving cell measurement is fully offloaded by LR, the UE needs to activate MR to perform neighboring cell measurement under idle/inactive configuration. As a result, MR cannot be relaxed and power consumption of the UE would be increased [3]. Regarding the same issue, some companies [4][5] proposed that a necessity of discussion on neighboring cell measurement relxation with MR when a UE is configured with idle/inactive measurement. Also, another company mentioned [6] to study cell reselection with LR.

Observation 2: According to IDLE spec, MR RRM relaxation of neighboring cell measurements cannot be allowed with idle/inactive configuration.

Based on the issues above, RAN2 can discuss the need and feasibility to support neighboring cell measurement using OFDM-based LR. We can send an LS to ask RAN4 to investigate the feasibility of the neighboring cell measurement using OFDM-based LR.

Proposal 1: RAN2 to discuss the need and feasibility to support neighbour cell measurements using OFDM-based LR.

Proposal 2: Ask to RAN4 whether it is feasible to support neighbour cell measurements using OFDM-based LR.

# 3. Conclusion

Observation 1: In urban enviroments, MR RRM relaxation of neighboring cell measurements cannot be operated (e.g., triggered reselection) due to urban radio conditions.

Observation 2: According to IDLE spec, MR RRM relaxation of neighboring cell measurements cannot be allowed with idle/inactive configuration.

Proposal 1: RAN2 to discuss the need and feasibility to support neighbour cell measurements using OFDM-based LR.

Proposal 2: Ask to RAN4 whether it is feasible to support neighbour cell measurements using OFDM-based LR.

# 4. References

[1] RP-243266, Way forward on Rel-19 LP-WUS separate band issues, RANP#106

[2] R2-2410086, LP-WUS and RRM measurements, Ericsson, RAN2#128.

[3] R2-2410510, Discussion on RRM measurement relaxation and offloading, InterDigital, RAN2#128.

[4] R2-2410556, RRM measurement relaxation in RRC_IDLE/INACTIVE, Nokia, RAN2#128.

[5] R2-2410633, Discussion on RRM measurement relaxation and offloading, Sharp, RAN2#128.

[6] R2-2410378, Discussion on RRM aspects for LP-WUS/WUR, Sony, RAN2#128.

[7] 3GPP 38.304, NR User Equipment (UE) procedures in idle mode and RRC inactive state.