3GPP TSG RAN2 Meeting #129                                                                R2-2501134

17th Feb. – 21st Feb. 2025, Athens, Greece

Agenda item:	8.4.4

Source:	Samsung

Title:	Procedures for LP-WUS in RRC Connected Mode

Document for:	Discussion & Decision

Introduction

Energy conservation has emerged as an important issue in recent mobile communication network environments. Accordingly, RAN2 starts with the new Work Item (WI), Low-power wake-up signal and receiver for NR (LP-WUS/WUR) [1][2].

During the previous meeting #127bis and #128, with respect to Procedures for LP-WUS in RRC Connected Mode agenda, RAN2 agreed as follows:

#127bis Option 1-2 related For Option 1-2, LP-WUS monitoring is performed at least outside legacy C-DRX Active Time. FFS if the legacy drx-onDurationTimer is started or not if the new timer is configured in Option 1-2. In option 1-2, a new timer triggered by LPWUS is introduced. When this new timer is running, UE is in C-DRX active time. When UE is not in C-DRX active time, UE goes back to LPWUS monitoring. When UE is in C-DRX active time, UE PDCCH monitoring behaviors related to other legacy DRX timers (except for drx-onDurationTimer) are not affected. #128 Option 1-2 (Handling of drx-onDurationTimer, periodic CSI/L1-RSRP reporting, etc.) The drx-onDurationTimer is not started with Option 1-2 LP-WUS. For Option 1-2, network can configure whether UE reports periodic CSI/L1-RSRP during the time given by the configured drx-onDurationTimer, for the case when UE is outside C-DRX active time. For option 1-2, if UE receives DRX command MAC CE or Long DRX command MAC CE, UE stops the new timer triggered by LP-WUS. Option 1-1 and 1-2 Don’t support Option 1-1 and Option 1-2 simultaneously configured for the same UE.

And, RAN1 made the below related summary at the meeting #118bis and #119 w.r.t. various LP-WUS procedure options[3][4]:

#118bis Agreement For Option 1-1 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective. - LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer - UE is configured with legacy C-DRX configurations as Rel-18 - UE behaviors related to CDRX active time triggered by all legacy DRX timers are not affected unless included in this proposal - No impact on RRM/RLM/BFD measurement requirements is assumed - For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP) - Periodic CSI/L1-RSRP is not reported during the time given by the configured drx-onDurationTimer if UE is not indicated to wake-up - Periodic CSI/L1-RSRP is periodically reported during the time given by the configured drx-onDurationTimer regardless if UE is indicated to wake-up or not - Note: Periodic CSI/L1-RSRP is reported during CDRX active time as in legacy specification Agreement Confirm following working assumption Working Assumption From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE Note: Above can be revisited considering RAN2 decisions. Agreement - For LP-WUS CONNECTED mode operation, Option 1-2 can be configured separately from Option 1-1 - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE Agreement - Regarding the potential restriction for LP-WUS configuration in relation with C-DRX configuration for option 1-2 of LP-WUS CONNECTED mode operation, from RAN1 perspective, - AltOpt2: At least LP-WUS periodicity is no larger than long C-DRX cycle, FFS other restrictions - Introducing monitoring window is not precluded #119 Agreement - For LP-WUS MOs in connected mode for Option 1-1, following is considered further discussion and possible down-selection - Approach 1: - LP-WUS MOs, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s). - UE selectively monitors LP-WUS in the LP-WUS MOs identified by other parameters such as time offset prior to the start of drx-onDurationTimer. - Approach 2: - LP-WUS MOs, including periodicity and offset, are identified based on the slot that drx-onDurationTimer would start and a new RRC parameter indicating the time offset until the slot that drx-onDurationTimer would start. - FFS: An RRC parameter indicates the duration where the UE monitors LP-WUS [consecutively in time] - FFS: the consecutive LP-WUS MOs in time identified by the RRC parameter are associated with a same C-DRX cycle; - FFS: LP-WUS MOs in time identified by the RRC parameter provides consistent indication for the same C-DRX cycle of the UE, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS MOs associated with the same C-DRX cycle. - For LP-WUS MOs in connected mode for Option 1-2, following is considered further - Approach 1: - LP-WUS MOs, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s). - UE monitors LP-WUS in the LP-WUS MOs and starts a new timer for PDCCH monitoring triggered by LP-WUS, after a time offset. - Approach 2: - Periodicity/offset of the slot that the new timer for PDCCH monitoring may be potentially triggered by LP-WUS are configured by RRC parameters. - LP-WUS MOs, including periodicity and offset, are identified based on the slot that new timer would start and a new RRC parameter indicating the time offset until the slot that new timer would start. - FFS: An RRC parameter indicates the duration where the UE monitors LP-WUS [consecutively in time] - FFS: the consecutive LP-WUS MOs in time identified by the RRC parameter are associated with the same slot that the new timer would start; - FFS: LP-WUS MOs in time identified by the RRC parameter provides consistent indication for the same slot that the new timer would start, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS MOs associated with the same slot that the new timer would start. Agreement For Option 1-2 of LP-WUS CONNECTED mode operation, for explicit indication to stop the new timer for PDCCH monitoring in Option 1-2 and drx-InactivityTimer for the DRX group, RAN1 identified the following potential solution - Use existing DRX Command MAC CE and Long DRX Command MAC CE - When UE is not in C-DRX active time, UE monitors LP-WUS Send LS to RAN2 to ask the feasibility of the above potential solution. LS in R1-2410909. Conclusion For Option 1-1 of LP-WUS CONNECTED mode operation, RAN1 assumes existing DRX Command MAC CE and Long DRX Command MAC CE can be used to stop drx-onDurationTimer and/or drx-InactivityTimer for the DRX group - When UE is not in C-DRX active time, UE monitors LP-WUS Send LS to RAN2 to ask whether RAN2 has same understanding or not. LS in R1-2410909. Agreement For Option 1-2, for LP-WUS CONNECTED mode operation, the periodic CSI/L1-RSRP reporting operation is same as Rel-16 DCP, that is the UE can be configured with a parameter to enable/disable periodic CSI/L1-RSRP reporting, respectively: - If the parameter is NOT configured: - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is not reported. - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time. - If the parameter is configured: - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the time given by the configured drx-onDurationTimer in DRX-Config for the case when UE is outside C-DRX active time. - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time. - FFS: Whether RAN1 spec impact is required.

In this contribution, we would like to discuss the detailed procedures and configurations to support LP-WUS in RRC IDLE/INACTIVE mode.

Discussion

- LS from RAN1: LS on LP-WUS operation in CONNECTED mode

In this meeting, RAN2 received an LS from RAN1[5] as follows:

Conclusion - For Option 1-1 of LP-WUS CONNECTED mode operation, RAN1 assumes existing DRX Command MAC CE and Long DRX Command MAC CE can be used to stop drx-onDurationTimer and drx-InactivityTimer for the DRX group - When UE is not in C-DRX active time, UE monitors LP-WUS - Send LS to RAN2 to ask whether RAN2 has same understanding or not. LS in R1-2410909. Agreement - For Option 1-2 of LP-WUS CONNECTED mode operation, for explicit indication to stop the new timer for PDCCH monitoring in Option 1-2 and drx-InactivityTimer for the DRX group, RAN1 identified the following potential solution - Use existing DRX Command MAC CE and Long DRX Command MAC CE - When UE is not in C-DRX active time, UE monitors LP-WUS - Send LS to RAN2 to ask the feasibility of the above potential solution. LS in R1-2410909.. Regarding the above conclusion on Option 1-1, RAN1 would like to ask RAN2 whether RAN2 has same understanding or not. Regarding the above agreement on Option 1-2, RAN1 would like to ask RAN2 the feasibility of the above potential solution.

For Option 1-1, we concur that the DRX timer mechanism remains consistent with the existing NR framework, aligning well with our comprehension.

Nevertheless, regarding the UE monitoring of LP-WUS during periods when the UE is not within C-DRX active times, we propose additional considerations:

- It might not necessarily imply continuous monitoring of LP-WUS beyond these designated intervals.

- Instead, upon receiving relevant configurations from the network, UEs shall monitor LP-WUS occasions accordingly.

Proposal 1. For Option 1-1 of LS from RAN1, RAN2 understands as follows:

- The timers, drx-onDurationTimer and drx-InactivityTimer, of the DRX group will be stopped once a DRX Command MAC CE or a Long DRX Command MAC CE is received (adheres to the current specifications)

- Outside of CDRX active times, UEs perform scheduled monitoring of LP-WUS according to predefined parameters configured by the network.

Regarding Option 1-2, RAN2 made agreements that a new timer is introduced for this option. When this new timer is running, UE is in C-DRX active time. And, during the active time, UE PDCCH monitoring behaviors related to other legacy DRX timers (except for drx-onDurationTimer) are not affected.

Hence, drx-InactivityTimer of the DRX group will be stopped once a DRX Command MAC CE or a Long DRX Command MAC CE is received (adheres to the current specifications).

In light of the newly-introduced timer, even though this timer is not a C-DRX timer, we may also consider this timer to be managed with the existing MAC CEs as proposed by RAN1.

Proposal 2. For Option 1-2 of LS from RAN1, RAN2 understands that the following solutions are feasible as:

- The timers, new timer for Option 1-2 and drx-InactivityTimer of the DRX group, will be stopped once a DRX Command MAC CE or a Long DRX Command MAC CE is received (adheres to the current specifications)

- Option 1-1 and 1-2

Based on the discussions that have been carried out in RAN1 and RAN2, it has become apparent that the main and most significant use case for LP-WUS in the RRC Connected mode will revolve around  'PDCCH monitoring which is triggered by LP-WUS in conjunction with the C-DRX configuration'.

For a quick summary, we capture the each options examples from RAN1’s discussion [6]:

Option 1-1: Replacing DCP following legacy configurations

Figure 1: LP-WUS monitor procedure in Option 1-1

Option 1-2: UE performs LP-WUS monitoring outside at least legacy C-DRX active time, and PDCCH monitoring may be additionally triggered

Figure 2: LP-WUS monitor procedure in Option 1-2 with ‘duty-cycled’ LP-WUS

Figure 3: LP-WUS monitor procedure in Option 1-2 with ‘continuous’ LP-WUS

Based on the discussions from RAN2#127bis, RAN2 progressed and established the precise PDCCH monitoring timing as follows:

Observation 1. RAN2 defines when the UE starts monitor PDCCH as follows:

- Option 1-1: If LP-WUS addressed to UE or UE's subgroup is detected and the legacy drx-onDurationTimer is started

- Option 1-2: If LP-WUS addressed to UE or UE's subgroup is detected and the new timer is started. FFS on drx-onDurationTimer.

The remaining question is whether this timer is newly configured or utilizes an existing traditional timer such as drx-onDurationTimer. It is noteworthy that if the abovementioned timer for Option 1-2 corresponds to drx-onDurationTimer and the LP-WUS periodicity is comparable to the DRX cycle, then Option 1-2 might be equivalent to Option 1-1.

Observation 2. If the above timer for Option 1-2 is the same as drx-onDurationTimer and the LP-WUS periodicity is similar as DRX cycle, the Option 1-2 could be the same as Option 1-1

In the meantime, when the LP-WUS enabled UE is configured with both Option 1-2 and legacy DRX, then the UE operating with LP-WUS option 1-2 is not required to monitor the PDCCH during the legacy DRX active time.

Proposal 3. When the LP-WUS enabled UE is configured with both Option 1-2 and legacy DRX, then the UE operating with LP-WUS option 1-2 is not required to monitor the PDCCH during the legacy DRX active time.

- 3 Conclusion

RAN2 is requested to discuss and agree to the following proposals:

Observation 1. RAN2 defines when the UE starts monitor PDCCH as follows:

- Option 1-1: If LP-WUS addressed to UE or UE's subgroup is detected and the legacy drx-onDurationTimer is started

- Option 1-2: If LP-WUS addressed to UE or UE's subgroup is detected and the new timer is started. FFS on drx-onDurationTimer.

Observation 2. If the above timer for Option 1-2 is the same as drx-onDurationTimer and the LP-WUS periodicity is similar as DRX cycle, the Option 1-2 could be the same as Option 1-1

Proposal 1. For Option 1-1 of LS from RAN1, RAN2 understands as follows:

- The timers, drx-onDurationTimer and drx-InactivityTimer, of the DRX group will be stopped once a DRX Command MAC CE or a Long DRX Command MAC CE is received (adheres to the current specifications)

- Outside of CDRX active times, UEs perform scheduled monitoring of LP-WUS according to predefined parameters configured by the network.

Proposal 2. For Option 1-2 of LS from RAN1, RAN2 understands that the following solutions are feasible as:

- The timers, new timer for Option 1-2 and drx-InactivityTimer of the DRX group, will be stopped once a DRX Command MAC CE or a Long DRX Command MAC CE is received (adheres to the current specifications)

Proposal 3. When the LP-WUS enabled UE is configured with both Option 1-2 and legacy DRX, then the UE operating with LP-WUS option 1-2 is not required to monitor the PDCCH during the legacy DRX active time.

- References

[1] RP-234056 Low-power wake-up signal and receiver for NR (LP-WUS/WUR), CMCC

[2] 3GPP TR38.869 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on low-power wake up signal and receiver for NR (Release 18)

[3] Draft_Minutes_report_RAN1#118bis_v020, Draft Report of 3GPP TSG RAN WG1 #118bis v0.2.0, RAN1

[4] Draft_Minutes_report_RAN1#119_v020, Draft Report of 3GPP TSG RAN WG1 #119 v0.2.0, RAN1

[5] R1-2410909 LS on LP-WUS operation in CONNECTED mode

[6] R1-2407378 FL summary #2 on LP-WUS operation in CONNECTED mode, NTT DOCOMO, RAN1