3GPP TSG RAN2 Meeting #129                                                                R2-2501135

17th Feb. – 21st Feb. 2025, Athens, Greece

Agenda item:	8.11.2

Source:	Samsung

Title:	Random access in SBFD

Document for:	Discussion & Decision

Introduction

The Rel-19 WID Evolution of Duplex operation introduces NR specification support for gNB-side SBFD within a TDD carrier, enhancements to inter-gNB and inter-UE CLI handling, and BS RF and RRM requirements [1].

In the Rel-19 WID, it is assumed that SBFD is deployed at the gNB side and that the TDD UE operates in half-duplex. Subband full-duplex (SBFD) operation on a time-domain resource is limited to FDM in non-overlapping DL and UL subbands. SBFD enhancements for SBFD-aware UEs assume SBFD operation for NR single carrier in FR1 or FR2-1. From the perspective of the SBFD-aware UE, SBFD operation is assumed within a single configured DL and UL BWP pair with aligned center frequencies. For the semi-static indication of frequency/time locations of the SBFD subbands to SBFD-aware UEs on a legacy DL or F symbol/slot, it is assumed that one UL subband is configured for SBFD operation in an SBFD symbol.

During the previous meetings #128, RAN2 agreed as follows:

CFRA The RO type is indicated by NW for CFRA. FFS on signaling (can FFS for the SI request case if needed). CBRA FFS on the following options Option 1 - Upon initiation of RACH procedure for a SBFD-aware UE, network provides the indication on the prioritization of the additional ROs over legacy RO. If there is no such indication from the NW, FFS on the following mechanism - UE select legacy RO or SBFD RO based on SSB RSRP, or - UE select the legacy RO, or - UE select the SBFD RO, or - Other metrics than SSB RSRP. Option 2 UE select legacy RO or SBFD RO based on SSB RSRP if such condition is configured, and if not configured, then UE can prioritize one type of the ROs, FFS which one. RACH configuration Only one RACH configuration option (i.e., either RACH configuration Option 1 with Alt 1-1 or RACH configuration Option 2) is supported in a cell.

In this contribution, we would like to discuss the detailed random access in SBFD symbols in RRC Connected and Idle/Inactive mode.

Discussion

To increase the SBFD symbol utilization, random access in SBFD symbols are proposed and studied. Random access in SBFD symbols increases the number of available UL transmission opportunities in the TDD UL-DL frame configuration period for the PRACH/preamble transmissions and reduces the random access latency.

According to RAN1 discussion so far, there could be a RACH Occasion (RO) within a SBFD symbol. And such RO may start from SBFD symbol and ending in non-SBFD symbol either in the same slot or across different slots can be valid based on network configuration. Figure 1 illustrates an example of RO across two SBFD slots.

Utilizing such RO in SBFD symbols, RAN1 and RAN2 require to design/ extend the existing random access procedure such as RO configuration, Msg.1/preamble transmission, and UE power ramping behaviour.

Figure 1 An example of RO across SBFD slots

Early Indication

RAN2 needs to discuss and decide on the necessity of an 'Early Indication' mechanism for Release 19 SBFD. Similar to previous releases of NR that supported an early indication mechanism for REDCAP, we believe Msg 3-based early indication mechanism could be considered for Release 19 SBFD, in addition to the obvious Msg 1-based early indication which is already agreed as a RAN2 understanding.

The reason behind considering an early indication mechanism is the same as for previous NR features; namely, the inability to promptly exchange or retrieve UE radio access capabilities until later during the initial access procedure once the UE identity has been verified and NAS and AS authentication have successfully completed.

The example of NR standalone mode operation for FR1 TDD 30kHz is considered in below Figure 2. Any UE in RRC_IDLE must connect to the network and first perform the NAS initial access procedure (“with stored context”) before DL/UL data scheduling.

Figure 2 NAS initial access procedure (“with stored context”) in NR SA mode

It should be considered that about 8-10 DL/UL RRC/NAS messages must be exchanged between the network and the UE during NAS initial access procedure. If an early indication type of mechanism becomes available for SBFD-aware UEs, then any PUSCH containing RRC and NAS messages during the transition from RRC_IDLE to RRC_CONNECTED mode can already be transmitted usint the SBFD UL subband following RACH Message 5 (RRC Setup Complete + NAS Registration Request) upon entering RRC_CONNECTED mode. Hence, an early indication could reduce the initial access delay significantly.

This is because SBFD UL subbands can only be configured for the UE by the RRC re-configuration msg which itself can occur only after the UE has been identified and authenticated by gNB and AMF.

As a candidate for early indication (EI) of SBFD-aware UEs, we consider both Msg1-based EI which could rely on RACH/preamble resource and Msg3-based EI that could rely on Msg.3 (or later) transmissions.

When RACH configuration option 1 is configured or indicated through SIB1, the choice of ROs used for random access attempts by SBFD-aware UEs depends on the RO selection and validation rules. If an SBFD-aware UE utilizes a legacy RO, Early Indication may require a Layer 3 (L3)-based approach.

It is also crucial to consider that feature independence must be achievable for deploying SBFD on the network side. Operators may opt to deploy SBFD solely for operations involving UEs in RRC_CONNECTED mode, meaning they may choose not to leverage the random access in SBFD resources feature. Consequently, Early Indication upon transitioning to RRC_CONNECTED mode would still necessitate reliance on an L3-based solution.

Proposal 1. RAN2 supports the Msg3-based Early Indication for Rel-19 SBFD-aware UEs.

RACH Resource Selection

Considering a User Equipment (UE) supporting the SBFD feature, it could make use of both legacy Random Access Opportunities (RO) and SBFD RO for random access. In situations where there are two ROs configured, the UE must determine which RO to utilize for its random access.

To ensure that the SBFD-aware UE benefits from this additional SBFD RO, we should first establish whether such a UE should be capable of utilizing the legacy RO. When considering the relationship between the SBFD RO and the legacy RO, we might explore the possibility of introducing rules, conditions, or prioritization mechanisms among the ROs. Given these considerations, we recommend that RAN2 agreed as follows:

FFS on the following options Option 1 - Upon initiation of RACH procedure for a SBFD-aware UE, network provides the indication on the prioritization of the additional ROs over legacy RO. If there is no such indication from the NW, FFS on the following mechanism - UE select legacy RO or SBFD RO based on SSB RSRP, or - UE select the legacy RO, or - UE select the SBFD RO, or - Other metrics than SSB RSRP. Option 2 UE select legacy RO or SBFD RO based on SSB RSRP if such condition is configured, and if not configured, then UE can prioritize one type of the ROs, FFS which one.

In more detail, UE could select one type of ROs, based on 1) network configured priority indication, or 2)  network configured conditions are met, or 2) existing(configured) prioritization rule.

Considering configuration simplicity and UE flexibility, we support option 2:

Proposal 2. Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE select legacy RO or SBFD RO based on SSB RSRP if such condition is configured, and if not configured, then UE can prioritize one type of the ROs, FFS which one.

In addition to the above prioritization rule, we believe that there exists an SBFD-aware UE capable of utilizing both SBFD ROs and legacy ROs, allowing it to take advantage of the fastest available RO. In order to provide low latency support for this UE, we suggest that the network should indicate such behavior. Proposal 3 suggests that upon initiating the CBRA RACH process for an SBFD-aware UE, the network can instruct the UE to use the nearest RO for its initial access. The specific details regarding FFS signaling are yet to be determined.

Proposal 3. Upon initiation of CBRA RACH procedure for a SBFD-aware UE, network can indicate the UE to utilize the nearest RO for low latency.

Type-2 Random Access

In the Type-2 random access procedure, RACH Msg.1 and Msg.3 (PUSCH) are transmitted together as MsgA. The RACH Msg.2 (RAR) and Msg.4 (for contention resolution) are transmitted together as MsgB. Type-2 is applicable for UEs in RRC_CONNECTED and in RRC_IDLE/INACTIVE modes. Both CBRA and CFRA are possible for Type-2 random access. For MsgA transmission, the gNB configures the MCS and PUSCH allocation. For MsgB reception, a new RNTI is defined. The Type-2 procedure reduces signaling overhead and decreases RA procedure delay.

In Rel-16 NR, the random access causes for Type-1 are considered as valid for Type-2 random access. No new/additional random access cause is introduced for Type-2.

When Type-2 random access is available and configured in the serving cell, the UE selects the Type-1 or Type-2 procedure at the initiation of the random access procedure based on the network configuration.

Most expected design and specification impact to support Type-1 random access procedure in SBFD symbols occur with respect to the Msg.1/preamble transmission such as RO configuration/validation, SSB-RO mapping rules, and UE power ramping behavior. Support of the Type-2 procedure for MsgA transmission shares these common design aspects. For (Type-2) PUSCH in MsgA in SBFD symbols, network configuration can be used similar to existing Rel-16 NR principles.

Therefore, we do not see significantly increased specification effort to also support Type-2 random access in SBFD symbols in addition to the Type-1 procedure.

Proposal 4: For SBFD-aware UEs, support the Type-2 random access procedure in SBFD symbols.

Conclusion

RAN2 is requested to discuss and agree to the following proposals:

Proposal 1. RAN2 supports the Msg3-based Early Indication for Rel-19 SBFD-aware UEs.

Proposal 2. Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE select legacy RO or SBFD RO based on SSB RSRP if such condition is configured, and if not configured, then UE can prioritize one type of the ROs, FFS which one.

Proposal 3. Upon initiation of CBRA RACH procedure for a SBFD-aware UE, network can indicate the UE to utilize the nearest RO for low latency.

Proposal 4: For SBFD-aware UEs, support the Type-2 random access procedure in SBFD symbols.

- References