3GPP TSG-RAN WG2 Meeting #129 					     R2-2501137

Athens, Greece, February 17-21, 2025

Source:	Continental Automotive

Title:                       Further discussion on NW side data collection

Agenda Item:         	8.1.3

Document for:	Discussion and Decision

# Introduction

In RAN2#128 meeting the following agreements were made for NW side data collection:

1.	Focus on the following three radio condition event-based logging

- L3 serving cell measurement based (e.g. X1/X2 similar to A1/A2)

- Beam based events (e.g. beam becomes top-1 beam and number of measurements is less than configured value)

- L1 beam level measurement

2.	Measurements on aperiodic CSI resources are not reported for NW sided data collection.

3.	Data collection is controlled by the network.   The UE will not autonomously stop when low power state is detected.

4.	The UE reports to the network when the power state is low.  We will not specify how the UE determines low power state.   The network should de-configure the data collection (this can be captured in stage 2).

5.	The UE reports to the network when buffer is or may become full.  FFS when it reports (before and/or after).

6.	The UE can report the reason for triggering of indication for the status (e.g. low power state, low memory).  FFS how this is signalled and if the reporting can be part of availability indication.

RAN2 #129 is set to discuss the type of data required to be collected for NW sided model and UE sided model (common to NW sided and different).

# Discussion

From NW perspective, the basic motivation of event triggered measurement is to save the air interface resources of sending collected data by avoiding collecting the useless data. it is beneficial in the following sense:

- NW flexibility to collect data in specific area/by specific UEs/for specific scenario/conditions, and

- UE flexibility to collect/log only required dataset.

The collected dataset maybe identified by an identifier for the UE side model to understand which dataset is used for the model training and which is to be used for model inference. The network may collect the data based on location, specific channel condition and provide these criteria to the UE such that the UE collects the dataset only when this criterion is fulfilled.

Proposal 1: Event triggered measurement is to be adopted for data collection as it is beneficial in the sense that: NW has the flexibility to collect data in specific area/by specific UEs/for specific scenario/conditions, and UE has the flexibility to collect/log only required dataset.

It is possible in certain scenarios that the collected dataset size is not suitable to be transferred between entities as it may result in overhead. In such cases there should be a method to reduce the overhead associated with dataset transfer. A simple approach could be to provide the collected dataset with some identifier which is associated to partition of the large dataset and the associated ID can be associated with a link if the dataset is from the OTT server. This will result in the reduction of overhead of transferring a big chunk of dataset over-the-air.

Proposal 2: Provide the collected dataset with some identifier which is associated to partition of the large dataset and the associated ID can be associated with a link if the dataset is from the OTT server.

# Conclusion

In this contribution, we discussed a few items that could be considered for further discussion related to NW side data collection.

Proposal 1: Event triggered measurement is to be adopted for data collection as it is beneficial in the sense that: NW has the flexibility to collect data in specific area/by specific UEs/for specific scenario/conditions, and UE has the flexibility to collect/log only required dataset.

Proposal 2: Provide the collected dataset with some identifier which is associated to partition of the large dataset and the associated ID can be associated with a link if the dataset is from the OTT server.

# References

[1] R2-128 ChairNotes_24-11-22_17-00_eom