3GPP TSG-RAN WG2 Meeting #129                                                             R2-2501145

Athens, Greece, Feb. 17th – 21rd, 2025

Agenda Item	:	8.2.3

Title	:	Discussion on random access for ambient IoT

Source	:	Google Ireland Limited

Work Item	:	Ambient_IoT_solutions

Document for	:	Discussion and Decision

# 1		Introduction

In this tdoc, we discussed the support of different random access types and discuss whether there is a need to indicate the random access type to device. We also support a backoff mechanism to reduce collision probability among devices.

# 2	Discussion

In RAN2 128, RAN2 discusses the down-selection and unification of 2-step and 3-step CBRA. It is still FFS whether to support only 3-step CBRA or the unified solution.

The benefit of 2-step CBRA is to reduce latency since it requires only two steps to complete a random access procedure. However, the channel utilization is low if a large number of devices contend to transmit a MsgA because the size of MsgA is larger than Msg1. The collision duration is longer for MsgA.

For ambient IoT, if no use case have a delay or QoS requirement, only 3-step CBRA needs to be supported. Otherwise, if there is delay-sensitive CN service, the unified solution can be supported.

Proposal 1:  If no use case have a delay requirement, only 3-step CBRA should be supported.

RAN2 agreed that a device determines the random access type (2step, 3step, CFRA) from the paging message.   It is FFS whether it is explicitly or implicitly.

Since Msg1 size and MsgA size are different, different resources are used to transmit Msg1 and MsgA. The reader can determine CBRA or CFRA from the different resources so there is no need to explicitly indicate CBRA and CFRA in a paging message.

Similarly, a device can also determine whether a CBRA or CFRA should be initiated as the resources to transmit upper-layer data for CFRA and the resources for transmit Msg1 are different. From the observation above, it does need to explicitly indicate random access type in a paging message.

Proposal 2:  No need to explicitly indicate random access type in a paging message.

There are also some discussion on slotted ALOHA in the [POST127] [033] discussion. Slotted ALOHA is a multiple access scheme based on the ALOHA multiple access. In ALOHA protocol, device can immediately transmit data when it has data in buffer. Slotted ALOHA improves the channel utilization of ALOHA by dividing time into equal-size slot. Device can only transmit data at the beginning of a time slot.

To spread a large amount of devices in time requires a backoff function. In the legacy random access procedure, gNB may indicate a BI to UE. UE uses the BI to determine which PRACH occasion to transmit a random access preamble. UE waits for the selected PRACH occasion to transmit a random access preamble. In RFID devices selects one of Q slots to transmit a 16-bits random ID.

Backoff function is also useful for ambient IoT, especially in the case that a large amount of IoT devices is near the reader. For example, an ambient IoT device can be attached to each item on a rack in a warehouse. In such a case, a reader should be able to indicate a backoff to spread devices’ transmissions in times.

The reader could indicate the backoff indicator in a paging message as in RFID, or alternatively, the reader could include the backoff indicator in Msg2 similar to legacy random access procedure.

Proposal 3:  Backoff function should be supported for Msg1/MsgA transmission.

Proposal 4:  Backoff indicator can be included in a paging message or Msg2.

# 3	Conclusions

In conclusion, we have the following proposals:

Proposal 1:  If no use case have a delay requirement, only 3-step CBRA should be supported.

Proposal 2:  No need to explicitly indicate access type in a paging message.

Proposal 3:  Backoff function should be supported for Msg1/MsgA transmission.

Proposal 4:  Backoff indicator can be included in a paging message or Msg2.