3GPP TSG-RAN WG2 Meeting #129	                                    R2-2501152

Athens, Greece, Feb. 17th – Feb. 21st, 2025

Agenda item:		8.1.3

Source:			Sharp

Title:		Discussion on NW-side Data Collection

Document for:	 	Discussion

- Introduction

During Rel-18, data collection for the training of NW-side models was discussed with the following agreements in TR 38.843 [1]. The following principles apply to all the currently identified used cases (i.e., the beam management (BM), the positioning, and the CSI compression and prediction use cases).

- A set of general data collection principles is expected to be considered for network-side model training. These include: - UE to support data logging - UE to report the collected data periodically, event-based, and on-demand - The UE memory, processing power, energy consumption, signalling overhead should be considered

In previous RAN2#128 meeting, the following agreements were made [2].

- Focus on the following three radio condition event based logging - L3 serving cell measurement based (e.g. X1/X2 similar to A1/A2) - Beam based events (e.g. beam becomes top-1 beam and number of measurements is less than configured value) - L1 beam level measurement - Measurements on aperiodic CSI resources are not reported for NW sided data collection. - Data collection is controlled by the network. The UE will not autonomously stop when low power state is detected. - The UE reports to the network when the power state is low. We will not specify how the UE determines low power state. The network should de-configure the data collection (this can be captured in stage 2). - The UE reports to the network when buffer is or may become full. FFS when it reports (before and/or after). - The UE can report the reason for triggering of indication for the status (e.g. low power state, low memory). FFS how this is signalled and if the reporting can be part of availability indication.

In our view, some issues are addressed for the BM use cases and can be further designed for the next step, while for the positioning use cases, it is still not clear how the NW-side data collection can be achieved. As a result, in this contribution, we would like to share our views on 1) Further details of the data collection in BM use cases (or general to BM use cases and positioning use cases) and 2) data collection designs specific to the positioning use cases.

- Discussion

UE Capability for Data Collection

First, since the measurements are performed by the UE, the data collection for NW-side model needs both the consent and the capability of the UE. That is, whether the UE measures the SSB/CSI-RS (for BM use cases) and PRS (for positioning use cases) and reports the result for NW-side data collection is based on the UE capability. The NW can provide configurations for NW-side data collection and obtain the measurement results for NW-side model only when the UE supports the function of NW-side data collection and is able to perform such function (e.g., has sufficient power and memory for data collection for NW-side model).

In our views, for BM use cases, the existing RRC messages UECapabilityEnquiry and UECapabilityInformation can be used to request/indicate the UE’s capabilities related to data collection for NW-side model, while the existing LPP messages ProvideCapabilities and RequestCapabilities can achieve the coordination for positioning use cases. Specifically for the positioning use cases, the UE may not be capable of some UE-based positioning methods (e.g., DL TDoA) to acquire a ground truth label, but may be able to perform the measurement and collect the data (e.g., RSTD). In this case, the UE can provide such information in the corresponding IEs (e.g., DL-TDOA-ProvideCapabilities) to indicate the detailed capabilities. Alternatively, a new IE specifically for AI/ML can be considered. For example, the LMF and the UE can exchange the AI/ML-related capabilities via AIML-RequestCapabilities and AIML-ProvideCapabilities messages. In our views, we consider the new IE for AI/ML would be beneficial to incorporate all the capabilities related to different positioning methods.

Additionally, it was agreed to have a minimum AS layer memory size in the last meeting. Considering that the UE’s memory status configured for data collection may vary, we propose that the UE includes the memory size information in the RRC/LPP message providing the capability information. The memory size information can be an absolute value or can be an estimated value (e.g., the maximum number of instances that a UE can log).

Proposal 1: For BM use cases, the UECapabilityEnquiry and UECapabilityInformation messages are used to request and provide the UE capabilities for data collection for NW-side model.

Proposal 2: For Pos use cases, new IEs AIML-RequestCapabilities and AIML-ProvideCapabilities are used to request and provide the UE capabilities for data collection for NW-side model.

Proposal 3: The UE can include the AS layer memory size information in the RRC/LPP message providing the UE capability information.

Event-Triggered Logging

For the process of NW-side data collection, the event-triggered mechanism is beneficial for two reasons. First, the signalling overhead can be decreased. The UE filters the measurement results with configured events to reduce the size of the logged data. As such, upon reporting, the signalling overhead of the reported data can be mitigated. Second, the NW can control the set of data to be collected for the flexibility of model choices. For example, the NW can configure the UE to collect the data under good channel quality and/or under poor channel quality to generate specific models. The model generated with data collected under good channel quality can be applied when the channel quality is stably good; similarly, the model generated with data collected under poor channel quality can be applied when the channel quality is stably poor. As for other situations, the NW can use a generalized model, which is generated with mixed data. It should be noted that the NW-side data collection is a procedure to facilitate the NW-side model, so it is intuitive that the NW can determine which type of data it wants. According to the advantages mentioned above, event-triggered logging should be supported for NW-side data collection.

Observation 1: Event-triggered data collection reduces the signalling overhead and provides the flexibility for the network to collect specific data.

Observation 2: NW-side data collection is aimed at the data collection for NW-side model generation.

Proposal 4: Event-triggered logging is supported for NW-side data collection.

As mentioned in the previous paragraph, some basic situations like good channel quality or poor channel quality can be considered as the basic events. Since the UE may stay in stably good channel quality (e.g., in the cell center) or may stay in stably poor channel quality (e.g., in the cell edge), it is more reliable for the network to use a specific model rather than a generalized model.

Proposal 5: Different channel quality status can be considered as the basic events for event-triggered NW-side data collection.

When determining the fulfillment of the events, the channel quality can be a cell quality or a beam quality. In our view, using consolidation method would be a compromise and provides the flexibility of quality derivation. The legacy consolidation method can be considered as the baseline. Namely, a maximum RS number and a measurement threshold are given, and the UE derives the quality by averaging the measurement results of the RSs whose qualities are above the threshold. As an alternative design, a threshold on measurement quality and a threshold on the number of beams satisfying the events can be considered. For example, the UE starts the logging when the number of beams whose quality is above (or below in another case) the measurement quality threshold is above the beam number threshold.

Observation 3: Consolidation of beam qualities provides the flexibility when the UE determines whether the event is fulfilled.

Proposal 6: Consolidation of beam qualities is supported for event-triggered NW-side data collection.

Positioning-Specific NW-side Data Collection

For positioning use cases, the measurement configuration can take the legacy ProvideAssistanceData message as a baseline. In our view, the LMF should be capable of flexibly switching among different NW-side models corresponding to different positioning methods (e.g., TDOA, AOD, Multi-RTT). As a result, LMF needs to collect data with different measurement quantities (e.g., RSTD, RSRP, RxTxTimeDiff, etc.) in different additional conditions, and the LMF needs to explicitly indicate the required measurement quantities in the measurement configuration. To allow the LMF to dynamically configure the measurement quantities, we propose to use a new type of ProvideAssistanceData message (e.g., AIML-ProvideAssistanceData) to configure the UE what to measure. Similar to the legacy ProvideAssistanceData message, the AIML-ProvideAssistanceData message contains PRS resource configurations; additionally, it also configures whether one or more of the measurement quantities (i.e., RSTD, RSRP, RTT) need to be measured by the UE. In the AIML-ProvideAssistanceData message, the associated ID (or the area ID or CGI) which identifies the NW-side additional conditions can also be provided.

Observation 4: LMF should be capable of flexibly switching between different NW-side models leveraging different positioning methods, so it may need to collect data with different measurement quantities in different additional conditions.

Proposal 7: For Positioning use cases, a new type of ProvideAssistanceData message (e.g., AIML-ProvideAssistanceData) is used for the LMF to configure the measurements, where the configuration can include the required measurement quantity, the PRS resource set, and the associated ID (or area ID or Cell Global Identity).

According to RAN2#127bis agreements, the periodic and event-based reporting is not supported for BM use case. In our view, the same concept can be applied to the positioning use cases for the reporting behavior; namely, on-demand reporting is supported, while periodic and event-triggered reporting is not supported. Also, since the event-triggered logging is supported, the UE’s indication of data availability would be pursued.

Proposal 8: For Pos use cases, RAN2 to support on-demand reporting. Additionally, RAN2 to support the indication of data availability.

As for the report configuration, the legacy RequestLocationInformation message can be taken as a baseline. Considering some possible AI/ML-specific parameters that can be leveraged for the data collection, we propose a new type of RequestLocationInformation message (e.g., an AIML-RequestLocationInformation message) as the report configuration. The AIML-RequestLocationInformation message) as the report configuration can include the following information:

- A counter to limit the number of logged instances.

- A condition to trigger the UE to start logging.

- An indication whether the ground truth label obtained by the UE should be included in the report.

- The measurement quantities to be reported.

For the above-mentioned information, the first and the second bullets define the logging behavior from the perspective of UE memory and from the perspective of the channel status. Depending on the UE’s capability to obtain the ground truth label, the LMF can determine whether to include the indication to request that the UE reports the ground truth label along with the measurement results. Additionally, since multiple measurement quantities can be configured for the UE to measure, it would be flexible for the LMF to decide which measurement quantities are needed at some given time. In addition, since the time at which the UE obtains the measurement result (e.g., the RSTD) and the time at which the UE obtains the ground truth label (e.g., the location) may be different, the LMF can provide a validity period to the UE. The UE can then associate the measurement results with the ground truth label according to the validity period. By doing so, the timestamps of the measurement result and the ground truth label are not needed to be reported.

Observation 5: For Pos use cases, the UE may obtain the ground truth label by itself.

Proposal 9: For Pos use cases, use a new type of RequestLocationInformation message (e.g., an AIML-RequestLocationInformation message) as the report configuration.

Proposal 10: For Pos use cases, the report configuration further contains: 1) a counter limiting the number of logged instances, 2) a condition that triggers the UE to start logging, 3) an indication whether to report the ground truth label, and 4) the measurement quantities to be reported.

Proposal 11: For Pos use cases, the LMF provides a validity period for the UE to associate the measurement result and the ground truth label.

Enhancements of On-Demand Report

It is agreed that on-demand reporting is supported for NW-side data collection. Also, for BM case, the UEInformationRequest/UEInformationResponse messages are agreed to be used, and the UE can indicate the availability of logged data to the network to trigger the on-demand request. Then, the next step is how the availability is indicated; i.e., which signalling is used and whether other information is included.

In our view, UAI is a candidate for the data availability indication. With the operation of prohibit timer, the UE can send a UAI indicating there is available data when the timer expires, to reduce the signalling overhead. In addition, the UE can also include the size of the buffered data in the UAI to inform the NW.

Proposal 12: UAI is used for the indication of data availability for NW-side data collection. The UE can additionally include the size of the buffered data in the UAI.

Also, the UE can be configured to collect data under different conditions. In such cases, since it is up to the NW to determine which data to be collected, the NW can indicate the UE to only report the data associated with a specific condition. The indication can be included in the UEInformationRequest message.

Proposal 13: The NW can indicate the UE to only report the data associated with a specific condition via the UEInformationRequest message.

As for the pos use cases, based on the analysis in Section 2.3, it is desirable to have on-demand reporting and an indication of data availability. For the data availability indication, it can be achieved by adding the indication in the ProvideLocationInformation message. As for the on-demand request/report procedure, the messages RequestLocationInformation and ProvideLocationInformation can be reused.

Proposal 14: For Pos use cases, the following procedure can be considered for on-demand reporting:

- Step1: UE indicates the availability of data to the LMF via ProvideLocationInformation.

- Step2: LMF requests the logged data via RequestLocationInformation

- Step3: UE transfers the logged data to the LMF via ProvideLocationInformation

In a case that on-demand measurement report is supported, a UE may face the problem that the logged measurement results increase upon every logging instance while the network does not request the measurement results. To prevent the UE from the overwhelming size of logged measurement results, a dropping rule is needed. In our view, although this can be up to UE’s implementation, an upper bound of logging time (like 48 hours in logged MDT) can be discussed.

Observation 6: For both BM and Pos use cases, a dropping method is needed to prevent the UE from the overwhelming size of logged measurement results.

Proposal 15: An upper bound of logging time (e.g., 48 hours in logged MDT case) is specified for the use case of data collection for NW-side model.

Conclusion

Based on the above, we have the following observations and proposals.

Observation 1: Event-triggered data collection reduces the signalling overhead and provides the flexibility for the network to collect specific data.

Observation 2: NW-side data collection is aimed at the data collection for NW-side model generation.

Observation 3: Consolidation of beam qualities provides the flexibility when the UE determines whether the event is fulfilled.

Observation 4: LMF should be capable of flexibly switching between different NW-side models leveraging different positioning methods, so it may need to collect data with different measurement quantities in different additional conditions.

Observation 5: For Pos use cases, the UE may obtain the ground truth label by itself.

Observation 6: For both BM and Pos use cases, a dropping method is needed to prevent the UE from the overwhelming size of logged measurement results.

Proposal 1: For BM use cases, the UECapabilityEnquiry and UECapabilityInformation messages are used to request and provide the UE capabilities for data collection for NW-side model.

Proposal 2: For Pos use cases, new IEs AIML-RequestCapabilities and AIML-ProvideCapabilities are used to request and provide the UE capabilities for data collection for NW-side model.

Proposal 3: The UE can include the AS layer memory size information in the RRC/LPP message providing the UE capability information.

Proposal 4: Event-triggered logging is supported for NW-side data collection.

Proposal 5: Different channel quality status can be considered as the basic events for event-triggered NW-side data collection.

Proposal 6: Consolidation of beam qualities is supported for event-triggered NW-side data collection.

Proposal 7: For Positioning use cases, a new type of ProvideAssistanceData message (e.g., AIML-ProvideAssistanceData) is used for the LMF to configure the measurements, where the configuration can include the required measurement quantity, the PRS resource set, and the associated ID (or area ID or Cell Global Identity).

Proposal 8: For Pos use cases, RAN2 to support on-demand reporting. Additionally, RAN2 to support the indication of data availability.

Proposal 9: For Pos use cases, use a new type of RequestLocationInformation message (e.g., an AIML-RequestLocationInformation message) as the report configuration.

Proposal 10: For Pos use cases, the report configuration further contains: 1) a counter limiting the number of logged instances, 2) a condition that triggers the UE to start logging, 3) an indication whether to report the ground truth label, and 4) the measurement quantities to be reported.

Proposal 11: For Pos use cases, the LMF provides a validity period for the UE to associate the measurement result and the ground truth label.

Proposal 12: UAI is used for the indication of data availability for NW-side data collection. The UE can additionally include the size of the buffered data in the UAI.

Proposal 13: The NW can indicate the UE to only report the data associated with a specific condition via the UEInformationRequest message.

Proposal 14: For Pos use cases, the following procedure can be considered for on-demand reporting:

- Step1: UE indicates the availability of data to the LMF via ProvideLocationInformation.

- Step2: LMF requests the logged data via RequestLocationInformation

- Step3: UE transfers the logged data to the LMF via ProvideLocationInformation

Proposal 15: An upper bound of logging time (e.g., 48 hours in logged MDT case) is specified for the use case of data collection for NW-side model.

Reference

- TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface, December 2023

- Report of 3GPP TSG RAN WG2 meeting#128, November 18-22, 2024

- Report of 3GPP TSG RAN WG2 meeting#127bis, October 14-18, 2024